3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215879

# Electronic Meeting, Oct 10th – 19th, 2022

Source:	ZTE Corporation

Title:	Discussion on RRM performance requirements for NR SDT

Agenda Item:	4.8.2

Document for:	Discussion

# Introduction

In August, RAN5 has sent the reply LS to RAN4 on the test feasibility of UE MO data transmission in RRC_INACTIVE [1]:

For UE to initiate reliable MO Small Data transmission in RRC_INACTIVE, RAN5 discussed the testability aspects of SDT and agreed on a solution to use ‘UE Test Loop Function operating in UE test loop mode B’ defined in TS 36.509 with “T_delay_modeB” timer configured, as per test requirement.

With the confirmation from RAN5, RAN4 needs to complete the corresponding RRM test cases for NR SDT in Q4 [3][4]. In this contribution, we discuss some aspects when designing RRM test cases for NR SDT.

# Discussion

UE test loop mode B is specified as [2]:

### 5.4.4 UE test loop mode B operation #### 5.4.4.1 Void #### 5.4.4.2 Reception of IP PDUs when UE is in E-UTRA or NB-IoT mode Upon receiving a PDCP SDU (=IP PDU) when operating in E-UTRA or NB-IoT mode with UE Test Loop Mode B active the UE shall: 1> if T_delay_modeB timer is running: 2> buffer the received PDCP SDU 1> else 2> if BUFFER_IP_PDUs is TRUE 3> buffer the received PDCP SDU 3> Start T_delay_modeB timer 2> else 3> submit the received PDCP SDU without any modification of the IP header to the UL TFT handling SAP for transmission in uplink. See note. Note 1: The UL TFT function in the UE is mapping IP PDUs received from SDF to EPS bearer/radio bearer as configured by SS in the UL TFT IE sent as part of the EPS bearer establishment procedures. See figure 5.1-2. Note 2: Size of the received PDCP SDUs in downlink shall be bit strings that are byte aligned (i.e. multiple of 8 bits) according to TS 36.323 [24] clause 6.2.1. The PDCP SDUs themselves shall contain a valid IPv4 or IPv6 Header in accordance with the PDN type (Ref. TS 24.301 [36]) accepted by the UE. #### 5.4.4.3 Expiry of T_delay_modeB timer when UE is in E-UTRA or NB-IoT mode When timer T_delay_modeB expires when UE is operating in E-UTRA or NB-IoT mode and has UE Test Loop Mode B active then the UE shall: 1> submit the buffered PDCP SDUs in the same order as received (first-in-first-out) and without any modification of the IP header to the UL TFT handling SAP for transmission in uplink. See note 1. 1> set BUFFER_IP_PDUs to FALSE Note 1: The UL TFT function in the UE is mapping IP PDUs received from SDF to EPS bearer/radio bearer as configured by SS in the UL TFT IE sent as part of the EPS bearer establishment procedures. See figure 5.1-2. NOTE 2: After the PDCP SDU buffer becomes empty the loopback will return any received PDCP SDU in uplink directly as specified in clause 5.4.4.2. In order to reactivate the loopback delay and PDCP SDU buffering the SS shall deactivate UE test loop B function first. …

And the value of T_delay_modeB (8-bit) is set as [2]:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T7 | T6 | T5 | T4 | T3 | T2 | T1 | T0 | octet 1 |

T7..T0 = value of T_delay_modeB timer 0..255 seconds (binary coded, T7 is most significant bit and T0 least significant bit), which means the minimum non-zero value for T_delay_modeB corresponds to 0x01, i.e., 1 second.

Observation 1: The minimum non-zero value for T_delay_modeB is 1 second.

Since TC#1 (TA validation fails) and TC#3 (TA validation success) are combined in one test as illustrated in Fig. 1, there are four possible outcomes of the test as shown in the table below, and only one of them indicates a test-pass with the rest as test-fails.

| TC#3 | TC#1 | Notes |
| --- | --- | --- |
| TA validation success, CG-SDT transmission occurs | TA validation fails, CG-SDT transmission not occurs | Test pass |
| TA validation success, CG-SDT transmission occurs | TA validation success, CG-SDT transmission occurs | Test fails |
| TA validation fails, CG-SDT transmission not occurs | TA validation fails, CG-SDT transmission not occurs | Test fails |
| TA validation fails, CG-SDT transmission not occurs | TA validation success, CG-SDT transmission occurs | Test fails |

Fig. 1, Combining TC#1 and TC#3

Observation 2: Only one of the four possible outcomes of the combined test indicates a test-pass.

No matter the order of TC#1 and TC#3, it should be guaranteed that T_delay_modeB expires at the moment of T2, which means the time duration between T2 and T1 should be larger than T_delay_modeB.

Observation 3: The setting of T_delay_modeB and time duration between T1 and T2 should guarantee that the timer T_delay_modeB expires at the moment of T2.

Proposal 1: RAN4 to consider the above observations when designing RRM test cases for NR SDT.

Once the combined tests (TC#1+TC#3, TC#2+TC4) are defined, NR SDT performance requirements should be declared as completed.

Proposal 2: Declare 100% completion for NR SDT performance part once the combined tests are defined.

# Conclusion

In this paper, we have the following observations and proposal for NR SDT RRM test cases:

Observation 1: The minimum non-zero value for T_delay_modeB is 1 second.

Observation 2: Only one of the four possible outcomes of the combined test indicates a test-pass.

Observation 3: The setting of T_delay_modeB and time duration between T1 and T2 should guarantee that the timer T_delay_modeB expires at the moment of T2.

Proposal 1: RAN4 to consider the above observations when designing RRM test cases for NR SDT.

Proposal 2: Declare 100% completion for NR SDT performance part once the combined tests are defined.

# Reference

R5-225279, Reply LS on the feasibility of testing UE initiated SDT data transmission in RRC_INACTIVE, Qualcomm

TS 36.509 v17.0.0, Special conformance testing functions for User Equipment (UE)

R4-2214280, Email discussion summary for [104-e][230] NR_SmallData_INACTIVE, Moderator(ZTE)

R4-2213341, Way forward on RRM requirements and test cases for NR SDT, ZTE