3GPP TSG-RAN WG4 Meeting # 104bis-e										R4-2216131

Electronic Meeting, October 10 – October 19, 2022

Source: 	vivo

Title: 	Discussion on test related issue of beam correspondence

Agenda Item:	6.7.3.3

Document for:	Discussion

- Introduction

In the last meeting, it was agreed that BEAM_LOCK is not available during non-RRC_CONNECTED state and several issues was listed for further discussion [1]:

Way forward/FFS:

- FFS: discuss if and how UE Tx beam should/would change during IA and what are the impact on the test method.

- FFS: Feasibility of testing UEs in Inactive mode

- FFS: Feasibility of triggering SDT in test mode for different UE implementations

- FFS: Whether and how UE Tx beam should/would change without beam lock function during IA when test equipment is holding RAR to achieving UE max output power.

- FFS: How to achieve and keep max power in testing

- FFS: Number of tests, test time implications and proposed test time reductions

- FFS: polarization aspects without beam lock function in testing

In this contribution, we provide our views on several issues.

- Discussion

  - Necessity and feasibility of SDT testing

In [2], the LS from RAN5 provide a method to enable the test of SDT by using “UE Test Loop Function operating in UE test loop mode B” and UE can enter RRC_INACTIVE by TE send RRC release, so it seems both maintain UE in RRC_CONNECTED and trigger SDT during test are feasible.

Observation 1: Both test UEs in RRC_INACTIVE mode and trigger SDT are feasible.

In addition, we also agreed that:

- BC test for RRC_INACTIVE (if applicable) and RRC_IDLE is at UE maximum output power

It makes the test become more complicated because the power control for initial access, RA-SDT and CG-SDT may be different and different configuration is needed, and multiple test cases need to be defined, then the test time and complexity is also hard to reduce. However, one question here is that whether all cases (initial access, RA-SDT, CG-SDT) need to be tested one by one. The beam correspondence performance is mainly affected by the consistency between Tx and Rx chain and RSRP measurement accuracy, and the consistency between Tx and Rx chain is the characteristic for hardware itself and should be similar for different cases. As for the RSRP measurement accuracy, all these cases perform SSB measurement and the performance is expected to be close. One potential impact is DRX configuration, but we think the it is not need to verify the requirement under different DRX configuration. In short, we think the it is redundant to verify each case, and as long as one of the cases can pass the test, the other cases are considered to satisfy the requirement as well. Comparing to SDT, the initial access is mandatory procedure for all UE, so we think only verify the beam correspondence under initial access is enough.

Proposal 1: No need to test beam correspondence performance of SDT, and once the UE can pass the beam correspondence test for initial access, the SDT can be considered to meet the requirements as well.

  - Keep max power for IA

In [2], we discuss that we can make the UE achieve max output power by holding RAR and configuring specific power control parameter. In order to ensure the UE to maintain the max output power during test, it is better to repeat the power control for each test point and there are some detail behaviors need to be considered. In TS 38.321, the behavior of msg1 transmission is defined:

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

This means that the power ramp up only happens after the msg1 retransmission is triggered, and the retransmission depends on the parameter “ra-ResponseWindow”. If the UE can not receive the RAR with the monitor window, UE will realize that the IA fa until the PREAMBLE_TRANSMISSION_COUNTER is larger than preambleTransMax and the beam is not expected to change within this procedure. The value of ra-ResponseWindow and preambleTransMax is recorded in TS38.331:

Observation 2: The msg1 retransmission time depends on the configuration of preambleTransMax and ra-ResponseWindow.

For each test point, the IA procedure should be initialized, which means the retransmission time only need to guarantee the time of finishing one test point.

Proposal 2: For each test point, the IA procedure should be initialized and the preambleTransMax and ra-ResponseWindow need to guarantee the retransmission time is larger than the time of one test point.

As for the keep UE at max output power, the ideal situation is that UE can transmit the msg1 with max output power at the first time. Based on the PRACH power control formula:

[dBm],

Once we can set the  is larger than , the UE will transmit the msg1 with Pcmax and no need for ramp up based on the formula.

Observation 3: The msg1 may be transmitted with max output power at the first time by a specific configuration.

- Conclusion

Observation 1: Both test UEs in RRC_INACTIVE mode and trigger SDT are feasible.

Observation 2: The msg1 retransmission time depends on the configuration of preambleTransMax and ra-ResponseWindow.

Observation 3: The msg1 may be transmitted with max output power at the first time by a specific configuration.

Proposal 1: No need to test beam correspondence performance of SDT, and once the UE can pass the beam correspondence test for initial access, the SDT can be considered to meet the requirements as well.

Proposal 2: For each test point, the IA procedure should be initialized and the preambleTransMax and ra-ResponseWindow need to guarantee the retransmission time is larger than the time of one test point.

- References

- R4-2214454	WF on BC in RRC_INACTIVE and initial access, Nokia, RAN4#104e

- R5-225279, Reply LS on the feasibility of testing UE initiated SDT data transmission in RRC_INACTIVE, Qualcomm