3GPP TSG-RAN WG4 Meeting #104bis-e													R4-2216200

Electronic meeting, October 10-19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	General considerations for the study of sub-band full-duplex (SBFD) in RAN4

Agenda item:	6.17.1

Document for:	Approval

- Introduction

The new RAN1/RAN4 study item on evolution of duplex operation for NR TDD systems in unpaired spectrum was adopted [1]. The assumptions are listed as follows:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

While the work item objectives are the following:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

This document builds on the discussion and agreements reached in RAN4#104-e and focuses on general aspects regarding non-overlapping subband full duplex (SBFD) with especial focus on the scope of RAN4 work on SBFD feasibility and performance evaluation.

# Discussion

As stated in the work item description  shown in Section 1, the focus of RAN4 work should be on evaluating co-existence on adjacent channel with respect to the legacy operation, as well as studying the feasibility and impact on RF requirements due to  SBFD. The initial discussions on the topic took place in RAN4#104-e, which resulted in multiple agreements and a way forward regarding RF feasibility, adjacent channel co-existence study, and simulation assumptions as captured in references [2], [3], [4], respectively. In addition, a LS response to RAN1 was agreed [5] which included, among other things, some recommendations on RF modeling aspects, including approximate self interference suppression values (or ratio of self interference, RSI, as the term adopted in RAN1) to be assumed as a starting point for system-level evaluations.

Overall, due to urgency in RAN4 to provide a LS response to RAN1 (to avoid slowing down RAN1’s progress on the topic), there was not proper assessment of the feasibility of the provided parameters (e.g. RSI values) in the LS response. In our view, the handling of the gNB self-interference (SI) can be regarded as one of the biggest challenges to be addressed in order to make SBFD feasible. SI is a problem regardless of the scenario/deployment type, but it is certainly an even larger problem for UMa deployments due to high gNB DL transmit power and the use of active antenna systems (AAS) with beamforming and Multi-user MIMO (MU-MIMO) capabilities as further elaborated in our companion contributions. If evaluations in the SI are not handled correctly with appropriate modeling of feasible RF implementation and interference cancellation techniques, SBFD deployed based on incorrect assumptions may in fact deteriorate the performance as well as power and spectral efficiency compared to static TDD. As this is envisioned to increase the gNB complexity, size and cost, a fair and thorough study is needed to see whether the benefits of SBFD versus static and dynamic TDD justify the increased complexity.

Proposal 1: As the next step, RAN4 should do a proper assessment of the feasibility of SBFD RF requirements, taking at least the following aspects into consideration:

- Feasibility of the different interference cancellation techniques, with special focus on their applicability to massive-MIMO high-power macro base stations.

- Additional receiver aspects such as receiver linearity, receiver blocking (LNA overload), etc.

- Indirect system performance degradation due to e.g. one or more of: lower BS antenna beamforming gain,  lower number of transmit/receive chains, reduction in the number of MIMO layers, etc.

- Cost, size and power consumption aspects of SBFD RF transceivers.

- Site deployment constraints, e.g. due to high clutter in the vicinity.

Proposal 2: Sufficiently large gain under realistic assumptions should be observed from SBFD as compared to fixed and dynamic TDD to justify the additional hardware complexity required for SBFD.

Similarly, a initial set of parameters for the RAN4 coexistence evaluation was discussed and agreed in [5]. Here, we would like to highlight the importance of selecting the scenarios according to market relevance, instead of selecting scenarios which can be considered a-priori favourable for SBFD co-existence evaluation which would lead to misleading conclusions and potentially in-field issues. One example is the grid shift considerations for adjacent channel evaluations, where at least 0% grid shift (co-located BS, e.g. different operators using the same mast) should be considered as a mandatory case during the evaluation. This also includes having a realistic assumption for the coupling gain between co-located cells from the different operators.

Proposal 3: It is necessary to do evaluations with realistic scenarios and parameters to avoid misleading conclusions and potentially in-field issues.

# Conclusion

In this contribution the impact of RF self-interference at gNB, and inter-subband CLI at both gNB and UE during sub-band non-overlapping full duplex operation was discussed. Following proposals were made:

Proposal 1: As the next step, RAN4 should do a proper assessment of the feasibility of SBFD RF requirements, taking at least the following aspects into consideration:

- Feasibility of the different interference cancellation techniques, with special focus on their applicability to massive-MIMO high-power macro base stations.

- Additional receiver aspects such as receiver linearity, receiver blocking (LNA overload), etc.

- Indirect system performance degradation due to e.g. one or more of: lower BS antenna beamforming gain,  lower number of transmit/receive chains, reduction in the number of MIMO layers, etc.

- Cost, size and power consumption aspects of SBFD RF transceivers.

- Site deployment constraints, e.g. due to high clutter in the vicinity.

Proposal 2: Sufficiently large gain under realistic assumptions should be observed from SBFD as compared to fixed and dynamic TDD to justify the additional hardware complexity required for SBFD.

Proposal 3: It is necessary to do evaluations with realistic scenarios and parameters to avoid misleading conclusions and potentially in-field issues.

# References

[1] RP-221352, “Study on Evolution of NR Duplex Operation”, CMCC

[2] R4-2214377, “WF on feasibility study from RF perspective”, Samsung

[3] R4-2214378, “WF on adjacent channel co-existence study”, Samsung

[4] R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, CMCC

[5] R4-2214376, “Reply LS on interference modelling for duplex evolution”

[6] R4-2214777, “Work plan for Rel-18 SI: Study on evolution of NR duplex operation”, Samsung, CMCC