3GPP TSG-RAN WG4 Meeting #104bis-e                                      R4-2216237

Electronic Meeting, October 10 – 19, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on the co-existence study for NR duplex operation

Agenda Item:	6.17.2.1

Document for:	Approval

Introduction

Since it was agreed to involve RAN4 into the feasibility study on adjacent channel co-existence with legacy TDD system for NR duplex operation, in RAN4#104-e, initial discussion on the simulation assumptions has been done. Related agreements can be found in the WF [1]. Hence in this contribution, we will provide our further views on the simulation assumptions and share some results.

Discussion

On grid shift

In the last meeting, the agreement regarding grid shift can be found as below:

Further study following candidate options for grid shift in SLS and analysis. - Option 1: 100% - Option 2: 0% - Option 3: 10% - Option 4: TBA

Judging from TR 38.828, the simulation results were mainly provided for grid shift 100%. If other grid shift value needs to be analysed, we think it is better to clarify the definition of it first. The following figure is provided for better explanation.

Fig. 1 Explanation to the grid shift of base station

As we all know, 100% grid shift can be realised by a coordinate translation along axis-x (or axis-y). According to the topology, the distance from the shifted base station to each of the surrounding base station is identical, as well to the angular offset between the shifted base station and the original one. As for the grid shift value other than 100%, how to define the shift shall be clarified first since it will impact the corresponding evaluation results for sure.

Proposal 1: The definition of non-zero grid shift value other than 100% shall be clarified first, if it is necessary to be introduced for Rel-18 duplex co-existence evaluation.

On indoor/outdoor UE ratio

According to the WF, there is a FFS point for UE ratio:

Further study with following candidate options: - Option 1-1: Re-use TR 38.828, which means FR1 Macro-to-Macro uses 20% indoor and 80% outdoor; FR2 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor; - Option 1-2: For Macro-to-Macro cases of both FR1 and FR2, assume 80% indoor and 20% outdoor ratio.

While the second option is the current RAN1 assumption, which seems to be a routine choice also in ITU evaluation and TR 38.802, the first option is what has been applied during Rel-16 study for CLI co-existence. As for now, we can focus on Option 1-1. The other option can be considered if it is necessary in the future.

Proposal 2: For UE ratio evaluation assumption, reuse TR 38.828 for now, which means FR1 Macro-to-Macro uses 20% indoor and 80% outdoor; FR2 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor.

On UE antenna and Tx power

For FR2, there is a FFS point as below:

Further study with following candidate options: - Option 1: Re-use TR 38.828 assumptions. i.e. FR2 max Tx 13.4dBm (peak eirp 22.4dBm), min Tx -40dBm, with antenna configuration in the table of annex 2 - Option 2: FR2 max Tx 23dBm (peak eirp 43dBm) with (M, N, P)=(1, 4, 2), 2 panels antennas and element gain as 1.5 dBi

As for Option 2, we feel the “max Tx 23dBm (peak eirp 43dBm)” might be referred to the FR2 PC3 UE’s max TRP defined by RAN4 and max peak EIRP referred by RAN4 from regulator. Anyway we think the intention here is that FR2 PC3 UE transmission power requirement will be used for the evaluation. Thus, we think reuse TR 38.828 assumption is enough for this issue.

Proposal 3: Reuse the TR 38.828 assumptions for FR2 UE antenna and Tx power.

Initial simulation results

According to the WF in [2], Urban Macro -> Urban Macro is the high priority scenario for FR1. Meanwhile, legacy TDD DL as victim and SBFD as aggressor is the co-existence study case with high priority. Thus we would like to provide initial simulation results as below. Most of the simulation assumptions are aligned with WF in [1].

Fig. 2 FR1 Macro-Macro SBFD co-ex with legacy TDD: SINR and Throughput CDF curve

Table 1. Initial results for SBFD co-ex with legacy TDD in FR1 Macro-Macro scenario

| FR | Deployment Scenario (Aggressor -> Victim) | Aggressor | Victim | Observation Point | SNR degradation (dB) | Throughput degradation (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Macro -> Macro | NR, 100MHz SBFD Subband config {DU}: 80MHz DL + 20MHz UL | NR, 100MHz, DL | 5% | 0.1 | 2.04 |
|  |  |  |  | 50% | 0.08 | 0.39 |
|  |  |  |  | 95% | 0.03 | 0 |

Observation: According to the TR 38.828’s evaluation criteria, the initial results for FR1 Macro-Macro SBFD co-ex with legacy TDD show that the existing ACIR requirements provide sufficient interference reduction in adjacent channel for legacy TDD.

Conclusion

In this contribution we discuss on co-existence study for NR duplex operation. According to the contribution, we have the following observations and proposals:

Observation: According to the TR 38.828’s evaluation criteria, the initial results for FR1 Macro-Macro SBFD co-ex with legacy TDD show that the existing ACIR requirements provide sufficient interference reduction in adjacent channel for legacy TDD.

Proposal 1: The definition of non-zero grid shift value other than 100% shall be clarified first, if it is necessary to be introduced for Rel-18 duplex co-existence evaluation.

Proposal 2: For UE ratio evaluation assumption, reuse TR 38.828 for now, which means FR1 Macro-to-Macro uses 20% indoor and 80% outdoor; FR2 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor.

Proposal 3: Reuse the TR 38.828 assumptions for FR2 UE antenna and Tx power.

- References

- R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, CMCC, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.

- R4-2214378, “WF on adjacent channel co-existence study”, Samsung, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.