GPP TSG-RAN WG4 Meeting # 104-e	R4-2216269

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	7.5.6

Source: 	Huawei, HiSilicon

Title: 	Discussion RRM requirements for IoT NTN

Document for:	Discussion

# Introduction

The Rel-18 RAN4 WID was approved in [1] to define requirements for Rel-17 IoT NTN [2]. In Rel-17 IoT NTN, RAN1/2/3 specifications were defined to enable NB-IoT and eMTC operating over NTN. In last RAN4 meeting, the RRM impact was initially discussed [3]. In this paper, we further provide our views on remaining issues.

# Discussion

## 2.1 General

The impact of Doppler shift was discussed for NGSO, which was already well considered in NR NTN discussion. The status of IoT NTN are summarized as follows:

__Issue 1-3-4: measurement capability on number of NGSO satellites - FFS For both NB and M1 in NGSO, the number of target satellites UE needs to monitor [per carrier] is 2 including serving LEO satellite. - FFS whether to introduce UE capability for the number of target satellites the UE can monitor per carrier including serving LEO satellite, which can be up to 4 __Issue 1-3-5: For NGSO, Doppler shift impact in Multiple NGSO satellites - RAN4 shall investigate Doppler shift impact on the overlapping receiving window from multiple intra-frequency and multiple inter-frequency satellites on IoT NTN - Except NB-IoT in Connected mode - FFS the following proposals - the scaling factor for measurement of multiple LEO satellites should also apply - For eMTC over NTN, RAN4 not to define scheduling restriction due to RRM measurement __Issue 1-5: UE capability introduced in R17 NR NTN - FFS whether to introduce the following UE capability, for LTE IoT NTN in Rel-18 - Enhanced RRM requirements for measurements in IDLE and INACTIVE modes - Relaxed cell reselection on GEO - Parallel measurements on multiple LEO satellites

In NR NTN, corresponding capabilities were introduced to indicate that whether UE can perform parallel measurement on multiple NGSO satellites (25-5), and whether UE supports additional number of satellites per carrier (25-7). Thus, for IoT NTN, though the UE is less powerful than that in NR NTN, it is reasonable to adopt the same approach as NR NTN which is to introduce UE capabilities on number of NGSO that UE can monitor per carrier and whether UE can perform parallel measurement on multiple NGSO satellites.

Proposal 1: Introduce UE capabilities on number of NGSO that UE can monitor per carrier and whether UE can perform parallel measurement on multiple NGSO satellites.

For applicability of DRX/eDRX cycle length and enhanced coverage, the status are summarized as follows:

__Issue 1-3-6: DRX/eDRX, HD-FDD applicability - FFS the applicability of DRX/eDRX cycle length and PTW length for LEO due to change in satellite coverage (t-Service). - The existing TN requirements can be used as a starting point. __Issue 2-2-4: NGSO, cell Re-selection in Enhanced Coverage - For Enhanced Coverage intra-/inter-frequency measurement, the existing TN requirement on Tmeasure, Tevaluate can be the baseline. - The exact values are FFS. - FFS the cell detection time (Tdetect).

As discussed in last meeting, for some long DRX/eDRX length, the overall cell detection delay will be extremely long which is not feasible in NGSO, especially in enhanced coverage. For normal coverage, it could be observe that the longest cell detection time is 102 seconds when DRX cycle length is 10.24 seconds. From our understanding, it can work in NGSO scenarios.

Observation 1: For normal coverage, requirements for cell detection for existing DRX cycles can still work in NGSO.

For eDRX, the total delay is complicated as it depends on the length of eDRX and PTW. Thus, for simplification and the flexibility of NW configurations, it is proposed that the requirements with eDRX applies when Tdetect,NB_Intra_NC is less than or equal to the maximum value of that without eDRX (DRX = 10.24 seconds).

Proposal 2: For normal coverage, existing TN requirements for DRX can be reused. When eDRX is configured, the requirements apply when Tdetect,NB_Intra_NC is less than or equal to the maximum value of that without eDRX (e.g. DRX = 10.24 seconds for NB-IoT).

For enhanced coverage, as discussed in last meeting, the cell detection time could be extremely long which may not be able to suit the NGSO scenarios. For instance, the cell detection time for NB-IoT with DRX length of 1.28 s, the delay could be 532 seconds, and it could be longer with larger DRX cycle length. It seems the requirements cannot well support NGSO with short coverage time. As commented by companies in last meeting, the requirements for serving cell measurement in enhanced coverage can still apply. Thus, it is proposed that for enhanced coverage, only define requirements for serving cell.

Proposal 3: For enhanced coverage, only define requirements when serving cell is in enhanced coverage.

#### Issue 1-3-2: information for the neighbor/target cell - Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.

In last meeting it was agreed that valid information for the neighbour/target cell should be made available of the mobility and measurement requirement to apply, which is same to NR NTN. It is noted that with current signaling for IoT NTN, neighbour/target cell cannot be provided to the UE, and RAN2 is waiting for feedback from RAN4.

Agreements:

- Introduce UL gap configuration for PUSCH/PUCCH/NPUSCH segmented transmission, based on the reported UE capability

- PUSCH and PUCCH segmented transmission use the same gap configuration.

- The changes in R2-2208574 are not pursued

- The changes in R2-2208294 are agreed

- Discussion on the introduction of cell ephemeris information in SIB31 is on hold until we receive feedback from RAN4 on this, if any

To facilitate RAN2 discussion, we suggest to send LS to RAN2 to inform the agreement in last meeting. A draft LS is attached in the Annex.

Proposal 4: Send LS to inform RAN2 the agreement on information for the neighbor/target cell from RAN4#104.

## 2.2 IDLE state mobility

For IDLE state mobility requirements, it was agreed that UE is not required to perform any cell search while out of coverage in discontinuous coverage which is shown as follows:

#### __Issue 2-1: Discontinuous Coverage - Define the RAN4 requirements based on the assumption that the UE is able to predict the coverage. - In IDLE mode or PSM mode, the Ues are not required to perform any cell search while out of coverage in discontinuous coverage. - FFS whether and how to define the requirement.

As commented during the discussion, RAN2 introduced a specific signaling (SystemInformationBlockType32-NB) about ephemeris parameters for neighbour cell and potentially serving satellite for coverage prediction which is shown as follow. After receiving the SI, if UE determines that it will enter out of coverage state after the current serving stops service, it makes no sense to perform cell search anymore. Instead, UE can stop neighbour cell measurement when receiving the SI instead of after entering out of coverage state.

#### – SystemInformationBlockType32-NB The IE SystemInformationBlockType32-NB contains satellite assistance information for prediction of discontinuous coverage. SystemInformationBlockType32-NB is only signalled in a NTN cell. SystemInformationBlockType32-NB information element -- ASN1START SystemInformationBlockType32-NB-r17 ::= SEQUENCE { satelliteInfoList-r17 SatelliteInfoList-r17 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ... } -- ASN1STOP

Proposal 5: UE is not required to perform neighbour cell measurement from the last slot of SI transmission which indicates that UE will be in out of coverage after Tservice when the serving cell stop serving the area.

For the requirements of Cell re-selection, the status are shown as follows:

__Issue 2-2-1b: NGSO, cell re-selection - RAN4 to discuss which eDRX/DRX cycles to exclude due to the impact of Tservice (cell stop time) for serving cell and neighbour cell measurements in LEO. - FFS whether to scaled up the existing TN delay requirements by KSatellite, where KSatellite is the number NGSO satellites. - FFS: In NGSO, the cat-M/NB UE in IDLE mode is allowed to meet the relaxed neighbour cell measurement requirements provided that the UE has met the relaxed monitoring criterion and the serving cell is not going to stop serving the area in the next L1 number of DRX cycles; L1 is FFS. - FFS: In NGSO, the cat-M/NB UE in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the relaxed monitoring criterion and the serving cell is not going to stop serving the area in the next L2 number of DRX cycles; L2 is FFS.

It was agreed that Tservice shall be considered for neighbour cell and serving cell measurement in LEO. For scaling up the delay by number of NGSO satellite, as analyzed above, the similar approach as NR NTN shall be adopted.

Proposal 6: The existing TN delay requirement shall be scaled up by number of NGSO satellites.

For relaxed neighbour cell measurement and relaxed serving cell measurement, the existing TN requirements are shown as follows:

Serving cell #### 4.6.2.1A Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS The UE which supports wakeUpSignal [2] shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in Section 4.6.2.1, provided the following conditions are met: - WUS has been configured in the serving NB-IoT cell using WUS-Config-NB-r15 [2], and - The serving cell measurement relaxation is signalled as n by the network using numDRX-CycleRelaxed-r15, and - Serving cell S criteria is met with at least 2 dB margin. - the relaxed monitoring criteria for neighbour cells in TS 36.304 [1] clause 5.2.4.12.1 is fulfilled, and , where the relaxation factor N is given by Table 4.6.2.1A-1. Otherwise the requirements defined for the configured DRX cycle length in Section 4.6.2.1 shall apply. Neighbour cell If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6.2.2-1 and Table 4.6.2.2-2.

It is reasonable to add the additional conditions for Tservice, where UE shall exit relaxed neighbour/serving cell measurement when the serving cell is going to stop serving the area. Thus, again similar approach in NR NTN is preferred where Ttrigger is used. Thus, it is proposed that to add the conditions that the time span to Tservice when the serving cell stops service is longer than Ttrigger.

Proposal 7: For relaxed serving cell and neighbour cell measurement, add following condition:

-   Time span to Tservice when serving cell stops service is longer than Ttrigger,

Where Ttrigger = max(Tdetect,NB_Intra_NC , Pcarrier * Tdetect,NB_Inter_NC) for NB and Ttrigger = max(Tdetect,EUTRAN_Intra_NC, Kcarrier*Tdetect,EUTRAN_Inter_NC) for eMTC.

For TA validation for PUR, the status are summarized as follows；

#### __Issue 2-6: Transmission using preconfigured uplink resources (PUR) - Continue discussion the following aspect in the next meeting - whether to reuse the legacy RSRP-based TA validation for PUR in IoT NTN - The UE update the uplink timing for transmitting on PUR using the configured TA command according to TS 36.211 v17.2.0 i.e. transmission of uplink radio frame number from the UE starts TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs - The UE assumes TA is valid provided that the following conditions are met, otherwise the TA is considered invalid: - Satellite assistance information (SAI) is valid i.e. T317 has not expired and - Current time of the UE is at least DT seconds earlier than t-Service.

Regarding whether UE can assume the TA is valid or not, it is already reflected in RAN2 spec TS36.331 as follows:

### 5.3.18 T317 expiry The UE shall: 1> if in RRC_CONNECTED: 2> inform lower layers that the UL synchronisation is lost; 2> start timer T318; 2> acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) as specified in 5.2.2; 2> upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT): 3> stop timer T318; 3> inform lower layers that the UL synchronisation is restored; NOTE: SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

Observation 1: RAN2 has defined the rules when synchronization is lost.

Then, back to the issue whether RAN4 to use the TN RSRP-based TA validation for PUR, as commented in last meeting, the legacy RSRP-based approach is to reflect the changing of TA indirectly as UE has no idea about the changing of TA. However, in IoT NTN, it is assumed that UE is always equipped with GNSS, which means UE can determine the changing of TA directly with much more higher accuracy than the RSRP-based approach. We fails to see the necessity to define requirements for IoT NTN.

Proposal 8: Don’t define RSRP-based TA validation for PUR in IoT NTN.

## 2.3 CONNECTED state mobility

For Random access, it is proposed to define requirements for UE specific TA reporting during Random access. Based on RAN2 agreement, TA reporting can be configured via SI ta-Report in MSG3/MSG5. From RRM requirement’s perspective, the behavior is already clearly defined in TS36.321 and TS36.331. Thus, only general description and reference to RAN2 spec is enough to be captured in RAN4 spec.

Observation 2: Generic description about UE specific TA reporting and reference to RAN2 spec is sufficient in RAN4 spec.

#### __Issue 3-1: RRC Re-establishment and RRC release with redirection - Continue discussion the following aspect in the next meeting - Whether the requirements of RRC Re-establishment and RRC release with redirection of TN can apply to IoT NTN not not. - the impact of sharing factor regarding to number of NGSO satellites - FFS on impact of serving cell coverage (Tservice) on RRC re-establishment and RRC connection release with redirection.

In last meeting the RRC re-establishment and re-direction requirements are discussed for NTN in email [214]. The conclusion is that the scaling factor related to number of NGSO satellites is considered. The reason is that during these procedures, UE would perform cell search on the target carriers, and that may need to be done separately for each target cell due to different Doppler shifts.

The same scaling needs to be considered in IoT NTN as well. For example, in M1 re-direction requirement the search time is defined as 960ms for CEModeA. When UE searches multiple satellites on the target carrier, the time needs to be scaled with number of LEO satellites.

Proposal 9: Scaling factor related to number of NGSO satellites should be considered in RRC Re-establishment and redirection requirements.

#### __Issue 3-3: M1, E-UTRAN Handover - For eMTC (M1) over NTN, define E-UTRAN Handover requirements by re-using TN HO requirements for NTN as baseline - Note: RAN4 can further discuss and identify potential NTN specific impact #### __Issue 3-4: M1, CHO requirements - Option 1: define CHO requirements for M1 in NTN - FFS no need to consider time or location based CHO - Option 2: CHO requirements for M1 should be introduced first in the Terrestrial IoT

For HO, some companies suggested to consider possible GNSS fix during HO which is being discussed in RAN2. We understand there has been no agreement in RAN2. We suggest RAN4 to take the TN requirements as baseline until we receive further information from RAN2.

For CHO, we support option 1. The WI is for IoT NTN, so it is clear out of scope to define any requirement for IoT TN. For time and location based CHO, as commented in last meeting, RAN2 has not defined time or location CHO for IoT NTN in Rel-17, so they should not be considered in RAN4 requirements.

Proposal 10: RAN4 to define CHO requirements for M1 in NTN. No need to consider time or location based CHO.

## 2.4 Timing requirements

For gradual timing adjustment, the discussion in RAN4#104-e are summarized as follows:

#### __Issue 4-1-2: Gradual timing adjustment - FFS For gradual timing adjustment, the reference timing shall be (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Ts before the downlink timing of the reference cell. Clarify the adjustment with “apart from a change of NTA,UE-specific and NTA,common” - FFS the gradual timing adjustment figures (max step size, max, min rate etc) - FFS the impact of “segmented UL transmission

Sub-topic 3-1: timing adjustment during a UL repetition period - The following proposals are discussed in this meeting and can be further discussed. - Proposal 1: RAN4 shall clarify the IoT NTN behaviour in TS 36.133 for Rel-17 to allow UEs to adjust TA during the ongoing repetition. - Proposal 1a: The restriction on UL transmission adjustment shall be updated according to RAN1 LS on per-segment TA pre-compensation. - Proposal 2: RAN4 shall specify the IoT NTN behaviour in TS 36.133 to ensure the UE keeps a constant TA value within each segment - Proposal 3: Segmented UL transmission can be covered by NTN UE transmit timing requirements, i.e. Te_NTN. FFS whether and how to capture in RAN4

Based on the discussion in last meeting, companies had different interpretations on UE behavior based on RAN1’s LS. The very controversial discussion in last RAN4 meeting is whether UE is allowed to adjust TA during a segment. According to RAN1 spec TS 36.213 clause 16.1.2 which is shown below, the time and frequency pre-compensation is down per uplink segment, which means UE shall keep a constant TA within one segment. In this way, there could be big step accumulated timing drift after the transmission of the segment, and the quantity of adjustment for the next segment could be large which may overlap with the previous one. This is way RAN1 had introduced the transmission gap.

For a NB-IoT UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the quantity Nsegmentprecompensation is provided by system information, as specified in 3GPP TS 36.331.

Observation 2: RAN1’s agreements imply that UE shall not do time pre-compensation within one segment.

The above RAN1 agreements may have impacts on RAN4 spec on the restrictions on uplink timing adjustment during one repetition period, which is provided below:

TS 36.133 clause 7.20 When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

In TN scenarios, the motivation to have above restrictions is that the autonomous adjustment during repetition may have impact on UL demodulation performance as eNB may need to proceed multiple repetitions before it can assess the timing of UE. The restriction is valid in TN IoT where the mobility is negligible. However, in IoT NTN, if such restriction applies to IoT NTN UE no matter on the whole repetition period or one per segment basis, it means large timing drift is experienced at eNB side. For instance, at the beginning of a segment, the timing error of UL transmission could be ranged from [-Te, Te] considering the UE implementation margin. During the segment, eNB will experience constant timing drift caused by the dramatic changing of the propagation delay, and at the end of the segment, the accumulated timing error could be Te + D, where D is the total timing during the segment.

Observation 3: If UE is not allowed to perform time pre-compensation during a segment, gNB will experience large timing drift during one segment.

An essential problem is whether UE is allowed do pre-compensation during one segment which seems contradicts with RAN1’s agreement. With UE specific TA adjustment, the timing error shall meet Te requirements for each transmission. For instance, for transmission on a segment, UE shall adjust the transmit timing constantly during the segment based on UE specific TA. From eNB side, the timing drift caused by moving of satellite is eliminated for each subframe of the segment. Besides, the large step timing adjustment can also be avoided.

Observation 4: If UE is allowed to perform time pre-compensation during a segment, timing drift caused by moving of satellite is eliminated.

Proposal 11: UE is allowed to perform time pre-compensation during a segment.

Based on analysis above, it is more reasonable that UE can perform time pre-compensation during a segment. Then the following question is how to define transmit timing requirements for IoT NTN and whether the restriction on UE autonomous timing adjustment is valid. From our understanding, the similar approach as NR NTN transmit timing requirement can be adopted. For IoT NTN, the reference point for the UE initial transmit timing shall be the downlink timing of the serving cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs.

Proposal 12: For IoT NTN, the reference point for the UE initial transmit timing shall be the downlink timing of the serving cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs.

For the gradual timing adjustment, similar statement as NR NTN can be adopted. Current requirements can apply apart from the change of NTA,adjUE due to satellite position update and NTA,adjcommon.

Proposal 13: Current requirements on gradual timing adjustment can apply excluding the change of NTA,adjUE due to satellite position update and NTA,adjcommon .

Since UE is allowed to perform timing pre-compensation, the restriction on autonomous uplink timing adjustment during an ongoing repetition period does not apply to NTN IoT.

Proposal 14: The restriction on autonomous uplink timing adjustment during an ongoing repetition period does not apply to NTN IoT.

## 2.5 RLM requirements

For RLM requirements, the status are summarized as follows:

#### __Issue 4-4: RLM - For NB GEO, the existing TN RLM requirements apply, as in 7.23 - For NB NGSO, FFS the impact of satellite assistance information validity and t-Service - For M1 GEO, the existing TN RLM requirements apply, as in 7.19 - For M1 NGSO, FFS the impact of satellite assistance information validity and t-Service - FFS the following proposal - Option 1: The UE in DRX shall evaluate the RLM according non-DRX requirements provided that the following conditions are met, otherwise the UE is allowed to evaluate following DRX requirements: - Satellite assistance information (SAI) is valid i.e. T317 has not expired and - Current time of the UE is at least DT seconds earlier than t-Service.

For the FFS point about DRX and non-DRX, based on the comments in last meeting, the motivation is to speed up the RLM evaluation before Tservice. However, how the proposal is formed seems that after serving cell stops serving or T317 expires, UE can perform RLM slower (in DRX mode). We do not fully understand the motivation to differentiate DRX/non-DRX based RLM based on t-Service. (e.g. fast RLM before t-Service and slow RLM after t-Service).

Proposal 15: Existing TN RLM requirements apply for GEO/NGSO.

## 2.6 Measurement

#### __Issue 1-3-5: For NGSO, Doppler shift impact in Multiple NGSO satellites - RAN4 shall investigate Doppler shift impact on the overlapping receiving window from multiple intra-frequency and multiple inter-frequency satellites on IoT NTN - Except NB-IoT in Connected mode - FFS the following proposals - the scaling factor for measurement of multiple LEO satellites should also apply - For eMTC over NTN, RAN4 not to define scheduling restriction due to RRM measurement

#### __Issue 5-1-2: M1, Measurement requirement - eMTC over NTN, re-use the TN measurement delay requirements for NTN as baseline, and the scaling factor for measurement of multiple LEO satellites should also apply - FFS the following proposals - For M1 in GEO, the existing M1 TN intra frequency measurement requirements apply, as in 8.13.2.1 for CE mode A and 8.13.3.1 for CE mode B - For M1 in GEO, the existing M1 TN inter frequency requirements apply, as in 8.13.2.6 for CE mode A and 8.13.3.5 for CE mode B. - For M1 in NGSO, the delay requirements are scaled up by the number NGSO satellites.

In NR NTN, scaling factor is introduced in the measurement requirement for NGSO/LEO. The reason is that the difference in the receive frequency of different LEO satellites can be up to 50kHz due to very high speed of satellite movement, and in such cases UE may not be able to receive signals from multiple satellites simultaneously. The same issue exists for IoT NTN, so the scaling factor for measurement of multiple LEO satellites should also apply. Except the scaling factor for measurement of multiple LEO satellites, we think the TN measurement requirements can be re-used.

Proposal 16: Define M1 measurement requirements as follows.

- For M1 in GEO, the existing M1 TN intra frequency measurement requirements apply, as in 8.13.2.1 for CE mode A and 8.13.3.1 for CE mode B

- For M1 in GEO, the existing M1 TN inter frequency requirements apply, as in 8.13.2.6 for CE mode A and 8.13.3.5 for CE mode B.

- For M1 in NGSO, the delay requirements are scaled up by Ksat, which is NsatNsat,simul,

  - Nsat is the number of LEO satellites to be measured for the MO,

  - Nsat,simul is the number of LEO satellites that UE can measure in parallel for one MO.

#### __Issue 5-2: M1, Measurement Gap - For M1 over NTN, a single MG is considered as baseline for RRM measurement. - FFS whether to consider 2 MGs.

We do not think two MGs are needed for IoT NTN. Two MGs are introduced in NR NTN because SMTCs for same or different MO may not be covered by the same MG. However, in IoT NTN the measurement is based on PSS/SSS/CRS, and UE can always measure all the target cells with a single MG regardless of the propagation delay difference between them, as such a single MG is enough.

In addition, RRM measurement, including both intra- and inter-frequency, in M1 are based on MGs. This means there is no need to define scheduling restriction due to measurement. In NR NTN, scheduling restriction is defined only for measurement performed outside MG which do not exist for M1.

Proposal 17: For M1 measurement, only single MG is considered. RAN4 not to define scheduling restriction due to RRM measurement

#### __Issue 5-3: Connected mode channel quality report - For GEO, existing TN requirements apply - For NB, as in 8.14.4 - For M1, as in 8.13.2.8 for CE-A, 8.13.3.8 for CE-B - FFS LEO

#### __Issue 2-4: Channel quality report for UE Category M1 in idle mode - For M1 in GEO, the existing TN requirements of channel quality report for in idle mode apply, as in 4.7.3. - FFS M1 in LEO.

In last meeting, some companies suggested to discuss which specific time instance the CQR corresponds to. In our understanding, this is already clarified in the existing TN requirement, e.g. below is a copy from cl. 8.14.4, i.e. the CQR is corresponding to the time periods of the NPDCCH which carries the UL grant. As such, we think the TN requirement can be re-used.

The reported NPDCCH repetition level shall be derived from the channel quality measured over the NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier.

Proposal 18: For channel quality report in both idle and connected mode, the existing TN requirements are re-used also for LEO.

# Conclusions

Proposal 1: Introduce UE capabilities on number of NGSO that UE can monitor per carrier and whether UE can perform parallel measurement on multiple NGSO satellites.

Observation 1: For normal coverage, requirements for cell detection for existing DRX cycles can still work in NGSO.

Proposal 2: For normal coverage, existing TN requirements for DRX can be reused. When eDRX is configured, the requirements apply when Tdetect,NB_Intra_NC is less than or equal to the maximum value of that without eDRX (e.g. DRX = 10.24 seconds for NB-IoT).

Proposal 3: For enhanced coverage, only define requirements when serving cell is in enhanced coverage.

Proposal 4: Send LS to inform RAN2 the agreement on information for the neighbor/target cell from RAN4#104.

Proposal 5: UE is not required to perform neighbour cell measurement from the last slot of SI transmission which indicates that UE will be in out of coverage after Tservice when the serving cell stop serving the area.

Proposal 6: The existing TN delay requirement shall be scaled up by number of NGSO satellites.

Proposal 7: For relaxed serving cell and neighbour cell measurement, add following condition:

-   Time span to Tservice when serving cell stops service is longer than Ttrigger,

Where Ttrigger = max(Tdetect,NB_Intra_NC , Pcarrier * Tdetect,NB_Inter_NC) for NB and Ttrigger = max(Tdetect,EUTRAN_Intra_NC, Kcarrier*Tdetect,EUTRAN_Inter_NC) for eMTC.

Observation 1: RAN2 has defined the rules when synchronization is lost.

Proposal 8: Don’t define RSRP-based TA validation for PUR in IoT NTN.

Observation 2: Generic description about UE specific TA reporting and reference to RAN2 spec is sufficient in RAN4 spec.

Proposal 9: Scaling factor related to number of NGSO satellites should be considered in RRC Re-establishment and redirection requirements.

Proposal 10: RAN4 to define CHO requirements for M1 in NTN. No need to consider time or location based CHO.

Observation 2: RAN1’s agreements imply that UE shall not do time pre-compensation within one segment.

Observation 3: If UE is not allowed to perform time pre-compensation during a segment, gNB will experience large timing drift during one segment.

Observation 4: If UE is allowed to perform time pre-compensation during a segment, timing drift caused by moving of satellite is eliminated.

Proposal 11: UE is allowed to perform time pre-compensation during a segment.

Proposal 12: For IoT NTN, the reference point for the UE initial transmit timing shall be the downlink timing of the serving cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs.

Proposal 13: Current requirements on gradual timing adjustment can apply excluding the change of NTA,adjUE due to satellite position update and NTA,adjcommon .

Proposal 14: The restriction on autonomous uplink timing adjustment during an ongoing repetition period does not apply to NTN IoT.

Proposal 15: Existing TN RLM requirements apply for GEO/NGSO.

Proposal 16: Define M1 measurement requirements as follows.

- For M1 in GEO, the existing M1 TN intra frequency measurement requirements apply, as in 8.13.2.1 for CE mode A and 8.13.3.1 for CE mode B

- For M1 in GEO, the existing M1 TN inter frequency requirements apply, as in 8.13.2.6 for CE mode A and 8.13.3.5 for CE mode B.

- For M1 in NGSO, the delay requirements are scaled up by Ksat, which is NsatNsat,simul,

  - Nsat is the number of LEO satellites to be measured for the MO,

  - Nsat,simul is the number of LEO satellites that UE can measure in parallel for one MO.

Proposal 17: For M1 measurement, only single MG is considered. RAN4 not to define scheduling restriction due to RRM measurement

Proposal 18: For channel quality report in both idle and connected mode, the existing TN requirements are re-used also for LEO.

# References

[1] RP-221556 New WID: WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks

[2] RP-211601 NB-IoT/eMTC support for Non-Terrestrial Networks

[3] R4-2214350 WF on LTE IoT NTN RRM requirements, MediaTek inc

[4] R4-2214289 Email discussion summary for [104-e][240] LS_reply

[5] R4-2214353 WF on UL Segmented Transmission for UL synchronization for IoT NTN, MediaTek inc.

# Annex: draft LS on information for neighbor/target cell in IoT NTN

3GPP TSG-RAN WG4 Meeting #104-bis-e R4-220xxxx Electronic Meeting, 10 – 19 October, 2022 Title: LS on information for neighbor/target cell in IoT NTN Response to: Release: Release 18 Work Item: LTE_NBIOT_eMTC_NTN_req-Core Source: RAN4 To: RAN2 Cc: Contact Person: Name: Li Zhang E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: In LTE_NBIOT_eMTC_NTN_req WI, RAN4 is specifying RRM requirements for NB-IoT and eMTC operation over NTN functionality defined in the Rel-17 LTE_NBIOT_eMTC_NTN work item. In RAN4#104-e, the following agreement [1] is reached on information for neighbor/target cell in IoT NTN #### Issue 1-3-2: information for the neighbor/target cell - Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE. RAN4 respectfully asks RAN2 to take the above information into account in the future work for IoT NTN. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account in the future work for IoT NTN. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #105 Nov. 14 – Nov.18, 2022 Toulouse, France RAN WG4 Meeting #106 Feb. 27 – Mar.03, 2023 Athens, Greece