3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2216311

Electronic Meeting, October 10 – October 19, 2022

Title: 	Discussion on FR2 eHST impact on RRM

Source: 	Huawei, HiSilicon

Agenda item:	6.12.5

Document for:	Discussion

Introduction

The R18 WI on enhanced NR support for high speed train scenario in FR2 is to be discussed in the upcoming meeting. The objectives are duplicated as below [1]. This contribution provides preliminary analysis on the FR2 eHST impact on RRM core requirements.

- Only train roof-mounted high power devices with target applicable carrier frequency up to 30GHz and up to 350km/h velocity are considered in this WI - Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4] - Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4] - Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2]. - Note: RAN1/RAN2 work can be triggered by RAN4 LS.

Discussion

- Measurement requirements of Supporting CA

One target of R18 FR2 eHST WI is to support intra-band CA. From measurement perspective, UE only needs to measure the serving SCells in the same band as PCell according to the existing principle specified in 9.2.3.2(duplicated as below).

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

That’s to say, in intra-band CA scenario UE shall measure on PCC (including PCell and its neighbor cells) and serving SCells. Then the question is whether PSS/SSS detection requirements on SCC need to be specified in intra-band CA FR2 HST scenario, as the timing can refer to PCell and there is no need to identify neighbor cells on SCC.

Proposal 1: To further study whether PSS/SSS detection requirements on SCC need to be specified in intra-band CA FR2 HST scenario.

UE shall perform L3 measurement on serving SCells, then L3 measurement period on both active SCell and deactivated SCells need to be specified. The measurement period for intra-frequency measurement without/with gap specified in R17 FR2 HST can be reused forthe measurement period for activated SCell in R18 FR2 HST at least for open deployment scenarios. Tunnel scenario is also one target deployment. The RX beam scaling factor under tunnel scenario needs further study.

Proposal 2: The measurement period for intra-frequency measurement without and with gap specified in R17 FR2 HST can be reused to the measurement period for activated SCell in R18 FR2 HST at least for open deployment scenarios.

Proposal 3: The RX beam scaling factor under tunnel scenario needs further study.

Regarding the measurement on the deactivated SCell, as the WI focus on CPE, power consumption is not a critical issue. So we doubt whether SCell will be deactivated in FR2 HST scenario.

Proposal 4: Further study whether SCell is deactivated in FR2 HST scenario.

Regarding time period for time index acquirement, we focus on intra-band CA where there is a PCell in the band. The timing difference for intra-band contiguous CA in FR2 is 130ns and for intra-band non-contiguous is 260ns. The MRTD is less than min (2 SSB symbols, 1 PDSCH symbol). Therefore the time index of SCell can be derived from the PCell in the same band. Therefore the time period of time index on SCell may not need to be specified in intra-band CA FR2 scenario.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| FR2-2 | TBD |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

Proposal 5: The time period of time index on SCell may not need to be specified in intra-band CA FR2 scenario.

- SCell activation/deactivation delay

Whether RAN4 needs to specify SCell activation/deactivation delay in R18 FR2 eHST depends on whether SCell is deactivated in FR2 scenario.

Proposal 6: Whether RAN4 needs to specify SCell activation/deactivation delay in R18 FR2 eHST depends on whether SCell is deactivated in FR2 scenario.

Simultaneous multi-panel operation

The objective regarding simultaneous multiple-panel operation in FR2 HST is duplicated as below:

- Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception

For FR2 multi-Rx chain DL reception in R18, RAN4 has agreed that the total searcher number for CA capable UE is still assumed as 2 for cell detection and L3 measurements, which means same searcher capability as R15. Based on the searcher assumption, at most one searcher is available for L3 measurements on one CC. Simultaneous L3 measurements with different beam directions on one CC will require that multiple searchers are available for L3 measurements on one CC, which is not aligned with the agreed searcher assumption in RAN4.

From R15, it is common understanding that for non-CA UE, the searcher number is 1. So it is not possible for supporting simultaneous L3 measurement for non-CA capable UE as well.

Proposal 7: In R18, simultaneous multi-panel operation is not supposed to be applied for L3 RRM measurements.

In FR2, UE is assumed to use rough beam for L3 measurements and use fine beam for L1 measurements and data receptions. The beams for simultaneous multiple-panel operation are assumed as fine beam. Hence, UE is still not assumed to perform L1 measurement and L3 measurement simultaneously. The sharing factor between L1 andL3 still need to be kept in L3 RRM measurement requirements.

Proposal 8: In R18, the sharing factor between L1 and L3 measurements needs to be kept in L3 RRM measurement requirements.

For R18 multi-Rx DL reception, the typical application scenario is for multi-TRP transmissions. Supporting of simultaneous DL reception from different directions is considered for TRP specific L1 measurements. Simultaneous L1 measurements on two RSs from different TRPs can be assumed. In HST scenario, if RRHs are used as TRPs for a serving cell, then simultaneous L1 measurements on two RSs from different RRHs of the serving cell can be assumed. UE is able to activate two panels at a time if the two RSs are simultaneously received on two different antenna panels.  Then the question is how UE determines whether the two RRHs are on UE different sides or on UE same side.

Proposal 9: If RRHs are used as different TRPs for a serving cell, UE can be assumed to support simultaneous L1 measurements on two different QCL-typeD RSs from different RRHs of the serving cell, and the precondition is that two RSs are simultaneously received on two different antenna panels.

Conclusions

This contribution provides the consideration on R18 enhanced NR support for high speed train scenario in FR2. The following proposals are provided:

Proposal 1: To further study whether PSS/SSS detection requirements on SCC need to be specified in intra-band CA FR2 HST scenario.

Proposal 2: The measurement period for intra-frequency measurement without and with gap specified in R17 FR2 HST can be reused to the measurement period for activated SCell in R18 FR2 HST at least for open deployment scenarios.

Proposal 3: The RX beam scaling factor under tunnel scenario needs further study.

Proposal 4: Further study whether SCell is deactivated in FR2 HST scenario.

Proposal 5: The time period of time index on SCell may not need to be specified in intra-band CA FR2 scenario.

Proposal 6: Whether RAN4 needs to specify SCell activation/deactivation delay in R18 FR2 eHST depends on whether SCell is deactivated in FR2 scenario.

Proposal 7: In R18, simultaneous multi-panel operation is not supposed to be applied for L3 RRM measurements.

Proposal 8: In R18, the sharing factor between L1 and L3 measurements needs to be kept in L3 RRM measurement requirements.

Proposal 9: If RRHs are used as different TRPs for a serving cell, UE can be assumed to support simultaneous L1 measurements on two different QCL-typeD RSs from different RRHs of the serving cell, and the precondition is that two RSs are simultaneously received on two different antenna panels.

Reference

- RP-213565, New WID on Further NR mobility enhancements, MediaTek

- R4-2103678, WF on FR1 HST RRM, CMCC