3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216548

E-meeting,10th Oct – 19th Oct,2022

Agenda item:	7.5.4

Source: 	ZTE Corporation

Title: 	Further discussion on SAN RF requirements for IoT over NTN

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221556] on NB-IoT/eMTC core & performance requirements for NTN was approved as one of Rel-18 RAN4 package. During the last RAN4 meeting, we have reached good consensus on SAN RF requirements for IoT over NTN, however there are still some remaining issues left for further discussions. In this contribution,we we want to share some further considerations on these issues.

| Requirement | Requirement clause of 36.108 | Updates to support SA NB-IoT | Updates to support SA Cat-M1 |
| --- | --- | --- | --- |
| DL RS power | Not exist | DL RS power was E-UTRA specific. Motivation and the requirement value to be discussed next meeting. | DL RS power was E-UTRA specific. Motivation and the requirement value to be discussed next meeting. |
| ACLR | 6.6.3 | Depend on co-existing discussion/simulation | Depend on co-existing discussion/simulation |
| Operating band unwanted emissions | 6.6.4 | Depend on co-existing discussion/simulation | Depend on co-existing discussion/simulation |
| Dynamic range | 7.3 | - Add NB-IoT channel BW - For GEO, to follow the same agreement without requirement defined. - For LEO, - Reuse the same FRC and SNR for standalone NB-IoT - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - FFS for IoT Level for IoT over NTN - Ruse the implementation margin as 2.5dB; | Add 1.4MHz channel BW For GEO, to follow the same agreement without requirement defined. For LEO, - Reuse the same FRC A2-1 and SNR for 1.4MHz LTE BW - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse IoT 15dB for NR over NTN - Ruse the implementation margin as 2.5dB; |
| ACS | 7.4.1 | Depend on co-existing simulation | Depend on co-existing simulation |

Discussion

## 2.1. DL RS power

DL RS power accuracy requirement was introduced in UTRA for CPICH channel from UE demod perspective and this requirement was inherited for E-UTRA DL RS power accuracy, however during the Rel-15 NR work item phase, this requirement was removed with the confirmation from UE vendors, therefore the practical usage of this requirement might be not so strong. Based on the previous discussions, we are open for the discussion of its necessity. If UE chipset vendors have strong interest to include this requirement for SAN RF and then we propose to reuse the existing requirement from LTE BS.

Proposal 1: to reuse the existing requirement from LTE BS if UE chipset vendors have strong preference/interest to include this requirement for SAN RF.

## 2.2. ACLR/ACS

For SAN DL ACLR requirement, it is mainly dependent on the coexistence case 3. In case 3, NTN SAN DL transmission would cause the interference to TN UE. The effective ACLR of NTN SAN is according to the section 6.3 of TR 36.802[3]. The initial simulation result is shown as the following table.

| Case 3 | ACLR [dB] | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Rural TN_AAS_DL | Average throughput loss | 22.88 | 10.10 | 3.80 | 1.25 | 0.41 | 0.13 | 0.05 | 0.01 |
|  | Cell edge through loss | 31.78 | 13.49 | 4.91 | 1.49 | 0.50 | 0.16 | 0.05 | 0.02 |
| GEO_Rural TN_non_AAS_DL | Average throughput loss | 31.09 | 13.88 | 5.25 | 1.80 | 0.60 | 0.15 | 0.06 | 0.01 |
|  | Cell edge through loss | 53.77 | 26.93 | 10.44 | 3.41 | 0.89 | 0.29 | 0.11 | 0.05 |
| GEO_Urban_macro TN_AAS_DL | Average throughput loss | 0.24 | 0.09 | 0.04 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | 0.04 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| GEO_Urban_macro TN_non_AAS_DL | Average throughput loss | 0.28 | 0.08 | 0.02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | 0.23 | 0.19 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| LEO600KM_Rural TN_AAS_DL | Average throughput loss | 65.07 | 43.80 | 24.39 | 10.97 | 4.16 | 1.39 | 0.44 | 0.15 |
|  | Cell edge through loss | 84.35 | 62.85 | 34.10 | 13.73 | 4.78 | 1.44 | 0.52 | 0.14 |
| LEO600KM_Rural TN_non_AAS_DL | Average throughput loss | 81.39 | 59.52 | 33.93 | 15.03 | 5.71 | 1.83 | 0.50 | 0.17 |
|  | Cell edge through loss | 100.00 | 100.00 | 57.48 | 29.79 | 11.85 | 4.23 | 1.39 | 0.42 |
| LEO600KM Urban_macro TN_AAS_DL | Average throughput loss | 2.47 | 0.84 | 0.28 | 0.13 | 0.04 | 0.02 | 0.00 | 0.00 |
|  | Cell edge through loss | 1.32 | 0.40 | 0.17 | 0.08 | 0.05 | 0.00 | 0.00 | 0.00 |
| LEO600KM Urban_macro TN_non_AAS_DL | Average throughput loss | 2.58 | 0.86 | 0.33 | 0.14 | 0.05 | 0.02 | 0.00 | 0.00 |
|  | Cell edge through loss | 3.89 | 0.91 | 0.05 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
| LEO1200KM_Rural TN_AAS_DL | Average throughput loss | 65.02 | 43.60 | 24.52 | 11.09 | 4.36 | 1.53 | 0.43 | 0.17 |
|  | Cell edge through loss | 84.37 | 62.61 | 34.51 | 14.18 | 4.92 | 1.69 | 0.45 | 0.14 |
| LEO1200KM_Rural TN_non_AAS_DL | Average throughput loss | 81.37 | 59.34 | 33.96 | 15.14 | 5.52 | 1.95 | 0.64 | 0.18 |
|  | Cell edge through loss | 100.00 | 100.00 | 57.16 | 29.37 | 11.36 | 3.94 | 1.41 | 0.48 |
| LEO1200KM Urban_macro TN_AAS_DL | Average throughput loss | 2.34 | 0.82 | 0.29 | 0.08 | 0.02 | 0.01 | 0.00 | 0.00 |
|  | Cell edge through loss | 0.91 | 0.21 | 0.05 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
| LEO1200KM Urban_macro TN_non_AAS_DL | Average throughput loss | 2.87 | 0.96 | 0.27 | 0.11 | 0.03 | 0.01 | 0.00 | 0.00 |
|  | Cell edge through loss | 3.96 | 0.81 | 0.10 | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 |

For ACS requirement for SAN supporting IoT over NTN, this still depends on the simulation results for Case 2 and Case 6, more simulation results would be provided for more concrete proposals for SAN Rx ACS requirements.

Proposal 2: to define ACLR as 24dB for LEO SAN class and ACLR as 14dB for GEO SAN class without 100kHz frequency gap next to 1s adjacent carriers.

## 2.3. SAN DL UEM

Based on the simulation results for coexistence scenario 2 and initial proposal 2 for SAN ACLR requirement, we think that the existing NR SAN OBUE requirement could be also applicable for IoT over NTN SAN. Basically we have two reasons to support this proposal. Firstly, the ACLR requirement for SAN supporting IoT over NTN could be the same as that of SAN supporting NR over NTN. Secondly, OBUE requirement is somehow bandwidth agnostic defined, in other words, this OBUE requirement could be also scaled to 200kHz for IoT over NTN and 1.4MHz for eMTC over NTN.

Table 6.6.4.2-1: SAN LEO and GEO Classes OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BWChannel | 0.002 MHz f_offset < 2× BWChannel + 0.002 MHz | maxSE limit, PSDchannel – ΔSat_ClassdB-40×log10 f_offset-0.002BWChannel×2+1dBm | 4kHz |
| NOTE 1: PSDchannel = Prated,c – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause 6.6.5. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [9], Annex 5 OoB domain emission limits for space services. NOTE 4: ΔSat_ClassdB=0 dB for GEO class and ΔSat_ClassdB=3 dB for LEO class. |  |  |  |

Proposal 3: to reuse the existing NR SAN OBUE requirement for IoT over NTN SAN OBUE requirements

## 2.3. IoT level for dynamic range requirement for NB-IoT over NTN

Similar as Rel-17 NTN SAN, legacy LTE BS dynamic range requirement and FR2 BS dynamic range requirement discussion, we need to check the IoT level in different NTN deployment scenarios and the initial simulation results are shown in the following Figure 1/2/3/4/5/6. Based on the simulation results obtained so far, it could be found that IoT level@95% CDF for different deployment scenarios are also varying. The details of IoT level in the different deployment scenarios are summarized in the following table.

Figure 1a. IoT leve1 in GEO Rural scenario

Figure 1b. IoT level in GEO Urban macro scenario

Figure 1c. IoT leve1 in GEO Rural scenario with single tone 15kHz

Figure 1d. IoT level in GEO Urban macro scenario with single tone 15kHz

Figure 1e. IoT leve1 in GEO Rural scenario with single tone 3.75kHz

Figure 1f. IoT level in GEO Urban macro scenario with single tone 3.75kHz

Figure 2a. IoT leve1 in LEO1200km Rural scenario

Figure 2b. IoT level in LEO1200km Urban macro scenario

Figure 2c. IoT leve1 in LEO1200km Rural scenario with single tone 15kHz

Figure 2d. IoT level in LEO1200km Urban macro scenario with single tone 15kHz

Figure 2e. IoT leve1 in LEO1200km Rural scenario with single tone 3.75kHz

Figure 2f. IoT level in LEO1200km Urban macro scenario with single tone 3.75kHz

Figure 3a. IoT leve1 in LEO600km Rural scenario

Figure 3b. IoT level in LEO600km Urban macro scenario

Figure 3c. IoT leve1 in LEO600km Rural scenario with single tone 15kHz

Figure 3d. IoT level in LEO600km Urban macro scenario with single tone 15kHz

Figure 3e. IoT leve1 in LEO600km Rural scenario with single tone 3.75kHz

Figure 3f. IoT level in LEO600km Urban macro scenario with single tone 3.75kHz

Table 1. summarized IoT level for IoT over NTN

|  | Multiple tone | Single tone 15kHz | Single tone 3.75kHz | Recommendation |
| --- | --- | --- | --- | --- |
| GEO Rural scenario | 4.7 | 11.2 | 18.2 | 22 |
| GEO Urban scenario | 7.0 | ~12 | 22 |  |
| LEO1200KM Rural scenario |  |  |  | 24 |
| LEO1200KM Urban scenario |  |  | 24 |  |
| LEO600KM Rural scenario |  |  |  | 24 |
| LEO600KM Urban scenario |  |  | 24 |  |

Based on the initial simulation results summarized in the above table 1, it could be found that IoT level as 24dB for both GEO and LEO should be sufficient to test its performance.

It should be noted that we still observe the high IoT level compared for GEO scenario especially for single tone 3.75kHz case which is different from the Rel-17 NR over NTN.

Proposal 4: to agree the IoT level as 24dBc for GEO and LEO dynamic rang requirements.

Conclusions

In this contribution, we want to share some further views on SAN RF requirement for IoT over NTN and proposals are made as following:

Proposal 1: to reuse the existing requirement from LTE BS if UE chipset vendors have strong preference/interest to include this requirement for SAN RF.

Proposal 2: to define ACLR as 24dB for LEO SAN class and ACLR as 14dB for GEO SAN class without 100kHz frequency gap next to 1s adjacent carriers.

Proposal 3: to reuse the existing NR SAN OBUE requirement for IoT over NTN SAN OBUE requirements.

Proposal 4: to agree the IoT level as 24dBc for GEO and LEO dynamic rang requirements.

References

- RP-221556, WID on NB-IoT/eMTC core & performance requirements for NTN

- R1-2200042, LS on DL synchronization enhancements for IoT NTN, RAN1 (Jan-22)

- R4-2214382,WF on SAN RF requirement for IoT over NTN, ZTE, approved.