3GPP TSG-RAN WG4 Meeting #104-bis-e                                     R4-2216660

Electronic Meeting, Oct 10 – Oct 19, 2022

Source: 	Huawei, Hisilicon

Title: 	Discussion on UE supporting CA_n39A-n41A with simultaneous Rx/Tx

Agenda Item:	5.26.2

Document for:	Approval

Introduction

A new WID Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 was agreed in last RAN meeting [1], the objective and scope is as follows:

1. Identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values. Note 1: Band combinations considered in this WI have to be introduced first via basket WIs (see 2.3) or completed in previous releases if necessary. Note 2: Whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case. 2. Align the specification treatment of simultaneous Rx/Tx capability for CA, SUL, MR-DC and NR-DC band combinations.

This contribution provides analysis on simultaneous Rx/Tx operation for CA_n39-n41.

Discussion

Existing requirements for CA_n39-n41

In 38.101-1, the requirements for CA_n39-n41 are defined as follows

Table 1 NR CA configurations and bandwidth combinations sets defined for CA_n39-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n39A-n41A | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n39A-n41C | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
| CA_n39A-n41(2A) | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS0 |  |

Table 2 ΔTIB,c due to CA_n39-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n39-n41 | n39 | 02 |
|  | n41 | 02 |
|  | n39 | 0.53 |
|  | n41 | 0.53 |
| NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

Table 3 ΔRIB,c due to CA_n39-n41

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n39-n41 | n39 | 0.22 |
|  | n41 | 0.22 |
|  | n39 | 0.23 |
|  | n41 | 0.23 |
| NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

Table 4 Requirements for uplink CA_n39-n41

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |

Observation 1: From the existing specification, it is observed that requirements defined in 38.101-1 for CA_n39-n41 are only based on non-simultaneous Rx/Tx operation condition.

Reference architectures

The delta Tib and Rib requirements can be mapped to the UE reference architectures in Figure 1 and Figure 2 [2].

Figure 1: Architecture 1

Figure 2: Architecture 2

Though the current requirements are specified with condition of not supporting simultaneous Rx/Tx operation for CA_n39-n41, the UE reference architecture 2 itself can support such operation. The RF performance depends on the filter and diplexer rejection at frequency range of band n39 and n41.

It is noted that the requirements and operation limitations for CA-n39-n41 are inherited from LTE, and there is no more detailed analysis for simultaneous Rx/Tx for this specific band combination for NR as what has been done for LTE CA. The reference architectures in Figure 1 and 2 were proposed for a long time, which are kind of obsolete compared to the latest UE implementation. Nowadays, duplexer/multiplexer in the RF front end module is a more popular implementation architecture for this band combination. The reference architecture 3 is given in Figure 3.

Figure 3: Architecture 3

Observation 2: Multiplexer is similar to the implementation for FDD bands. Smaller delta Tib and Rib can be considered for UE supporting simultaneous Rx/Tx operation.

Proposal 1: It is proposed to consider new delta Tib and Rib values for simultaneous Rx/Tx for CA_n39-n41.

MSD analysis

Architecture 3 is used in the following MSD analysis on simultaneous Rx/Tx operation for CA_n39-n41.

Table 1 and Table 2 are the estimated REFSENS degradation for the simultaneous Rx/Tx working condition. The duplexer isolation is based on survey of component vendors.

Table 1: Estimated n41 REFSENS degradation

| RFIC n39 Tx noise in n41 (dBm/Hz) | -150 |
| --- | --- |
| PA n39 Tx noise in n41 (dBm/Hz) | -145 |
| PA gain in n41 (dB) | 18 |
| PA output noise in n41 (dBm/Hz) | -132 |
| SPDT switch loss in n41 (dB) | 0.5 |
| Duplexer isolation at n41 (dB) | 40 |
| Tx noise at ant port (dBm/Hz) | -172.5 |
| Rx noise figure assumption (dB) | 9 |
| Sensitivity degradation (dB) | 0.7 |

Table 2: Estimated n39 REFSENS degradation

| RFIC n41 Tx noise in n39 (dBm/Hz) | -150 |
| --- | --- |
| PA n41 Tx noise in n39 (dBm/Hz) | -135 |
| PA gain in n39 (dB) | 18 |
| PA output noise in n39 (dBm/Hz) | -130 |
| SPDT switch loss in n39 (dB) | 0.4 |
| Duplexer isolation at n39 (dB) | 40 |
| Tx noise at ant port (dBm/Hz) | -170.4 |
| Rx noise figure assumption (dB) | 9 |
| Sensitivity degradation (dB) | 1.1 |

Observation 3: From the MSD calculation, it can be seen that sensitivity degradation is small with the latest implementation capability for CA_n39-n41 supporting simultaneous Rx/Tx operation.

Proposal 2: It is proposed to define MSD requirement for CA_n39-n41 in order to support simultaneous Rx/Tx operation.

Conclusion

This contribution provides analysis on Simultaneous Rx/Tx for CA_n39-n41.

Observation 1: From the existing specification, it is observed that requirements defined in 38.101-1 for CA_n39-n41 are only based on non-simultaneous Rx/Tx operation condition.

Observation 2: Multiplexer is similar to the implementation for FDD bands. Smaller delta Tib and Rib can be considered for UE supporting simultaneous Rx/Tx operation.

Observation 3: From the MSD calculation, it can be seen that sensitivity degradation is small with the latest implementation capability for CA_n39-n41.

Proposal 1: It is proposed to consider new delta Tib and Rib values for simultaneous Rx/Tx for CA_n39-n41.

Proposal 2: It is proposed to define MSD requirement for CA_n39-n41 in order to support simultaneous Rx/Tx operation.

- References

[1] RP-222648, New WID Simultaneous RxTx inter-band combinations in Rel-18, Huawei, HiSilicon, RAN WG e-Meeting#97, September 12 – 16, 2022

[2] TR 36.852-12 v12.2.0 LTE-Advanced inter-band Carrier Aggregation (CA); Two downlinks, one uplink (2DL/1UL) (Release 12)