3GPP TSG-RAN WG4 Meeting # 105										 R4-2219264

Toulouse, France, November 14 – November 18, 2022

Title: 	Discussion on CA_n8-n20-n28

Source: 	Xiaomi

Agenda item:	8.2.4.2

Document for:	Approval

- Introduction

This contribution gives the text proposal on CA_n8-n20-n28 for TR 38.872.

- Annex

*****************************************Start of the TP*****************************************

## 5.3	CA_n8-n20-n28

### 5.3.1	UE RF architecture assumption

The following UE RF architectures can be assumed to analyze and study for CA_n8-n20-n28.

2 antenna, 3 antenna, 4 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

Figure 5.3.1-1: Reference RF architecture with two low band antenna for CA_n8-n20-n28

Figure 5.3.1-2: Reference RF architecture with three low band antenna for CA_n8-n20-n28

Figure 5.3.1-3: Reference RF architecture with four low band antenna for CA_n8-n20-n28

### 5.3.2	CA_n8-n20

#### 5.3.2.0	General

According to the current spec TS 38.101-1, DL CA_n8A-n20A_BCS0 has been specified. Thus, some of RF requirements can be reused as below. Both DL_n8A-n20A_BCS0 and DL_n8A-n20A_UL_n8A-n20A are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n8A-n20A.

#### 5.3.2.1	Operating bands for CA

Table 5.3.2.1-1:  CA band combination of band n8+n20

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz | – | 915 MHz |
| n20 | 832 MHz | – | 862 MHz |

#### 5.3.2.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n8A-n20A_BCS0 has been specified.

#### 5.3.2.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n8-n20 as we didn’t specify them in current spec.

#### 5.3.2.4	∆TIB,c and ∆RIB,c values

For CA_n8-n20, the TIB,c and RIB,c values have been specified in TS 38.101-1.

#### 5.3.2.5	REFSENS requirements

As CA_n8-n20 has been specified into current spec TS 38.101-1, there is no harmonics, cross band isolation and harmonics mixing exception.

#### 5.3.2.6	Maximum output power for UL inter-band CA

Power class 3 is assumed for UL CA_n8-n20.

#### 5.3.2.7	UE co-existence studies for UL inter-band CA

Table 5.3.2.7-1 lists Band n8 + Band n20 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.3.2.7-1: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 83 | 18 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 844 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592 | 2692 | 2544 | 2639 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 | 1913 | 1581 | 1706 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 | 3607 | 3376 | 3501 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 36 | 166 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2568 | 2413 | 2828 | 2658 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 826 | 666 | 1081 | 916 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 | 4363 | 4352 | 4522 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4416 | 4304 | 4469 |

Based on Table 5.3.2.6-1, 3rd and 5th order IMD may also fall into Rx frequencies of bands n8 or band n20.

Table 5.3.2.7-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.3.2.7-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n20 | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8, 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 5.3.2.8	REFSENS requirements for UL inter-band CA

IMD3 MSD of DC_20_n8A in n20 and n8 is already specified in TS 38.101-3 and can be reused for CA_n8-n20

### 5.3.3	CA_n8-n28

#### 5.3.3.0	General

According to the current spec TS 38.101-1, DL CA_n8A-n28A_BCS0 has been specified. Thus, some of RF requirements can be reused as below. Both DL_n8A-n28A_BCS0 and DL_n8A-n28A_UL_n8A-n28A are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n8A-n28A.

#### 5.3.3.1	Operating bands for CA

Table 5.3.3.1-1:  CA band combination of band n8+n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

#### 5.3.3.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n8A-n28A_BCS0 has been specified as below.

Table 5.3.3.2-1: Supported bandwidths per CA band combination of band n8+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n28A | CA_n8A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |

#### 5.3.3.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n8-n28 as we didn’t specify them in current spec.

#### 5.3.3.4	∆TIB,c and ∆RIB,c values

For CA_n8-n28, the TIB,c and RIB,c values have been specified in TS 38.101-1.

#### 5.3.3.5	REFSENS requirements

As CA_n8-n28 has been specified into current spec TS 38.101-1, there is no harmonics, cross band isolation and harmonics mixing exception.

#### 5.3.3.6	Maximum output power for UL inter-band CA

Power class 3 is assumed for UL CA_n8-n28.

#### 5.3.3.7	UE co-existence studies for UL inter-band CA

Table 5.3.3.7-1 lists Band n8 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.3.3.7-1: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Based on Table 5.3.3.7-1, there is no IMD issue for CA_n8-n28.

Table 5.3.3.7-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.3.3.7-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n28 | E-UTRA Band 20, 31, 34, 38, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79, n100 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

#### 5.3.3.8	REFSENS requirements for UL inter-band CA

There is no REFSENS exception due to IMD interference for CA_n8A-n28A.

### 5.3.4	CA_n20-n28

#### 5.3.4.1	General

According to the current spec TS 38.101-1, DL_n20A-n28A_UL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS1 have already been specified. Thus, all of the related RF requirements have been analysed in clause 6.31 from TR 38.716-02-00 and can be reused as below. Both DL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS0 are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n20A-n28A.

1) The operating bands for CA_n20-n28 were specified in table 5.2A.2.1-1 from TS 38.101-1 with note 2 that The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.

2) The UL/DL configurations and bandwidth combination sets for DL_n20A-n28A_UL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS1 were specified in table 5.5A.3.1-1g from TS 38.101-1.

3) 23dBm Power class 3 for UL CA_n20A-n28A was specified in Table 6.2A.1.3-1 from TS 38.101-1.

4) 0.5dB TIB,c for CA_n20-n28 was specified in Table 6.2A.4.2.3-1 from TS 38.101-1.

5) Spurious emissions for UE co-existence for Inter-band CA_n20-n28 were specified in Table 6.5A.3.2.3-1 from TS 38.101-1.

6) Referring to UE co-existence studies in clause 6.31.2.1 from TR 38.716-02-00, there are no harmonic/harmonic mixing or IMD issues affecting own Rx frequencies of either band n20 or n28. Therefore, there is no need to specify the additional REFSENS exceptions for CA_n20-n28.

7) 0dB RIB,c for CA_n20-n28 was specified in Table 6.31.1.4-2 from TR 38.716-02-00.

### 5.3.5	CA_n8-n20-n28

#### 5.3.5.1	Operating bands for CA

Table 5.3.5.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

It’s noted that the following sentence was clarified in clause 5.5A.0 from TS 38.101-1.

“For a higher order band combination of which CA_n20-n28 is a subset, the frequency range in band n28 is restricted for the higher order band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.”

Thus, this frequency range restriction is also applicable to CA_n8-n20-n28.

#### 5.3.5.2	Channel bandwidths per operating band for CA

Table 5.3.5.2-1: Supported bandwidths per CA band combination of band n8+n20+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n20A-n28A | CA_n8A-n20A CA_n8A-n28A CA_n20A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 |  |

#### 5.3.5.3	∆TIB,c and ∆RIB,c values

For CA_n8-n20-n28, the TIB,c and RIB,c values are given in table 5.3.5.3-1 and table 5.3.5.3-2.

Table 5.3.5.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.6 |
|  | n20 | 0.5 |
|  | n28 | 0.5 |

Table 5.3.5.3-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.2 |
|  | n20 | 0 |
|  | n28 | 0.1 |

#### 5.3.5.4	UE co-existence studies for UL inter-band CA

Table 5.3.5.4-1 lists Band n8 + Band n20 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n28.

Table 5.3.5.4-1: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 83 | 18 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 844 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592 | 2692 | 2544 | 2639 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 | 1913 | 1581 | 1706 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 | 3607 | 3376 | 3501 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 36 | 166 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2568 | 2413 | 2828 | 2658 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 826 | 666 | 1081 | 916 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 | 4363 | 4352 | 4522 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4416 | 4304 | 4469 |

Table 5.3.5.4-2 lists Band n8 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n20.

Table 5.3.5.4-2: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Table 5.3.5.4-3 lists Band n20 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n8.

Table 5.3.5.4-3: Band n20 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 733 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 159 | 99 | 1535 | 1595 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 931 | 1021 | 544 | 634 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367 | 2457 | 2238 | 2328 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1763 | 1883 | 1247 | 1367 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199 | 3319 | 2941 | 3061 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 198 | 318 | 3070 | 3190 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2100 | 1950 | 2745 | 2595 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 535 | 385 | 1180 | 1030 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 | 3794 | 4031 | 4181 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773 | 3923 | 3902 | 4052 |

Based on Table 5.3.5.4-1, 3rd and 5th order IMD may also fall into Rx frequencies of third DL band n28 when CA_n8-n20 is the UL configuration.

Based on Table 5.3.5.4-2, there is no IMD interference which may also fall into Rx frequencies of third DL band n20 when CA_n8-n28 is the UL configuration.

Based on Table 5.3.5.4-3, 3rd order IMD may also fall into Rx frequencies of third DL band n8 when CA_n20-n28 is the UL configuration.

#### 5.3.5.5	REFSENS requirements

TBD.

*****************************************End of the TP*****************************************