3GPP TSG-RAN WG4 Meeting #105	R4-2219840

# Electronic Meeting, November 14 – 18, 2022

Source: 	Qualcomm Incorporated

Title: 	Updated Simulation Results for FR2-2 UE Demodulation PDCCH and PBCH

Agenda item:	6.3.6.2.2

Document for:	Discussion

- Introduction

In RAN4#104-bis-e [2], requirement tables and configurations were agreed for PDCCH and PBCH demodulation requirements for UEs that support FR2-2[1]. In this paper, we share our alignment and impairment results for the definition of the proposed PDCCH and PBCH requirements.

# PDCCH Alignment and Impairment Results

Based on the agreed simulation assumptions and the options provided in [3] for the requirements definition, we provide here the PDCCH SNR Alignment and impairment results for FR2-2 test cases, at 1% misdetection rate.

| Test number | SCS(kHz)/ CBW(MHz | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA30-200 | 1_0 | 40 | 1x2 Low | 3.9 | 6.4 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA30-650 | 1_1 | 56 | 1x2 Low | 0.9 | 3.4 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA30-200 | 1_1 | 56 | 2x2 Low | -2.7 | -0.2 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA30-650 | 1_0 | 40 | 2x2 Low | -5.9 | -3.4 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 2.2 | 4.7 |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low | -1.3 | 1.2 |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | -5.3 | -2.8 |

# PBCH

Based on the agreed simulation assumptions and the options provided in [3] for the requirements definition, we provide here the PBCH SNR Impairment results for FR2-2 test cases, at 1% misdetection rate.

| Test number | SCS/CBW | Antenna configuration | Propagation condition | SS/PBCH block index | Impairment SNR @1% Pm-bch (dB) |
| --- | --- | --- | --- | --- | --- |
| 1-1a | 120kHz/100MHz | 1x2 Low | TDLA30-650 | unknown | -5.6 |
| 1-1b | 120kHz/100MHz |  | TDLA30-200 |  | -5.7 |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 |  | -5 |

- Conclusions

Alignment and Impairment SNR results for FR2-2 PDCCH and PBCH demodulation requirements are included in this contribution.

- References

[1] RP-213540, “Revised WID:  Extending current NR operation to 71 GHz”, 3GPP TSG RAN Meeting #94-e, Electronic Meeting, December 06-17, 2021

[2] R4-2217391, “WF on FR2-2 UE demodulation requirements”, 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, October 10 – 19, 2022;

[3] 3GPP TR 38.884, “Study on enhanced test methods for FR2 NR UEs (Release 18, v18.2.0)”;