3GPP TSG-RAN WG4 Meeting # 106	R4-2301832

Athens, Greece, February 27 – March 3, 2022

Title: 	Consideration on enhanced CHO configurations

Source: 	Huawei, HiSilicon

Agenda item:	9.23.6

Document for:	Discussion

Introduction

One objective of R18 WI on Further NR Mobility Enhancements starts is CHO including target MCG and candidate SCGs for CPC/CPA.

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline - To specify RRM core requirements for the following, as necessary [RAN4]: - L1/L2-based inter-cell mobility - Enhanced CHO configurations addressed by this WI

This contribution provides some considerations on this objective from RRM perspective.

Discussion

“CHO only” and “CHO including target MCG and target SCG” are supported in Rel-17. In R17 for the latter case, the handover execution condition only focus on the target candidate PCell, and there are no triggering conditions on the PSCell. In R18 mobility enhancement WI, one aim is to support CHO with candidate SCGs. As per the current RAN2 progress, the execution conditions of the Target PCell would be configured together with the associated execution conditions of the candidate PSCells to the UE. However it is still under discussion in RAN2 that whether CHO triggering evaluation and CPC/CPA triggering evaluation are concurrent or sequential. The delay requirements for concurrent triggered case and sequential triggered case would be quite different. More discussion are needed in RAN2. RAN4 can wait for the conclusion of RAN2 and afterwards define corresponding requirements accordingly.

- RAN2 119 - Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17. - CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported. - FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential. Chair: NOTE that the above agreements are NOT intended to describe the Stage3 signalling details.

Proposal 1: Whether and how RAN4 define requirements for CHO with candidate SCG can wait RAN2’s conclusion on whether CHO evaluation and CPC/CPA evaluation are concurrent or sequential.

Conclusions

This contribution provides some preliminary consideration on CHO including target MCG and candidate SCGs for CPC/CPA.

Proposal 1: Whether and how RAN4 define requirements for CHO with candidate SCG can wait RAN2’s conclusion on whether CHO evaluation and CPC/CPA evaluation are concurrent or sequential.

Reference

[1].	RP-223520, Revised WID on Further NR mobility enhancements, MediaTek Inc., Apple