3GPP TSG-TSG RAN4 Meeting #106	R4-2302610

# Athens, Greece, 27th Feb – 3rd March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0472 | rev | <-> | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction CR on PUCCH JCE requirements for TS 38.141-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Perf |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The BS demoulation requirement for PUCCH JCEenhancement has been defined in Rel-17. The related big CR has been endorsed in the last meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction the clause number in section 8.3.12 and 8.3.13 Correction the minimum requirement tables in section 8.3.12 and 8.3.13 Remove the [] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is inconsistence of PUCCH JCE requirement for FR2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.12 and 8. 3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.3.12	Performance requirements PUCCH format 1 with DM-RS bundling

#### 8.3.12.1	NACK to ACK detection

##### 8.3.12.1.1	Definition and applicability

The performance requirement of PUCCH format 1 with DM-RS bundling for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1 % or less. The probability of false detection of the ACK shall be 1 % or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause 8.1.2.2.

##### 8.3.12.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], clause 11.3.1.8.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], clause 11.3.2.7.

##### 8.3.12.1.3	Test purpose

The test shall verify the receiver's ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.12.1.4	Method of test

###### 8.3.12.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: for single carrier: M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

###### 8.3.12.1.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.12.1.4.2-1.

Table 8.3.12.1.4.2-1: Test parameters for PUCCH format 1 with DM-RS bundlling

| Parameter | Test 1 [TDD] | Test 2 [FDD] |
| --- | --- | --- |
| Eample TDD UL-DL pattern (Note1) | 15/30 kHz SCS: 7D1S2U, S=6D:4G:4U 60/120 kHz SCS: DDSUU, S=10D:2G:2U or DSUUU, S=10D:2G:2U |  |
| Number of information bits | 2 | 2 |
| Number of PRBs | 1 | 1 |
| Number of symbols | 14 | 14 |
| Intra-slot frequency hopping | disabled | disabled |
| Inter-slot frequency hopping | disabled | disabled |
| Hopping ID | 0 | 0 |
| Initial cyclic shift | 0 | 0 |
| First symbol | 0 | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 | 0 |
| Number of slots for PUCCH repetition | 2 | 8 |
| PUCCH-TimeDomainWindowLength | 2 | 8 |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and PUCCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUCCH is only scheduled for the actual TDW including 2 consecutive UL slots. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in clause 8.3.12.1.5.1 and 8.3.12.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.12.1.4.2-2

Table 8.3.12.1.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O (Note 4) | 15 kHz | 5 | -83.5 – ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 kHz | 10 | -80.6 – ΔOTAREFSENS dBm / 8.64 MHz |
| BS type 2-O (Note 5) | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  | 120 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. NOTE 4: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	The signal generator sends random codewords from applicable codebook, in regular time periods that conform with the required actual Time Domain Window (aTDW) needed by the tests (see subclause 8.2.xxx). The following statistics are kept: the number of ACK bits falsely detected in the idle periods and the number of missed ACK bits. Each falsely detected ACK bit in the idle periods is accounted as one error for the statistics of false ACK detection, and each missed ACK bit is accounted as one error for the statistics of missed ACK detection.

Note that the procedure described in this clause for ACK missed detection has the same condition as that described in clause 8.3.12.1.4.2 for NACK to ACK detection. Both statistics are measured in the same testing.

##### 8.3.12.1.5	Test Requirement

###### 8.3.12.1.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1 % and the fraction of NACK bits falsely detected as ACK shall be less than 0.1 % for the SNR listed in table 8.3.12.1.5.1-1 and table 8.3.12.1.5.1-2.

Table 8.3.12.1.5.1-1: Minimum requirements for PUCCH format 1 with DM-RS bundling with 15 kHz SCS

| Test Number | Number of TX antennas | Number ofdemodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 5 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 | -2.4 |
| 2 | 1 | 2 | Normal | TDLA30-10 | -8.5 |

Table 8.3.12.1.5.1-2: Minimum requirements for PUCCH format 1 with DM-RS bundling with 30 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 10 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 | -2.4 |
| 2 | 1 | 2 | Normal | TDLA30-10 | -8.1 |

###### 8.3.12.1.5.2	Test Requirement for BS type 2-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in tables 8.3.12.1.5.2-1 and table 8.3.12.1.5.2-2.

Table 8.3.12.1.5.2-1: Minimum requirements for PUCCH format 1 with DM-RS bundling with 60 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 | -2.7 |

Table 8.3.12.1.5.2-1: Minimum requirements for PUCCH format 1 with DM-RS bundling with 120 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-75 | -2.6 |

#### 8.3.12.2	ACK missed detection

##### 8.3.12.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause 8.1.2.

##### 8.3.12.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], clause 11.3.1.8.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], clause 11.3.2.7.

##### 8.3.12.2.3	Test purpose

The test shall verify the receiver's ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.12.2.4	Method of test

###### 8.3.12.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: for single carrier: M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

###### 8.3.12.2.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.12.2.4.2-1.

Table 8.3.12.2.4.2-1: Test parameters for PUCCH format 1 with DM-RS bundlling

| Parameter | Test 1 [TDD] | Test 2 [FDD] |
| --- | --- | --- |
| Eample TDD UL-DL pattern (Note1) | 15/30 kHz SCS: 7D1S2U, S=6D:4G:4U 60/120 kHz SCS: DDSUU, S=10D:2G:2U or DSUUU, S=10D:2G:2U |  |
| Number of information bits | 2 | 2 |
| Number of PRBs | 1 | 1 |
| Number of symbols | 14 | 14 |
| Intra-slot frequency hopping | disabled | disabled |
| Inter-slot frequency hopping | disabled | disabled |
| Hopping ID | 0 | 0 |
| Initial cyclic shift | 0 | 0 |
| First symbol | 0 | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 | 0 |
| Number of slots for PUCCH repetition | 2 | 8 |
| PUCCH-TimeDomainWindowLength | 2 | 8 |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and PUCCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUCCH is only scheduled for the actual TDW including 2 consecutive UL slots. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.2.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in clause 8.3.12.2.5-1and 8.3.12.2.5-2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.12.2.4.2-2.

Table 8.3.12.2.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O (Note 4) | 15 kHz | 5 | -83.5 – ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 kHz | 10 | -80.6 – ΔOTAREFSENS dBm / 8.64 MHz |
| BS type 2-O (Note 5) | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  | 120 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. NOTE 4: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	The signal generator sends random codewords from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits falsely detected in the idle periods and the number of missed ACK bits. Each falsely detected ACK bit in the idle periods is accounted as one error for the statistics of false ACK detection, and each missed ACK bit is accounted as one error for the statistics of missed ACK detection.

Note that the procedure described in this clause for ACK missed detection has the same condition as that described in clause 8.3.12.1.4.2 for NACK to ACK detection. Both statistics are measured in the same testing.

##### 8.3.12.2.5	Test Requirement

###### 8.3.12.2.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1 % and the fraction of correctly detected ACK bits shall be larger than 99 % for the SNR listed in table 8.3.12.2.5.1-1 and table 8.3.12.2.5.1-2.

Table 8.3.12.2.5.1-1: Minimum requirements for PUCCH format 1 with DM-RS bundling with 15 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 5 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 | -3.3 |
| 2 | 1 | 2 | Normal | TDLA30-10 | -8.3 |

Table 8.3.12.2.5.1-2: Minimum requirements for PUCCH format 1 with DM-RS bundling with 30 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 10 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 | -3.2 |
| 2 | 1 | 2 | Normal | TDLA30-10 | -7.9 |

###### 8.3.12.2.5.2	Test Requirement for BS type 2-O

The fraction of falsely detected ACK bits shall be less than 1 % and the fraction of correctly detected ACK bits shall be larger than 99 % for the SNR listed in table 8.3.12.2.5.2-1 and table 8.3.12.2.5.2-2.

Table 8.3.12.2.5.2-1: Minimum requirements for PUCCH format 1 with DM-RS bundling with 60 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 | -4.2 |

Table 8.312.2.5.2-2: Minimum requirements for PUCCH format 1 with DM-RS bundling with 120 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | correlation matrix (annex G) | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-75 | -4.3 |

### 8.3.13	Performance requirements for PUCCH format 3 with DM-RS bundling

#### 8.3.13.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause 8.1.2.2.

#### 8.3.13.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], clause 11.3.1.9.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], clause 11.3.2.10.

#### 8.3.13.3	Test purpose

The test shall verify the receiver's ability to detect UCI under multipath fading propagation conditions for a given SNR.

#### 8.3.13.4	Method of test

##### 8.3.13.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table.4.6-1).

##### 8.3.13.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.13.4.2-1.

Table 8.3.13.4.2-1: Test parameters for PUCCH format 1 with DM-RS bundlling

| Parameter | Test 1 [TDD] | Test 2 [FDD] |
| --- | --- | --- |
| Eample TDD UL-DL pattern (Note1) | 15/30 kHz SCS: 7D1S2U, S=6D:4G:4U 60/120 kHz SCS: DDSUU, S=10D:2G:2U or DSUUU, S=10D:2G:2U |  |
| Modulation order | QPSK | QPSK |
| Intra-slot frequency hopping | disabled | disabled |
| Number of PRBs | 1 | 1 |
| Number of symbols | 14 | 14 |
| The number of UCI information bits | 16 | 16 |
| Number of slots for PUCCH repetition | 2 | 8 |
| PUCCH-TimeDomainWindowLength | 2 | 8 |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and PUCCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUCCH is only scheduled for the actual TDW including 2 consecutive UL slots. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in clause 8.3.13.5.1 and 8.3.13.5.2 for BS type 1-O and BS type 2-O respectively, and the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.13.4.2-2.

Table 8.3.13.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O (Note 4) | 15 kHz | 5 | -83.5 – ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 kHz | 10 | -80.6 – ΔOTAREFSENS dBm / 8.64 MHz |
| BS type 2-O (Note 5) | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  | 120 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. NOTE 4: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	The signal generator sends random codewords from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits falsely detected in the idle periods and the number of missed ACK bits. Each falsely detected ACK bit in the idle periods is accounted as one error for the statistics of false ACK detection, and each missed ACK bit is accounted as one error for the statistics of missed ACK detection.

Note that the procedure described in this clause for ACK missed detection has the same condition as that described in clause 8.3.12.1.4.2 for NACK to ACK detection. Both statistics are measured in the same testing.

#### 8.3.13.5	Test requirement

##### 8.3.13.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1 % and the fraction of correctly detected ACK bits shall be larger than 99 % for the SNR listed in table 8.3.13.5.1-1 and table 8.3.13.5.1-2.

Table 8.3.13.5.1-1: Required SNR for PUCCH format 3 with 15 kHz SCS

| Test Number | Number of TX | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | antennas |  |  | correlation matrix (annex G) | configuration | 5 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 Low | No additional DM-RS | -0.9 |
|  |  |  |  |  | Additional DM-RS | 0.5 |
| 2 | 1 | 2 | Normal | TDLA30-10 Low | No additional DM-RS | -4.7 |
|  |  |  |  |  | Additional DM-RS | -5.2 |

Table 8.3.13.5.1-2: Required SNR for PUCCH format 3 with 30 kHz SCS

| Test Number | Number of TX | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | antennas |  |  | correlation matrix (annex G) | configuration | 10 MHz |
| 1 | 1 | 2 | Normal | TDLA30-10 Low | No additional DM-RS | 0.8 |
|  |  |  |  |  | Additional DM-RS | 0.6 |
| 2 | 1 | 2 | Normal | TDLA30-10 Low | No additional DM-RS | -4.6 |
|  |  |  |  |  | Additional DM-RS | -5 |

##### 8.3.13.5.2	Test Requirement for BS type 2-O

The fraction of falsely detected ACK bits shall be less than 1 % and the fraction of correctly detected ACK bits shall be larger than 99 % for the SNR listed in table 8.3.13.5.2-1 and table 8.3.135.2-2.

Table 8.3.13.5.2-1: Required SNR for PUCCH format 3 with 60 kHz SCS

| Test Number | Number of TX | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | antennas |  |  | correlation matrix (annex G) | configuration | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-75 Low | No Additional DM-RS | 0.7 |
| 2 | 1 | 2 | Normal | TDLA30-75 Low | Additional DM-RS | 0.4 |

Table 8.3.13.5.2-2: Required SNR for PUCCH format 3 with 120 kHz SCS

| Test Number | Number of TX | Number of demodulation branches | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | antennas |  |  | correlation matrix (annex G) | configuration | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-75 Low | No Additional DM-RS | 1.3 |
| 2 | 1 | 2 | Normal | TDLA30-75 Low | Additional DM-RS | 0.5 |

<End of Change 1>