3GPP TSG-RAN WG4 Meeting #106bis-e                	R4-2304100

Electronic Meeting, April 17 – 26, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Simulation results of 8Rx CSI requirements

Agenda item: 	5.5.4.1.2

Document for: 	Discussion

Introduction

The achieved agreements from previous RAN4 meetings (RAN4#105 and RAN#106) are detailed in [1] and [2].

In this paper we provide the results of our conducted simulations.

Discussion

We provide in this section the initial simulation results for CQI based on simulation assumption showed in Table 1.

Table 1: Simulation assumption for CQI

| CQI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| Bandwidth |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |  |  |
|  | CSI-RS | slot | 10-janv |  |  |  |
|  | periodicity and offset |  |  |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig | slot | 10-janv |  |  |  |
|  | periodicity and offset |  |  |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping |  | (4, 9) |  |  |  |
|  | (kCSI-IM,lCSI-IM) |  |  |  |  |  |
|  | CSI-IM timeConfig | slot | 10/1 |  |  |  |
|  | periodicity and offset |  |  |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-i1/CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 10-sept |  |  |  |
| aperiodicTriggeringOffset |  |  | Not configured |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | -2,1 |  |  |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | -4,1 |  |  |  |
|  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  | RI Restriction |  | 1000 |  |  |  |
| Physical channel for CSI report |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |
| Measurement channel |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |
| - (N1,N2)=(2,1); (O1,O2)=(4,1) |  |  |  |  |  |  |
| - ReportQuantity: CRI/Ri/i1/CQI |  |  |  |  |  |  |
| - Two-one-TypeI-SinglePanel-Restriction: 0b00000001 |  |  |  |  |  |  |
| - TypeI-SinglePanel-codebookSubsetRestriction-i2: 0x0000000000000001 |  |  |  |  |  |  |

The table below presents  the result of Median CQI assuming 4T8R FR1 40M 30kHz.

| CQI TDD FR1 (static condition) | SNR | CQI | Count | Percent | Median | Prob (Median CQI-1<CQI< Median CQI+1) |
| --- | --- | --- | --- | --- | --- | --- |
| CQI TDD 40M 30kHz FR1 4T8R UE | 18 | 11 | 2 | 0.05 | 12 | 100 |
|  |  | 12 | 3998 | 99.95 |  |  |
|  | 16 | 11 | 3795 | 94.875 | 11 | 100 |
|  |  | 12 | 205 | 5.125 |  |  |
|  | 14 | 10 | 3338 | 83.45 | 10 | 100 |
|  |  | 11 | 662 | 16.55 |  |  |
|  | 12 | 9 | 75 | 1.875 | 10 | 100 |
|  |  | 10 | 3925 | 98.125 |  |  |
|  | 10 | 8 | 2390 | 59.75 | 8 | 100 |
|  |  | 9 | 1610 | 40.25 |  |  |
|  | 8 | 7 | 3978 | 99.45 | 7 | 100 |
|  |  | 8 | 22 | 0.55 |  |  |
|  | 6 | 6 | 3526 | 88.15 | 6 | 100 |
|  |  | 7 | 474 | 11.85 |  |  |
|  | 4 | 5 | 3981 | 99.525 | 5 | 100 |
|  |  | 6 | 19 | 0.475 |  |  |
|  | 2 | 3 | 317 | 7.925 | 4 | 100 |
|  |  | 4 | 3621 | 90.525 |  |  |
|  |  | 5 | 62 | 1.55 |  |  |
|  | 0 | 2 | 58 | 1.45 | 3 | 100 |
|  |  | 3 | 3941 | 98.525 |  |  |
|  |  | 4 | 1 | 0.025 |  |  |
|  | -2 | 1 | 243 | 6.075 | 2 | 100 |
|  |  | 2 | 3740 | 93.5 |  |  |
|  |  | 3 | 17 | 0.425 |  |  |
|  | -4 | 0 | 723 | 18.075 | 1 | 100 |
|  |  | 1 | 3244 | 81.1 |  |  |
|  |  | 2 | 33 | 0.825 |  |  |
|  | -6 | 0 | 3899 | 97.475 | 0 | 100 |
|  |  | 1 | 101 | 2.525 |  |  |

Conclusion

In this contribution we provided the results of our conducted simulation campaign to define the 8 Rx CSI requirements.

References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023