3GPP TSG-RAN WG4 Meeting # 106-bis-e							       R4-2304207

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.14.2.3

Source: 	CMCC

Title: 	Discussion on ATG UE Rx requirements

Document for:	Discussion

# 1. Introduction

RAN4#106 meeting had made some agreements for ATG UE Rx requirements. But there are still some issues that need to be discussed in the WF [1]. At this meeting, we will provide our views on the remaining issues.

# 2. Discussion

After the last meeting, most of the ATG UE Rx requirements have been defined, or wait for the co-existence results. The following Table 1 shows the link budget to calculate the maximum input level. Based on the current deployment, the distance between ATG UE and BS would be larger than 3km. But considering the possibility of ATG serving the aircraft directly above in the future still exists, this paper still uses 3km as the minimum distance.

Table 1. Maximum input level

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Carrier frequency | 2GHz | 4GHz |
| Distance | 3km | 3km |
| Path Loss | 108dB | 114dB |
| BS output power | 46dBm | 53dBm |
| BS antenna gain can assume | 25.1dBi | 25.1dBi |
| ATG UE antenna gain | 0dBi | 17dBi |
| Maximum input level | -37dBm | -19dBm |

Proposal 1: Maximum input level is -37dBm for 2GHz and -19dBm for 4GHz.

We defined two maximum input level requirements for the different antenna types, and this way could be also used to define other ATG UE Rx requirements. So we suggest defining at least two kinds of power classes based on the antenna types.

Proposal 2: Define at least two kinds of power classes based on the antenna types. One for omni-direction and the other for antenna array.

# 3. Conclusions

In this contribution, ATG UE Rx requirements are discussed with the following proposals:

Proposal 1: Maximum input level is -37dBm for 2GHz and -19dBm for 4GHz.

Proposal 2: Define at least two kinds of power classes based on the antenna types. One for omni-direction and the other for antenna array.

# 4. References

[1] R4-2303645, WF on ATG UE RF requirements, Huawei, HiSilicon