3GPP TSG-RAN WG4 Meeting #106 bis-e                   	                                  R4-2304255

# Electronic meeting, April 17 – April 26, 2023

Agenda item:	5.19.2

Source:	Intel Corporation

Title:	Views on the remaining issues for the specification of absolute physical layer throughput requirements

Document for:	Discussion

- Introduction

In RAN4 #106 requirements for physical layer throughput were discussed and way forward [1] was approved. Along with HARQ introduction to test equipment (TE) side, there a few remaining issues for the specification of ATP requirement and we provide our views.

- Remaining Issues

## HARQ Transmission

### The main purpose of H-ARQ is to achieve reliable residual BLER to guarantee desired level of higher layer retransmission. The simulation results in [2] also shows the marginal gain in throughput and initial BLER. After last meeting, offline tentative agreement on retransmission scheduling has been made as below by email discussion.

o For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; for precoder, follow the latest UE reported PMI whose rank is same as the initial transmission.

Proposal 1-1: As in WF [1], investigate the collected HARQ simulation results and check the validity of HARQ enablement with tentatively agreed transmission scheduling scheme for test equipment (TE) operation.

Proposal 1-2: After HARQ results are aligned in the 1st round discussion, company are encouraged to submit impairment results for the 2nd round discussion.

## ATP specifications

### Phy Layer TP test metric

As in WF the previous meeting [1] captured as below, RAN4 would continue discussion with the updated simulation results with HARQ enable or not.

- Average SNR of impairments results to achieve T% of maximum throughput + X dB margin - Use Gspan = [2.5] dB to check if the results are aligned - Use X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM - The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs. - Discuss based on the updated simulation results in the next meeting - Whether X dB margin is applied to alignment results or impairment results - Whether the proposed X dB values are agreeable or not

Proposal 2: Consider single X dB margin in test metric decision, which is to be applied to impairment results since MCS are not fixed under link adaptation in fading channels.

### Test point T (%) selection

Although most of company have a concern on the performance misalignment at rank transition region, we think that it is also possible to go with Option 3. If performance misalignment is a real problem, it can be checked based on simulation alignment results. If rank transition region is not proper test point based on Gspan criteria, we may consider next available T (%) point with 5% step which satisfy Gspan criteria.

- Test SNR selection criteria - Option 1 - Cover both low and higher modulation order/layer - Option 2 - For 2Rx: Choose one in rank 1 and one in rank 2 - For 4Rx: Choose both T points in rank 2 region, one in the medium SNR away from rank transition region, and one close to 20 dB (peak SNR). - Option 2a: Set of SNR with no/frequent rank transitions - Option 2b: - For 4Rx: Choose 1 SNR point in high SNR region. - Option 3 - Choose the SNRdominant RI transition where major of simulation results shows median RI change - For 2Rx, add mid-point in [0 ~ SNRdominant RI transition] range - For 4Rx, add mid-point in [SNRdominant RI transition ~ 20] range - Test points T(%) based on the SNR selection criteria - Option 1: - For FR1 2Rx, T% = (10% or 15%) and (40% or larger) - For FR1 4Rx. T% = (10% or 15% or 20%) and (45% or larger) - For FR2 2Rx, T% = (10% or 15% or 20%) and (40% or larger) - Option 2: - For FR1 2Rx, T% = 10% and 40% - For FR1 4Rx, T% = 15% and 60% - For FR2 2Rx, T% = 10% and 40% - Option 3: - For FR1 2Rx, T% = 10% and 35% - For FR1 4Rx, T% = 20% and 55% - For FR2 2Rx, T% = 10% and 35% - Option 4: - For FR1, T% = 10% and 40% - For FR2, T% = 10% and 35% - Option 5: - Trimming to T (%) with 5% granularity based on Option 3 for SNR selection - For FR1 2Rx, T% = 15% and 30% - For FR1 4Rx, T% = 15% and 40% - For FR2 2Rx, T% = 20% and 35% - Discuss in the next meeting with the following aspect based on the updated simulation results - SNR options considering uniqueness of test SNR coverage - Tentative agreement on T (%) based on simulation results for alignment considering Gspan and margin - Confirm T (%) based on simulation results with impairment. It does not preclude the possibility of adjustment with [+- 5% steps] from alignment perspective.

Proposal 3-1: Consider Option 3 as a starting point of test T (%) selection.

Following Option 3, the results are as below based on HARQ-Off simulation alignment results in [2]. The same rule can be applied to HARQ enable results, too.

1) 1st iteration

| Configuration |  |  | SNR |  | T’ (%) |  | Trim T (%) |  | SNR (dB) |  | Gspan (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RI transition | Additional Point | T’1 | T’2 | T1 | T2 | SNR1 | SNR2 | Gspan1 | Gspan2 |
| FR1 | 2 Rx | FDD | 16 dB | (0+16) /2 = 8 dB | 16.4 | 31.8 | 15 | 30 | 7.1 | 15.2 | 1.8 | 2.9 |
|  |  | TDD | 14 dB | (0+14) /2 = 7 dB | 14.0 | 26.0 | 15 | 25 | 7.5 | 13.6 | 1.6 | 2.0 |
|  | 4 Rx | FDD | 4 dB | (4+20) /2 = 12 dB | 15.0 | 38.3 | 15 | 40 | 4.0 | 12.5 | 2.3 | 1.9 |
|  |  | TDD | 4 dB | (4+20) /2 = 12 dB | 15.2 | 38.7 | 15 | 40 | 3.9 | 12.4 | 2.6 | 2.2 |
| FR2 | 2 Rx | TDD | 14 dB | (0+14) /2 = 7 dB | 19.8 | 36.3 | 20 | 35 | 7.2 | 11.6 | 3.5 | 2.8 |

2) 2nd iteration

Under 2.5 dB Gspan, there are three cases failed and we selected neighbour T(%) values with satisfying Gspan less than equal to 2.5 dB.

| Configuration |  |  | SNR |  | T’ (%) |  | Trim T (%) |  | SNR (dB) |  | Gspan (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RI transition | Additional Point | T’1 | T’2 | T1 | T2 | SNR1 | SNR2 | Gspan1 | Gspan2 |
| FR1 | 2 Rx | FDD | 14 dB | (0+14) /2 = 7 dB | 14.9 | 27.6 | 15 | 20 | 7.1 | 10.1 | 1.8 | 2.0 |
|  |  | TDD | 12 dB | (0+12) /2 = 6 dB | 14.0 | 26.0 | 15 | 25 | 7.5 | 13.6 | 1.6 | 2.0 |
|  | 4 Rx | FDD | 2 dB | (2+20) /2 = 11 dB | 15.0 | 38.3 | 15 | 40 | 4.0 | 12.5 | 2.3 | 1.9 |
|  |  | TDD | 2 dB | (2+20) /2 = 11 dB | 15.2 | 38.7 | 25 | 40 | 7.5 | 12.4 | 2.3 | 2.2 |
| FR2 | 2 Rx | TDD | 12 dB | (0+12) /2 = 6 dB | 18.0 | 30.1 | 15 | 35 | 4.5 | 13.6 | 2.5 | 2.3 |

Proposal 3-2 RAN4 to decide whether 2 test points or 1 test point based on the simulation collection results.

Depending on alignment results, only single point might be meaningful and feasible. For example, two test points satisfying Gspan in FR1 2 Rx. are close each other since there 25% ~ 35% point are ruled out under Gspan of 2.5 dB set-up.

Table 1. FDD 2x2 T(%) with HARQ Off [2]

## Applicability and release dependency

No new UE functionality beyond Rel-15 is required for ATP test. Thus, ATP requirement with link adaptation should be applicable for all NR UEs without any new applicability rules.

Proposal 4: The ATP requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15.

## Work Split for CRs

Once the test point in T (%) values are agreed, next step would be CR preparation. We can consider either Split 1 or Split 2 depending on participation of the interested company.

The section numbering would be like as below according to the agreement in the last meeting [1].

5.X Absolute downlink physical layer throughput with link adaptation 5.X.1 1 Rx requirement (Void) 5.X.2 2 Rx requirement 5.X.2.1 FDD 5.X.2.2 TDD 5.X.1 4 Rx requirement 5.X.1.1 FDD 5.X.1.2 TDD Table 5.X.1.2.xx Test purpose … Table 5.X.1.2.xx Test parameters .. Table 5.X.1.2.xx Minimum performance for ATP requirements Test num; BW / SCS; TDD UL-DLpattern.; Propagation condition; Correlation matrix and antenna config; Reference value. Fraction of max. TP T (%); SNR (dB). 1-1; 100 / 120; TDLA30-5; 2 x 4; [15]; xx.x. 1-2; 100 / 120; TDLA30-5; 2 x 4; [45]; xx.x. 7.X Absolute downlink physical layer throughput with link adaptation 7.X.1 1 Rx requirement (Void) 7.X.2 2 Rx requirement 7.X.2.1 FDD (Void) 7.X.2.2 TDD

Proposal 5: Consider work split for CRs in this meeting and company are encouraged to take CR work split in the 1st round discussion.

| Section |  |  | Split 1 | Split 2 |
| --- | --- | --- | --- | --- |
| FR1 (5.X) | 1 Rx | Void | NA | NA |
|  | 2 Rx | FDD | Company A [Intel] | Company A |
|  |  | TDD |  | Company D |
|  | 4 Rx | FDD | Company B | Company E |
|  |  | TDD |  | Company B |
| FR2 (7.X) | 1 Rx | Void | NA | NA |
|  | 2 Rx | TDD | Company C | Company C |

- Conclusion

Proposal 1-1: As in WF [1], investigate the collected HARQ simulation results and check the validity of HARQ enablement with tentatively agreed transmission scheduling scheme for test equipment (TE) operation.

Proposal 1-2: After HARQ results are aligned in the 1st round discussion, company are encouraged to submit impairment results for the 2nd round discussion.

Proposal 2: Consider single X dB margin in test metric decision, which is to be applied to impairment results since MCS are not fixed under link adaptation in fading channels.

Proposal 3-1: Consider Option 3 as a starting point of test T (%) selection.

Proposal 3-2 RAN4 to decide whether 2 test points or 1 test point based on the simulation collection results

Proposal 4: The ATP requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15.

Proposal 5: Consider work split for CRs in this meeting and company are encouraged to take CR work split in the 1st round discussion.

- References

- R4-2302930 WF on PHY Layer TP requirements, Intel

- R4-2300484 Simulation results of absolute physical layer throughput requirements with link adaptation, Intel