3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2304386

# Electronic Meeting, April 17 – April 26, 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on Enhanced CHO configurations

Agenda item:	5.25.5

Document for:	Discussion

# Introduction

In this paper, we share our view on the view of CHO with CPC per RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18.

# Discussion

RAN2 #121 agreements [1] - Agreement RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18

Unlike to HO with PScell, CHO and CPC delay requirement include Tevent_DU for uncertainty of delay to check the condition is met from decoding RRC command. TEventDU for CHO and CPC can start at the same time. However, it is unknown when UE finish to evaluate the conditions for CHO and CPC as they are independent process. By assuming same starting point but different ending point of evaluation process of CHO and CPC, we identify the potential interruption between two processes which is illustrated at Figure 1.

<Figure1. Example scenario to illustrate potential interruptions>

As shown in Figure1, measurements for CHO or CPC can be interrupted by UE performing RFtune during Tprocessing for the other process. Moreover, when UE is performing Tprocessing for CHO or CPC, it can be interrupted when UE initiate RFtune for the other process.

Proposal: Before defining requirements, RAN4 shall discuss possible interruptions from multiple scenarios due to performing CHO and CPC simultaneously.

By assuming Tevent_DU for CHO and CPC are started at the same time but done in different time, there can be potential interruptions such as

- RF tune during CHO or CPC can interrupt to perform the measurement for the other process of CHO or CPC.

- RF tune may not start at the same time. During Tprocessing, it can be interrupted due to RF tune for the other process of CHO or CPC.

# Conclusion

Proposal: Before defining requirements, RAN4 shall discuss possible interruptions from multiple scenarios due to performing CHO and CPC simultaneously.

By assuming Tevent_DU for CHO and CPC are started at the same time but done in different time, there can be potential interruptions such as

- RF tune during CHO or CPC can interrupt to perform the measurement for the other process of CHO or CPC.

- RF tune may not start at the same time. During Tprocessing, it can be interrupted due to RF tune for the other process of CHO or CPC.

# References

[1] R4-2303176, WF on NR Mobility Enhancements RRM requirements (part 2), Apple, RAN4 #106.