3GPP TSG-RAN4 Meeting #106bis-e	                                                           R4-2304418

Online, April 17 – April 26, 2023

Title:	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR

Source:	CATT

Agenda Item:	5.10.3.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, RRM requirements for UEs reporting NeedForGapsInfoNR IE were discussed and the conclusions were captured in the WF [1]. This contribution further discusses the open issues on the topic and presents our views.

2 Discussion

2.1 Interruption requirements when UE reports ’no-gap’

In last meeting, it was agreed to introduce additional R18 signalling to differentiate UE supporting no gap with interruption and a LS [2] was sent to RAN2 to request the signalling design. It means RAN2 need to define a UE signaling to indicate whether the interruption is needed when UE report “no gap”. How to define the signaling (on top of R16 signaling “NeedForGapInfoNR” or introduce a totally new R18 signaling which is decoupled with “NeedForGapInfoNR”) is up to RAN2. With the agreements, RAN4 needs to define the interruption requirements for the case that “no gap with interruption” is indicated.

For the framework of the interruption requirements, it was agreed in last meeting that the interruption length is should be defined and FFS whether to define interruption ratio or interruption location. We think the interruption ratio can be defined to guarantee the system performance which is similar as legacy requirements. But it is not necessary to define the interruption location which is up to UE implementation.

For the length of each interruption, it should be defined based on RF retuning time (0.5ms for FR1 and 0.25ms in FR2) and there is no reason to use a longer time to define the interruption. And the interruption length is defined as the number of interrupted slots.

For the interruption ratio, we think a total allowed ratio for the measurement without gap is enough and no need to assume that the interruption exists on each SMTC occasion. And the existing requirements for the measurement on deactivated SCC can be reused i.e. allow up to [0.5%] probability of missed ACK/NACK.

Proposal 1: For the case when UE reporting “no gap with interruption”, each interruption length should be defined as the number of interrupted slots corresponding to the RRT (0.5ms in FR1 and 0.25ms in FR2).

Proposal 2: RAN4 to define the interruption ratio for UE indicating no gap with interruption and not to define the interruption location.

Proposal 3: For the case when UE reporting “no gap with interruption”, UE is allowed an interruption on PSCell or any activated SCell with up to [0.5%] probability of missed ACK/NACK.

2.2 Requirement for measurement without gap without interruption

The following case differentiation is used in this discussion:

- Case 1: without gap and no interruption (e.g. ’[TBD1]’ indicated in [TBD new signaling])

- Case 2: without gap but interruption allowed (e.g. ’[TBD2]’ indicated in [TBD new signaling])

It was agreed to take the measurement delay requirements in section 9.3.9 of TS38.133 (inter-freq w/o gap) as starting point but the possible clarifications as below if still open.

- Issue 1-2-2: Requirement for inter-freq measurement without gap when no interruption (Inter-f case 1) < Way forward >: - FFS on: - Proposal 1: - to update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘no-gap’ via interFreq-needForGap - Proposal 2: - updates/clarification on CSSFoutside_gap - Proposal 3: - Define measurement reporting delay requirements for UEs indicating no-gap with interruption considering both deriveSSB-IndexFromCellInter-r17 enabled and disabled

We think the proposals are not exclusive to each other and all of them are reasonable. Firstly, the requirements for inter-frequency measurement without gap without interruption when UE reporting [high layer signalling for no gap no interruption] should be defined in section 9.3.9 of TS 38.133 and the corresponding inter-frequency layers need to be counted in CSSF calculation.

On the other hand, in previous meeting, the UE capability indicating the support of deriveSSB-IndexFromCellInter-r17 for non-NCSG UE was introduced in R17 TEI. This should also be considered in the inter-frequency measurement without gap. But it has been implemented in the current requirements in section 9.3.9 of TS 38.133, so we understand there is no additional work in this WI for this issue.

Proposal 4: Reusing the measurement delay requirements in section 9.3.9 of TS 38.133 (inter-freq w/o gap) for the inter-freq measurement without gap without interruption (case 1) with the following updates:

- update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates [high layer signalling for no gap no interruption]

- updates/clarification on CSSFoutside_gap to count the inter-frequency layers on which UE indicates [high layer signalling for no gap no interruption]

2.3 Requirement for measurement without gap with interruption

- Issue 1-2-1 Requirement for intra/inter-freq measurement without gap when interruption allowed (case 2) < Way forward/Agreement >: - Option 1: Apple, MTK - Can be FFS after RAN4 agree how to define the interruption (length, location or ratio) - Option 2:vivo, Huawei, Ericsson, - The deactivated SCell measurement except the measCycleSCell can be a start point - Option 2a: Huawei - Measurement cycle larger than 160ms can be considered - Option 3: CATT, Xiaomi, Qualcomm, Nokia, - For inter-f case 2,take requirements in 38.133, clause 9.3.9 (inter-freq w/o gap) as a starting point - For intra-f case 2, Take requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) as a starting point for the definition of requirements - Option 3a: Nokia, - considering both deriveSSB-IndexFromCellInter-r17 enabled and disabled - Option 4: OPPO, ZTE, MTK - Take requirements NCSG requirements in TS38.133 clause 9.2.7 and 9.3.10 as a starting point for intra-f and inter-f case2 respectively.

For case 2 in which interruption is allowed, we think the same measurement delay requirements as case 1 can be used since there are still no gap and no NCSG in this case and the requirements are based on the resources periodicity. The only difference is the introduction of interruption requirements.

For the suggestion to define the requirements based on configured measurement cycle, we see some issues to be considered. Firstly, additional signaling on the measurement cycle is needed. Secondly, if the signaling is defined, it only applies to the case when UE report no gap no interruption for inter-frequency measurement. But actually, this case it not different to other inter-frequency measurement without gap cases, there is no reason to define different NW configuration processes which increase the NW complexity.

Proposal 5: For case 2 in which interruption is allowed, the same measurement delay requirements as case 1 can be used for intra-frequency and inter-frequency measurement respectively.

2.4 UE behavior

Since we have introduced NeedForGapNCSG which can also indicates measurement without gap, the relation between NeedForGapsInfoNR  and NeedForGapNCSG need to be clarified. This issue has been discussed in last meeting but there is no consensus.

- Issue 1-3-1: Mapping between NeedForGap and NCSG capabilities when UE supports both of them < Way forward >: - FFS on: - [NeedForGapsInfoNR] and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE at the same time - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG - Issue 1-3-2: UE behaviors mismatch between UE and NW < Way forward >: - FFS on: - No impact on Rel-18 NFG requirements because of mismatch scenarios where either UE or NW support Rel-17 or earlier release. - The requirements of Rel18 NFG will not be applicable to these mismatch scenarios - Rel-17 UE which supports NCSG in a Rel-16 NW which only supports NeedForGaps - Rel-16 UE which supports NeedForGaps in a Rel-17 NW which supports NCSG - Both UE and NW support NCSG and NeedForGaps - Others are not precluded

Generally we think these two UE are performing the same function, and there is no need to report both. When UE support NCSG, the reporting of NeedForGapNCSG is enough, no need to repeatedly report  NeedForGapsInfoNR . If unfortunately both capabilities are reported, we think the report for gap and no-gap should be aligned. Otherwise, they will cause confusion between UE and gNB understanding.

Also since it is possible to define a totally new R18 signaling to indicate measurement without gap with interruption and without interruption which is decoupled with R16 NeedForGapsInfoNR, both of the signaling report will not impact the introduced R18 requirements. In this case, the clarification on the relation of the two capabilities is a R17 issue and not related to this WI.

For the impact on Rel-18 NFG requirements due to mismatch between UE and NW, it depends on the signalling design from RAN2. If the signalling is defined on top of R16 capability NeedForGapsInfoNR, the R18 requirements will apply only to the UE supporting R16 capability. But if the signaling is defined as total new and decoupled with R16 capability, then there is no impact on the R18 requirements due to mismatch of the supported release. So RAN4 can postpone the issue until RAN2 design is clear.

Proposal 6: NeedForGapsInfoNR and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE.

Proposal 7: RAN4 to discuss issue 1-3-2 the impact on R18 requirements due to mismatch of supported capabilities after RAN2 signaling design is clear.

- Issue 1-3-3: Impacts on the legacy UE behavior < Agreement >: - Proposal 1: Intel, Nokia, ZTE, CATT - Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2 < Way forward >: - FFS on: - Proposal 2: Qualcomm - It is up to UE what reporting capability is used for reporting when both R17 and R18 reporting capability are supported - Proposal 3: Nokia - Indication of “no-gap” as part of needForGaps or needForGapsNCSG means no-gap Case 1 (no gap without interruption) - Proposal 4: E/// - Rel-16 UE is assumed to need interruption since no new interruption indication bit will be reported.

For the impact on legacy UE behavior, it is also related to the RAN2 signaling design. If the signalling is defined on top of R16 capability NeedForGapsInfoNR, whether interruption is needed when UE reports NeedForGapsInfoNR is based on the signaling indication. But if the signaling is defined as total new and decoupled with R16 capability, then requirements apply when UE report R16 capability NeedForGapsInfoNR and no interruption should be assumed. So RAN4 can postpone the discussion until RAN2 design is clear.

Proposal 8: RAN4 to discuss issue 1-3-3 the impact on legacy UE behavior after RAN2 signaling design is clear.

2.5 Scheduling availability

For the scheduling availability for the measurement without gap (for both with interruption and without interruption), the requirements in section 9.2.5.3 and section 9.3.9.3 can be reused for the intra-frequency measurement and inter-frequency measurement respectively. The scheduling restriction is applied for both case 1 and case 2.

Proposal 9: Take the similar requirements for intra-/inter-frequency measurement without gaps (TS38.133 section 9.2.5.3 and section 9.3.9.3) as baseline to define scheduling availability and the requirements apply to both case 1 (with interruption) and case 2 (with interruption).

3 Summary

This contribution discusses the RRM requirements for UEs reporting the NeedForGapsInfoNR IE. The following proposals are made.

Proposal 1: For the case when UE reporting “no gap with interruption”, each interruption length should be defined as the number of interrupted slots corresponding to the RRT (0.5ms in FR1 and 0.25ms in FR2).

Proposal 2: RAN4 to define the interruption ratio for UE indicating no gap with interruption and not to define the interruption location.

Proposal 3: For the case when UE reporting “no gap with interruption”, UE is allowed an interruption on PSCell or any activated SCell with up to [0.5%] probability of missed ACK/NACK.

Proposal 4: Reusing the measurement delay requirements in section 9.3.9 of TS 38.133 (inter-freq w/o gap) for the inter-freq measurement without gap without interruption (case 1) with the following updates:

- update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates [high layer signalling for no gap no interruption]

- updates/clarification on CSSFoutside_gap to count the inter-frequency layers on which UE indicates [high layer signalling for no gap no interruption]

Proposal 5: For case 2 in which interruption is allowed, the same measurement delay requirements as case 1 can be used for intra-frequency and inter-frequency measurement respectively.

Proposal 6: NeedForGapsInfoNR and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE.

Proposal 7: RAN4 to discuss issue 1-3-2 the impact on R18 requirements due to mismatch of supported capabilities after RAN2 signaling design is clear.

Proposal 8: RAN4 to discuss issue 1-3-3 the impact on legacy UE behavior after RAN2 signaling design is clear.

Proposal 9: Take the similar requirements for intra-/inter-frequency measurement without gaps (TS38.133 section 9.2.5.3 and section 9.3.9.3) as baseline to define scheduling availability and the requirements apply to both case 1 (with interruption) and case 2 (with interruption).

4 References

- R4-2303305	WF on Rel-18 NR measurements without gaps, Intel, RAN4#106

- R4-2303306 LS on measurements without gap, Intel,CATT, RAN4#106

- R4-2302772	Topic summary for [106][214] NR_MG_enh2_part2, Intel, RAN4#106