3GPP TSG-RAN4 Meeting #106-bis-e	R4-2304569

# Online, 17th – 26th April 2023

Source:				Ericsson

Title:					NTN enhancements – SAN RF requirements

Agenda item:		5.27.3

Document for:		Approval

# Introduction

This document discusses the SAN RF requirements in the scope of the NTN enhancements WI ([1]), based on the Way Forward [2] agreed in last RAN4#106 meeting.

# Discussion

## SAN RF overview

Based on last RAN4#106 way forward [2], the following table gives a snapshot of the SAN RF requirements status. The conducted requirements are not listed as it was agreed to prioritize radiated requirements.

The requirements in red would require further studies, the ones in orange are waiting for the simulations results and the ones in black are either not applicable or known (even they might still require some effort).

| OTA Tx requirements | Status |
| --- | --- |
| Radiated transmit power | Based on manufacturer declaration |
| OTA SAN output power | Based on manufacturer declaration |
| OTA Output power dynamics |  |
| OTA RE power control dynamic range | NA |
| OTA Total power dynamic range | Based on TS 38.104 section 9.4.3.3. |
| OTA Transmit ON/OFF power | NA |
| OTA Transmitted signal quality |  |
| OTA Frequency error | Based on TS 38.104 section 9.6.1 |
| OTA Modulation quality | Based on TS 38.104 section 9.6.2 for QPSK and 16QAM, 64QAM FFS |
| OTA Time alignment error | NA |
| Unwanted emissions |  |
| OTA Occupied bandwidth | Based on TS 38.104 section 9.7.2 |
| OTA relative ACLR | Based on co-existence study outcomes |
| OTA absolute ACLR | NA |
| OTA OBUE | Based on related ITU recommendation SM.1541-6. |
| OTA TX spurious emissions | General requirement: based on ITU-R SM.329. Protection of the SAN receiver: NA |
| OTA TX intermodulation | NA |
| OTA Rx requirement | Status |
| OTA sensitivity | NA |
| OTA Reference sensitivity level | Based on TS 38.104 section 10.3 The range of EIS values applicable to the NTN SAN deployment in FR2 requires further study. |
| OTA Dynamic range | Based on IoT simulations outcomes |
| OTA ACS | Based on co-existence study outcomes |
| OTA In-band blocking | NA |
| OTA Out of band blocking | To be further studied |
| OTA Receiver spurious emission | NA |
| OTA Receiver intermodulation | NA |
| OTA In-channel selectivity | Based on IoT simulations outcomes |
| co-location requirements | NA |

It should be noted that the WF also aligned  [2] the assumptions for FR2 with the ones taken for FR1, i.e. the SAN doesn’t support multi-band, nor non-contiguous multi-carrier operations. Note that these agreements were not made for technical reason but to simplify and speed up the specification effort, this could be then reconsidered in a future release.

To make this clear and avoid any ambiguity, we then propose to capture this in the following proposals:

Proposal1: Like for FR1, the SAN doesn’t support multi-band operation above 10 GHz in Rel-18.

Proposal2: Like for FR1, the SAN doesn’t support non-contiguous multi-carrier operation above 10 GHz in Rel-18.

## New requirements

Regulators have specified additional requirements for GSO and NGSO FSS space stations transmitting in the proposed NTN Ka-bands. FCC has specified power flux density limits for GSO space station in the 17.7-19.7 GHz, and for space station in the 18.6-18.8 GHz (47 CFR 25.208).

Such requirements should be introduced in RAN4 specifications for the corresponding satellite NTN bands n511 and n510.

Proposal3: RAN4 should further discuss on introducing SAN power flux density requirements (n511 and n510 specific requirements) based on regulations limits.

# Conclusion

In this contribution, we analyzed the remaining open issues for the SAN RF requirements and operation above 10GHz. We also made some clarification on the Rel-18 scope. We made the following proposals:

Proposal1: Like for FR1, the SAN doesn’t support multi-band operation above 10 GHz in Rel-18.

Proposal2: Like for FR1, the SAN doesn’t support non-contiguous multi-carrier operation above 10 GHz in Rel-18.

Proposal3: RAN4 should further discuss on introducing SAN power flux density requirements (n511 and n510 specific requirements) based on regulations limits.

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2302877, WF for above 10GHz SAN RF requirements, Thales