3GPP TSG-RAN WG4 Meeting # 106-bis-e		R4-2304818

# Online, April 17 – April 26, 2023

Agenda item:			5.27.5

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for NTN enhancement

Document for:	Discussion

# Introduction

In this contribution, our views on the mobility enhancement impact on RRM requirement are provided.

# Mobility enhancement impact on RRM requirement

According to the latest WID [2],

4.1.4	NTN-TN and NTN-NTN mobility and service continuity enhancements

This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility.

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

  - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

  - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

one of objectives is NTN-TN and NTN-NTN mobility and the scope is provided as below.

NTN-TN enhancements

The TN coverage is agreed to be broadcasted in RAN2#121 [4]

Agreements:

- TN coverage area information will be associated to the frequency information.

- RAN2 adopts explicit description of geographical TN area, and focuses on the following options for further discussion, taking the signalling overhead into account (FFS on the accuracy of the information):

Option 1: The corresponding geographical area information is provided by network with location coordinates of area center and radius.

Option 2: a boundary line is provided by network in the format of a list of location coordinates, additionally an indication can be used to indicate which side is the TN side

Option 6: for each TN area, a list of locations is provided by network, and the corresponding close shape could be illustrated by a polygon connecting these points within the list.

Agreements:

- As a baseline, broadcast signalling is used to provide the information on the TN coverage area for UEs supporting NTN.

- Also based on the signalling overhead of the broadcast solution, RAN2 will further consider the option that UE-specific update can be optionally be provided via dedicated signalling, overriding the broadcast configuration (FFS if via RRC or higher layers. FFS on the validity time, if provided by RRC)

Agreements:

- We don’t introduce additional cell reselection prioritization rules for NTN vs TN in Rel-18 (e.g. per service type, per mobility state, or per UE type) on top of what specified in Rel-17

Proposal 1: RAN4 to discuss the requirement impact on cell reselection when TN coverage area information is provided.

NTN-NTN enhancements

The following agreements on  earth-moving are agreed in RAN2#121 [4]

Agreements:

- In R18, for earth-moving system, satellite with steerable beam is not considered as part of mobility enhancement in NTN.

- A serving cell reference location and a distance threshold/radius will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris).

- For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell

Agreements:

- For earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused, i.e., for cell reselection in earth-moving cell, UE initiates measurements when its location to serving cell reference location is larger than the configured distance threshold.

Observation 1: RAN2 agreed for earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused.

Proposal 2: RAN4 to discuss the RRM requirement for location-based cell reselection for earth-moving cell.

# Summary

The observations and proposals in this contribution are summarized:

Observation 1: RAN2 agreed for earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused.

Proposal 1: RAN4 to discuss the requirement impact on cell reselection when TN coverage area information is provided.

Proposal 2: RAN4 to discuss the RRM requirement for location-based cell reselection for earth-moving cell.

# Reference

- R4-2303238, WF on NR NTN enhancement RRM requirements, Qualcomm, Feb. 2023.

- RP-230809, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, RP#99 meeting, Mar. 2023.

- R2-2211101, Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online.

- R2-2301901, Report from Break-Out Session on NTN, IoT NTN and RedCap, Vice Chairman (ZTE), RAN2#121, Feb. 2023.