3GPP TSG-RAN WG4 Meeting # 106-bis-e                              R4-2305023

# Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.10.2.2

Source:	China Telecom

Title:	Discussion on Case 1 requirements for Pre-MG and concurrent MG

Document for:	Discussion

- Introduction

In RAN4 #106 meeting, RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG were discussed further and the WF [1] was approved.

In this paper, we present our views about Case 1 RRM requirements for the combination of Pre-configured MG and concurrent MG issues captured in the WF [1].

- Discussion

Regarding Case 1 RRM requirements for the combination of Pre-configured MG and concurrent MG, there are issues about Pre-MGs activation/deactivation procedure, and collision handling being discussed in this paper.

Pre-MGs activation/deactivation procedure

Status in the WF [1]:

## Issue 3-2-1: [Case 1] Define definitions for simultaneous and non-simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG < Agreement >: - Definitions for simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation - In simultaneous case, the multiple Pre-MGs activation/deactivation duration are fully or partially overlapping (before any potential delay extension) in time. - In non-simultaneous case, the multiple Pre-MGs activation/deactivation duration are not overlapping (before any potential delay extension) in time. - FFS the requirements, e.g., triggered by the same or different commands. Issue 3-2-4: [Case 1] Whether to extend the delay for simultaneous activation/deactivation for Pre-MG+Pre-MG < Wayforward >: - FFS. Issue 3-2-5: [Case 1] Whether to extend the delay for non-simultaneous activation/deactivation for Pre-MG+Pre-MG < Agreement >: - For the non-simultaneous two Pre-MGs activation/deactivation case, the existing Rel-17 Pre-MG (de)activation delay requirements can be reused.

As agreed in the WF, in simultaneous case, the multiple Pre-MGs activation/deactivation duration are fully or partially overlapping (before any potential delay extension) in time, and whether to extend the delay for simultaneous activation/deactivation for Pre-MG + Pre-MG is FFS. In non-simultaneous case, the multiple Pre-MGs activation/deactivation duration are not overlapping (before any potential delay extension) in time, and for the non-simultaneous two Pre-MGs activation/deactivation case, the existing Rel-17 Pre-MG (de)activation delay requirements can be reused.

In our views, in simultaneous multiple Pre-MGs activation/deactivation case, if the multiple Pre-MGs activation/deactivation are triggered by the same event, the existing Rel-17 Pre-MG (de)activation delay requirements can be reused. If the multiple Pre-MGs activation/deactivation are triggered by the different events, the second Pre-MG is (de)activated before the first Pre-MG (de)activation procedure is complete, the delay requirement for Pre-MG + Pre-MG is expected to be extended.

Proposal 1: In simultaneous multiple Pre-MGs activation/deactivation case, if the multiple Pre-MGs activation/deactivation are triggered by the same event, the existing Rel-17 Pre-MG (de)activation delay requirements can be reused.

Proposal 2: In simultaneous multiple Pre-MGs activation/deactivation case, if the multiple Pre-MGs activation/deactivation are triggered by the different events, the delay requirement for Pre-MG + Pre-MG is expected to be extended.

Collision handling

Status in the WF [1]:

- Issue 3-3-1: [Case 1] Required changes for Pre-MG on collision < Wayforward >: - No consensus on whether to consider deactivated Pre-MG in a collision. - FFS whether an additional capability is needed if collisions on Pre-MG is only considered when Pre-MG is activated.

In our opinion, for the combination of Pre-configured MG and concurrent MG, when the Pre-MG is deactivated, there is only legacy MG being used, e.g., the Type-2 MG being used, thus it’s not necessary to consider collisions between deactivated Pre-MG and the legacy MG. In our understanding, when a Pre-MG with high priority is deactivated, it can depend on UE implementation or NW decision whether to active the deactivated Pre-MG in order to measure the associated measurement with this Pre-MG.

We stick to follow the previous agreement that collisions between Pre-MG and other MG is only considered when the Pre-MG is activated. When the Pre-MG is activated, we think that Rel-17 collision mechanism between concurrent measurement gaps can be reused and it’s not necessary to introduce an additional capability.

Proposal 3: RAN4 to stick to the agreed baseline that collisions between Pre-MG and other MG is only considered when the Pre-MG is activated.

Proposal 4: Rel-17 collision mechanism between concurrent measurement gaps can be reused and it’s not necessary to introduce an additional capability.

- Conclusion

The contribution presents our views about Case 1 RRM requirements for the combination of Pre-configured MG and concurrent MG, with the following proposals:

Proposal 1: In simultaneous multiple Pre-MGs activation/deactivation case, if the multiple Pre-MGs activation/deactivation are triggered by the same event, the existing Rel-17 Pre-MG (de)activation delay requirements can be reused.

Proposal 2: In simultaneous multiple Pre-MGs activation/deactivation case, if the multiple Pre-MGs activation/deactivation are triggered by the different events, the delay requirement for Pre-MG + Pre-MG is expected to be extended.

Proposal 3: RAN4 to stick to the agreed baseline that collisions between Pre-MG and other MG is only considered when the Pre-MG is activated.

Proposal 4: Rel-17 collision mechanism between concurrent measurement gaps can be reused and it’s not necessary to introduce an additional capability.

- References

- R4-2303197, WF on Rel-18 NR MG enhancements, MediaTek inc., RAN4 #106, Feb. 27th – March 3rd, 2023.