3GPP TSG-RAN WG4 Meeting # 106-bis	R4-2305463

# Online, April 17-April 26, 2023

Title: 	Discussions on SDR requirements for 8Rx

Source: 	Huawei, HiSilicon

Agenda item:	5.4.5.1.2

Document for:	Discussion

Background

As per [1], RAN4 agreed to provide SDR simulation results in this meeting to determine the maximum achievable for each modulation order. In this paper, we provide our simulation results and views on remain open issues.

Discussions

PDCCH only has 1 MIMO layers, if 8Tx is used, the codebook contain 64 precoding matrixes many of which leads to zero input on receiving antenna and it is difficult to describe the precoding matrix restriction due to the large size of codebook. To be simple, we suggest to only use 2Tx to transmit PDCCH with PMI index limited to 0 and 2. It is noted that no external noise is added so PDCCH can always be decoded successfully.

Observation 1: There are 64 precoding matrixes in (8Tx, 1 layer) codebook, many of which leads to zero input on reviving antenna.

Proposal 1: Use 2Tx to transmit PDCCH for SDR test

Simulation results for 64QAM, 256QAM and 1024QAM are captured in Figure 2-1:

Figure 2-1: Simulation results for SDR test

Table 2-2 summarizes the target SNR@85% of max TP for each modulation

Table 2-1: Summary of Target SNR.

| Modulation and Rank | MCS | SNR(dB)@85% of max TP |
| --- | --- | --- |
| 64QAM, Rank8 | 25 | 18.2 |
|  | 26 | 19.7 |
|  | 27 | 20.9 |
|  | 28 | 22.9 |
| 256QAM, Rank8 | 24 | 23.9 |
|  | 25 | 25.6 |
|  | 26 | 26.8 |
|  | 27 | 28.8 |
| 1024QAM, Rank2 | 23 | 22.9 |
|  | 24 | 24.8 |
|  | 25 | 27.1 |
|  | 26 | Inf |
| 1024QAM,Rank4 | 23 | 26.0 |
|  | 24 | 27.9 |
|  | 25 | Inf |
|  | 26 | Inf |

We assume the upper bound of SNR is 25dB. Hence, we propose the following:

Proposal 2: Consider following MCS configuration and MIMO layers combination for each modulation

- Define SDR requirements for 64QAM with 8 layers up to MCS28

- Define SDR requirements for 256QAM with 8 layers up to MCS24

- Define SDR requirements for 1024QAM with 2 layers up to MCS24

Conclusion

In this paper we provide our views on 8Rx SDR test. The proposals and observations are:

Observation 1: There are 64 precoding matrixes in (8Tx, 1 layer) codebook, many of which leads to zero input on reviving antenna.

Proposal 1: Use 2Tx to transmit PDCCH for SDR test

Proposal 2: Consider following MCS configuration and MIMO layers combination for each modulation

- Define SDR requirements for 64QAM with 8 layers up to MCS28

- Define SDR requirements for 256QAM with 8 layers up to MCS24

- Define SDR requirements for 1024QAM with 2 layers up to MCS24

- 4   Reference

- R4-2302942 WF for 8Rx UE performance requirements, Huawei, HiSilicon