3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305512

Online, April 17 – April 26, 2023

Agenda Item:	5.5.4.1.3

Source:	Ericsson

Title:	Remaining issues on CQI reporting for 8Rx in FR1

Document for:	Discussion

# 1	Introduction

Last meeting #106, some progress has been achieved in this WI. In this meeting, we go through the remaining issues that still need to be addressed as stated in [1]. Below, we summarize the remaining open issues on the general aspects and PDSCH requirements’ definition.

Issue 4-1: Simulation assumptions for CQI requirmeents - Antenna configuration: XP 4T8R - (N1,N2)=(2,1); (O1,O2)=(4,1) - ReportQuantity: CRI/Ri/i1/CQI - Two-one-TypeI-SinglePanel-Restriction: 0b00000001 - TypeI-SinglePanel-codebookSubsetRestriction-i2: 0x0000000000000001 Issue 4-2: Summary of simulation assumptions for CQI requirements Companies are encouraged to bring simulation results based on assumptions listed in Table 4-1 Issue 4-3: SNR for CQI requirements - Option 1: [2, 3] dB and [8, 9]dB - Option 2: [4, 5] dB and [10, 11]dB - Option 3: [1, 2] dB and [7, 8] dB - Other options are not precluded

It is worth mentioning that we carried out simulations in [2] to draw our proposals.

# 2	CQI reporting requirements

In WF [1], we summarized the simulation parameters in the following Table.

Table 2-1 Simulation parameters for CQI reporting .

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| andwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,0 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-i1/CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

Following the restriction on the codebook, the precoding matrix is given by

W= 14111-1111-1111-1-1-1-11

Based on the simulation results provided in [2], we note that 256QAM can be tested since the corresponding its maximum SNR levels is at 30.45 dB. Consequently, we would like to encourage the companies to boost the modulation order to 256QAM instead of 64QAM.

Fig. 1 – Median CQI as function of SNR. CQI reporting under static condition for 8 Rx UE in FR1.

Observation 1: We prefer proposing test points using 256QAM as the highest modulation order for the CQI reporting with Rank 4 based on the measurement channel as specified in Table A.4-3 using TBS.3-4 in TS 38.101-4.

Proposal 1: Define the CQI reporting requirements for 8Rx UE under static conditions while considering Rank 4, QAM256 as the highest modulation order and based on the measurement channel as specified in Table A.4-3 using TBS.3-4 in TS 38.101-4, with test points

- 256QAM is at SNR = 16/17 dB (CQI 12).

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Observation 1: We prefer proposing test points using 256QAM as the highest modulation order for the CQI reporting with Rank 4 based on the measurement channel as specified in Table A.4-3 using TBS.3-4 in TS 38.101-4.

Proposal 1: Define the CQI reporting requirements for 8Rx UE under static conditions while considering Rank 4, QAM256 as the highest modulation order and based on the measurement channel as specified in Table A.4-3 using TBS.3-4 in TS 38.101-4, with test points

- 256QAM is at SNR = 16/17 dB (CQI 12).

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# References

- R4-2302942, “WF on 8RX UE performance requirements”, Huawei.

- R4-2305513, “Simulation results for CQI reporting for 8Rx in FR1”, Ericsson.

# Appendix	Simulation assumption

## A.1	CQI reporting in static condition

### We use the following tables in TS 38.101-4

Table A.4-3: Mapping of CQI Index to Information Bit payload (CQI table 2, Rank 3 and Rank 4)

| TBS Scheme |  |  |  | TBS.3-1 | TBS.3-2 | TBS.3-3 | TBS.3-4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |  |  |
| Number of PDSCH MIMO layers |  |  |  | 3 | 4 | 3 | 4 |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |  |  |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |  |  |
| 1 | 0.2344 | 0 | QPSK | 4360 | 5896 | 8976 | 11784 |  |  |
| 2 | 0.3770 | 1 |  | 7048 | 9480 | 14344 | 18976 |  |  |
| 3 | 0.8770 | 3 |  | 16392 | 22032 | 33816 | 45096 |  |  |
| 4 | 1.4766 | 5 | 16QAM | 27656 | 36896 | 56368 | 75792 |  |  |
| 5 | 1.9141 | 7 |  | 35856 | 48168 | 73776 | 98376 |  |  |
| 6 | 2.4063 | 9 |  | 45096 | 60456 | 92200 | 122976 |  |  |
| 7 | 2.7305 | 11 | 64QAM | 51216 | 67584 | 104496 | 139376 |  |  |
| 8 | 3.3223 | 13 |  | 62504 | 81976 | 127080 | 167976 |  |  |
| 9 | 3.9023 | 15 |  | 73776 | 98376 | 147576 | 196776 |  |  |
| 10 | 4.5234 | 17 |  | 83976 | 112648 | 172176 | 229576 |  |  |
| 11 | 5.1152 | 19 |  | 96264 | 127080 | 196776 | 262376 |  |  |
| 12 | 5.5547 | 21 | 256QAM | 104496 | 139376 | 213176 | 278776 |  |  |
| 13 | 6.2266 | 23 |  | 116792 | 155776 | 237776 | 319784 |  |  |
| 14 | 6.9141 | 25 |  | 129128 | 172176 | 262376 | 352440 |  |  |
| 15 | 7.4063 | 27 |  | 139376 | 184424 | 278776 | 376896 |  |  |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-2 of TS 38.214 [12] |  |  |  |  |  |  |  |  |  |