3GPP TSG-RAN WG4 Meeting # 106-bis	R4-2305570

Electronic Meeting, April 17th – 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Core Requirements for LPHAP Use Case 6 in NR Positioning

Agenda item:	5.23.3.3

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning includes the enhancements for LPHAP.

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In RAN4#106, the PRS measurement requirements for LPHAP use case 6 in RRC_IDLE is discussed.

Agreements: - Existing PRS measurement requirements in RRC_INACTIVE state defined in 38.133 Rel-17 can be reused for defining the corresponding PRS measurement requirements in RRC_IDLE state.

RRM core requirements are discussed in this contribution and few proposals on further proceedings are made.

Discussion

Impacts to RRM core requirements for some of the above enhancements are discussed in this section.

RRM Impacts

RAN4 will define the measurement period and reporting requirements for DL PRS measurements in the LPHAP use-case 6 scenario both in RRC_INACTIVE and RRC_IDLE states.

In RRC_INACTIVE state, these requirements need to be specified for eDRX cycle > 10.24s. According to WID [1], the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. Therefore, the eRedCap UE’s requirements for eDRX cycle > 10.24 sec can be used as a baseline of the requirements for positioning of LPHAP use case 6.

The requirements for eDRX cycle > 10.24 sec in Rel-18 eRedCap WI is used as a baseline of the requirements for eDRX cycle > 10.24 sec in LPHAP use case 6.

Moreover, the measurement and measurement reporting requirements can be investigated considering the change of SRS positioning validity area for LPHAP in RRC_INACTIVE. For area-based SRS configuration, when a validity area consists of multiple cells, the UE can continue sending SRS without the update of SRS configurations if it remains within the coverage of any of the cells in the validity area. Therefore, SRS configuration could not change when moving to a different cell within a validity area. Otherwise, if the UE leaves the validity area, a new SRS configuration may be required for UE. In this case, we can investigate if any measurement period and measurement reporting requirement need to be specified.

RAN4 to investigate and specify the RRM impact, if any, due to the introduction of SRS positioning validity area.

Conclusion

The scope of RRM impacts for Rel-18 Expanded and Improved NR positioning is investigated in this contribution.

The following proposals are made.

- The requirements for eDRX cycle > 10.24 sec in Rel-18 eRedCap WI is used as a baseline of the requirements for eDRX cycle > 10.24 sec in LPHAP use case 6.

- RAN4 to investigate and specify the RRM impact, if any, due to the introduction of SRS positioning validity area.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation, RAN#99

- Annex

The agreements related to LPHAP positioning in RAN1#112 are captured below.

Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration. Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. - FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.