3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305706

Online, April 17th – 26th, 2023

Agenda Item:	5.7.3.3

Source:	MediaTek Inc.

Title: 	On Beam correspondence test issues

Document for:	Discussion

# Introduction

In RAN4#106, a WF was agreed on beam correspondence requirements for initial access and RRC_INACTIVE state [1]. Regarding test related issues, there are some issues still open:

## Sub-topic 3-1 Test issues in RAN4/RAN5 Issue 3-1-2: Which test issues are for further discussion within RAN4 - Proposals - Option 1: how to reach the MOP in IA - Option 2: feasibility of beam lock function in IA; - Option 3: testability of dual polarization in IA; - Option 4: others - WF - FFS Issue 3-1-3: What should be further informed to RAN5 for the test issues left to RAN5 - Proposals - Option 1: None - Option 2: Others - WF - FFS ## Sub-topic 3-2 Feasibility to achieve maximum output power - Proposals - Option 1: Feasible by holding RAR. - Option 2: Feasible already from the first preamble. - Option 3: Using a ra-ResponseWindow timer only for the last preamble transmission to reduce test time. - Option 4: we should carefully take care of ra-ResponseWindow parameter to make sure the EIRP testing has been finished based on max power before fourth re-transmission of PREAMBLE. - Option 5: - for beam correspondence of initial access and RRC_Inactive, Pcmax is not configured. - powerRampingStep is configured as 6dB, and further discuss if preambleReceivedTargetPower is configured as -100dBm or higher (before calibration). - further discuss if a calibration process is needed before test case is run for the test direction, so as to configure reasonable values for ss-PBCH-BlockPower and rsrp-ThresholdSSB - it is proposed to holding RAR and further discuss the value for preambleTransMax. - Option 6: Beam lock function could be used to require UE to transmit PRACH with PCMAX (Huawei) - WF - FFS ## Sub-topic 3-4 Polarization issues - Proposals - Option 1: DL polarizations during msg1 EIRP verification follow same practice as PUSCH EIRP testing. - Option 2: Beam lock function could be used to solve the polarization issue. - Option 3: Others - WF - Option 3: Wait for RAN5 on beam lock.

And RAN5 has sent the reply LS on RAN4’s questions on test issues related to beam correspondence in initial access [2]:

Question 1: whether it is necessary to introduce the UBF function in initial access for simplification of the test (note that the beam used in initial access may be different from the beam in connected mode)? Response: Yes, it would be necessary to evaluate and introduce a new UBF-like test function in initial access or update the existing UBF test function if the objective of the test is to lock the beam during initial access. The current UBF applies only in RRC Connected Mode. If the core requirements are specified for both Standalone (SA) and Non-Standalone (NSA), the implementation of test function has to be discussed separately. Question 2: RAN4 would also like to invite RAN5 to check if there is any other solution on testing the EIRP in initial access. e.g., for testing EIRP of Msg1. Response: There are currently no RF requirements in RAN5 that require testing of EIRP in initial access therefore this scope has not been discussed for conformance testing. New test procedure would have to be introduced to establish and maintain maximum power/peak EIRP during initial access. Question 3: RAN4 would also like to know how to ensure P-MPR is set to 0 during the test. Response: It’s specified that P-MPR is 0dB in conformance testing in 6.2.4 of TS 38.101-2. There is no other specific action in RAN5 to ensure P-MPR is set to 0.

In this contribution, we discuss further those open issues by taking into account RAN5’s reply.

# Discussion

## 2.1 Beamlock-like test function in initial access

According to RAN5’s reply, although RAN5 has not started works on Rel-18 FR2 enhancements, it is confirmed that a new UBF-like test function in initial access or update the existing UBF test function to lock the beam during initial access, and it is also confirmed that testing EIRP in initial access requires new test procedure to establish and maintain maximum power/peak EIRP during initial access.

Observation 1: According to RAN5’s reply, for FR2 enhancement requirements, either new UBL-like test function is introduced or the existing UBF test function is updated to lock the beam during initial access, and new test procedure is required as well to establish and maintain maximum power/peak EIRP during initial access.

With the confirmation from RAN5, RAN4 may assume beam lock function and there is no testability issue on polarization for beam correspondence requirements in initial access.

Proposal 1: RAN4 may assume beam lock function and no testability issue on polarization for beam correspondence requirements in initial access.

## 2.2 Achieving maximum output power

Reducing test time is important for testing requirements in initial access. If including RAR into the test, the test time will become much longer than approaches excluding RAR. Actually, with the introduction of beam lock function in initial access, UE may be aware that it is in test mode, thus the UE does not need to follow the ordinary (non-test mode) procedure, and it can directly transmit at its maximum output power in the first preamble.

Proposal 2: With the introduction of beam lock function in initial access, UE may transmit at its maximum output power in the first preamble, i.e., Option 2 and Option 6.

# Conclusion

In this contribution we have the following observation and proposals for beam correspondence test issues for initial access:

Observation 1: According to RAN5’s reply, for FR2 enhancement requirements, either new UBL-like test function is introduced or the existing UBF test function is updated to lock the beam during initial access, and new test procedure is required as well to establish and maintain maximum power/peak EIRP during initial access.

Proposal 1: RAN4 may assume beam lock function and no testability issue on polarization for beam correspondence requirements in initial access.

Proposal 2: With the introduction of beam lock function in initial access, UE may transmit at its maximum output power in the first preamble, i.e., Option 2 and Option 6.

# References

- R4-2303563	WF on beam correspondence requirements for initial access and RRC_inactive mode, Nokia

- R4-2304021  Response LS on testability for beam correspondence in initial access, Apple