3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305847

# E-meeting, April 17th – April 26th, 2023

# Title:	Updated NTN Simulation Parameters for above 10 GHz Coexistence Studies and Initial Simulation Results

# Source:	THALES, Magister Solutions Ltd

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	5.27.2

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

At RAN4#105 the following WFs have been approved:

- R4-2220239 (source: THALES, “WF for above 10GHz band definition and system parameters”) with the latest references for Ka-Band frequency bands;

- R4-2220241 (source: Samsung, “Simulation assumptions for NTN co-existence study in bands above 10GHz”) with the latest discussions for the coexistence studies.

Agreements R4-2220239 1/ Companies are invited to study (general) ITU/national regulations and bring contributions/provide more information at this meeting (RAN#105) and next RAN4 meeting. 2/ Since it has been agreed to define several NTN Ka-bands to address the diversity of spectrum allocation (see agreement 15/11/2022 during online session), moderator proposes recommendation based on Option 2b. Option 2b (as starting point for next meeting discussion): RAN4 to consider defining/continue to discuss at next meeting (all options have same priority, companies need more time to check): - n511 with consideration of US/FCC regulations. - [n512 with consideration of CEPT regulations.] Note 1: All companies to further check impacts of latest revision of ECC Decision(05)01 on ECC Decision(13)01 cited in the current WF. - [n510 with consideration of US/FCC regulations.] Note 1: Provide the 3GPP definition that specifies the prevention of the use of a fixed terminal in the Fixed Service Spectrum (FSS). Note 2: Also provide information of a movable NTN user terminal in FSS spectrum in the US. with the following considerations: - DL: 17.7-20.2 GHz (n512, n511, n510); - UL: 27.5-30.0 GHz (n512), 28.35-30.0 GHz (n511), 27.5-28.35 GHz (n510).

Agreements R4-2220241 Table 1.1-1 Aggressor and victim combination No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; NTN UL; TN UL. 2; TN with NTN; TN UL; NTN UL. 3; TN with NTN; NTN UL; TN DL. 4; TN with NTN; TN DL; NTN UL. 5; TN with NTN; TN DL; NTN DL. 6; TN with NTN; NTN DL; TN DL. 7; TN with NTN; NTN DL; TN UL. 8; TN with NTN; TN UL; NTN DL. NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified..

Moreover, at RAN4#106 the following agreements have been made:

Agreement 1: Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Furthermore, for the assumption on TN ACLR/ACS for co-existence simulation, the values for 17 GHz in below table are considered as starting point for co-existence simulation purpose yet other options not precluded.

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Agreement 2: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

The following WF documents shall be also taken into account:

- R4-2217468, WF on [313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, 10 - 19 October, 2022;

- R4-2220241, Way forward on [105][313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, November 14 – November 18, 2022;

- R4-2302878, WF for above 10GHz NTN-TN co-existence study (Samsung), 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023.

This contribution provides material for discussion with respect to Ka-Band NTN-TN coexistence simulations and NTN UE terminal and Satellite antenna parameters. In the contribution R4-2302535 (“NTN Simulation Parameters for above 10 GHz Coexistence Studies”, THALES), several NTN simulation parameters have been proposed for above 10 GHz coexistence studies. This contribution is to update those parameters from R4-2302535 (THALES) and to present initial simulation results.

# Discussion with respect to Coexistence Use Cases

First, coexistence parameters should be chosen to allow NTN broadband connectivity in above 10 GHz.

Proposal 1: Coexistence parameters (i.e. for SAN, NTN UE) should be chosen to allow NTN broadband connectivity in above 10 GHz.

This means for instance that RAN4 should consider sufficient transmission power, antenna gain, adapted RB allocation, to allow a reasonable link budget between NTN UE and SAN in above 10 GHz.

Proposal 2: RAN4 should consider sufficient transmission power, antenna gain, relevant NTN RB allocation, to allow a reasonable link budget (e.g. above 10 dB) between NTN UE and SAN in above 10 GHz.

Moreover, initial simulation results showed that FRF>1 is necessary to achieve this broadband connectivity because satellite beams are overlapping which may cause increase interference not corresponding to broadband NTN communication.

Proposal 12: Use (for NTN) an FRF>1 (e.g. 2 or 3) for NTN calibration and coexistence analysis in above 10 GHz.

Proposal 13: Preferably use (for NTN) an FRF=2 with two polarizations (RHCP, LHCP) for NTN communication/calibration/coexistence analysis in above 10 GHz.

Moreover, after a careful check of previous WFs (R4-2217468, R4-2220241, R4-2302878), it has been also noticed that no NF has been provided for TN UE and for TN BS. In this sense, we propose to consider TR 38.803 for further reference:

| Parameters | Urban macro | Dense urban |
| --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm |
| BS Noise figure in dB | Note 1 | Note 1 |
| UE Noise figure in dB | Note 1 | Note 1 |
| Handover margin | 3dB | 3dB |
| Note 1: For deriving ACIR/ACS values which are included in respond to WP5D, following NF are used in co-existence simulation study for both UE and BS. 30 GHz: 9 and 11 dB, 45 GHz: 11 and 13 dB. |  |  |

Proposal 14: Use TR 38.803 for TN parameters related to calibration and coexistence analysis with NTN in above 10 GHz.

Proposal 15: Use TN UE NF of 9 dB for the calibration and the coexistence analysis with NTN in above 10 GHz.

Proposal 16: Use TN BS NF of 11 dB for the calibration and the coexistence analysis with NTN in above 10 GHz.

Proposal 17: Use TN urban macro scenarios (and no TN dense urban) for the calibration and the coexistence analysis with NTN in above 10 GHz.

# Initial NTN simulation results in above 10 GHz

First, coexistence parameters should be chosen to allow NTN broadband connectivity in above 10 GHz.

NTN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UEs per cell | 10 |
| UE height | 1.5 m |
| UE Tx power | 3.0 dBW |
| UE antenna gain dBi | Tx = 42.9, Rx = 39.0 |
| UE antenna diameter | 0.60 m |
| UE noise figure | 2.1dB |
| Satellite antenna type | Circular aperture RAN4 THALES |
| UE antenna type | ITU-R S.465-6 |
| Shadowing | Enabled |
| Uplink Tx power control | Disabled |
| Cell selection | RSRP |
| Frequency re-use | 1 and 3 |

NTN Scenario Specific Parameters

|  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite Tx power | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite antenna gain dBi | 58.5 | 38.5 | 38.5 |
| Noise figure | 3.5 dB | 3.5 dB | 3.5 dB |
| G/T dB/K | 30.4 | 10.4 | 10.4 |
| Beamwidth deg | 0.1884 | 1.9012 | 1.9012 |

As seen from the next simulation results, FRF>1 is preferable for broadband NTN communications above 10 GHz.

3.1 Results for FRF1

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=1 is not preferable for broadband NTN communications above 10 GHz, since the expected SINR is quite low for broadband communication purpose.

3.2 Results for FRF3

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=3 is an option for broadband NTN communications above 10 GHz.

3.3 Results for FRF2 with dual polarization

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=2 with dual polarization is an option for broadband NTN communications above 10 GHz.

Observation 1: FRF=1 is not preferable for broadband NTN communications above 10 GHz, since the SINR values are limited due to inter-neighbour beam interference.

Observation 2: FRF=3 is an option for broadband NTN communications above 10 GHz.

Observation 3: FRF=2 with dual polarization is an option for broadband NTN communications above 10 GHz.

# Initial TN simulation results in above 10 GHz

TN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UE height | 1.5 m |
| gNB antenna element gain dBi | 5.5 |
| gNB antenna elements | 16x8 |
| gNB antenna beamwidth deg | Horizontal = 90.0, Vertical = 90.0 |
| Shadowing | Enabled |
| UE Tx power | 22.4 dBm EIRP |
| Tx power control | R4-2120671, CLxile = 71.0 + 10*log10(200/BW [Mhz]) dB |
| UE antenna type | 3GPP TR38.901, 2x2 antenna array with 5.0 dBi element gain |
| gNB antenna type | 3GPP TR38.901, 16x8 antenna array with RAN4 agreements |

1/ Coupling loss

2/ Carrier signal power

3/ SINR

# Proposals resubmitted/updated from R4-2302535

Proposal 3: RAN4 to update the coexistence simulations TDoc with the simulation scope mentioned below:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |  |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |  |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |  |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |  |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |  |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |  |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |  |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |  |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

Proposal 4: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB). Either Option 1 or Option 2 (optimised with respect to secondary lobes) can be used, with a preference for Option 2:

| Options | Equations | Half-power beam-width 2θ-3dB Value |
| --- | --- | --- |
| Option 1 (see TR 38.811) | Fθ=2 J1uu | 2θ-3dB=2×Arcsin1.616×λπD |
| Option 2 (new equation) | Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=2×Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 5: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB):

| Equations (Option 2) | Half-power beam-width 2θ-3dB Value |
| --- | --- |
| Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=2×Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 6: Moreover, the following set of SAN parameters are proposed for Ka-Band in DL (Set-1 from TR 38.821):

Ka-Band DownLink (i.e. ~17 GHz for DL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 5,9 | 0,6 | 0,6 |
| Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |
| Satellite Tx max Gain (dBi) | 58,5 | 38,5 | 38,5 |

Proposal 7: Moreover, the following set of SAN parameters are proposed for Ka-Band in UL (Updated Set-1 from TR 38.821):

Ka-Band UpLink (i.e. ~27 GHz for UL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,66 | 0,36 | 0,36 |
| G/T max (dB/K) | 30,4 | 10,4 | 10,4 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Satcom Repeater Noise Figure (dB) | 3,5 | 3,5 | 3,5 |

Proposal 8: RAN4 shall consider only parabolic/dish antenna NTN UE for coexistence studies (worst case).

Proposal 9: RAN4 shall use for coexistence studies the NTN UE antenna pattern as described by the following relation:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

FudB=10 logFu2

Proposal 10: With respect to NTN UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-6:

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

Proposal 11: For secondary NTN UE lobes, RAN4 shall use ITU-R S.465-6 limit as reference for coexistence studies instead of complete antenna pattern with all the side lobes (which would require intensive simulations):

Proposal 12: RAN4 to select between [Option1] and [Option2] to define the NTN UE parameters:

[Option1] For the coexistence analysis in Ka-Band, RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

[Option2] RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

# Conclusions

Proposal 1: Coexistence parameters (i.e. for SAN, NTN UE) should be chosen to allow NTN broadband connectivity in above 10 GHz.

Proposal 2: RAN4 should consider sufficient transmission power, antenna gain, relevant NTN RB allocation, to allow a reasonable link budget (e.g. above 10 dB) between NTN UE and SAN in above 10 GHz.

Proposal 3: RAN4 to update the coexistence simulations TDoc with the simulation scope mentioned below:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |  |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |  |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |  |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |  |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |  |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |  |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |  |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |  |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

Proposal 4: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB). Either Option 1 or Option 2 (optimised with respect to secondary lobes) can be used, with a preference for Option 2:

| Options | Equations | Half-power beam-width 2θ-3dB Value |
| --- | --- | --- |
| Option 1 (see TR 38.811) | Fθ=2 J1uu | 2θ-3dB=2×Arcsin1.616×λπD |
| Option 2 (new equation) | Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=2×Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 5: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB):

| Equations (Option 2) | Half-power beam-width 2θ-3dB Value |
| --- | --- |
| Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=2×Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 6: Moreover, the following set of SAN parameters are proposed for Ka-Band in DL (Set-1 from TR 38.821):

Ka-Band DownLink (i.e. ~17 GHz for DL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 5,9 | 0,6 | 0,6 |
| Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |
| Satellite Tx max Gain (dBi) | 58,5 | 38,5 | 38,5 |

Proposal 7: Moreover, the following set of SAN parameters are proposed for Ka-Band in UL (Updated Set-1 from TR 38.821):

Ka-Band UpLink (i.e. ~27 GHz for UL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,66 | 0,36 | 0,36 |
| G/T max (dB/K) | 30,4 | 10,4 | 10,4 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Satcom Repeater Noise Figure (dB) | 3,5 | 3,5 | 3,5 |

Proposal 8: RAN4 shall consider only parabolic/dish antenna NTN UE for coexistence studies (worst case).

Proposal 9: RAN4 shall use for coexistence studies the NTN UE antenna pattern as described by the following relation:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

FudB=10 logFu2

Proposal 10: With respect to NTN UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-6:

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

Proposal 11: For secondary NTN UE lobes, RAN4 shall use ITU-R S.465-6 limit as reference for coexistence studies instead of complete antenna pattern with all the side lobes (which would require intensive simulations):

Proposal 12: RAN4 to select between [Option1] and [Option2] to define the NTN UE parameters:

[Option1] For the coexistence analysis in Ka-Band, RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

[Option2] RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

Observation 1: FRF=1 is not preferable for broadband NTN communications above 10 GHz, since the SINR values are limited due to inter-neighbour beam interference.

Observation 2: FRF=3 is an option for broadband NTN communications above 10 GHz.

Observation 3: FRF=2 with dual polarization is an option for broadband NTN communications above 10 GHz.

Proposal 12: Use (for NTN) an FRF>1 (e.g. 2 or 3) for NTN calibration and coexistence analysis in above 10 GHz.

Proposal 13: Preferably use (for NTN) an FRF=2 with two polarizations (RHCP, LHCP) for NTN communication/calibration/coexistence analysis in above 10 GHz.

Proposal 14: Use TR 38.803 for TN parameters related to calibration and coexistence analysis with NTN in above 10 GHz.

Proposal 15: Use TN UE NF of 9 dB for the calibration and the coexistence analysis with NTN in above 10 GHz.

Proposal 16: Use TN BS NF of 11 dB for the calibration and the coexistence analysis with NTN in above 10 GHz.

Proposal 17: Use TN urban macro scenarios (and no TN dense urban) for the calibration and the coexistence analysis with NTN in above 10 GHz.

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220241, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- Ka-Band Low Noise Amplifier Sub-System Module for Communication Satellite payload”, S. K. Jain and all.

- ITU-R Rec. P.618-12.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

-END-