3GPP TSG-RAN WG4 Meeting # 106bis-e 	R4-2305884

Online, April 17 – 26, 2023

Source: 	Huawei, HiSilicon, Nokia, Ericsson

Title: 	TP on SI summary

Agenda Item:	5.2.2

Document for:	Approval

1 Introduction

In the last meeting the summary of the SI was discussed and a set of agreed proposals which will form the summary were agreed in a WF [1].

These are:

  - Proposal 1: the following points for the technical feasibility are captured in the summary:

    - Multi-band implementations with percentage BW of up to 19.5% are feasible.

    - Multi-band implementations with greater percentage BW’s may be feasible in the future.

  - Proposal 2: the following points for frequency groups are captured in the summary

    - Technical feasibility to frequency groups are captured in the Technical Report

    - Frequency groups and/or band pairing restrictions are not needed in the technical specification

  - Proposal 3: Update the TS to include FR2 multi-band using the same approach as FR1 multi-band, re-using FR1 definitions.

    - The BS requirements may need to be revisited in the future release for percentage BW greater than 19.5%.

2 TP for TR 38.877

<start of TP>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[3]	3GPP R4-2212622: “Multi-band BS in mm wave”, Ericsson

[4]	Rusek, Fredrik, et al. "Scaling up MIMO: Opportunities and challenges with very large arrays." IEEE signal processing magazine 30.1 (2012): 40-60.

[5]	Fager, Christian, et al. "Linearity and efficiency in 5G transmitters: New techniques for analyzing efficiency, linearity, and linearization in a 5G active antenna transmitter context." IEEE Microwave Magazine 20.5 (2019): 35-49.

[6]	Tervo, Nuutti, et al. "Analyzing the effects of PA variations on the performance of phased array digital predistortion." 2018 IEEE 29th Annual International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC). IEEE, 2018.

[7]	Jalili, Feridoon, et al. "Linearization trade-offs in a 5G mmWave active phased array OTA setup." Ieee Access 8 (2020): 110669-110677.

[8]	Khan, Bilal, et al. "Statistical Digital Predistortion of 5G Millimeter-Wave RF Beamforming Transmitter Under Random Amplitude Variations." IEEE Transactions on Microwave Theory and Techniques 70.9 (2022): 4284-4296.

[9]	Ng, Eric, et al. "Digital predistortion of millimeter-wave RF beamforming arrays using low number of steering angle-dependent coefficient sets." IEEE Transactions on Microwave Theory and Techniques 67.11 (2019): 4479-4492.

[10]	Larsson, Erik G., and Liesbet Van der Perre. "Out-of-band radiation from antenna arrays clarified." IEEE Wireless Communications Letters 7.4 (2018): 610-613.

[11]	Haider, Muhammad Furqan, et al. "Predistortion-Based Linearization for 5G and Beyond Millimeter-Wave Transceiver Systems: A Comprehensive Survey." IEEE Communications Surveys & Tutorials (2022).

[12]	ADI, Technical article, “Why Millimeter Wave Requires a Different Approach to DPD and How to Quantify Its Value”

[13]	Abdelaziz, Mahmoud, et al. "Digital predistortion for hybrid MIMO transmitters." IEEE Journal of Selected Topics in Signal Processing 12.3 (2018): 445-454.

[14]	Design and linearization of concurrent dual-band Doherty PA with frequency-dependent power ranges, Chen W H, Bassam S A, Li X, et al. IEEE Trans Microw Theory Tech, 2011, 59:2537–2546

[15]	Wu, Qian, et al. "Digital Predistortion for Concurrent Dual-Band Millimeter Wave Analog Multibeam Transmitters." IEEE Transactions on Circuits and Systems II: Express Briefs 69.3 (2021): 1747-1751.

[16]	Li, Yue, Xiaoyu Wang, and Anding Zhu. "Sampling rate reduction for digital predistortion of broadband RF power amplifiers." IEEE Transactions on Microwave Theory and Techniques 68.3 (2019): 1054-1064.

[17]	Yu, Chao, et al. "Linear-decomposition digital predistortion of power amplifiers for 5G ultrabroadband applications." IEEE Transactions on Microwave Theory and Techniques 68.7 (2020): 2833-2844.

[18]	3GPP R4-2219154: “Further discussion on phase shifters”, Huawei, HiSilicon.

[19] 	28/38 GHz Dual-band Dual-polarized Highly Isolated Antenna for 5G Phased Array Applications, Chenhao Chu et.al, University College Dublin, IEEE IWS 2019: Sixth IEEE MTT-S International Wireless Symposium, Guangzhou, China, 19-22 May 2019 https://researchrepository.ucd.ie/bitstream/10197/11051/2/Antenna%20for%205G%20Phased%20Array_IWS2019_CChu.pdf

[20] 	A Compact Dual-Band and Dual-Polarized Millimeter-Wave Beam Scanning Antenna Array for 5G Mobile Terminals，Yuqi He et.al，Xidian University, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=9500212

[21] 	Dual-Band Dual-Polarized Microstrip Antenna Array Using Double-Layer Gridded Patches for 5G Millimeter-Wave Applications, Wangyu Sun et.al, IEEE Transactions On Antennas And Propagation, Vol. 69, No. 10, October 2021

[22]	A 32-Element 28/39 GHz Dual-Band Dual-Beam 5G Phased-Array with 40 dBm EIRP and Simultaneous 64 QAM Operation, Shufan Wang et al., University of California San Diego, USA, 2022 IEEE/MTT-S International Microwave Symposium, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=9865316

[23]	Antenna diversity in mobile communications, R. G. Vaughan et al., IEEE Transactions on Vehicular Technology, Vol. 36, No. 4, November 1987, https://ieeexplore.ieee.org/document/1623508

[24]	R4-2215413: "General consideration on mmWave multi-band BS".

[25]	An Instantaneously Broadband Ultra-Compact Highly Linear PA with Compensated Distributed-Balun Output Network Achieving >17.8dBm P1dB and >36.6% PAEP1dB over 24 to 40GHz and Continuously Supporting 64-/256-QAM 5G NR Signals over 24 to 42GHz, Fei Wang, Hua Wang, ISSCC 2020 / session 24 / RF & mm-Wave power amplifiers / 24.6, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=9063157

[26]	A 26-to-39GHz Broadband Ultra-Compact High-Linearity Switchless Hybrid N/PMOS Bi-Directional PA/LNA Front-End for Multi-Band 5G Large-Scaled MIMO System，ISSCC 2022 / Session 19 / power amplifiers and building blocks / 19.4, Jeongsoo Park1,

[27] 	A Mm-Wave Wideband MIMO RX With Instinctual Array-Based Blocker/Signal Management for Ultralow-Latency Communication, Min-Yu Huang, et al., IEEE Journal Of Solid-State Circuits, Vol. 54, No. 12, December 2019

[28] 	A Power-Efficient 24-to-71GHz CMOS Phased-Array Receiver Utilizing Harmonic-Selection Technique Supporting 36dB  Inter-Band Blocker Rejection for 5G NR, Jian Pang et al., ISSCC 2022 / Session 27 / mm-Wave & Sub-6GHz receivers and transceivers for 5g radios / 27.2, https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=9731619

[29]	A 22–44-GHz Phased-Array Receive Beamformer in 45-nm CMOS SOI for 5G Applications With 3–3.6-dB NF, Li Gao et al., IEEE Transactions on Microwave Theory and Techniques, Vol. 68, No. 11, November 2020

[30]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing, Part 2: Radiated conformance testing".

<next change>

7	Summary and further work

The feasibility study for the essential RF components and sub-systems for an FR2-1 multi-band system were carried out and found that:

  - Multi-band implementations with percentage BW of up to 19.5% are feasible.

  - Multi-band implementations with greater percentage BW’s may be feasible in the future.

Within these percentage BWs some design blocks such as the PA, and RF front end components are capable of true multi-band performance, other blocks and specifically the phase shifters used for beam forming the different bands need to be applied to separate band specific solutions. There are currently no proposed solutions to enable a wide band beam former to steer beams for different bands in different directions.

Antennas and antenna arrays are limited to certain maximum percentage BW’s however MB solutions using multiple antenna arrays have been investigated and solutions within the agreed percentage BW’s above have been identified.

Other considerations, i.e., on a need of DPD and complications of digital and analogue designs for a wideband / multiband BS are also analysed.

Considering the wide range of FR2-1 operating bands and potential combinations FR2-1 operating bands have been considered within frequency groups with limited percentage BW’s as shown in table 5.1-1 and feasibility analysed within these groups. However, whilst practical limits exist which may limit multi-band solutions to be within these groups no restrictions need to be used in the technical specification and as such frequency groups do not need to be defined with the updates of the technical specifications. On the other hand, as highlighted in clause 6.2 study of solutions across frequency groups may be further studied and the use of additional multi-band requirements may be considered in the future for percentage BW’s greater than 19.5%.

For updating the technical specifications, FR2-1 multi-band requirements are to be added to the existing NR 3GPP TS 38.104 [2]and 3GPP TS 38.141-2 [30] technical specifications, using the same approach as FR1 multi-band as described in clause 6. The existing MB RIB definition as well as the explanations on BS capable of supporting operation in multiple operating bands with different implementations at the RIB in 38.104 [2] for FR1 can also apply to FR2-1.

<end of TP>

References

[1] R4-2302902, WF for SI summary, Huawei, HiSilicon