3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305890

Online, 17th ‒ 26th April 2023

Title: 	WF on FR1 4Tx demodulation requirement

Agenda Item:	5.5.4.2

Source: 	Ericsson

Document for:	Approval

# Topic 1: PUSCH requirement

Issue 1-1-1: Bandwidth selection between 20MHz and 50MHz

Agreement:

  - For 15kHz:

    - Introduce requirements for 5MHz and 50MHz, FFS on 20MHz

  - For 30kHz:

    - Introduce requirements for 10MHz and 100MHz, FFS on 40MHz

Issue 1-1-2: MCS

Agreement:

  - For TDLB100-400: MCS2

  - For TDLA30-10: MCS 17

  - For TDLC300-100: MCS12

Issue 1-1-3: Antenna configuration

Agreement:

  - Define requirements for both 4Rx and 8Rx.

Issue 1-1-4: Whether to define same requirement for PUSCH mapping type A and B

Agreement:

  - Define separate requirements for PUSCH mapping type A and mapping type B.

Issue 1-1-5: Precoding for vehicular cases

Agreement:

  - Only consider TPMI index 0 for all 4Tx PUSCH demodulation requirements.

Issue 1-1-6: Applicability rule

Way forward:

- Option 1:

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the 4 connectors and the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

- Option 2:

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

Issue 1-2-1: Testing limit

  - No OTA 4Tx demodulation requirements will be defined.

Issue 1-1-7: Specification structure

- For the cases of bandwidth 50MHz, add a new table to capture the requirement

- For the cases of agreed bandwidths other than 50MHz, adding 4Tx requirements to the same table as 1Tx/2Tx requirements.

# Simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth 15kHz SCS: 5MHz and 50MHz, FFS on 20MHz 30kHz SCS: 10MHz and 100MHz, FFS on 40MHz |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Channel model and MCS |  | TDLB100-400: MCS2 TDLC300-100: MCS12 TDLA30-10: MCS17 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

# Work split

| CR split |  |  |
| --- | --- | --- |
| Specification | Content part | Responsible |
| 38.104 | PUSCH requirement | Nokia |
| 38.141-1 | PUSCH requirement | ZTE |
| 38.104, 38.141-2 | FRC Tables | Ericsson |
| 38.141-1 | FRC Tables | Huawei |
| 38.141-1 | Applicability rule, manufactory declaration, measurement uncertainty and test tolerance | Samsung |