3GPP TSG-RAN WG4 Meeting#106bis                               R4-2305894

E-meeting, April 17 – April 26,2023

Title: 	WF on ATG BS RF requirements

Agenda Item:	     5.14.6

Source: 	ZTE

Document for:	Approval

Agreement

#### Issue 1-1: Descriptions for the ATG BS class

Tentative agreements:

Proposal 2: ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m

#### Issue 1-2:  Additional supported bands

Tentative agreements:

- Add TDD bands n39 as the operating band of ATG.

#### Issue 1-3-1:  ATG BS ACLR

Tentative agreements:

- [To reuse legacy FR1 ACLR (i.e. 45dBc) requirement]

#### Issue 1-3-2:  ATG BS ACS

Tentative agreements:

- [To reuse legacy FR1 ACS (i.e. 46dBc) requirement]

#### Issue 1-4:  Whether to define Tx IM for ATG BS?

Tentative agreements:

- The existing requirement in TS 38.104 for Tx IM requirements could be reused.