3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2305900

Electronic Meeting, 17 April –26 April, 2023

Title: 	WF on FRC parameters for link level simulation for added FRC

Agenda Item:	5.15.6

Source: 	ZTE Corporation

Document for:	Approval

Issue 1-3: FRC parameters to be used for link level simulation for added FRC for 3MHz channel bandwidth receiver dynamic range and ICS

- Agreement:

Table 1: Added FRC parameters for FR1 in-channel selectivity

| Reference channel | G-FR1-A1-20 |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Allocated resource blocks | 9 |
| CP-OFDM Symbols per slot (Note 1) | 12 |
| Modulation | QPSK |
| Code rate (Note 2) | 1/3 |
| Payload size (bits) | [808] |
| Transport block CRC (bits) | 16 |
| Code block CRC size (bits) | - |
| Number of code blocks - C | 1 |
| Code block size including CRC (bits) (Note 3) | [824] |
| Total number of bits per slot | [2592] |
| Total symbols per slot | [1296] |
| NOTE 1: UL-DMRS-config-type = 1 with UL-DMRS-max-len = 1, UL-DMRS-add-pos = 1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity. NOTE 3: Code block size including CRC (bits) equals to K’ in sub-clause 5.2.2 of TS 38.212 |  |

Table 2: Added FRC parameters for FR1 dynamic range

| Reference channel | G-FR1-A2-15 |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Allocated resource blocks | 15 |
| CP-OFDM Symbols per slot (Note 1) | 12 |
| Modulation | 16QAM |
| Code rate (Note 2) | 2/3 |
| Payload size (bits) | [5504] |
| Transport block CRC (bits) | [24] |
| Code block CRC size (bits) | - |
| Number of code blocks - C | 1 |
| Code block size including CRC (bits) (Note 3) | [5528] |
| Total number of bits per slot | [8640] |
| Total symbols per slot | [2160] |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211. NOTE 2: MCS index 16 and target coding rate = 658/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K’ in sub-clause 5.2.2 of TS 38.212 . |  |