3GPP TSG-RAN WG4 Meeting #106bis-e                                                    R4-2305907  
Online, April 17 – April 26, 2023

Agenda item:	 5.16.3

Source:	 vivo

Title:	Updated working procedure for Rel-18 TRP TRS requirements

Document for:	 Approval

- Introduction

RAN4#106 meeting agrees an initial framework to guide Rel-18 TRP TRS requirement work [1]. This contribution provides further views on refinement of working procedure for Rel-18 TRP TRS requirements.

- Discussion

RAN4#106 meeting agrees the initial working procedure for 1Tx requirement work, but the 2Tx scope and other aspects (e.g. UE information disclosure, bands down-selection) are not decided.

2Tx lab alignment is within Rel-18 working scope, which can be considered at a later stage and started after the finalization of 2Tx test method and 1Tx lab alignment activity.

For Rel-18 TRP TRS requirements, we have the following suggestions:

- Both PC3 and PC2 are considered, if the band has already support PC2 BEFORE Rel-18.

- Down-selection of bands should be done, to make sure all the test cases can be finalized before WI deadline. Contribution [2] provides a good starting point for RAN4 discussion.

- UE information disclosure should be decided before submitting the measurement results. Contribution [3] provides a good starting point for RAN4 discussion.

How to define PC3 requirements needs more discussions:

- O1: PC3 defined based on PC2 results with X dB offset

- O2: based on PC3 measurement results

Regarding the required percentage of UE data pool (e.g. number of vendors, percentage of UE production year, etc), before making specific requirements, feedback from interested companies are quite important, i.e. vendors, operators, and most importantly, volunteered test labs.

The decision for requirement scope should be made based on a reasonable consideration of acceptable test burden within Rel-18 timeline.

- Updated Working procedure for Rel-18 TRP TRS requirements (for approval)

  - Working procedure for Rel-18 AC Lab Alignment Campaign

- The purpose of Lab Alignment Campaign is to ensure there is no unexpected lab deviation and establish full trust and confidence on the results.

- Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should be accredited under ISO 17025 (ISO 17025 accredited labs) and have any of 3GPP TS 37.544, CCSA YD/T 1484.6, and CTIA OTA Test Plan listed on its accreditation scope.

  - Participating lab should have anechoic chamber(s) ready to support testing based on 3GPP TS 38.161.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TS 38.161;

- Test cases for Lab Alignment Campaign:

  - Test bands: n28, n78; one low and one high band is sufficient

  - Number of test cases per band: BHHL and BHHR at low/mid/high channel;

  - Use scenario: Head and Hand phantom (Talk mode), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 1Tx as phase 1, 2Tx as phase 2 can be started after UL-MIMO TRP test method is concluded.

- Lab Alignment Device (LAD):

  - Reuse LADs (LAD1 and LAD2) from Rel-17 lab alignment activity

- Test results submitting:

  - Using the same worksheet template in [TBD] to submit the measurement results

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of lab alignment data pool from ≥3 labs submitted before [TBD] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as X*MU (X is TBD) as baseline. MU value is the expanded MU for BHH (to be defined in RAN5).

  - The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty.

  - How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LAD delivery scheme

    - Decide LAD delivery scheme after all the test lab and LAD information being confirmed (after the confirmation of volunteered labs).

  - LAD measurement time in each test lab: finalize LAD measurement within [TBD] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems

  - Working procedure for TRP TRS Performance Test Campaign

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: Table 1 as starting point, further down-selection needed;

  - Use scenarios: Both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip Hand as first priority, PDA hand is not precluded if sufficient devices are available

  - Operation mode: SA with 1Tx  as phase 1, study how to specify EN-DC requirements based on SA measurement results. SA with 2Tx as phase 2.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm) as 1st priority; Size 2 as 2nd priority

  - DUT capability: support for Bands [TBD] those listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: [2020-2023]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3,depends on corresponding band. for bands support both PC2 and PC3, both requirements are needed. For bands do not support HPUE, only PC3 requirements

- Test results submitting:

  - UE information disclosure. At least all the supported bands information and production year should be shared

    - Other UE information disclosure (and thresholds) depends on further discussions, which can be added based on agreements.

  - Using the same worksheet template in [TBD] to submit the measurement results for Rel-18 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity)

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-18.

  - Method of limits derivation: per-band Data driven approach

  - The value at [TBD] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, study how to specify PC3 requirement based on finalized PC2 requirements.

- Test lab procedures

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - Newly defined Coarser measurement grid can be used for TRP TRS measurement

Table 1: targeted TRP/TRS OTA requirements for Rel-18

| Aspect / Feature |  | TRS |  | 1Tx TRP PC3 |  | 1Tx TRP PC2 |  | 2Tx TRP PC2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Browse | Talk | Browse | Talk | Browse | Talk | Browse | Talk |
| AC Test method and prelim MU |  | R17 | R18 MU | R17 | R18 MU | R17 | R18 MU | R18 | R18 |
| n1 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 | R18 | R18 |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p |  |  |
| n3 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p |  |  |  |  |
| n5 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p |  |  |  |  |
| n7 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p |  |  |  |  |
| n8 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p |  |  |  |  |
| n28 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p |  |  |  |  |
| n41 | 72 < w ≤ 92 mm | R17 | R18 | R18 | R18 | R17 | R18 | R18 Phase2 | R18 Phase2 |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p |
| n77 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 | R18 | R18 | R18 Phase2 | R18 Phase2 |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p |
| n78 | 72 < w ≤ 92 mm | R17 | R18 | R18 | R18 | R17 | R18 | R18 Phase2 | R18 Phase2 |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p |
| n79 | 72 < w ≤ 92 mm | R18 | R18 | R18 | R18 | R18 | R18 |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p | R18 2p |  |  |
| Note: bands n1/n5/n8/n28/n41/n78/n77 for Size 1 (72 < w ≤ 92 mm) are listed as 1st priority for Rel-18 requirements. Further down-selection is needed. |  |  |  |  |  |  |  |  |  |

Proposal: Approve the updated working procedure for Rel-18 TRP TRS requirements in Section 3 of this contribution.

- Conclusion

In this contribution, we propose the working procedure for Rel-18 TRP TRS requirement work:

Proposal: Approve the updated working procedure for Rel-18 TRP TRS requirements in Section 3 of this contribution.

# References

- R4-2302920, Working procedure for Rel-18 TRP TRS requirements, vivo, RAN4 #106

- R4-2300351, On TRP TRS requirement development prioritization, Apple, RAN4 #106

- R4-2300669, Device’s information disclosure for FR1 TRP-TRS and MIMO OTA (Rel-18), Telecom Italia S.p.A., RAN4 #106