3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2305914

Electronic Meeting, 17 April –26 April, 2023

Source:	China Telecom

Title:	WF on advanced receiver for MU-MIMO scenario

Agenda Item:	5.19.3

Document for:	Approval

Introduction

This document provides the way forward on advanced receiver for MU-MIMO scenario of Rel-18 NR demodulation requirement evolution WI.

The previous approved WFs are listed as below:

  - R4-2302929 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106

<Topic #1: Assumptions for the advanced receiver for MU-MIMO>

Sub-topic 1-1 Reference receiver assumptions

Issue 1-1-1: Reference receiver assumption for R-ML

Candidate options:

  - Option 1: UE perform RML algorithm for serving and all co-scheduled UEs in the cell

  - Option 2: R-ML receiver in terms of total layer (serving + interfering) and modulation order

  - Option 3: UE performs joint detection on layers of one additional co-scheduled UE in addition to its own layers on the same frequency and time resource as its own allocation

  - Option 4: Limit the number of co-scheduled UE is no more than 1 and the number of interference layers are no more than 2

Way forward

  - This issue is highly related to how UE could obtain each requirement information and how NWA is designed.

  - Discuss how to obtain each of the needed parameters.

Issue 1-1-2: Reference receiver

Candidate options:

  - Option 1: Down-select R-ML as a candidate reference receiver to define phase II requirements

  - Option 2: To be decided later

Way forward

  - To be discussed in the next meeting.

Sub-topic 1-2 Discussion on the required information (High priority for Q2)

Sub-topic 1-2-1: Timeline

Issue 1-2-1-1: Timeline for the required information studying

- Agreement:

  - RAN4 should prioritize NWA related topics (especially for DCI related signalling) for this WI in Q2.

Sub-topic 1-2-2: Information required for both E-MMSE-IRC and R-ML

Issue 1-2-2-1: The presence of co-scheduled UE

- Proposals on how this information could be obtained by the UE:

  - Option 1: Blind detection should be considered

  - Option 2: By assistant information signalling

  - Option 3: Through signalling of other parameters or by blind detection to the DMRS port information to be discussed

- Proposals on how the NWA is signalled (if introduced):

  - Option 1: DCI-based signalling jointly with modulation order and DMRS port

  - Option 2: DCI-based signalling jointly with only modulation order

  - Option 3: 1 bit RRC signalling to indicate whether the DMRS port is used for the co-scheduled UE

Way forward

  - Postpone this issue after the agreements of other information are reached.

Issue 1-2-2-2: The DMRS sequence information for the co-scheduled UE

Way forward

  - For the DMRS configuration parameter including: DMRS type, DMRS additional position, maximum length:

    - Restriction already exists in RAN1 specification (TS38.214), thus signaling is not needed.

  - For the scrambling ID and nSCID information:

    - Assume same as that of the target UE agreed as RAN4 default assumption

    - FFS on the signaling to inform UE if RAN4 default assumption not valid for any of the co-scheduled UE:

- Option 1: 1-bit RRC signaling

- Option 2: Implied by DCI signaling on modulation order (if introduced)

- Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits

Issue 1-2-2-3: The DMRS port information for the co-scheduled UE

GTW agreements on Apr 17th:

  - Dedicated DCI signaling is not preferred for the DMRS port information

  - FFS whether assistant RRC signalling can be introduced to reduce the BD complexity and/or maintain reasonable CE performance for target UE

  - Companies are encouraged to further evaluate BD performance including the detailed assumption:

    - Number of co-scheduled UE for BD

    - Time/frequency location of co-scheduled UE

Way forward

  - Study the BD performance together with FDRA information as recommended in Issue 1-2-2-8.

  - FFS whether to introduce additional assistant RRC signalling to restrict the BD complexity.

Issue 1-2-2-4: PRB bundling size for the co-scheduled UE

Way forward

  - UE needs to know the PRB bundling size of co-scheduled UEs if different from target UE

  - How could be obtained

    - Assume the PRB bundling size of co-scheduled UEs is same with that of target UE

    - FFS on the signaling to inform UE if RAN4 default assumption not valid:

- Option 1: 1-bit RRC signaling

- Option 2: No signaling is required.

- Option 3: Implied by DCI signaling on modulation order (if introduced)

Issue 1-2-2-5: DMRS power boosting for the co-scheduled UE

Way forward

  - DMRS power boosting should be the same for both target and the co-scheduled UE.

  - FFS on the signaling to inform UE if RAN4 default assumption not valid:

    - Option 1: 1-bit RRC signaling

    - Option 2: Implied by DCI signaling on MO (if introduced)

- Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits

    - Option 3: No signaling is required.

Issue 1-2-2-6: The transmission power ratio of co-scheduled users PDSCH to own PDSCH

Agreements:

  - No need for the target UE to know the transmission power ratio of co-scheduled PDSCH to own PDSCH

Issue 1-2-2-7: Time domain resource allocation information of the co-scheduled UE

Way forward

  - UE assumes the same PDSCH symbols are allocated to the target and the co-scheduled UEs

  - FFS on the signaling to inform UE if RAN4 default assumption not valid, by:

    - Option 1: 1-bit RRC signalling

    - Option 2: Implied by DCI signaling on MO (if introduced)

- Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits

    - Option 3: No signaling is required

Issue 1-2-2-8: Frequency domain resource allocation information of the co-scheduled UE

Way forward

  - UE should know the FDRA information of the co-scheduled UE

  - Study the R-ML performance with BD to the DMRS port information as well as the FDRA information of the Co-UE (Low priority for the May meeting). Evaluation assumptions are as below:

    - 1 Co-UE

    - Detection granularity – up to UE implementation

    - Select the following two cases based on the agreed simulation assumption:

- Case 2 (rank 1+1 2T2R QPSK interference TDLC300-100 random precoding)

- Case 9 (rank 2+2 4T4R 64QAM interference TDLA30-10 orthogonal precoding)

    - Two cases for the FDRA of the co-UE:

- Full CHBW allocation (52PRBs)

- Partial CHBW allocation (0~24 PRBs)

    - Note: Assume that the R-ML has known all the other required information and all the agreed default assumptions are valid

  - Companies are encouraged to bring simulation results.

Sub-topic 1-2-3: Additional information required for R-ML

Issue 1-2-3-1: The modulation order information of the co-scheduled UE

Way forward

- The following additional assumptions to the R-ML receiver can be agreed:

  - Within each PRB/PRG, UE applies R-ML to all interference layers with prior information that all interference layers have same modulation order

  - FFS whether to consider the case with interference layers have different modulation orders within one or more PRBs.

- Evaluation assumptions of the MO BD study:

  - 1 Co-UE

  - Detection granularity – up to UE implementation

  - Following cases:

    - Rank 1+1, 2T2R, MCS 13 for the target UE, QPSK interference, TDLC300-100, random precoding

    - Rank 2+2, 4T4R, MCS 17 for the target UE, 16QAM interference, TDLA30-10, orthogonal precoding

    - Rank 1+1, 2T2R, MCS 13 for the target UE, 16QAM interference TDLC300-100 random precoding (Optional)

  - Full CHBW allocation (52PRBs) FDRA of the co-UE:

  - Note: Assume that the R-ML also needs to perform DMRS port and FDRA information BD and all the agreed default assumptions are valid.

  - Companies are encouraged to bring simulation results for the next meeting.

- With this MO BD study, the following is not precluded:

  - The possibility of full signalling of modulation order and/or other information.

  - The possibility of non-dynamic NWA signalling (i.e., non-DCI) solutions.

- For this MO BD study, companies are encouraged to take all proposals from Issue 1-2-3-2 into consideration.

Issue 1-2-3-3: RS location information of the co-scheduled UE

Way forward

  - UE can assume the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE

  - FFS whether to consider RRC signalling to inform UE whether the default assumption is needed

Issue 1-2-3-2: Content of the network signalling on modulation order (for information)

- Proposal 1:

| Signalling Overhead |  | DCI signalling Info. | Antenna Port = 1000 + DMRS Port (P) |
| --- | --- | --- | --- |
| 2 bits | maxMIMO-Layers = 2 | 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM | P = {0, 1} |
| 6 bits | maxMIMO-Layers = 4 | For 3 interfering ports in cyclic order from desired layer, each 2 bits represent as 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM | P = {0, 1, 2, 3} |

- Proposal 2: Use examples listed in below as start point

  - Example 1:

| Bitfield | coUE modulation order | UE behavior |
| --- | --- | --- |
| 000 | No UE which has same DMRS sequence with target paired | SU-MIMO (MMSE-IRC) |
| 001 | PRB aligned (Single coUE modulation per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection with only 4 hypotheses (UE dependent) |
| 010 | Bandwidth aligned, QPSK | R-ML without modulation order detection |
| 011 | Bandwidth aligned, 16-QAM | R-ML without modulation order detection |
| 100 | Bandwidth aligned, 64-QAM | R-ML without modulation order detection |
| 101 | Bandwidth aligned, 256-QAM | R-ML without modulation order detection |
| 110 | Bandwidth aligned, 1024-QAM | R-ML without modulation order detection |
| 111 | PRB not aligned (Multiple coUE modulations per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection (UE dependent) |

- Proposal 3: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs, or signal the maximum MCS index table (64QAM, 256QAM or 1024QAM table) used among all the (potential) co-scheduled UEs.

- Proposal 4: Include blind detection opportunities for modulation order detection using semi-static DUT resource allocation, where DUT is not scheduled with PDSCH, but co-scheduled UEs are scheduled with PDSCH.

- Proposal 5: Introduce DCI signaling to assist blind detection, such as 1 bit modulation order set information for each co-scheduled UE to reduce search space, for example, when gNB decide 16QAM for a co-scheduled UE, then gNB transmit 1 bit to indicate the modulation order of this co-scheduled UE is in the set {16QAM, 64QAM}

Way forward

  - To be discussed in the next meeting if introduced

Sub-topic 1-2-4: Signalling for network assistant information if introduced

Issue 1-2-4-1: Signalling for the network assistant information (If introduced)

Candidate options:

  - Option 1: Only consider RRC or MAC-CE based network assistance signalling

  - Option 2: DCI

  - Option 3: FFS once it is agreed which information is to be signalled

  - Option 4: Some of the information could be carried by DCI and others carried by higher layer

Way forward

  - Discuss separately for each parameter

Issue 1-2-4-2: Granularity of the network assistant signalling (If introduced)

Candidate options:

  - Option 1: The granularity of the network assistant signalling should be the wideband

  - Option 2: FFS until it is agreed which information is to be signalled

Way forward

  - Further discuss.

<Topic #2: Simulation assumptions and results>

Issue 2-1-1: The number of co-scheduled UEs

Candidate options:

  - Option 1: 1 co-scheduled UE as higher priority

  - Option 2: Cover 2 co-scheduled UEs at least in phase I study

Way forward

  - Keep the previous meeting agreement, and continue discussion in the next meeting.

Issue 2-1-2: Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE

Way forward

  - For 2Rx: rank 1+1

  - For 4Rx: Only consider rank 2+2

Issue 2-1-3: Rank allocation for the target and co-scheduled UEs, with 2 co-scheduled UEs

Candidate options:

  - Option 1: Rank 1+1+1

Way forward

  - Maintain the last meeting agreement: Option 1 for interested companies to bring evaluation results in initial study stage

Issue 2-1-4: MCS for the target UE

Way forward

  - Cover MCS 13 for rank 1 and rank 2 for initial simulation

  - Cover MCS 17 for rank 2 for initial simulation

  - Further discuss whether to cover MCS 4 for rank 1 in the next meeting

  - The assumption can be updated later based on available results.

Issue 2-1-5: Modulation order for the co-scheduled UE

Way forward

  - Maintain last meeting status from the modulation order perspective:

For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose) for initial simulation

    - For rank 1+1: QPSK (high priority)

    - For rank 2+2: 64QAM (high priority)

    - For rank 2+2: QPSK (high priority)

    - For rank 1+1 (64QAM) +1 (QPSK) (lower priority)

    - For rank 1+1 (64QAM) +1 (16QAM) (lowest priority)

    - Other options on the modulation order for co-scheduled UE are not precluded.

    - These assumptions can be updated in the next meeting based on available simulation results.

Issue 2-1-6: Channel model

Way forward

  - For initial study

    - TDLC300-100 when the rank of the target UE is 1

    - TDLA30-10 when the rank of the target UE is 2

Issue 2-1-7: Antenna correlation

Way forward

  - Rank 1+1: ULA medium

  - Rank 2+2: ULA Low

  - Rank 1+1+1: XPL medium (Interested companies can bring simulation results)

Issue 2-1-8: Precoder selection target and co-scheduled UEs

Way forward

  - For phase I study

    - For rank 1+1: Random PMI selection

    - For rank 2+2: Orthogonal PMI selection

Issue 2-1-9: PDSCH resource allocation for the target and co-scheduled UE

Candidate options:

  - Candidate scenarios:

    - Scenario 1: Maximum transmission bandwidth configuration for all UEs.

    - Scenario 2: Maximum transmission bandwidth configuration for the target UE and partial transmission bandwidth configuration for the co-scheduled UEs.

    - Full OFDM symbol allocation for both scenarios.

  - Option 1: Cover both scenario 1 and scenario 2 in the phase I study

    - Option 1A: Consider the following scenario for the study of R-ML and E-IRC with blind detection to the required information

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~19 allocation with 16QAM transmission.

- Co-scheduled UE2 with PRB40~51 allocation with QPSK transmission.

- Full OFDM symbol allocation.

  - Option 1B:

| Allocation | Layer 1 | Layer 2 | Layer 3 | Layer 4 |
| --- | --- | --- | --- | --- |
| PRB 0 ~ PRB19 | Target UE |  | UE3 64QAM | UE1 16QAM |
| PRB20 ~ PRB39 |  |  |  | None |
| PRB40 ~ PRB51 |  |  | UE2 QPSK |  |

  - Option 1C

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~26 allocation.

  - Option 2: Only consider scenario 1

  - Option 3: FFS based on the conclusion for Issue 1-2-2-8

Way forward

  - For initial simulation in phase I, cover scenario 1

  - FFS whether to cover scenario 2, based on the discussion outcome in the NWA part.

<TP work split>

| TP to TR38.878 on | Responsibility |
| --- | --- |
| Scenario and interference modelling | MediaTek |
| Receiver structure | ZTE |
| Analysis on the required information | Intel |
| Parameters for link level evaluation | Huawei |
| Link level simulation results | Ericsson |
| Summary of link level evaluation | Nokia |
| Conclusions | China Telecom |

Note: According to the work plan, these TPs will be approved in RAN4#108 in Aug 2023.