3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305917

Electronic meeting 17th April ‒ 26th April, 2023

Title: 	WF on implementation feasibility of SBFD: Co-channel Co-site/inter-site interference

Agenda Item:	5.20.4

Source: 	Ericsson

Document for:	Approval

# General agreement on inter-sector interference analysis framework

Agreements:

  - For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors.

    - There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study.

  - For the proposed inter-sector table, the wrongly used term “RSIC shall be corrected.

  - The same framework can be applied for co-existence study

The following agreement applies also to co-site co-channel inter-sector analysis:

  - Add subsection (e.g., one subsection per company) to allow companies’ technical input for at least WA BS, and other BS classes if the need justified. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc.

  - Summary sub-section shall be considered to harmonize the common understating from RAN4 if possible and summarize the input from companies.

# Template for capturing co-site inter-sector co-channel interference impact

<Way forward>:

- Proposal for table name:

  - Co-site inter-sector co-channel interference table

- Proposal for table format:

| FR1 (or FR2-1) |  |  |  | Company-A |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm |
| Number of co-site co-channel sectors considered |  |  |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc |  | xxx dBc |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc |  | xxx dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  |  |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI-frequency, RX= ⑥ dBc |  | xxx dBc |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |
|  |  |  | Rx IM3 contribution (dBm) | Note: Receiver linearity may depend on both inter-sector interference and self-interference… each company can explain how they calculate IM3 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | Note: Each company to explain if/how they have separated CSSI and SI when considering IM3 |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI-beam = ⑨ dBc |  |  | xxx dB |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |
|  | Digital processing interference supression capability |  |  |  |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | xxx dBc |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW |
| Calculated Desensitization (dB) |  |  |  | xxx dBm |
| SBFD configuration |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |

- Proposal for capturing technical concerns / motivations:

  - These should be captured in the rows of the table and/or within the company specific TPs to the TR.