3GPP TSG-RAN WG4 Meeting#106bis                               R4-2305931

E-meeting, April 17 – April 26,2023

Title: 	WF on system parameters and RF requirements

Agenda Item:	      5.29.7

Source: 	ZTE

Document for:	Approval

Agreement for GTW discussion

Issue 1-1: System parameter for NCR-MT

- Agreement:

  - For the system parameter of NCR-MT, refer to the UE specification (TS38.101-1/2)

  - Take IAB-MT approach as starting point i.e. to set the channel bandwidth per operating band to be optional for NCR-MT

Issue 1-2: NCR-MT feature list

- Agreement:

  - Further discuss NCR-MT feature list, FFS whether feature list required or not for NCR-fwd from RAN4 perspective

  - Proposal 1: to focus on the mandatory UE capability firstly and to check whether this is still applicable for NCR-MT and identify some features which are not applicable for NCR-MT; [for the rest of UE features, it should be optional for NCR-MT].

Issue 1-3-1: NCR Class

- Agreement:

  - For NCR-fwd, following NCR classes introduced

    - DL/UE side: wide-area, medium range and local area

    - UL/BS side: Wide-area and local area

    - Above NCR classes applicable for all NCR types

  - NCR classes for NCR-MT

    - [Wide-area and local area]

  - FFS whether NCR class can be declared separately for NCR-fwd, and NCR-MT

Issue 1-3-3: RF diagram for NCR

- Agreement: Option 2 [CATT] as starting points, further work on the refinement.

Issue 2-1: Power control for NCR-Fwd

. Proposals

  - Proposal 1: there is no need for power control requirements for NCR fwd-link. [CMCC,R4-2304201]

- Agreement: Proposal 1 agreed.

Issue 2-2: RF requirements for NCR-Fwd type 1-C

Proposals

  - Proposal 1: The conducted requirement for repeater 1-C specified in sub-clause 6 of TS 38.106 can be reused for NCR type 1-C Fwd.  [CATT, R4-2304445]

- Agreement: Proposal 1 agreed

  - Further check whether FDM operation allowed or not; if allowed, further check any impact on the emission requirements for FDM operation

Issue 2-3: RF requirements for NCR-Fwd type 2-O

Proposals

  - Proposal 1: The radiated requirement for repeater 2-O specified in sub-clause 7 of TS 38.106 can be reused for NCR type 2-O Fwd.  [CATT, R4-2304445]

- Agreement: Proposal 1 agreed

  - Further check whether FDM operation allowed or not; if allowed, further check any impact on the emission requirements for FDM operation

Issue 3-1-1: Power control for NCR-MT

  - Option 1: to support the NCR-MT power control;

- Agreement: Option 1 agreed

Issue 3-1-2: bandwidth related

  - Proposal 1: Proposal 1: Define NCR-MT RF requirements based on carrier/NCR-MT channel bandwidth instead of passband/nominal channel bandwidth. [CATT, R4-2304446]

- Agreement: Proposal 1 agreed

Agreement for offline GTW discussion

For NCR-Fwd type 1-H:

|  | Agreement |
| --- | --- |
| RF requirements |  |
| Repeater output power | - Agreement: - for DL part: - To follow the option 2; - For UL part: - FFS |
| Frequency stability | Agreement: Reuse frequency stability for repeater type 1-C specified in sub-clause 6.3.2 of TS 38.106. |
| Out of band gain | Agreement: Reuse out of band gain for repeater type 1-C specified in sub-clause 6.4.2 of TS 38.106. |
| Adjacent Channel Leakage Power Ratio | Agreement: Reuse the Rel-17 repeater type 1-C requirements for NCR-Fwd type 1-H and 1-O; [relative value] for DL absolute ACLR limit, to follwo the option 2 scaling factor agreed for repeater output power; |
| Operating band unwanted emissions | Agreement: reuse operating band unwanted emissions for repeater type 1-C specified in sub-clauses 6.5.3.2.1 - 6.5.3.2.6 of TS 38.106 as basic limit; for DL emission power leve, follow the option 2 scaling factor agreed for repeater output power; |
| Transmitter spurious emissions | - Agreement: - Further study on scaling factor for transmitter spurious emission requirement only. |
| Receiver spurious emissions | - Agreement: - Further study on scaling factor for receiver spurious emission requirement only. |
| Error Vector Magnitude | Agreement: To reuse the Rel-17 repeater type 1-C requirement f or each TAB connector of NCR-Fwd type 1-H; |
| Input intermodulation | Agreement: To reuse the Rel-17 repeater type 1-C requirement for each TAB connector of NCR-Fwd type 1-H; |
| Output intermodulation | Agreement: To reuse the Rel-17 repeater type 1-C requirement for each TAB connector of NCR-Fwd type 1-H; |
| Adjacent Channel Rejection Ratio (ACRR) | Agreement: Reuse the Rel-17 repeater type 1-C requirements |
| Transmit ON/OFF power and transition period | - Agreement: - For NCR-MT and NCR-Fwd switch ON-OFF together, proposed to have further study for it; - For NCR-MT and NCR-Fwd switch ON-OFF individually, propose to Reuse the Rel-17 repeater type 1-C requirements; |
| OTA output power(EIRP) | Agreement: Define the requirement based on EIRP manufacturer declaration, with the accuracy of the declared output power value (Prated,p,EIRP). Reuse the ±2.2dB and ±2.7dB from the BS type 1-H in sub-clause 9.2.2 of TS 38.104 for normal test conditions and extreme test condition, respectively. |

For NCR-Fwd type 1-O:

|  | Agreement |
| --- | --- |
| RF requirements |  |
| Repeater output power | - Agreement: - for DL part: - To follow the option 2 with 9dB scaling factor; - For UL part: - FFS |
| Frequency stability | Agreement: Reuse OTA frequency stability for repeater type 1-C specified in sub-clause 7.3.2 of TS 38.106. |
| Out of band gain | Agreement: 1) Reuse the Rel-17 repeater type 1-C requirements for NCR-Fwd type 1-H and 1-O; 2) define the gain for OOB gain as the ratio of TRP output power to directional input power in the same manner as for 2-O. |
| Adjacent Channel Leakage Power Ratio | Agreement: 1) for relative ACLR value, to reuse the Rel-17 repeater type 1-C requirements for NCR-Fwd type 1-H and 1-O 2) for absolute ACLR limits in the downlink part, to follow the 9dB scaling factor, however for uplink, it's FFS |
| Operating band unwanted emissions | Agreement: for NCR-Fwd downlink OBUE requirement, to follow the 9dB scaling factor, for NCR-Fwd uplink OBUE requirement, FFS on the scaling factor; |
| Transmitter spurious emissions | - Agreement: - Further study on scaling factor for transmitter spurious emission requirement only. |
| Receiver spurious emissions | - Agreement: - Further study on scaling factor for receiver spurious emission requirement only. - For receiver spurious emission requirement, we need to wait for the conclusion of type declaration of NCR-MT and NCR-Fwd. |
| Error Vector Magnitude | Agreement: - Further study |
| Input intermodulation | Agreement: - Further study |
| Output intermodulation | Agreement: - RAN4 should discuss further how to define and apply co-location requirements (for emissions and output intermodulation) considering the complexities of needing both a reference co-location antenna and of supplying a stimulus signal to the repeater. |
| Adjacent Channel Rejection Ratio (ACRR) | FFS |
| Transmit ON/OFF power and transition period | FFS |