3GPP TSG-RAN WG4 Meeting # 106-bis-e 												R4-2305932

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			5.29.3

Source:	ZTE Corporation

Title:	WF on Network Controlled repeater EMC testing

Document for:	Approval

Background

This WF captures the agreements and open issues reached in the [106-bis-e][312] NR_netcon_repeater_RF e-mail discussion summary for NR repeater EMC at the RAN4 #106-bis-e meeting.

Way forward

The agreements for the following open issues are as follows:

Issue 1-1-1:  Whether implement EMC test on both NCR-MT and NCR-Fwd?

  - Tentative agreements: Yes. NCR-MT and NCR-Fwd should be considered separately in NCR EMC requirements due to the different role they played in NCR communication link.[ZTE, R4-2304682]

- Discussion: ZTE, Ericsson, Nokia agree with the tentative agreements. Huawei mentions that EMC test shall apply per DUT, not per its logical entity. Wait for more mature discussion on RF side first.

- Agreement: EMC test should be applied to both NCR-MT and NCR-Fwd, but the exact test configuration should wait for RF discussion.

Issue 1-1-2:  Whether set Tx/Rx or UL/DL requirements for NCR-MT and NCR-Fwd?

  - Option 1: Define Tx/Rx requirements for NCR-MT and UL/DL requirements for NCR-Fwd. [ZTE, R4-2304682]

  - Option 2: Others, please provide reasons.

- Discussion: All companies agree to take option 1 as a starting point.

- Agreement: Option 1 can be take as a starting point.

Issue 1-2:  Shall we extend the scope of TS38.114 to cover type 1-H and 1-O NCR?

- Proposals

  - Tentative agreements: Yes. Extend the scope to include type 1-H and 1-O repeaters and also fill the missing EMC test content for them including definitions, test conditions, exclusion band, test configurations, test level and limits for emission and immunity EMC test, etc. [Nokia, R4-2304529]

- Discussion: All companies agree with the tentative agreements.

- Agreement: Extend the scope to include type 1-H and 1-O repeaters and also fill the missing EMC test content for them including definitions, test conditions, exclusion band, test configurations, test level and limits for emission and immunity EMC test, etc.

Issue 1-3-1:  Core part of NCR EMC

- Proposals

  - Proposal:Reuse the test requirements in NR repeater and make some adaptable changes including but not limited to type 1-H and 1-O NCR requirements. [ZTE, R4-2304682]

- Agreement: The proposal can be take as a starting point. More detailed test requirements and adaptable changes related with type 1-H and 1-O NCR should be further studied.

Issue 1-3-2:  Perf part of NCR EMC

- Proposals

  - Proposal: Perf part of NCR EMC would be different with NR repeater. i.e. performance criteria, exclusion band and test configuration etc. Therefore more study is need. [ZTE, R4-2304682; Nokia, R4-2304529]

- Agreement: Perf part of NCR EMC is required FFS. Core part should be discussed first.

Issue 1-4:  Test feasibility for “mixed” type of NCR

- Proposals

  - Proposal: From EMC testing perspective, it is feasible to have different types for NW side and UE side at fwd. link for FR1 NCR.[Ericsson, R4-2305188]

- Agreement: FFS.

- Reference

[1] draft R4-231xxx_Email discussion summary for [106bis-e][312] NR_netcon_repeater_RF