3GPP TSG-RAN WG4 Meeting # 106bis-e   	R4-2305934

Electronic Meeting, 17 April –26 April, 202

Title: 	WF on NCR-MT demodulation requirements

Agenda Item:	5.29.6

Source: 	ZTE Corporation

Document for:	Approval

- Demodulation requirements

Sub-topic 1-1 General

Issue 1-1-1: Duplex mode

Agreements:

- Consider TDD for both NR FR1 and FR2, FDD for NR FR1.

Issue 1-1-2: Whether define new requirements for FR1

Agreements:

- Define new requirements for FDD/15 KHz and TDD/30 KHz for 1 layer only by reusing/adapting requirements for IAB-MT.

  - 10M/15kHz; 40M/30kHz

  - FFS whether need to consider additional CHBW 5MHz/15kHz

Issue 1-1-3: Whether define new requirements for FR2

Agreement:

- Reuse IAB-MT requirements for NCR-MT demodulation and only consider Rank 1 with 100MHz/120kHz

- FFS new requirements on PDCCH/PDSCH for signaling of Access link beam change indication

Issue 1-1-4: Reference channels

Agreements:

- Define single-slot FRCs similar as IAB-MT.

- Issue 1-1-5: Reference signals

Agreements:

- Do not define SSB, TRS, CSI-RS configurations as a part of demodulation performance test parameters or FRC

- Issue 1-1-6: TDD and FDD pattern

Agreements:

- Default pattern can be reused from IAB-MT.

- Issue 1-1-7: Others

Agreements:

- RAN4 shall assume the same operating frequency band for both the C-link NCR-MT and the backhaul link of NCR-Fwd, when defining demodulation requirements for the C-link.

- Issue 1-1-8: Test for NCR-MT

Agreements:

- As with IAB-MT, there shall be no restriction on the testing style for NCR-MT, but the BS-style testing shall be assumed by default.

- Issue 1-1-9: Test scope

Agreements:

- Define NCR-MT performance requirements based on Rel-15 UE performance requirements.

Sub-topic 1-2 PDSCH requirements

Issue 1-2-1: How to define PDSCH requirements

Agreements:

- Both 70% and 30% throughput cases agreed as baseline assumption for the test cases reused from IAB-MT requirements

  - Other values higher than 70% not precluded

- FFS for the test metric of new test case for specifically for MAC-CE Access link beam indication transmitted over PDSCH if introduced

Issue 1-2-2: MCS

Agreements:

- QPSK and 16QAM

Issue 1-2-3: Rank

Agreements:

- Rank 1

Issue 1-2-4: Antenna configuration

Agreements:

- 2x2 and 2x4 for FR1; 2x2 for FR2

- Issue 1-2-5: Propagation conditions

Agreements:

- TDLA30-10

- Issue 1-2-6: Mapping type

Agreements:

- Mapping type A

- Issue 1-2-7: PRB bundling size

Agreements:

- 2

- Issue 1-2-8: Receiver type

Agreements:

- MMSE-IRC

- Issue 1-2-9: Overlapped CSI-RS

Agreements:

- Do not consider CSI-RS overlapped with PDSCH.

- Issue 1-2-10: Co-existence with LTE CRS

Agreements:

- Do not consider PDCSH Co-existence with LTE CRS.

Sub-topic 1-3 PDCCH requirements

- Issue 1-3-1: How to define PDCCH requirements

Agreements:

- Reuse UE requirements for FDD and TDD

- FFS for adapt PDCCH requirement for testing of DCI type 5_0 used for access link beam change indication.

- Issue 1-3-2: Coreset for FDD

Agreements:

- Option 1:

  - 1TX: coreset duration 2

  - 2TX: coreset duration 1

- Option 2:

  - 1TX: coreset duration 1

  - 2TX: coreset duration 1

- Issue 1-3-3: Coreset for TDD

Agreements:

- coreset duration 1

- Issue 1-3-4: Aggregation Level

Agreements:

  - 1TX: aggregation level 2 and 4

  - 2TX: aggregation level 8

- Issue 1-3-5: Propagation condition

Agreements:

- TDLA30-10

- Issue 1-3-6: Antenna configuration

Agreements:

- 1/2 TX; 2/4 RX

Sub-topic 1-4 PBCH requirements

- Issue 1-4-1: Whether define PBCH requirements

Agreements:

- Do not define PBCH requirements for NCR-MT.

Sub-topic 1-5 SDR requirements

- Issue 1-5-1: Whether to define SDR requirements

Agreements:

- Do not define SDR requirements for NCR-MT.

- CSI requirements

Sub-topic 2-1 CQI requirements

- Issue 2-1-1: Whether define CQI requirements

Agreements:

- Yes

  - Reuse the existing requirements for CQI.

- Issue 2-1-2: Propagation condition

Agreements:

- static channel(AWGN)

- Issue 2-1-3: CSI-RS configuration and CSI reporting

Agreements:

- Periodic

- Issue 2-1-4: Antenna configuration

Agreements:

- 2x2, 2x4 (FR1) and 2x2 (FR2).

Issue -2-1-5: CQI table

Agreements:

- Table 2.

Issue 2-1-6: Others

Agreements:

- Wideband

Sub-topic 2-2 PMI requirements

Issue 2-2-1:Whether define PMI requirements

Agreements:

- FFS whether need to define PMI reporting requirements

Sub-topic 2-3 RI requirements

Issue 2-3-1: Whether define RI requirements

Agreements:

- Do not define RI requirements for NCR-MT