3GPP TSG-RAN WG4 Meeting#106bis                             R4-2305938

E-meeting, April 17 – April 26,2023

Source:	ZTE Corporation

Title:	TP for TS 36.181: Clause 4.7~4.8

Agenda Item:	6.8.3.5

Document for:	Approval

- Introduction

- This contribution provides a text proposal to TS36.181 [1] on sub-clause 4.7~4.8.

- Reference

- R4-2302451, TS 36.181 SAN conformance testing skeleton and work-split arrangement, Huawei, approved.

- Text Proposal

----- Start of TP -----

4.7	Test configurations

4.7.1	General

The test configurations shall be constructed using the methods defined below, subject to the parameters declared by the manufacturer for the supported RF configurations as listed in subclause 4.6.8. The test configurations to use for conformance testing are defined for each supported RF configuration in subclause 4.11.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 6.1.1.

4.7.2	Test signal used to build Test Configurations

4.7.3	ETC1: Contiguous spectrum operation.

The purpose of test configuration ETC1 is to test all SAN requirements excluding CA occupied bandwidth.

For ETC1 used in receiver tests only the two outermost carriers within each supported operating band need to be generated by the test equipment.

4.7.3.1	ETC1 generation

ETC1 shall be constructed on a per band basis using the following method:

-	Declared maximum SAN RF Bandwidth supported for contiguous spectrum operation shall be used;

-	Place 1.4 MHz carrier adjacent to the lower SAN RF Bandwidth edge and place a 1.4 MHz carrier adjacent to the upper Base Station RF Bandwidth edge.

-	For transmitter tests, select as many 1.4 MHz carriers that the SAN supports within a band and fit in the rest of the declared maximum SAN RF Bandwidth. Place the carriers adjacent to each other starting from the upper SAN RF Bandwidth edge. The nominal carrier spacing defined in subclause 5.7 shall apply;

All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

4.7.3.2	ETC1 power allocation

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power Prated,t according to the manufacturer’s declaration in subclause 4.6.8.

4.7.4	ETC6: Contiguous spectrum operation.

The purpose of the ETC6 is to test NB-IoT standalone multi-carrier aspects.

4.7.4.1	ETC6 generation

ETC6 is constructed using the following method:

-	The SAN RF Bandwidth shall be the declared maximum SAN RF Bandwidth.

-	Place a NB-IoT carrier at the upper edge and a NB-IoT carrier at the lower edge of SAN RF Bandwidth.

-	For transmitter tests, add NB-IoT carriers at the edges using [600 kHz] spacing until no more NB-IoT carriers are supported or no more NB-IoT carriers fit.

4.7.4.2	ETC6 power allocation

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power Prated,t according to the manufacturer’s declaration in subclause 4.6.8.

4.8	Applicability of requirements

4.8.1	General

4.8.2	Requirement set applicability

4.8.3	Applicability of test configurations for single-band operation

----- End of TP -----