3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305947

# Electronic Meeting, 17 April – 26 April, 2023

Source: 	NEC

Title: 	TP to TS 36.181: Reference measurement channels

Agenda Item:	6.8.3.5

Document for:	Approval

Introduction

This contribution provides the text proposal to TS 36.181 for reference measurement channels.

Discussion

In the last meeting, RAN4 agreed to align the FRC numbering across NR BS demodulation specifications [1].

Although the agreements in [1] were on NR BS FRC numbering, we think the concepts should be applied to E-UTRA BS/SAN FRC numbering.

The points to be adopted are below:

- Align the FRC numbering and the corresponding annex numbers in the specifications among E-UTRA BS and SAN demodulation specifications

- For FRCs not used in the specification, add notes in the annex to inform the FRCs not used in the specifications

Based on the policies above, text proposal for the annex on reference measurement was prepared. FRCs are copied from E-UTRA BS core specifications [2], As FRC numbers are not aligned among BS core/conformance specifications [2], [3] and SAN core specification [4], we plan to provide CRs to correct them based on the concept in [1].

This is a revision of R4-2304716.

Annexes for the FRCs for demodulation performance requirements have been removed.

Conclusions

Based on the above discussion, the following proposal was formulated:

Proposal 1: Approve the attached TP to TS 36.181.

References

[1] R4-2302993, WF for FRC numbering alignment across specifications 38.104/38.141-1/38.141-2, Keysight, NTT DoCoMo, NEC

[2] TS 36.104

[3] TS 36.141

[4] TS 36.108

Text proposal

--------------Start of text proposal-------------

Annex A (normative):
Reference measurement channel

The parameters for the reference measurement channels are specified in clause A.1 for E-UTRA reference sensitivity and in-channel selectivity and in clause A.2 for dynamic range.

A schematic overview of the encoding process for the E-UTRA reference measurement channels is provided in Figure A-1.

E-UTRA receiver requirements in the present document are defined with a throughput stated relative to the Maximum throughput of the FRC. The Maximum throughput for an FRC equals the Payload size * the Number of uplink subframes per second. For FDD, 1000 uplink sub-frames per second are used.

The parameters for the reference measurement channels are specified in clause A.14 for NB-IoT reference sensitivity and in clause A.15 for dynamic range.

A schematic overview of the encoding process for the NB-IoT reference measurement channels is provided in Figure A-2.

NB-IoT receiver requirements in the present document are defined with a throughput stated relative to the Maximum throughput of the FRC. The Maximum throughput for an FRC equals the Payload size / (Number of Resource Unit * time to send one Resource Unit).

Figure A-1. Schematic overview of the encoding process

Figure A-2. Schematic overview of the encoding process for NB-IoT

A.1	Fixed Reference Channels for reference sensitivity and in-channel selectivity (QPSK, R=1/3)

Note:	Fixed reference channels A1-2, A1-3, and A1-5 to A1-9 are not used in this specification.

The parameters for the reference measurement channels are specified in Table A.1-1 for reference sensitivity and in-channel selectivity.

Table A.1-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A1-1 | A1-2 | A1-3 | A1-4 | A1-5 | A1-6 | A1-7 | A1-8 | A1-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 6 | 15 | 25 | 3 | 9 | 12 | 24 | 101 | 102 |
| DFT-OFDM Symbols per subframe | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 600 | 1544 | 2216 | 256 | 936 | 1224 | 2088 | 1032 | 1032 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Coded block size including 12bits trellis termination (bits) | 1884 | 4716 | 6732 | 852 | 2892 | 3756 | 6348 | 3180 | 3180 |
| Total number of bits per sub-frame | 1728 | 4320 | 7200 | 864 | 2592 | 3456 | 6912 | 2880 | 2880 |
| Total symbols per sub-frame | 864 | 2160 | 3600 | 432 | 1296 | 1728 | 3456 | 1440 | 1440 |
| NOTE 1: For reference channel A1-8, the allocated RB's are uniformly spaced over the channel bandwidth at RB index N, N+5, N+10, ..., N+45 where N = {0, 1, 2, 3, 4}. NOTE 2: For reference channel A1-9, the allocated RB's are uniformly spaced over the channel bandwidth at RB index N, N+10, N+20, ..., N+90 where N = {0, 1, 2, … 9}. |  |  |  |  |  |  |  |  |  |

A.2	Fixed Reference Channels for dynamic range (16QAM, R=2/3)

Note:	Fixed reference channels A2-2 to A2-5 are not used in this specification.

The parameters for the reference measurement channels are specified in Table A.2-1 for dynamic range.

Table A.2-1 FRC parameters for dynamic range

| Reference channel | A2-1 | A2-2 | A2-3 | A2-4 | A2-5 |
| --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 6 | 15 | 25 | 101 | 102 |
| DFT-OFDM Symbols per subframe | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate | 2/3 | 2/3 | 2/3 | 2/3 | 2/3 |
| Payload size (bits) | 2344 | 5992 | 9912 | 4008 | 4008 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 24 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 2 | 1 | 1 |
| Coded block size including 12bits trellis termination (bits) | 7116 | 18060 | 14988 | 12108 | 12108 |
| Total number of bits per sub-frame | 3456 | 8640 | 14400 | 5760 | 5760 |
| Total symbols per sub-frame | 864 | 2160 | 3600 | 1440 | 1440 |
| NOTE 1: For reference channel A2-4, the allocated RB's are uniformly spaced over the channel bandwidth at RB index N, N+5, N+10, ..., N+45 where N = {0, 1, 2, 3, 4}. NOTE 2: For reference channel A2-5, the allocated RB's are uniformly spaced over the channel bandwidth at RB index N, N+10, N+20, ..., N+90 where N = {0, 1, 2, … 9}. |  |  |  |  |  |

--------------Next text proposal-------------

A.14	Fixed Reference Channels for NB-IOT reference sensitivity (π/2 BPSK, R=1/3)

The parameters for the reference measurement channels are specified in Table A.14-1 for reference sensitivity.

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |

A.15	Fixed Reference Channels for NB-IoT dynamic range (π/4 QPSK, R=2/3)

The parameters for the reference measurement channels are specified in Table A.15-1 for NB-IoT dynamic range.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |

--------------End of text proposal-------------