3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305965

Electronic Meeting, 17 April ‒ 26 April 2023

Title: 	WF on IoT NTN SAN demodulation requirements

Agenda Item:	6.8.8

Source: 	Ericsson

Document for:	Approval

# Topic #1: General

  - Sub-topic 1-1: pre-compensation gap

Agreement: Whether to consider the multi-segment scenario in the SAN demodulation requirements

  - No (UL channel transmission duration should not exceed TxDuration parameter)

  - Sub-topic 1-2: Frequency offset drift

Agreement: Doppler offset drift modeling

  - Apply Doppler offset drift model on PUSCH, PUCCH, NPUSCH format 1 and NPUSCH format 2.

  - Not consider Doppler offset drift model for PRACH and NPRACH performance requirements.

  - Use fixed (the largest) frequency offset value for the SAN demodulation requirement. See the table below for the additional largest Doppler shift.

| Channel | Repetition | Tx duration in one segment | Largest frequency offset value (Hz) |
| --- | --- | --- | --- |
| eMTC PUSCH CE mode A | 8 | 8ms | 4 |
| eMTC PUSCH CE mode B | 256 | 256ms | 128 |
| eMTC PUCCH format 1a | 8 | 8ms | 4 |
| NPUSCH format 1, 3.75KHz, 1 tone | 4 | 256ms | 160 |
| NPUSCH format 1, 15KHz, 12 tones | 16 | 16ms | 8 |
| NPUSCH format 2, 3.75KHz, 1 tone | 16 | 128ms | 64 |
| NPUSCH format 2, 15KHz, 1 tone | 16 | 32ms | 16 |

  - Sub-topic 1-3: Timing offset drift

Agreement: Whether to consider timing offset drift model

  - The total timing offset doesn’t exceed the CP.

  - Not consider timing offset drift model in TS36.108 but consider it in TS36.181

Agreement: Timing offset drift model

  - Apply the additional transmitting timing drift to PUSCH, PUCCH, NPUSCH format 1 and NPUSCH format 2.

  - Not consider additional transmitting timing drift for PRACH and NPRACH performance requirements

  - Apply the gradual transmission timing offset model with step size Δt. See the table below for the step size per channel.

| Channel | Repetition | Tx duration in one segment | Step size Δt |
| --- | --- | --- | --- |
| eMTC PUSCH CE mode A | 8 | 8ms | [0.01] µs per subframe |
| eMTC PUSCH CE mode B | 256 | 256ms | [0.01] µs per subframe |
| eMTC PUCCH format 1a | 8 | 8ms | [0.01] µs per subframe |
| NPUSCH format 1, 3.75KHz, 1 tone | 4 | 256ms | [0.32] µs per RU |
| NPUSCH format 1, 15KHz, 12 tones | 16 | 16ms | [0.01] µs per RU |
| NPUSCH format 2, 3.75KHz, 1 tone | 16 | 128ms | [0.08] µs per RU |
| NPUSCH format 2, 15KHz, 1 tone | 16 | 32ms | [0.02] µs per RU |

  - Sub-topic 1-4: Requirement scope

Agreement: Requirement scope with timing offset drift model

  - Specify the SAN type 1-O radiated demodulation performance requirements for both NB-IoT and eMTC, by reusing the conducted performance requirements.

# Topic #2: SAN demodulation requirement for eMTC

  - Sub-topic 3-1: PUSCH requirements

Way forward: Updated simulation assumptions for PUSCH CE mode A (for alignment purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode A |
| --- | --- |
| Max number of HARQ transmissions | 4 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLC5-5 Option 2: NTN-TDLA100-5 |
| FRC | TS36.104 A3-2 |
| System BW | 1.4MHz only |
| Fraction of maximum throughput | 70% |

Way forward: Simulation assumption for PUSCH CE mode B (for alignment purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode B |
| --- | --- |
| Max number of HARQ transmissions | 2 |
| RV sequences | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 256 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-1 |
| System BW | 1.4MHz only |
| Fraction of maximum throughput | 70% |

  - Sub-topic 3-2: PUCCH requirements

Way forward: Simulation assumption for PUCCH format 1a (for alignment purpose)

  - The ACK missed detection probability shall not exceed 1%

  - Assume TxDuration is same as the number of repetitions

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-5 |
| System BW | 1.4MHz |

  - Sub-topic 3-3: PRACH requirements

Way forward: Simulation assumptions for PRACH requirements (for alignment purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Frequency hopping: OFF

  - Assume TxDuration is same as the number of repetitions

  - Timing estimation error: 2.08us

  - RAN4 will set the same propagation channel for PUCCH format 1a and PRACH fading scenario.

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-5 | 270Hz | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

# Topic #3: SAN demodulation requirement for NB-IoT

  - Sub-topic 4-1: NPUSCH format 1 requirements

Way forward: Simulation assumptions for NPUSCH format 1 requirements (for alignment purpose)

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 | TS36.104 A16-1 | 4 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1 | TS36.104 A16-5 | 16 | 70% |

  - Sub-topic 4-2: NPUSCH format 2 requirements

Way forward: Simulation assumptions for NPUSCH format 2 requirements (for alignment purpose)

  - The DTX to ACK probability, i.e., the probability that ACK is detected when nothing is sent per NPUSCH format 2 transmission, shall not exceed 1% per NPUSCH format 2 transmission.

  - The ACK missed detection probability shall not exceed 1%.

  - Assume TxDuration is same as the number of repetitions.

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 |

  - Sub-topic 4-3: NPRACH requirements

Way forward: Simulation assumptions for NPRACH demodulation requirements (for alignment purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Assume TxDuration is same as the number of repetitions

  - Timing estimation error: 3.646us

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | NTN-TDLA100-1 | 200Hz | YES | YES |
| 1 | 1 and 2 | 16 | NTN-TDLA100-1 | 200Hz | YES | YES |

# Topic #4: CR work split

  - TS 36.108: Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception

| Clause number | Clause name | Company name |
| --- | --- | --- |
| 8.1 | Conducted performance requirements - General | Ericsson |
| 8.2 | Performance requirements for PUSCH |  |
| 8.3 | Performance requirements for PUCCH |  |
| 8.4 | Performance requirements for PRACH |  |
| 8.5 | Performance requirements for NPUSCH format 1 | Samsung |
| 8.6 | Performance requirements for NPUSCH format 2 |  |
| 8.7 | Performance requirements for NPRACH |  |
| 11.1 | Radiated performance requirements - General | Ericsson |
| 11.2 | Performance requirements for PUSCH |  |
| 11.3 | Performance requirements for PUCCH |  |
| 11.4 | Performance requirements for PRACH |  |
| 11.5 | Performance requirements for NPUSCH format 1 | Samsung |
| 11.6 | Performance requirements for NPUSCH format 2 |  |
| 11.7 | Performance requirements for NPRACH |  |
| A.5 | Fixed Reference Channels for PUSCH transmission | Ericsson |
| A.6 | PRACH Test preambles | Ericsson |
| A.7 | Fixed Reference Channels for NPUSCH format 1 transmission | Samsung |

  - TS 36.181: Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node conformance testing

| Clause number | Clause name | Company name |
| --- | --- | --- |
| TBD | Conducted performance requirements - General | Nokia |
| TBD | Performance requirements for PUSCH |  |
| TBD | Performance requirements for PUCCH |  |
| TBD | Performance requirements for PRACH |  |
| TBD | Performance requirements for NPUSCH format 1 | Huawei |
| TBD | Performance requirements for NPUSCH format 2 |  |
| TBD | Performance requirements for NPRACH |  |
| TBD | Radiated performance requirements - General | Nokia |
| TBD | Performance requirements for PUSCH |  |
| TBD | Performance requirements for PUCCH |  |
| TBD | Performance requirements for PRACH |  |
| TBD | Performance requirements for NPUSCH format 1 | Huawei |
| TBD | Performance requirements for NPUSCH format 2 |  |
| TBD | Performance requirements for NPRACH |  |
| TBD | Fixed Reference Channels for PUSCH transmission | Nokia |
| TBD | PRACH Test preambles | Nokia |
| TBD | Fixed Reference Channels for NPUSCH format 1 transmission | Huawei |
| TBD | Measurement of Performance Requirements | Huawei |