3GPP TSG-RAN WG4 Meeting # 106-bis-e 	R4-2305972

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			5.2.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [106bis-e][302] FS_NR_BS_RF_evo

Document for:	Information

Introduction

This email thread discuss the SI on NR BS RF requirement evolution. The contributions are in agenda 5.2, which includes:

- Topic #1: General

- Topic #2: Feasibility of FR2-1 multi-band BS

- Topic #3: SI summary

- Topic #4: Others

Topic #1: General

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2305298 | Huawei, HiSilicon | TR 38.877 v0.3.0 |
| R4-2305299 | Huawei, HiSilicon | TP for Clause 3: abbreviations The contribution provides a TP for Clause 3 to add some missing abbreviations. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1

Issue 1-1: TR update 38.877 v0.3.0

- Proposals

  - Option 1: Approve the TR 38.877 v0.3.0 (R4-2305298)

  - Option 2: Revise R4-2305298

- Recommended WF

  - Option 1

Issue 1-2:  TP for Clause 3

- Proposals

  - Option 1: Approve the TP

  - Option 2: Revise R4-2305299

- Recommended WF

  - Option 1

Companies views’ collection for 1st round

Open issues

Collection of comments:

Issue 1-1: TR update 38.877 v0.3.0

| Company | Comments |
| --- | --- |
| Murata | Option1 |
| Huawei | Option 1 |
| Nokia | Option 1 |
| NEC | Option 2. Issue on table of contents where only up to second level section titles are shown and pages are not correct (styles of section headers issue?). |
| Huawei | To NEC Ok, we can update the contents |
| Ericsson | OK to approve (option 1) |

Issue 1-2:  TP for Clause 3

| Company | Comments |
| --- | --- |
| Murata | Option1 |
| Huawei | Option 1 |
| Nokia | Option 2: - Could also change to capital A for amplifier in 'PA - Power amplifier'. - The TP also missed to capture some abbreviations in the approved TP (e.g., TRX, AAS, VGA, etc). There are also new abbreviations in the newly submitted TPs. Those can also be captured if the TPs are approved. |
| NEC | Option 1 |
| Huawei | To Nokia It is ok to change for the first and second points. For the new abbreviation in the newly submitted TP, it is better to make update in those corresponding TPs, to reduce the cross check. |
| Ericsson | OK to approve (option 1) |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: TR update 38.877 v0.3.0 | The TR update is to be revised. |
| Issue 1-2: TP for Clause 3 | The TP is to be revised. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
| Revision of R4-2305298 | Company A: Company B: |
| Revision of R4-2305299 | Company A: Company B: |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2305882 (Revision of R4-2305298) | Agreeable |
| R4-2305883 (Revision of R4-2305299) | Agreeable |

Topic #2: Feasibility of FR2-1 multi-band BS

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304118 | Nokia, Nokia Shanghai Bell | TP to TR 38.877: Phase shifter and antenna |
| R4-2304632 | Murata Manufacturing Co Ltd. | TP to TR 38.877: Antenna array |
| R4-2304633 | Murata Manufacturing Co Ltd. | FR2-1 multi-band antenna analysis |
| R4-2304711 | NEC | TP to TR 38.877: Fractional bandwidth and percentage bandwidth |
| R4-2304712 | NEC | TP to TR 38.877: Corrections in DPD sections |
| R4-2305681 | Intel Corporation | Additional feasibility aspects for FR2-1 multi-band BS |
| R4-2305682 | Intel Corporation | TP for TR 38.877: Additional feasibility aspects |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2 –Feasibility of FR2-1 multi-band BS

Issue 2-1: Antenna array

- Proposal 1: TP to Clause 5.3.5 analog filtering technology in R4-2304632

- Proposal 2: TP to Clause 5.3.2 Single array bandwidth in R4-2304118

- Observations on antenna analysis from R4-2304633:

  - Observation 1:	From antenna performance perspective, the difference between single input and dual input of stacked patch multi-band antenna is small, and both types could achieve 4 to 5dBi element gain.

- Recommended WF

  - Discuss on the observation and indicate whether it is acceptable to capture it in the TR as RAN4 common understanding

  - Collect the comments on the TPs and merge the TPs

Issue 2-2: Phase shifters

- Proposal 1: TP to Clause 5.2.2 Phase Shifter in R4-2304118

- Recommended WF

  - Collect the comments on the TP and revise it if needed

Issue 2-3: Fractional bandwidth and percentage bandwidth

- Proposal 1: TP on Fractional bandwidth and percentage bandwidth in R4-2304711

- Recommended WF

  - Collect the comments on the TP and revise it if needed

Issue 2-4: Corrections in DPD sections

- Proposal 1: TP on Corrections in DPD sections in R4-2304712

- Recommended WF

  - Collect the comments on the TP and revise it if needed

Issue 2-5: Feasibility considerations for configuration options

- Proposal 1: TP on additional feasibility aspects in R4-2305682

- Recommended WF

  - Collect the comments on the TP and revise it if needed

Companies views’ collection for 1st round

Open issues

Collection of comments:

Issue 2-1: Antenna array

| Company | Comments |
| --- | --- |
| Huawei | We are ok with Proposal 1 and also the observation 1 in R4-2304633. For Proposal 2, for added Figure 5.3.2-3 and the corresponding text, we think the point is already in the preceding text “it should also be noted that the array is electrically shorter at lower frequencies than higher frequencies and this also affects the antenna directivity and gain.”– this perhaps does not really need expanding so extensively. We suggest to merge the TP and include the observation if agreed. |
| Nokia | For R4-2304632, the term 'LTCC' should be included in the Abbreviations clause. Reply to Huawei comment: Figure 5.3.2-1 and 5.3.2-2 in the approved TR were used to support the related points, and they were useful to achieve better understanding. Similarly, Figure 5.3.2-3 is aimed at supporting the point related to directivity variation, which is another important aspect needs to be considered in the antenna array designs. |
| NEC | We are ok with the TP in proposal 1, but it is on diplexer technology. We think it is more appropriate to introduce the text under 5.2 wideband RF architectures, rather than under 5.3 wideband antenna architectures. |
| Ericsson | For 4632, generally OK, the title may be better as “Diplexer technology” to be more clear. |

Issue 2-2: Phase shifters

| Company | Comments |
| --- | --- |
| Huawei | For the added example, we think the point is already made in the text. So the content here is correct but I think not needed. |
| Nokia | Reply to Huawei comment: Illustration is always good to visually elaborate the points, as some example simulation results in the TR did to improve the readability. On the other hand, the added example has additionally shown behaviour not clearly mentioned in the text, i.e., - It is feasible to steer the beams in the same directions for two separate bands if and only if the direction is at boresight. - A larger frequency separation of the bands results in a bigger error of the steering angle. |
| Ericsson | We are OK with the TP. |

Issue 2-3: Fractional bandwidth and percentage bandwidth

| Company | Comments |
| --- | --- |
| Murata | Agree with proposal |
| Huawei | In general we are ok with TP, but we would like to provide some additional text for the update. |
| Nokia | Track changes should be made on TR not another TP, also can simply refer to TS 38.104 for FBW definition instead of copying it here. |
| NEC | Reply to Nokia comment: Agree that track changes should be made on TR in general cases. However, at the time of submission, official version of TR 38.877 v0.3.0 was not available. Therefore, TP was on v0.2.0, although it was actually on the draft TR v0.3.0 as requested by the rapporteur. We believe we followed the right process. |
| Huawei | Do not have strong view, but we think the change mark based draft TR V 0.3.0 would help the reader to know the proposed change. |
| Ericsson | OK with the proposal |

Issue 2-4: Corrections in DPD sections

| Company | Comments |
| --- | --- |
| Huawei | Ok with the corrections |
| Nokia | Track changes should be made on TR not another TP. |
| Ericsson | OK with the proposal |

Issue 2-5: feasibility considerations for configuration options

| Company | Comments |
| --- | --- |
| Murata | Let me confirm a few points to add this configuration option. In case of Configuration#1 and #3, what is the “common active RF components” based on the definition of multi-band RIB? In case of Configuration#1, diplexer would generally be integrated in IC, is it feasible? In case of Configration#3, in my understanding, diplexer which implemented between PA and antenna is reversed implementation for lower and higher frequency range. |
| Huawei | In general I think the useful information from this proposed section should be merged into the appropriate RF sections – this is not really “other”. Configuration options may not be the right title. These are more like different design options. By the definition of Multi-band RIB, option 1 and option 3 are not MB RIB. Hence the corresponding text may not be relevant. Half the options are not MB and it’s a bit subjective. |
| Nokia | - Configurations #1 and #3 are not defined as MB RIB. Since Section 5.1 focuses more on multiband RIB or wideband BS, we propose to remove these two configurations. Otherwise, if these single-band RIB configurations would be captured in the TR, another configuration #5 where multiple single-band signals are routed to single-band RIBs/antenna arrays should also be included. - Reference [4] should be a new reference in the Reference clause. |
| Intel | Thank you for the comments, we address each below. Murata: In case of Configuration#1 and #3, what is the “common active RF components” based on the definition of multi-band RIB? In Configurations #1 and #3, the low and high designators can mean single-band or multi-band (as included in the notes under the figure), therefore the PA may be the common active component. Perhaps the following can be clarified: if the PAs in #1 and #3 cover multiple bands (that is, PAhigh covers more than one band and PAlow covers more than one band), would this not be considered MB? Using different subscripts (other than high and low) may be helpful; we are open to changing these. In case of Configuration#1, diplexer would generally be integrated in IC, is it feasible? How the diplexer is integrated is an implementation choice (it may be monolithically integrated with the PA, or there could be a hybrid integration implementation). Either way, it should be feasible. In case of Configration#3, in my understanding, diplexer which implemented between PA and antenna is reversed implementation for lower and higher frequency range. Yes, that is correct. This is an editorial error we need to fix (swap the diplexer images at the PA output). Huawei: In general I think the useful information from this proposed section should be merged into the appropriate RF sections – this is not really “other”. Configuration options may not be the right title. These are more like different design options. We are flexible on where this information is captured in the TR; including it in another section is ok. For the title, we are ok with changing configurations to design options. By the definition of Multi-band RIB, option 1 and option 3 are not MB RIB. Hence the corresponding text may not be relevant. Half the options are not MB and it’s a bit subjective. As previously mentioned, we would like to verify whether these could never be considered MB. Nokia: - Configurations #1 and #3 are not defined as MB RIB. Since Section 5.1 focuses more on multiband RIB or wideband BS, we propose to remove these two configurations. Otherwise, if these single-band RIB configurations would be captured in the TR, another configuration #5 where multiple single-band signals are routed to single-band RIBs/antenna arrays should also be included. We are ok with removing configurations #1 and #3 if needed, but we want to verify if even when each PA in those configurations cover multiple bands, these cannot be considered MB. As it may be beneficial to further illustrate the MB definition, we are also ok with keeping configurations #1 and #3 and adding configuration #5 (along with relevant text). - Reference [4] should be a new reference in the Reference clause. No problem, this reference is for the WF on SI summary (R4-2302902) |
| Nokia | Reply to Intel: We are ok with removing configurations #1 and #3 if needed, but we want to verify if even when each PA in those configurations cover multiple bands, these cannot be considered MB. As recorded in clause 6.1 of TR 38.877: As the same with FR1, the existing explanations on BS capable of supporting operation in multiple operating bands in 38.104 [xx] for FR1 are sufficient. The same descriptions can be applied to FR2-1, i.e. BS type 1-O and BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary: - All RIBs are single-band RIBs. - All RIBs are multi-band RIBs. - A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.. Therefore, it has been agreed that there are different implementations of BS capable of supporting operation in multiple operating bands. If the intention is to illustrate the possible implementations, then configuration #5 should also be included. |
| Ericsson | For the naming, how about “potential architecture options” ? Regarding the non-multi-band options, we do not see a harm to depict them, capture in the text that they are valid architectures but that those options would not be subject to multiband requirements (but rather multiple single band). |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1: Antenna array | From the GTW on April 19th - Agreement: - Observation 1 can be captured into revised TP. TP R4-2304632 need to be revised. - TP R4-2304118 approved |
| Issue 2-2: Phase shifters | From the GTW on April 19th - Agreement: - TP R4-2304118 approved |
| Issue 2-3: Fractional bandwidth and percentage bandwidth | From the GTW on April 19th • Agreement: TP R4-2304711 need to be revised. |
| Issue 2-4: Corrections in DPD sections | From the GTW on April 19th • Agreement: TP R4-2304712 is approved. |
| Issue 2-5: Feasibility considerations for configuration options | From the GTW on April 19th • Agreement: Further work on the revision of TP R4-2305682 |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |
| Revision of R4-2304632 | Nokia: Comments made on TP directly. Company B: |
| Revision of R4-2304711 | Nokia: Comments made on TP directly. Huawei: A update was made on the TP |
| Revision of R4-2305682 | Huawei: Comments are made on the TP directly. Nokia: Comments made on TP directly. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2305880 (Revision of R4-2304632) | The latest version is Agreeable |
| R4-2305881 (Revision of R4-2304711) | The latest version is Agreeable |
| R4-2305885 (Revision of R4-2305682) | The updated draft captures most of comments in the 2nd round. Moderator suggests to agree on the latest version if no further disagreement during final checking window, otherwise return to Wed GTW. |

Topic #3: SI summary

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304117 | Nokia, Nokia Shanghai Bell | TP to TR 38.877: Summary and conclusions |
| R4-2305301 | Huawei, HiSilicon | TP on SI summary |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3 – SI summary

Issue 3-1: SI summary

- Proposals

  - Option 1: TP in R4-2304117

  - Option 2: TP in R4-2305301

- Recommended WF

  - Collect the comments and merge the text from the two contributions

Companies views’ collection for 1st round

Open issues

Collection of comments:

Issue 3-1: SI summary

| Company | Comments |
| --- | --- |
| Nokia | - No need for band pairing restriction in TS should also be mentioned as agreed in WF; - additional feasibility aspects should also be captured in the summary; - agreements on reusing FR1 MB definitions and requirement approach should also be mentioned as agreed in WF; - FR2 MB BS requirements should be added to TS 38-141-2 instead of TS 38.141-1. |
| Huawei | To Nokia, Based on the agreed WF, we think we captured the above aspects already. e.g. No band pairing restriction aspect is actually captured in the TP “However, whilst practical limits exist which may limit multi-band solutions to be within these groups no restrictions need to be used in the technical specification and as such frequency groups do not need to be defined with the TS updates” And agree the requirements should added to 38.141-2. We are ok to merge the text from the two TPs. |
| Ericsson | Both proposals look generally OK. Regarding the lack of need to restrict to frequency band groups, slightly prefer the Huawei wording “do not introduct Frequency band groups” because discussing restrictions on band pairing etc. sounds like CA. However, it should be clarified that if new requirements are introduced for wider FBW then at that stage the FBG concept may be needed. Note that there is an NEC TP to change the term FBW to PBW. Regarding the recommendation to RAN, in our view it is better to capture that the study suggests that considering FBW<19.5%, a multi-band specification is feasible (with no need to restrict it) and a WI could be started. (Recommending a WI is rather strong, since the technical study has not demonstrated that the spec is absolutely needed because the alternative to do two single bands within the same enclosure is not viable or even not optimal). We are open to discuss a WI in RAN in any case. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1: SI summary | From the GTW on April 19th • Agreement: TP in R4-2304117 can be merged into revision of R4-2305301, TP R4-2305301 need to be revised with updated source company: Huawei, Nokia, Ericsson |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |
| Revision of R4-2305301 | Nokia: Comments made on TP directly. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2305884 (Revision of R4-2305301) | The latest version is Agreeable |

Topic #4: Others

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2305300 | Huawei, HiSilicon | Consideration on the co-location requirements |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4 – Others

Issue 4-1: Co-existence and co-location for FR2 BS

Summary from R4-2305300,

- Agreements as to the exclusion for co-location requirements in the 1st release of NR are captured in TR 38.817-2 but the background behind the decisions are not captured

- The work on MS-BS makes it clear that multiple FR2 bands in the same geographical area is a valid deployment scenario.

- Co-location is not an MB-BS issue as such as any MB-BS could be expected to have synchronised Tx and Rx across the bands.

- Potentially as there are now many more FR2 bands relying on regional regulation rather than incorporating in 3GPP requirements is no longer the best course of action.

- Proposals

  - to discuss whether co-existence and co-location in FR2 need to be studied in Rel-18 WI

- Recommended WF

  - Collect the comments

Companies views’ collection for 1st round

Open issues

Collection of comments:

Issue 4-1: Co-existence and co-location for FR2 BS

| Company | Comments |
| --- | --- |
| Nokia | 70 dB isolation may not be achievable for co-location as this requires 2.7 m separation with FSPL. |
| NEC | We do not think co-location and co-existence are FR2 MB-BS specific issues. They should be out of scope. |
| Huawei | To Nokia The exact value can be further studied in future WI. In our contribution we refer to contributions discussed in Rel-15. To NEC Strictly speaking, as mentioned in our paper, we agree it is not MB-BS issue as MB-BS is expected to be synchronized across the bands. While we think it is related and would like to seek other companies’ view on the future work. |
| Ericsson | There are a number of considerations for co-existence and co-location that should be taken into account. It may not be the right approach to base requirements on a worst case or near worst case that assumes beams pointing at one another. For both co-existence and co-location scenarios, the coupling level varies a lot depending on beam direction and beam isolation and an appropriate study to decide what is a meaningful statistical model of interference may be needed. Apart from an analysis of the interference conditions, consideration of the technology feasibility is also needed. Also, the co-location antenna model (based on a colocation reference antenna) for FR1 requirements is not obviously feasible for FR2 and there would need to be study on how to define and measure requirements. An alternative approach would be to define requirement levels as field strength or power density making requirements independent to any unique test antennas (e.g., CLTA). We note that there is study on co-location between operators as part of the SBFD study where it is claimed that site solutions can mitigate co-location interference (although we are not convinced that site solutions with isolation, distance etc. are generally feasible in practical deployments). With this in mind, we do not think that an objective on studying co-location should be added for the Rel-18 study as there is insufficient time remaining in the release to do the study properly. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 4-1: Co-existence and co-location for FR2 BS | No decision is needed. Moderator proposes to close the discussion. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2304117 | TP to TR 38.877: Summary and conclusions | Nokia, Nokia Shanghai Bell | Merged |  |
| R4-2304118 | TP to TR 38.877: Phase shifter and antenna | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2304632 | TP to TR 38.877: Antenna array | Murata Manufacturing Co Ltd. | Revised |  |
| R4-2304633 | FR2-1 multi-band antenna analysis | Murata Manufacturing Co Ltd. | Noted |  |
| R4-2304711 | TP to TR 38.877: Fractional bandwidth and percentage bandwidth | NEC | Revised |  |
| R4-2304712 | TP to TR 38.877: Corrections in DPD sections | NEC | Agreeable |  |
| R4-2305298 | TR 38.877 v0.3.0 | Huawei, HiSilicon | Revised |  |
| R4-2305299 | TP for Clause 3: abbreviations | Huawei, HiSilicon | Revised |  |
| R4-2305300 | Consideration on the co-location requirements | Huawei, HiSilicon | Noted |  |
| R4-2305301 | TP on SI summary | Huawei, HiSilicon | Revised | To updated source company: Huawei, Nokia, Ericsson |
| R4-2305681 | Additional feasibility aspects for FR2-1 multi-band BS | Intel Corporation | Noted |  |
| R4-2305682 | TP for TR 38.877: Additional feasibility aspects | Intel Corporation | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2305882 | TR 38.877 v0.3.0 | Huawei, HiSilicon | Agreeable |  |
| R4-2305883 | TP for Clause 3: abbreviations | Huawei, HiSilicon | Agreeable |  |
| R4-2305880 | TP to TR 38.877: Antenna array | Murata Manufacturing Co Ltd | Agreeable |  |
| R4-2305881 | TP to TR 38.877: Fractional bandwidth and percentage bandwidth | NEC | Agreeable |  |
| R4-2305885 | TP for TR 38.877: Additional feasibility aspects | Intel Corporation | Agreeable |  |
| R4-2305884 | TP on SI summary | Huawei, HiSilicon, Nokia, Ericsson | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Murata Manufacturing Co., Ltd. | Yoshiaki Hasegawa | yoshiaki.hasegawa@murata.com |
| NEC | Tetsu Ikeda | tetsu.ikeda@nec.com |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |
| Huawei | Liehai Liu | liuliehai@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)