3GPP TSG-RAN WG4 Meeting #106bis-e                                                        R4-2305976

Online, April 17th – 26th, 2023

Agenda item:			5.20.4

Source:	Moderator (Samsung)

Title:	Topic summary for [106bis-e][306] FS_NR_duplex_evo_Part1

Document for:	Information

# Introduction

This thread is on Rel-18 SI for Study on evolution of NR duplex operation, in which the following highlighted agenda items are supposed to be covered:

- Study on evolution of NR duplex operation [FS_NR_duplex_evo] - General and work plan [FS_NR_duplex_evo] - Study the feasibility of and impact on RF requirements [FS_NR_duplex_evo] # * R1-2302087 LS on interference modelling for duplex evolution FS_NR_duplex_evo Rel-18 to RAN4 - Adjacent channel co-existence evaluation [FS_NR_duplex_evo] - Implementation feasibility of SBFD [FS_NR_duplex_evo] - Feasibility of FR1 BS aspects [FS_NR_duplex_evo] - Feasibility of FR2 BS aspects [FS_NR_duplex_evo] - Feasibility of FR1 UE aspects [FS_NR_duplex_evo] - Feasibility of FR2 UE aspects [FS_NR_duplex_evo] - Impacts on BS RF requirements [FS_NR_duplex_evo] - Impacts on UE RF requirements [FS_NR_duplex_evo] - Summary of regulatory aspects [FS_NR_duplex_evo] - Moderator summary and conclusions [FS_NR_duplex_evo]

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Mustafa Emara | memara@qti.qualcomm.com |
| Samsung | Jackson Wang | h0809.wang@samsung.com |
| Kumu | Mayank Jain | mayank@kumunetworks.com |
| Intel | Mark Lehne | mark.a.lehne@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General and Reply LS to RAN1

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304036 | Spark NZ Ltd | RAN4 and RAN 1 simulation assumptions and performance degradation methodology should be aligned. |
| R4-2304535 | Nokia, Nokia Shanghai Bell | Proposal 1: Reply to RAN1 that in addition to the model proposed for the co-site inter-sector gNB-gNB adjacent-channel CLI modeling proposed in R1-2302087, the noise figure model proposed by RAN4 in R4-2302885, should also be considered. Proposal 2: Reply to RAN1 that ACLR and ACS values agreed for the co-site inter-sector co-channel inter-subband CLI can be reused for the co-site gNB-gNB adjacent channel case. Observation 1: Assumptions made for SBFD co-site inter-sector co-channel inter-subband are much higher than assumption used on current co-location BS isolation assumption of 30 dB. Proposal 3: RAN4 to discuss the range and typical values of co-site (zero grid shift) spatial isolation in light of the assumptions of the current requirements of 30 dB in FR1 and 50 dB for FR2 for co-site gNB-gNB adjacent channel. Proposal 4: Reply to RAN1 that the spatial isolation values agreed for co-site inter-sector gNB-gNB adjacent channel CLI, the following are proposed for macro BS: • FR1: 30 - 62 dB with TBD being typical values. • FR2: 50 - 75 dB-with TBD being typical values. Proposal 5: For the co-site gNB-gNB adjacent channel, no improvement over the spatial isolation due to isolating materials is considered. Proposal 6: RAN4 agrees with the RAN1’s model for the UE-UE co-channel inter-subband CLI when only large-scale fading is considered. Proposal 7: Reply to RAN1 that if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the working assumption on the model on UE-UE co-channel inter-subband CLI is inline with RAN4’s understanding. Proposal 8: Reply to RAN1 that the UE NF proposed in [3] model should be considered |
| R4-2304542 | Ericsson | Proposal 1 Clarify the UE ICS Proposal 2 Clarify that the IBE is applied per UE transmitter (with the per transmitter output power assumed) |
| R4-2304191 | CMCC | Observation 1: usually in commercial network, vertical isolation distance is larger than 3m between two operators’ antenna array when mounted on the same pole. Observation 2: based on testing results, we have following observation about the relationship between antenna isolation distance and CL. Compared with horizontal isolation, vertical spatial isolation almost dominant CL besides, such CL difference between horizontal and vertical isolation would narrow as isolation distance increases. when isolation distance is large enough, e.g. larger than 10m, the CL difference would almost disappear. Proposal 1: all the interference from three different inter-operator sectors could be assumed as the same. i.e. α_(adj,co-site-1)^ =α_(adj,co-site-2)^ =α_(adj,co-site-3)^ Proposal 2: the ACLR and ACS in agreement-3 for α_(adj,co-site-2)^ is suggested as below as starting point. when RAN4 finish co-existence simulation, such value may be updated accordingly. o when aggressor DL sub-band BW is wider than victim, equivalent ACLR is equal to normal ACLR (the same as legacy BS) o when aggressor DL sub-band BW is narrower than victim,  equivalent ACLR is equal to normal ACLR (the same as legacy BS) + X, where X equals to the ratio of victim DL sub-band BW to aggressor DL sub-band BW Proposal 3: inter-operator co-site deployment case would introduce severe blocking issue that’s the reason why RAN4 doesn’t include it into RAN4’s simulation. Proposal 4: RAN4 could confirm RAN1’s agreement-4. Observation 3: RAN4 needs further discussion on ICS modeling and whether it is feasible to model as flat. |
| R4-2304441 | CATT | Send an LS to RAN4 to inquire on the value of α_(adj,co-site-x)^ . Answer from RAN4: In current RAN4 adjacent channel co-existence simulation, grid shift uses 100% as baseline. No other spatial ISO is assumed except the antenna pattern. It’s suggested RAN1 follows RAN4 assumption in the SLS simulation. For the question related to the UE-UE co-channel inter-subband CLI modeling, Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding. Answer from RAN4: RAN1 agreement is inline with RAN4’s understanding. For the question related to the model of UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE, Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding. Answer from RAN4: RAN1 agreement is inline with RAN4’s understanding. RAN4 also would like to correct the NF model replied in R4-2302885 that FR2 NF model is missing. The NF model is corrected as below. The noise figure model is provided as below: X-axis: Total received power is the linear sum of all received power, including wanted signal, self-interference, inter-gNB interference and inter-sector interference. Y-axis: noise figure For FR1 WA BS, the values of A, B, C and D: A = -43dBm B = -25dBm C = 5dB D = 14dB For FR2 WA BS, the values of A, B, C and D: A = [-63]dBm B = [-45]dBm C = 10dB D = [19]dB |
| R4-2305074 | vivo | This contribution discusses interference modelling issues for duplex evolution in the LS [1]. Our proposed reply is as in the discussion part. |
| R4-2305202 | Samsung | Proposal 1: RAN4 reply the following information related to Agreement-3: Based on RAN1’s understanding on the co-site gNB-gNB adjacent-channel CLI modelling (i.e., reusing similar method as co-site inter-sector co-channel inter-subband CLI modeling), RAN4 would like to provide the understanding on α_(adj,co-site-x)^ (i.e., the interference suppression capability of co-site inter-sector adjacent-channel inter-subband CLI between the aggressor sector x and the victim sector) as 10*〖log〗_10 (α_(adj,co-site-x)^ )=〖spatial isolation〗_dB+10*log_10 (1/(1/ACLR_BS +1/ACS_BS )) in which ACLR_BS and ACS_BS: RAN4 agree to apply gNB ACLR and gNB ACS minimum requirement according to the RAN4 specification as the baseline for SBFD evaluation. For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4: FR1: 87-118dB with 100dB being typical value. FR2: 100-123dB with 113dB being typical value. Note: Companies has proposed that isolating materials between adjacent channel antennas and RF interference cancellation can provide additional spatial isolation. Proposal 2: RAN4 reply the following information related to Agreement-4: RAN4 confirm RAN1’s understanding on this model; For the in-channel selectivity (or subband selectivity) ICS_UE used in this modeling, the following values have been proposed in RAN4: 33dB for FR1 and 30dB for FR2; Note 1: based on the performance typical for legacy UE Proposal 3: RAN4 reply the following information related to Working Assumption: RAN4 confirm the same response for ICS_UE to Agreement-4 can be reused here. |
| R4-2305307 | Huawei, HiSilicon | On co-site gNB-gNB adjacent-channel CLI modelling, Answer from RAN4: For the new FR1 deployment band, if sub-band filter per operator is considered for macro BS, 10*〖log〗_10 (α_(adj,co-site-x)^ )=155 dB On UE-UE co-channel inter-subband CLI, Answer from RAN4: For system study, RAN4 agreed the following values for sub-band selectivity: FR1: 33 dB FR2: 30 dB On the update from R4-2302885, Update from RAN4: If analogue sub-band filter is adopted in the SBFD solution for FR1 macro BS, the blocking performance can be improved ~ 10 dB. The following values of A and B can apply to the agreed noise figure model. The values of A, B, C and D: A = -33dBm B = -15dBm C = 5dB D = 14dB |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Open issues and candidate options before e-meeting:

### Sub-topic 1-1: General

#### Issue 1-1-1: Simulation assumption alignment with RAN1

- [Moderator] Proposal from Spark (R4-2304036)

  - Proposal 1: RAN4 and RAN 1 simulation assumptions and performance degradation methodology should be aligned.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | This issue is also captured into email thread [308]. Now almost all the simulation parameters have been approved in RAN1 and RAN4. I’m afraid it’s too late to re-discuss simulation assumptions and performance degradation methodology again. |
| Nokia, Nokia Shanghai Bell | Small-scale fading is not considered in earlier simulations as it would lead to more pessimistic results. However, it should be discussed if this could be included. We agree that there is misalignment between RAN4 and RAN1 on certain simulation assumptions. Our intention since the beginning of the SI was to align RAN4 assumptions to the agreed RAN1, e.g., assume the same channel model for BS-BS links, UE clustering methodology, BS antenna configuration, etc. A similar initiative was also triggered by RAN1 during RAN1 #112 to align on parameters such as channel bandwidth, transmit power and noise figure. As result of this, some parameters were changed but there are still some inconsistencies, e.g., RAN4 adopts TR 38.803 equations for the BS-BS link while RAN1 uses TR 38.901. Given the time frame, we hardly see any further alignment between both WGs and our proposal is to clearly state the difference between RAN1 and RAN4 assumptions on the TR. We also agree that the simulation methodology between RAN1 and RAN4 is quite different, but we think each WG has different targets. In RAN1, we are interested in pure system performance, and we carefully model aspects such as time and frequency varying channel, link-adaptation, MIMO, dynamic scheduling, etc... On the other hand, RAN4 analyzes coexistence of technologies and assumes worst-case conditions with simplified and static assumptions. For instance, differently from RAN1, RAN4 do not model any small-scale fading and assumes full buffer traffic. We think that it is hard to find alignment between the methodologies of RAN1 and RAN4. |
| Spark | RAN 4 and RAN 1 have different simulation conditions and assumptions but this is not ideal. That being so, the results will also be different. How can this divergence bring confidence in the minds of operators and regulators who will need to gauge the performance impacts due to SBFD. |
| Qualcomm | Similar to our comment in the adjacent coexistence thread [308]. We had this discussion in RAN4 on the alignment between RAN1 and RAN4 and we agreed in RAN4 that due to the different objectives of the coexistence work in RAN1 and RAN4, each group should have freedom to define their parameters. It is naturally desirable to align general parameters (if possible) between RAN1 and RAN4, but given the timeline and effort in the calibration effort, we would like to propose to continue with the agreed parameters to progress the coexistence study in RAN4. |
| Samsung | For this duplex evolution SI, it had been agreed since #104bis meeting that RAN4 assumptions would not simply adopt RAN1 assumptions. The methodologies and purposes are different from two WGs, which is discussed, recognized and agreed for this work in RAN4. We agree that the simulation condition can be carefully captured in the TR. Specifically, we think the discussion comes from the lack of small scale fading as mentioned in R4-2300079. And the problem here is whether or not including small-scale fading has impact on ACIR co-existence evaluation, which is the intention. Whether or not the channel matrix provide the rank is not that relevant for this evaluation purpose. As RAN4 did for other coexistence study, small-scale fading are also not included. |
| Kumu | Agree with Qualcomm and Samsung. At this point, it is hard to align RAN1 and RAN4 parameters, especially since the evaluation goals are different. We don’t see the need to include small scale fading for RAN4 to evaluate feasibility of SBFD. |
| vivo | We had the same proposal in earlier meetings. It reached the consensus that simulation assumptions should be discussed case by case, not all simulation assumptions need to be aligned with RAN1 since the evaluation purpose slightly differs in RAN1 and RAN4. Furthermore, it is too late for this alignment. |
| CATT | We also support keeping the current simulation assumptions in each group if there’s no severe technical issues found. |

### Sub-topic 1-2: Response to Agreement-3 in R1-2302087

[Moderator] In Feb. RAN1 meeting, LS R1-2302087 has been approved which contains four RAN1 agreement, and accordingly confirmation and reply from RAN4 are expected.

[Moderator] The following Agreement-3 is contained in RAN1 LS.

Agreement-3 For SLS in RAN1, for co-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows. Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj - Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj. - Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale). - NDLRBadj is the total number of DL RBs in adjacent channel. - Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel. - αadj, co-site-x is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI between the aggressor sector x and the victim sector. - 10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - FFS the concrete value of αadj, co-site-x - ACLRBS and ACSBS are in linear scale. Send an LS to RAN4 to inquire on the value of αadj, co-site-x.

#### Issue 1-2-1: Sub-band filtering per operator and ACS/ACLR

- Proposal:

  - Option 1 (Huawei): it is proposed to consider the sub-band filter per operator for the new deployment band which can specify the new requirements from day one.

  - Option 2 (Samsung, vivo, CMCC, Nokia): ACLR_BS and ACS_BS: RAN4 agree to apply gNB ACLR and gNB ACS minimum requirement according to the RAN4 specification as the baseline for SBFD evaluation.

  - Option 2a (CMCC): such value may be updated accordingly.

      - when aggressor DL sub-band BW is wider than victim, equivalent ACLR is equal to normal ACLR (the same as legacy BS)

      - when aggressor DL sub-band BW is narrower than victim, equivalent ACLR is equal to normal ACLR (the same as legacy BS) + X,where X equals to the ratio of victim DL sub-band BW to aggressor DL sub-band BW

  - Option 3 (Ericsson): ACLR and ACS should be applied as per option 2, and it should be reminded that the blocking model should in addition be applied to the total of all RX power.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1 and Option 2 are not mutually exclusive. Option 1 is consider additional isolation from sub-band filter per operator to allow more SBFD deployments. We support also option 2a to consider the scaling factor. We do not agree Option 3, in our view, if we consider blocking model, it should replace ACS rather apply additionally. |
| CMCC | Option 1,2 could be merged. Option 1 is for future enhancement. Now using option 2 for simulation. Regarding option 3 NF modeling captured in RAN4 LS to RAN1, there are two different understandings in RAN1. First one, the NF modeling and ICS(ACS value) both should be considered, so non-linearity is the sum of NF and ACS value. another understanding is that only NF is enough without considering ICS (ACS value). for option 3, before we send this LS, we may need to have a further clarification for above issue. |
| Nokia, Nokia Shanghai Bell | Option 1: Sub-band filtering is not feasible due to added IL, increased complexity, decreased flexibility, added size and power consumption, additional thermal management, big transition BW, increased cost etc. Option 2: This should be selected for ACS/ACLR Option 2a: This can’t be applied to the RAN1 model where power spectral density is used (note P_TX_sector_perRB used). The scaling with the bandwidth is already applied and no need to scale the RAN4 ACLR values Option 3: We agree that the blocking model should be applied to the total RX power |
| Qualcomm | We support option 2 (and 2a, which we already conduct in the adjacent coex study). To our understanding the blocking model is already covered by the ACS. @Nokia: The ACLR scaling was discussed and agreed within RAN4 depending on the victim and aggressor’s relevant BW (WF in RAN4#105). RAN1 considers this scaling per RB as their modeling is done on a per RB basis. |
| Ericsson | In our understanding option2+option3 is according to existing agreements. |
| ZTE | For option 2, we are fine with it, for the option 2a ,we need to further discuss ACS together with ACLR, otherwise this not complete requirement. For the option 1, we did have see lots of feasibility study link budget has already considered the sub-band filtering, we think that this could be also been considered in the coexistence study. Since analog filter will not differentiate the interference towards itself or other sites. In addition,we noticed that the following minor ACI modelling between RAN1 and RAN4. even though difference is not big since DL bandwidth is 80MHz vs 100MHz. |
| Samsung | Option 2 and 2a are aligned with our understanding for ACLR and ACS to be used for co-site gNB-gNB adjacent-channel CLI modelling. For Option 1, we see subband filtering can be considered for new SBFD BS, and how the requirement shall be specified can be FFS in work item in future release (if any). For the option 3, it involves the different understanding of this issue and we are open for more discussion on this. |
| Kumu | Support option 2 and 2a. |
| Intel | We support option 2 and 2a, For new SBFD capable BS option 1 could be considered in the future. |
| CATT | Support to follow RAN4 assumptions in co-existence study. |

#### Issue 1-2-2: Spatial isolation for adjacent-channel inter-sector CLI

- Proposal:

  - Proposal 1 (CMCC): all the interference from three different inter-operator sectors could be assumed as the same. i.e. α_(adj,co-site-1)^ =α_(adj,co-site-2)^ =α_(adj,co-site-3)^

  - Proposal 2 (Samsung):

      - FR1: 87-118dB with 100dB being typical value.

      - FR2: 100-123dB with 113dB being typical value.

      - Note: Companies has proposed that isolating materials between adjacent channel antennas and RF interference cancellation can provide additional spatial isolation.

  - Proposal 3 (Nokia):

      - Reply to RAN1 that the spatial isolation values agreed for co-site inter-sector gNB-gNB adjacent channel CLI, the following are proposed for macro BS:

        - FR1: 30 - 62 dB with TBD being typical values.

        - FR2: 50 - 75 dB-with TBD being typical values.

      - For the co-site gNB-gNB adjacent channel, no improvement over the spatial isolation due to isolating materials is considered.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We need more evaluation on the spatial isolation for co-location inter-sector case. Option 3 may be too pessimistic. For example for FR1, the target band for FR1 is in high frequency, better isolation can be achieved. |
| CMCC | It seems option 1 is the common understanding. Regarding detailed value, at least co-site inter-operator could achieve better special isolation compared with self-interference since vertical isolated distance is larger for inter-operator case. the value for proposal 3 is even less than self-interference. |
| Nokia, Nokia Shanghai Bell | Proposal 1 is a fair assumption, but it is a simplified one. First of all, there is need to clarify the scenario considered by RAN1. Is RAN1 asking about a 0% grid shift scenario? In our view, this is what is meant by “co-site adjacent channel”. RAN4 has not agreed on the values for this case yet, and is using 100% grid shift as baseline. But a co-site deployment also needs to be studied. In TR 38.828, the isolation between co-located base stations (zero grid shift) is 30 dB in FR1 and 50-70 dB in FR2. Therefore, we see that the assumptions made for SBFD (inter-sector co-channel) are too high when compared to the values assumed in RAN4 requirements, and need to be discussed. Also, it should be noted that for co-site gNB-gNB adjacent channel, no improvement over the spatial isolation due to isolating materials is considered. |
| Qualcomm | We support proposal 2. Option 3 values are very pessimistic with a remarkable gap between the co-channel and adjacent inter-sector isolation. RAN4 has not considered the case with 0% grid shit due to the expected severe blocked that will be realized from this case. Note that in 38.828 indeed 30dB isolation was assumed, but this for the dynamic TDD which is fundamentally different to compare to SBFD capable gNBs due to the novel cancellation techniques that might be employed (e.g., spatial isolation, filtering, beam nulling, etc.). We agree with CMCC that at least co-site inter-operator case benefits from additional vertical isolation which leads to better spatial isolation numbers. |
| Ericsson | We do not expect significantly larger isolation between operators sectors compared to inter-sector for the same operator. Although other operators sectors may be located a bit further away than co-channel sectors, the isolation depends mainly on beam and side-lobe directions. All co-location requirements for operators BS on different bands assume 30dB isolation between BS and BS vendors need to use filters to ensure that co-existence actually works between bands. In our experience of site deployments, there are even some deployments where 30dB is too high. It does not make sense that isolation would be so much higher within the same band. We do not believe that building sites with isolating materials between different operators is generally practical for most sites taking into account the practicalities of different operators installations, site weight, form factor, wind load, weather proofing, planning permission and zoning etc. Sites today are already very crowded with equipment. We propose to use either the Nokia values (the lower end of which fairly represents what we actually experience for busy co-located sites today) or the same values as proposed for co-channel isolation. Alternatively, we could assume that SBFD is not operated on sites at which other operators are co-locaed and document this in the TR, as suggested below. |
| ZTE | Open for further discussions, we would also like to know how to get the additional 25dBc isolation (vertical separation or horizontal separation). This is done by the measurement or predictions? I did see some measurement results from CMCC LTE data, not yet from other companies. |
| Samsung | As proponent, we support proposal 2. And P1 is also okay to us. As illustrated in R4-2305202, considering the longer separation distance and also of the vertical placement, adjacent-channel inter-sector CLI (e.g., B2 to A1) compared to co-channel inter-sector CLI (e.g., A2 to A1) shall have larger spatial isolation, 20-30dB higher spatial isolation. For the spatial isolation of adjacent-channel inter-sector CLI like sector B1 to A1, it should be much higher than the spatial isolation for self-interference, which is assumed to be 80dB for FR1 and 87dB for FR2 as typical value. Considering the longer distance contribute 25dB more isolation, we assume the above proposed typical value for isolation can also be achievable. |
| Kumu | We support Proposal 2. Additionally, we propose editing the note in proposal 2 to include beam-nulling as follows: - Note: Companies have proposed that isolating materials between adjacent channel antennas and RF interference cancellation and / or beam nulling can provide additional spatial isolation. |
| Intel | We generally support Proposal 2, but our view is that these high isolation values require advanced techniques such as isolation materials. |
| CATT | We’re ok to reply RAN1 some value, but have some concerns on how RAN1 will use it. So prefer to reply RAN1 that RAN4 assumptions can be followed by RAN1. |

#### Issue 1-2-3: Drafted Reply

- Drafted reply with major difference for companies’ understanding on spatial isolation:

  - Option 1 (CMCC/CATT): The scenario evaluated by RAN1 is simulated by RAN4.

      - CMCC: Inter-operator co-site deployment case would introduce severe blocking issue that’s the reason why RAN4 doesn’t include it into RAN4’s simulation.

      - CATT: In current RAN4 adjacent channel co-existence simulation, grid shift uses 100% as baseline. No other spatial ISO is assumed except the antenna pattern. It’s suggested RAN1 follows RAN4 assumption in the SLS simulation.

  - Option 2 (Huawei):

      - For the new FR1 deployment band, if sub-band filter per operator is considered for macro BS,

        - 10*log10αadj,co-site-x=155 dB

  - Option 3 (vivo):

      - In the second part of reply LS [2], RAN4 provided the spatial isolation and the values 10*log10ACLRBS and  10*log10ACSBS for co-site gNB-gNB adjacent-channel CLI modelling. RAN1 can calculate the value of αadj, co-site-x based on RAN4’s reply.

Regarding spatial isolation values, the following values have been proposed for macro BS in RAN4:

      - FR1: 62-93dB with 75dB being typical values.

      - FR2: 75-98dB with 88dB being typical values.

The ACLR/ACS values for FR1 and FR2 are shown in the table below.

| Range | ACLR [dB] | ACS [dB] |
| --- | --- | --- |
| FR-1 | 45 | 46 |
| FR-2 | 28 | 24 |

  - Option 4 (Samsung): RAN4 reply the following information related to Agreement-3:

      - Based on RAN1’s understanding on the co-site gNB-gNB adjacent-channel CLI modelling (i.e., reusing similar method as co-site inter-sector co-channel inter-subband CLI modeling), RAN4 would like to provide the understanding on αadj, co-site-x (i.e., the interference suppression capability of co-site inter-sector adjacent-channel inter-subband CLI between the aggressor sector x and the victim sector) as

10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS

in which

  - ACLRBS and ACSBS: RAN4 agree to apply gNB ACLR and gNB ACS minimum requirement according to the RAN4 specification as the baseline for SBFD evaluation.

  - For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4:

    - FR1: 87-118dB with 100dB being typical value.

    - FR2: 100-123dB with 113dB being typical value.

    - Note: Companies has proposed that isolating materials between adjacent channel antennas and RF interference cancellation can provide additional spatial isolation.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | On BS ACS, it is the minimum requirement in the specification, we think we should also consider typical value in the evaluation, which is also mentioned in CMCC contribution R4-2304192. And in our analysis in R4-2216239, 62 dB receiver selectivity can be assumed for BS capable of SBFD operation, based on current BS hardware capability. |
| CMCC | Merge all options. For 0% grid shift case, It’s suggested to add more information that RAN4 doesn’t consider co-locate case due to blocking issue. For ACLR, use agreements in sub-topic 1-2-1 For isolation value, use agreements in 1-2-2 Regarding the ACS value, they should be much better compared with 46dB. 50dB is also conservative. 62dB proposed by HW is also OK for us. |
| Nokia, Nokia Shanghai Bell | Option 1: 100% has been agreed as baseline, but other values are not precluded according to the agreement in WF R4-2217466. In our view, RAN1 is asking about the values for a 0% grid shift in their LS. Furthermore, in our view, the case with 0% grid shift should be at least studied or captured in the TR, explaining that it case results in severe blocking issue and was not included in the study. Option 2: Sub-band filtering not feasible as explained in Issue 1-2-1. Option 3&4: We agree the way to calculate α_(adj,co-site-x) as described but there is a need to discuss the spatial isolation values. In TR 38.828, the isolation between co-located base stations (zero grid shift) is 30 dB in FR1 and 50-70 dB in FR2. Therefore, we see that the assumptions made for SBFD (inter-sector co-channel) are too high when compared to the values assumed in RAN4 requirements, and need to be discussed. |
| Qualcomm | We support option 1, 2 and 4, as we can also include information on how RAN4 considers the adjacent operator coexistence and clarify that for 0% grid shift severe blocking will be reached and RAN4 is not focusing on this. |
| Ericsson | See above for our proposal on spatial isolation. Alternatively, we would support the first part of option 1; that we capture in the TR that operation with co-sited operators is not feasible due to the blocking issue. This could be communicated ot RAN1. We do not believe that sub-band filter per oprator is generally workable; for one it means that each BS needs operator specific filters. Secondly it is not just the SBFD operator; also the victim operators would need filters. The filters would require BS replacement since the BS are AAS and have integrated filters. For the second part of option 1, as discussed in the other thread, 100% grid shift is actually an extreme that is not possible in a real deployment; it requires only two operators and that each operator is able to exactly place their BS in between the other operators BS (i.e. co-ordinates and has access to the right site locations, and that both operators networks still function satisfactory). The discussion on grid shift can be kept in the other thread (simulation assumptions) though. |
| ZTE | The problem might be RAN1 and RAN4 simulation case are not aligned at all. It’s better to reply RAN1 that co-site adjacent band scenario, RAN4 didn’t analysis it due to the blocking issue. To keep alignment between RAN1 and RAN4 on this scenario. |
| Samsung | We believe companies’ responses can be merged based on the above two issues. For CMCC/CATT’s proposal on providing the status that only 100% grid shift is included in RAN4 evaluation is okay to us, but whether or not a “severe” blocking problem will happen, we don’t believe we have conclusion on that, for which part we don’t agree because the methods to increase spatial isolation have not be studied or reviewed yet. |
| Intel | There is a wide range of proposals. We do not feel the interference is severe, but it is a potential concern. We would prefer to discuss further |
| CATT | Support ZTE comment. RAN4 did some analysis for the blocking issue, don’t need to bring this issue to RAN1 again. |

### Sub-topic 1-3: Response to Agreement-4 in R1-2302087

[Moderator] The following Agreement-4 is contained in RAN1 LS.

Agreement-4 For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB where - IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value). - PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value) - CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE - NULRB is the total number of UL RBs in the UL subband - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. - 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein, - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1 - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'. - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1. - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs) - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs. Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

[Moderator]: The following issues have overlapping with email thread [307]. Expect more coordination between moderators to make sure the discussion happens in one place.

#### Issue 1-3-1: Clarification on UE ICS (for large-scale fading model)

- Proposals:

  - Option 1 (Ericsson): Clarify the UE ICS.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Option 1: Agree |
| Qualcomm | Proposal to wait for the agreement on the UE feasibility track [307]. |
| Ericsson | The UE ICS is discussed in the UE thread. Preferably we could agree on one value for a reply. |
| ZTE | This could follow agreement reached in UE RF thread. |
| Moderator | Discuss in [307] firstly. |
| Intel | Agree to discuss further, after discussing in 307 |

#### Issue 1-3-2: Clarification on UE IBE

- [Moderator] Company asks to clarify the following method to be used based on IBE:

  - [R4-2304542, Ericsson] Regarding the IBE, the IBE is applicable per antenna connector at the UE, and so if the UE is assumed to have more than one transmit antenna then the IBE should be increased by a factor 10log(Nt). (However, the total power of the UE should be scaled to the power per transmitter and the combination of those two will cause no change to the relative IBE requirement).

- Proposals:

  - Option 1 (Ericsson): Clarify that the IBE is applied per UE transmitter (with the per transmitter output power assumed)

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | During all RAN1 LS, they want to know the IBE for all transmitters rather than per transmitter. If the common understanding is that IBE applies per UE transmitter, we should also translate IBE per transmitter to IBE for all transmitters. |
| Nokia, Nokia Shanghai Bell | Option 1: Agree |
| Qualcomm | We agree with the clarification provided in option 1. |
| ZTE | In RAN4, for FR1, we only consider the single RF branch with 23dBm, we don’t see the problem, for FR2, this could done by per polarization to victim receiver? |
| Samsung | Option 1 is also aligned with our understanding. But with the clarification, there is no change to how RAN1 use this IBE-based model in their simulation because with total UE power limited by P_t with Nt TX antenna for example, - At one TX antenna, the in-band emission will be P_t – 10*log10(Nt) – IBE - For the total impact from all antennas, the in-band emission should be Pt – IBE, which is how RAN1 did as far as we know. |
| Apple | We agree with Option 1. |
| vivo | We agree with Option 1. |
| Intel | Agree with the moderators WF. Discuss option 1 further |
| CATT | Agree with CMCC’s comment. |

#### Issue 1-3-3: Drafted Reply and ICS value (or value range)

- Drafted reply:

  - Option 1 (Samsung/Huawei): RAN4 reply the following information related to Agreement-4:

      - RAN4 confirm RAN1’s understanding on this model;

      - For the in-channel selectivity (or subband selectivity) ICSUE used in this modeling, the following values have been proposed in RAN4:

        - 33dB for FR1 and 30dB for FR2;

        - Note 1: based on the performance typical for legacy UE.

  - Option 2 (vivo):

      - Generally, the above agreement is inline with RAN4’s understanding.

      - In RAN4, we discussed UE co-channel Rx model and no final consensus was reached. For Sub-band/In-channel selectivity, that is ICSUE in RAN1’s model, the range 20~33 dB was proposed for FR1 and 20~34dB for FR2-1. RAN1 can refer to this range if no further RAN4 reply was received.

  - Option 3 (Nokia):

| Frequency range | ICS [dB] |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 23 |

- Recommended WF

  - Agree 33dB for FR1, and Further discussion for FR2-1.

| Company | Comments |
| --- | --- |
| CMCC | Recommended WF is OK for us. if no consensus, then option 2 which captures previous agreements. |
| Nokia, Nokia Shanghai Bell | Option 1&2: Agreed the response to confirm RAN1 understanding and FR1 ICS = 33dB but we propose 23dB for FR2-1. What is the justification for such higher FR2-1 values than stated in 38.101-2? |
| Qualcomm | We support option 1. To confirm, we should finalize this in [307]. |
| Ericsson | This discussion overlaps with the UE thread, it would be good to discuss and settle there. |
| ZTE | Same view as Ercisson, we are also fine with WF indeed. |
| Samsung | Discuss in [307] for the value firstly. |
| vivo | Wait for the conclusion in [307]. |

### Sub-topic 1-4: Response to Working Assumption in R1-2302087

[Moderator] The following working assumption is contained in RAN1 LS.

Working assumption: For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as: IInter-UE-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE, - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn - NR is the number of Rx chains and NT is the number of Tx chains - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1. - yn=y0n,y1n,…,yRnT, - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled - Iselectivity is modelled as frequency flat Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB - Pblocker=1NR*m∈Used UL RBsHCLImWsm2 - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm, - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1 - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB. - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled - NULRB is the total number of UL RBs in the UL subbands, - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

[Moderator]: The following issues have overlapping with email thread [307]. Expect more coordination between moderators to make sure the discussion happens in one place.

#### Issue 1-4-1: Clarification on UE ICS (for small-scale fading model)

- Proposals:

  - Option 1 (CMCC): RAN4 needs further discussion on ICS modeling and whether it is feasible to model as flat.

- Recommended WF

  - [Clarification from CMCC]: This proposal is for both small-scale fading model and large-scale fading model.

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | This issue can be merged with issue 1-3-3. We are OK for flat ICS with 33dB for FR1. |
| Qualcomm | Similar to the previous issue, this is discussed in [307]. |
| Ericsson | This is probably also best discussed in the UE thread. In our understanding the RF effects depend on the total input power to the UE, not the PSD. The distortion in the RX band can probably be seen as flat. |
| ZTE | We don’t need to discuss the UE ICS in small-scale fading model since RF requirement is not dependent on channel model used. |
| Samsung | Same as issue 1-3-3. |

#### Issue 1-4-2: Drafted Reply

- Proposals:

  - Option 1 (Samsung): RAN4 reply the following information related to Working Assumption:

      - RAN4 confirm the same response for ICS_UE to Agreement-4 can be reused here.

  - Option 2 (CATT):

      - RAN1 agreement is inline with RAN4’s understanding.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Option 1. |
| Nokia, Nokia Shanghai Bell | Agree with RAN4 confirming the model proposed by RAN1. |
| Ericsson | The ICS can be assumed to be the same regardless of the small/large scale fading between the UEs. |
| ZTE | Don’t see the discussion point here. To confirm RAN1 ‘s understanding is correct in the small scale part? |
| Samsung | The same response as the one used for Agreement- 4 can be reused here. |

### Sub-topic 1-5: Other information to be included in reply LS to RAN1

#### Issue 1-5-1: Updates on noise figure model for FR1 WA BS

[Moderator]: RAN4 sent a reply LS to RAN1 on interference modelling at last meeting in R4-2302885, where a noise figure model is provided.

- Proposals:

  - Option 1 (Huawei): If analogue sub-band filter is adopted in the SBFD solution for FR1 macro BS, the blocking performance can be improved ~ 10 dB. The following values of A and B can apply to the agreed noise figure model.

      - The values of A, B, C and D:

A = -33dBm

B = -15dBm

C = 5dB

D = 14dB

- Recommended WF

  - Related to Issue 2-3-2 (sub-band filtering’s impact), discussion on proposals/observations in 1st round shall be provided in Issue 1-5-1 by considering the information from Issue 2-3-2.

| Company | Comments |
| --- | --- |
| Huawei | We also think it is needed to discuss how to use the noise figure model and ACS model. In our view, at high blocking power, ACS should be replaced by noise figure model, since it already cover the nonlinear IM3, ADC dynamic and other RX impairments. If additional selectivity is added, it should be the capability of digital filtering, the value should be in the order of 60~80 dB. |
| CMCC | Option 1 is OK for us. we can have two modeling. One for implementation without analog sub-band filter. The other for implementation with analog sub-band filter. |
| Nokia, Nokia Shanghai Bell | Sub-band filtering not seen feasible as explained in Issue 1-2-1. |
| Qualcomm | We are ok with Huawei proposal to consider typical values not minimum requirements. Our preference to omit the subband filtering aspect and keep it implementation agnostic at this stage. |
| Ericsson | We do not believe that using filters to improve the blocking is a viable solution for a commercial BS as explained in our contribution R4-2304541. It would lead to operator specific, inflexible hardware, de-integration of the AAS, greater size, power consumption etc. Also, impacts of filter losses to the noise figure would need to be taken into account. |
| ZTE | We have different understanding with Huawei on ACS and noise model under the high blocking level. The digital filtering should not impacted by higher blocking levels since filter length has already been decided by the normal state. This is also under the discussion in RAN1. For sub-band filtering, we are fine to consider it since this has already been considered by many companies. Not only to Huawei. |
| Samsung | We think the discussion can hardly be converged if the discussion only focus on subband filtering is implementation feasible or not, since it is heavily dependent on companies’ choice and the amount of effort/expense they want to pay for the benefits of SBFD. Giving two sets of parameters can be one way to go to consider the different kinds of implementation. We agree with Huawei that how to use the noise figure model and ACS model shall be clarified firstly. |
| Kumu | Agree with Qualcomm on making the model implementation agnostic, since different companies have different techniques for preventing blocking (such as RF cancellation). |
| Intel | We could agree to include these values for reference, but we see the values already agreed without SB filtering are more universal and should be the first case considered. |
| CATT | Agree that some further thinking may be needed to reply RAN1, otherwise may bring trouble to them … |

#### Issue 1-5-2: Noise figure model for other FR1 BS Types

- Proposals:

  - Option 1 (CMCC):  it’s suggested to further check whether the NF modelling approved in last meeting applies for all the BS classes or only for WA? It only for WA, our suggestions for other classes are as below

      - For other BS classes, point A is suggested as the in-band blocking requirements as in 38.104, i.e. -38dBm for MR and -35dBm for LA and point B is suggested as NF, i.e. 10dB for MR and 13dB for LA.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Option 1. Besides, fixed NF value for MR and LA are also OK for us. besides, NF is very important factor for RAN1 simulation. it’s suggested to send the conclusion to RAN1 in this meeting. |
| Nokia, Nokia Shanghai Bell | Option 1 can be accepted. Noise figure of 10 dB and 13 dB for MR and LA BS respectively is in line with the RAN4 sensitivity requirements. In-band blocking levels can be used as the input power levels to trigger the NF increase in the model. |
| Qualcomm | We are ok with option 1. We agree also with CMCC to share with RAN1 the NF for different BS classes. |
| Ericsson | Our understanding is that the blocking model was for WA. The CMCC suggestion may be a reasonable approach. |
| ZTE | We are fine to further discuss the noise model for other BS class. We could start with CMCC’s approach. |
| Samsung | We are okay with Option 1 as starting point which can be further studied. |
| Intel | We see the agreed model as only applicable to WA BS. For LA BS, these are designed to tolerate higher blocker capability, so the WA BS model would be overly pessimistic. For MR BS, should be in-between. |
| CATT | The model in the reply LS is just WA. It should be clarified. Is it necessary to discuss further for MA and LA? |

#### Issue 1-5-3: Noise figure model for FR2-1 BS

[Moderator]: RAN4 sent a reply LS to RAN1 on interference modelling at last meeting in R4-2302885, but no conclusion for FR2-1

- Proposals:

  - Option 1 (CATT): For FR2 WA, the values of A, B, C and D:

      - A = [-63]dBm

      - B = [-45]dBm

      - C = 10dB

      - D = [19]dB

  - Option 2 (Qualcomm): FR2 BS NF:  For FR2-1 BS NF = 5 dB up to -52 dBm Pin, and then a sloped section rising 1 dB/dB with Pin > -52 dBm. (Note: 5dB vertical shift in the NF should be added on top of the figure, so curve start at 10dB instead of 5dB).

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | One question for option 2. The NF at first stage is 5dB which is different from what is assumed in simulation, i.e. 10dB NF. From our understanding, the NF at first stage should be the same as assumed NF in simulation. We can either update NF in simulation or update this NF modeling. |
| Qualcomm | Our model should start at 10 dB that is a correct aspect raised by CMCC, one would expect a 5dB vertical shift in the model, but the rest should be the same. We added a clarification in the proposal. |
| Ericsson | We are double checking, but the CATT proposal may be reasonable. Possibly the “A” and “B” values could be increased by around 5dB (whilst keeping the other values the same). For the Qualcomm proposal, it is not obvious why the starting NF is 5dB. Then the level at which NF starts to rise is around -55 to -50 dBm in our understanding, rising by around 9dB at -45 to -40 dBm as suggested by CATT. Also the RX blocking point should be captured, as in the CATT model. |
| Intel | We don’t values in this model. The A, B values are too pessimistic. We need to study further. |
| CATT | The proposal from our company was trying to align the FR1 model replied in last meeting. The exact value should be checked by companies. |

#### Issue 1-5-4: RAN4 part TR skeleton approved

[Moderator]: RAN4 approved the TR skeleton for RAN4 part in RAN#106.

- Proposals:

  - Option 1 (vivo): responses include:

      - In RAN4#106 meeting, TR skeleton of RAN4 part for TR 38.858 was also discussed and decided. Companies are encouraged to prepare TPs for interested topics in further meetings.

      - In RAN4#106 meeting, TR skeleton of RAN4 part for TR 38.858 was discussed and decided. Besides the implementation part, impact on RF requirements, adjacent channel co-existence evaluation and regulatory aspects will be in charge by RAN 4. The Sections for RAN4 part in TR 38.858 are listed in the following table. Companies are encouraged to prepare TPs for interested topics in further RAN4 meetings.

| No. | Section for TR 38.858 |
| --- | --- |
| 1 | 10.1 Background for analysis |
| 2 | 10.2 Feasibility of FR1 Wide Area BS aspects |
| 3 | 10.3 Feasibility of FR1 Medium Range BS aspects |
| 4 | 10.4 Feasibility of FR1 Local Area BS aspects |
| 5 | 10.5 Feasibility of FR2 BS aspects |
| 6 | 10.6. FR1 Feasibility of UE aspects |
| 7 | 10.7 FR2 Feasibility of UE aspects |
| 8 | 10.8 Summary |
| 9 | 11.1 Impact on BS RF requirements |
| 10 | 11.2 Impact on UE RF requirements |
| 11 | 12 Adjacent channel co-existence evaluation results |
| 12 | 13 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum |
| 13 | Annex <D>: Adjacent channel co-existence evaluation |

- Recommended WF

  - This part is just information sharing based existing RAN4 agreement, and moderator suggest to include them in the reply LS.

| Company | Comments |
| --- | --- |
| CMCC | Recommended WF is OK for us. |
| Qualcomm | We believe this should be communicated to RAN1 to update them about RAN4 updated skeleton and the planned sections and subsections. |
| Ericsson | Regarding the TP drafting, how to handle this during this meeting ? Our suggestion is to collect comments in this meeting, see further proposals in May (and updates by the proposing companies based on the comments) and then for the editors to make first attempts at merged versions after May. This gives 2H for merging and fine tuning. |
| ZTE | Fine with it, however it seems RAN1 didn’t inform us the skeleton yet, right? |
| Samsung | We are okay to include this in the reply LS. To response ZTE: RAN1 inform us about the updated TP in their LS: Agreement-1 Agree the updated TR for RAN1 in R1-2300997 in principle.. The TP handling methods proposed by Ericsson is aligned with us. |
| Kumu | Recommended WF is OK with us. |
| Apple | The recommended WF is OK. |
| vivo | It seems the Section for TR 38.858 we discussed in RAN4 is different with RAN1. Which do we use as baseline? |

#### Issue 1-5-5: UE NF value/model

- Proposals:

  - Option 1 (Nokia): Reply to RAN1 that the UE NF proposed in [R4-2304538] model should be considered.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

  - Related to email thread [307] on UE part discussion.

  - Need confirmation/coordination for where to discuss to avoid overlapping discussion scope.

| Company | Comments |
| --- | --- |
| Qualcomm | We prefer to discuss this in [307]. |
| Samsung | Discuss in [307] firstly. |

## Companies views’ collection for 1st round

### Open issues

Moderator: views are provided behind each sub-topic/issue.

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

### Sub-topic 1-1: General

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 General | Issue 1-1-1: Simulation assumption alignment with RAN1 Discussion status summary: - Most of the companies want to follow the original agreement (copied below for reference purpose), and realized the different objectives of the coexistence work in RAN1 and RAN4: Agreement Basic principle for the simulation assumption between RAN1 and RAN4 - Follow previous agreements and continue discussions on the remaining open co-ex assumptions to fulfil the RAN4 adjacent channel SBFD co-ex purpose. - There is no general principle for simulation assumption. Some common parameters may need to be aligned between RAN1 and RAN4 while others not.. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Moderator will suggest to RAN4 group to follow the existing agreement, and no more discussion on this “basic principle”. - If company found some particular parameters (regarded as “common parameters”) needs to be aligned between RAN1 and RAN4, it can be further discussed in case-by-case manner. |

### Sub-topic 1-2: Response to Agreement-3 in R1-2302087

| Sub-topic #1-2 Response to Agreement-3 in R1-2302087 | Issue 1-2-1: Sub-band filtering per operator and ACS/ACLR Discussion status summary: - Option 1: companies’ views are diverged. - Option 2: it is agreeable (based on existing agreement) Option 2a: Companies think it is applied in RAN4 simulation already but the scaling is already considered in RAN1’s equation. - Option 3: Agreed by Nokia, Ericsson, but different understanding from Huawei. Tentative agreements: - ACLRBS and ACSBS: - RAN4 agree to apply gNB ACLR and gNB ACS minimum requirement according to the RAN4 specification as the baseline for SBFD evaluation. - Better gNB ACS is achievable by typical implementation compared to minimum requirement: - e.g., ACSBS = 62dB Candidate options: - Alternative-1: ACLR and ACS should be applied as per option 2, and it should be reminded that the blocking model (noise figure over average total input power) should in addition be applied to the total of all RX power. - Alternative-2: If the blocking model (noise figure over average total input power) applied, no need to further consider the impact of ACS. - Note: The blocking model can be further improved (discussed in Issue 1-5-1). Recommendations for 2nd round: - Discussion based on the above tentative agreement and candidate options. - Further check the scaling methods used in RAN1 and RAN4: - RAN4 scaling method: seems the Option 2a is not aligned with below RAN4 agreement: R4-2220246: Agreement: frequency flat assumption with some detailed explanation as below - when aggressor BW is narrower than victim, e.g. SBFD gNB -> legacy TDD gNB - equivalent ACLR is equal to normal ACLR - when aggressor BW is wider than victim, e.g. legacy gNB -> SBFD gNB - total received interference = Ptx – (ACLR + the ratio of aggressor BW to victim BW) - for example, when aggressor is 100MHz and victim is 20MHz, the equivalent ACLR is 45+10*log10(100/20)=51.9dB R4-2302888: Agreement: Use flat ACS modelling in simulation for FR1 and FR2 gNB. - when aggressor BW is narrower than victim, e.g. SBFD gNB -> legacy TDD gNB - equivalent ACS is equal to normal ACS - when aggressor BW is wider than victim, e.g. legacy gNB -> SBFD gNB - total received interference = Ptx – (ACS - the ratio of aggressor BW to victim BW). Seems the above agreement is aligned with ZTE’s illustrative figure: - Question to the group: any follow-up action for the above difference? - For Option 1 (on sub-band filtering per operator for the new deployment), moderator suggest the preferred implementation can be provided in individual companies’ input (based on analysis framework). Suggestion/views are welcomed on how to proceed. |
| --- | --- |
|  | Issue 1-2-2: Spatial isolation for adjacent-channel inter-sector CLI Discussion status summary: - Proposal 1: supported by CMCC, Nokia, Samsung - Proposal 2: supported by Samsung, Qualcomm, Kumu, Intel - Proposal 3 (and Against P2): Ericsson, Nokia Tentative agreements: - All the interference from three different inter-operator sectors could be assumed as the same. i.e. α_(adj,co-site-1)^ =α_(adj,co-site-2)^ =α_(adj,co-site-3)^ - Spatial isolation for adjacent-channel co-site inter-sector interference: - FR1: (62+X)-(93+X)dB with (75+X)dB being typical values. - FR2: (75+X)-(98+X)dB with (88+X)dB being typical values. - X=[25] is added to the inter-sector isolation for co-channel inter-sector interference, because of additional spacing between adjacent-channel antennas. - Note: Companies has proposed that isolating materials between adjacent channel antennas and RF interference cancellation and/or beam nulling can provide additional spatial isolation. Candidate options: N/A Recommendations for 2nd round: - Discussion based on the above tentative agreement. - The following illustrative figure is copied here only for the purpose of facilitating the discussion. |
|  | Issue 1-2-3: Drafted Reply Discussion status summary: - In addition to Issue 1-2-1/2 discussion, it is suggested by CMCC/CATT: - For 0% grid shift case, it’s suggested to add more information that RAN4 doesn’t consider co-locate case due to blocking issue. Tentative agreements: N/A Candidate options: - Option 1: RAN4 also add more information that RAN4 doesn’t consider co-locate case due to blocking issue for 0% grid shift case. Recommendations for 2nd round: - Draft reply LS shall be drafted based on the conclusion from Issue 1-2-1 and 1-2-2. - Additionally, above option 1 can be further discussed. |

### Sub-topic 1-3: Response to Agreement-4 in R1-2302087

| Sub-topic #1-3 Response to Agreement-4 in R1-2302087 | Issue 1-3-1: Clarification on UE ICS (for large-scale fading model) Discussion status summary: - It is common understanding to discuss in [307] firstly. Tentative agreements: N/A. Candidate options: N/A. Recommendations for 2nd round: - No more discussion needed since it should be discussed in thread [307]. Reply LS shall capture the conclusion from [307]. |
| --- | --- |
|  | Issue 1-3-2: Clarification on UE IBE Discussion status summary: - Option 1 shall be the common understanding. Tentative agreements: - Clarify that the IBE is applied per UE transmitter (with the per transmitter output power assumed): - If total TX power (Pt) is equally allocated among the Nt transmitter for UE: - At one TX antenna, the in-band emission will be Pt – 10*log10(Nt) – IBE - For the total impact from all TX antennas, the in-band emission should be Pt – IBE. Candidate options: N/A Recommendations for 2nd round: - Further confirmation on the above tentative agreement. |
|  | Issue 1-3-3: Drafted Reply and ICS value (or value range) Discussion status summary: - Drafting can be based on the conclusion from thread [307] Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Drafting can be based on the conclusion from thread [307]. |

### Sub-topic 1-4: Response to Working Assumption in R1-2302087

| Sub-topic #1-4 Response to Working Assumption in R1-2302087 | Issue 1-4-1: Clarification on UE ICS (for small-scale fading model) Discussion status summary: - Seems it is the common understanding that UE ICS value is not dependent on channel model used. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - If it is the common understanding that UE ICS value is not dependent on channel model used, we can directly discuss Issue 1-4-2 directly and close this issue. |
| --- | --- |
|  | Issue 1-4-2: Drafted Reply Discussion status summary: - Seems RAN4 can confirm (1) the same response for ICS_UE to Agreement-4 can be reused here. (2) RAN1’s understanding on the modeling is aligned with RAN4. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - RAN4 reply the following information related to Working Assumption: - RAN4 confirm RAN1’s understanding on the UE-UE co-channel channel model in the working assumption. - RAN4 confirm the same response for ICS_UE to Agreement-4 can be reused here. |

### Sub-topic 1-5: Other information to be included in reply LS to RAN1

| Sub-topic #1-5 Others | Issue 1-5-1: Updates on noise figure model for FR1 WA BS Discussion status summary: - Different views on the feasibility of sub-band filtering as Issue 1-2-1. - Company suggested the discussion can be on the values in option 1 as the performance achievable by typical implementation. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Based on the conclusion on Issue 1-2-1, further discussion on the following performance improvement as implementation agnostic manner: - If analogue sub-band filter is adopted in the SBFD solution for FR1 macro BS, the blocking performance can be improved ~ 10 dB. The following values of A and B can apply to the agreed noise figure model. - The values of A, B, C and D: A = -33dBm B = -15dBm C = 5dB D = 14dB |
| --- | --- |
|  | Issue 1-5-2: Noise figure model for other FR1 BS Types Discussion status summary: - Option 1 from CMCC are agreed by most companies, at least as starting point for further study. - The noise figure model is agreed as below from last meeting: Tentative agreements: (revision based on CMCC’s proposal) - Noise figure model for other FR1 BS classes: - For MR BS class, the following noise figure model can be used as starting point: - (A, B, C, D) = (-38, -20, 10, 19) - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -38dBm. - Point C is suggested to follow the assumed noise figure, i.e. 10dB. - For LA BS class, the following noise figure model can be used as starting point: - (A, B, C, D) = (-35, -17, 13, 22) - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -35dBm. - Point C is suggested to follow the assumed noise figure, i.e. 13dB. Candidate options: N/A Recommendations for 2nd round: - Further check the above tentative agreement (revised from CMCC proposal). |
|  | Issue 1-5-3: Noise figure model for FR2-1 BS Discussion status summary: - Discussion is based on two proposals, and QC further clarify the proposal during the discussion. Tentative agreements: N/A Candidate options: - For FR2-1 BS, the value of A, B, C and D: - Option-1: - A = [-63]dBm - B = [-45]dBm - C = 10dB - D = [19]dB - Option-2: - A = -52dBm - C = 10dB - a sloped section rising by 1 dB/dB for Pin > -52 dBm Recommendations for 2nd round: - Further discussion based on the above option 1 and 2. |
|  | Issue 1-5-4: RAN4 part TR skeleton approved Discussion status summary: - No opposition on reply with RAN4 agreed TR skeleton for RAN4 relevant chapters. Tentative agreements: - In reply LS, RAN4 capture the agreed TR skeleton for RAN4 relevant chapters. Candidate options: N/A Recommendations for 2nd round: - Agree the above tentative agreement, and draft LS accordingly. |
|  | Issue 1-5-5: UE NF value/model Discussion status summary: - It is common understanding to discuss in [307] firstly. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - No more discussion needed since it should be discussed in thread [307]. Reply LS shall capture the conclusion from [307]. |

## Discussion on 2nd round (if applicable)

2nd round discussion is performed on draft LS directly.

# Topic #2: Implementation Feasibility of SBFD: FR1 BS

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304192 | CMCC | Proposal 1: the NF modelling for other FR1 classes is suggested as below with the same modeling curve but different ABCD value.  For FR1 MR: (A, B, C, D) = (-38, 10, -20, 19)  For FR1 MR: (A, B, C, D) = (-35, 13, -17, 22) Observation 2: commercial gNB could at least achieve 50dB Rx selectivity. Proposal 2: it’s suggested to at least use 50dB for inter-site gNB-gNB Rx selectivity. |
| R4-2304267 | Intel Corporation | Observation 1, nearby clutter can appear similar to self-interference leakage and also be treated by the interference cancellation algorithm. Observation 2, for interference cancellation, two effects can both degrade performance: 1) strong reflectors that require the cancellation algorithm to treat finite delays and 2) feedback delay that can occur when sectors are physically located at some distance apart. Observation 3: If we assume that the far out noise is always less than the adjacent channel power, then having multiple Tx operating on multi-carrier will have only small impact on the total leaked signal power. Proposal 1: In the case of multi-carrier, we should add analysis to account for the impact of far out noise. This will limit the effectiveness of potential digital IC since far out noise won’t be cancelled. Observation 4, since the maximum expected input from RSIC and self-interference leakage is on the order of ~-40dBm, the Rx will always be operating in a weak non-linear region where RMS power is the best way to measure signal power. Proposal 2, We should agree to model the gNB Rx for LNA and AGC with Option 1, RMS power of the input signal. Proposal 3: Add the Intel RSIC values in Table 1 to TR 38.858. Observation 5, the inter-sector interference is worse than the self-interference due to the lack of IC from Tx beam nulling. Methods to further improve the IC may be needed. Proposal 4: Add the Intel inter-sector interference values in Table 2 to TR 38.858. Observation 6, For MR BS and LA BS, where there is a more relaxed NF, it is easier to meet the required interference cancellation even though MR BS, and LA BS have worse isolation than WA BS. |
| R4-2304431 | CATT | TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects” |
| R4-2304433 | CATT | Observation 1: The Tx leakage and Rx contribution requirements to the SI can be divided to -96.8dBm each if each of them contributes to half of noise. Observation 2: The overall RSIC requirement for Tx leakage should be at least 117.8 dB for Local area SBFD BS. Observation 3: If Rx blocking in Tx subband is -35dBm, ACS requirement should be 64.8dB, which is very challenging. Observation 4: If IM3 contribution is -99.8dBm for Local area BS SBFD Rx noise, IIP3 should be at least -2.6 dBm, which is very challenging. Observation 5: If Rx blocking in Tx subband is -44dBm, ACS requirement should be 55.8dB. Observation 6: If Rx blocking in Tx subband is -44dBm, IIP3 should be -16dBm for Local area BS with SBFD capability. Observation 7: If Rx blocking in Tx subband is -44dBm, RSIC requirement should be 68dB for Local area BS with SBFD capability. Observation 8: From the analysis in table5, the following capabilities are needed, overall RSIC for Tx leakage capability is 118dB overall RSIC for blocking capability is 70dB ACS is 55dBc IIP3 is -16dBm The capabilities in observation 8 are improved compared with legacy LA BS, but it’s feasible from implementation point of view. |
| R4-2304540 | Nokia, Nokia Shanghai Bell | Proposal 1: gNB receiver saturation, non-linearity, and AGC model is based on RMS power of the input signal. Proposal 2: Include the provided TP into the TR 38.858. |
| R4-2304543 | Ericsson | TP to TR 38.858: Section 10.1 Background for analysis |
| R4-2304544 | Ericsson | TP to TR 38.858 section 10.2 Feasibility of FR1 Wide Area BS aspects |
| R4-2304882 | Qualcomm CDMA Technologies | TP to TR 38.858: Feasibility of wide area BS aspects |
| R4-2304883 | Qualcomm CDMA Technologies | Observation self-interference: Based on the combined RSIC analysis framework in RAN4+106, several companies have showed that the UL receiver degradation due to residual self-interference can be modeled by rise in the gNB noise floor as N= noise floor +1 dB. Proposal Self-interference: RAN4 to confirm the adoption of the impact of self-interference modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB) to study the impact of SBFD operation on the RF requirements. Observation Clutter measurements: For FR2, the measured 28/39GHz path loss between Tx and Rx antennas including clutter reflections is typically approximately 80 dB or better for empty conference room environment. • Higher path loss is generally observed for larger angular separation between Tx and Rx beams. Observation Digital cancellation and Clutter: The residual self-interference including both direct leakage and clutter echo can be cancelled using non-linear digital cancellation algorithm where additional 10dB-20dB of residual self-interference cancellation could be achieved via digital cancellation. Proposal Multi-carrier support: To progress the feasibility and coexistence work in RAN4, it is proposed to focus on single carrier case and capture high level information on multi-carrier support in the TR. Proposal Blocking model: For the agreed blocking model in RAN4#106, RAN4 to agree on the input of this model as the average total input power, which is the aggregate of the wanted signal and jammer at the receiver side. Proposal FR1 BS IIP3 model: For FR1 BS IM3 model, RAN4 to adopt a Pin-dependent (average total input power) piecewise linear model as shown in the Figure above to characterizes IIP3. The proposed model captures IM3 contributions and AGC impact on IIP3 which can be utilized by RAN4 to progress the SBFD feasibility work. Observation FR1 BS IM3: The IM3 contributions are not significant when the total input power signal + jammer is lower than -52 dBm. Observation ADC aspects: For FR1 BS, other distortions such as ADC quantization noise and distortions were considered in our simulation and measurements, and it was observed that ADC performance is not limiting. Similarly, phase noise and residual sideband are not significant contributors Proposal FR2 BS interference model with co-channel jammer: FR2 BS interference can be modelled as a fixed level of interference 34 dB below the total input power. Proposal co-channel CLI spatial isolation: For co-channel co-site inter-sector inter-gNB CLI, at least similar or improved spatial isolation compared to the self-interference framework (e.g., by means of additional electromagnetic absorbers between the different sectors or radiation mask) should be considered to provide sufficient inter-gNB CLI mitigation. Proposal co-channel CLI modelling: RAN4 to confirm the adoption of the impact of co-channel co-site inter-sector CLI modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB). The values could be updated if new insights are reached in the feasibility study. |
| R4-2305203 | Samsung | Observation 1: Samsung’s input for RSIC budget calculation for FR1 BS is provided in Table-1. Proposal 1: According to SIC budget calculation in Table-1, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation, beam nulling and digital IC applied, for FR1 BS. Proposal 2: For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors. - There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study. Observation 2: Samsung’s input for co-channel co-site inter-sector CLI for FR1 BS is provided in Table-1, in which the interference from one co-site sector can be suppressed to the level lower than noise floor by 5dB. Observation 3: Installing EM conjugated structure between sectors can provide additional inter-sector spatial isolation at the level of 25dB. Observation 4: Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR1 BS implementation. Observation 5: Digital IC is still technically feasible to cancel the residual co-channel co-site inter-sector interference. Observation 6: There are many other deployment-relevant methods to further improve inter-sector antenna isolation, including: - Larger horizontal distance between co-site sectors; - Vertical antenna arrangement for co-site sectors; - Different boresight angle directions, or different electrical tilts or combination thereof for co-site sectors. Proposal 3: RAN4 shall confirm RAN1 that additional spatial isolation can be achieved for co-site inter-sector co-channel CLI, with typical value at least 35dB. Proposal 4: There is limited necessity to further discuss the co-site inter-sector antenna isolation for BS types other than marco BS. Proposal 5: For SBFD feasibility study, the interference suppression capability of co-site inter-sector adjacent-channel inter-subband CLI between the aggressor sector x and the victim sector can be modeled by 10*〖log〗_10 (α_(adj,co-site-x)^ )=〖spatial isolation〗_dB+10*log_10 (1/(1/ACLR_BS +1/ACS_BS )) in which ACLR_BS and ACS_BS: RAN4 agree to apply gNB ACLR and gNB ACS minimum requirement according to the RAN4 specification as the baseline for SBFD evaluation. 〖spatial isolation〗_dB is the corresponding spatial isolation between the aggressor sector x and the victim sector. Proposal 6: For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4: FR1: 87-118dB with 100dB being typical value. Observation 7: For co-site inter-sector adjacent-channel CLI, additional spatial isolation can be provided by inserting the EM conjugated structure. Observation 8: For co-site inter-sector adjacent-channel CLI, RF interference cancellation can provide additional isolation. |
| R4-2305302 | Huawei, HiSilicon | Proposal 1: It is proposed that the scalling factor should be used in the budget calculation for the asymmetrical SBFD DL/UL configurations. Observation 1: Analogue sub-band filter after LNA can provide the needed suppression for the receiver parts after the filter, and the impacts to RX sensitivity due to insertion loss is negligible. Proposal 2: if analogue sub-band filter is adopted in the solution, the blocking performance can be improved at least 10 dB. The following values of A and B can apply to the agreed noise figure model. Our analysis and evaluation is sumarized in Table 2.3-1 for FR1 MR BS and LA BS. |
| R4-2305400 | ZTE Corporation | An excel table is provided for RSIC calculation. Wrong submission for the discussion paper in the zip file? |
| R4-2304400 (Moved from Topic#3 because wrong AI submitted) | Kumu Networks | Observation 1: Using 3GPP_38.901_Uma_LOS channel, we show that having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. This is the case even when beam-nulling is also used in mitigating inter-sector interference. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability. Observation 2: Considering mitigating inter-sector interference using beam-nulling , we show that the mean case rx sensitivity degradation is 0.6 dB. The degree of freedom remaining after self-interference cancellation (using beam-nulling and RF cancellation) provides up to 10 dB additional isolation when beam-nulling of inter-sector interference cancellation is applied. Observation 3: Combining inter-sector beam-nulling for inter-sector interference mitigation, and RF cancellation and beam-nulling for self-interference mitigation, the worst case RSIC capability is 151.64 dB and the mean case RSIC capability is 156.64 dB. Proposal 1: RF cancellation should be used in SBFD to mitigate self-interference pre Rx LNA in terms of minimizing non-linearity effects and overall self-interference residue. Proposal 2: We support using overall Rx sensitivity degradation of 1 dB for SBFD system simulation and evaluation. Proposal 3: Inter-sector interference reporting should consider beam-nulling based interference mitigation for feasibility analysis. |
| R4-2304541 (copied from Topic #4 ) | Ericsson | Observation 1 The use of filters in the early statges of the RF chain to improve IIP3 would cause a large increase in power consumption, size and weight and a decrease in the level of integration possible. Also the filtering would need to be tuned to a specific carrier and so would need to be operator specific and hardware inflexible. Observation 2 A “1dB criterion” for inter-sector interference would be on top of the self-interference. Observation 3 The receiver related distortion depends on the total input power to the receiver, including both self-interference and inter-sector interference. Observation 4 It is not possible to consider inter-sector receiver interference independently from self-interference. Observation 5 For the proposed inter-sector table, it may discussed whether the term “RSIC” is the right one. Observation 6 During a WI phase, there is a need to discuss whether all of a 1dB sensitivity degradation should be used for self-interference (so then inter-site and sector degradations come on top), or whether some margin of the 1dB should be considered for inter-site and inter-sector interference. Based on the discussion in the previous sections we propose the following: Proposal 1 Capture the range of interference levels/desensitization due to inter-sector interference (together with self-interference) under relevant assumptions. Proposal 2 The impact of (i) self-interference only and of (ii) combined self-interference+inter-sector interference should be evaluated. The impact of inter-sector interference could be interpreted as (ii) – (i). |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: BS aspects: deployment and CA support

#### Issue 2-1-1: Assumption on site deployment aspects

[Moderator] Agreement from last RAN4 meeting:

- FFS the effect of clutter on achievable RSIC performance:

  - FFS the clutter impact on digital IC in RSIC performance if needed

  - FFS the deployment options to alleviate the cluster impact in the rooftop deployment:

    - e.g., the different gNB sectors may need to be installed on separate poles at the opposite corners:

- Proposals/observations:

  - Observation 1 (Intel): Nearby clutter can appear similar to self-interference leakage and also be treated by the interference cancellation algorithm.

  - Observation 2 (Intel): For interference cancellation, two effects can both degrade performance: 1) strong reflectors that require the cancellation algorithm to treat finite delays and 2) feedback delay that can occur when sectors are physically located at some distance apart.

  - Observation 3 (Qualcomm): For FR2, the measured 28/39GHz path loss between Tx and Rx antennas including clutter reflections is typically approximately 80 dB or better for empty conference room environment.

  - Observation 4 (Qualcomm): The residual self-interference including both direct leakage and clutter echo can be cancelled using non-linear digital cancellation algorithm where additional 10dB-20dB of residual self-interference cancellation could be achieved via digital cancellation.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Observation 1, 3 and 4 is ok to us |
| Nokia, Nokia Shanghai Bell | In the worst case, the clutter may contribute to the AGC related NF increase, and the delay makes an analogue cancellation of echoes very complicated. Digital cancellation of echoes can only work if the receivers are operating within their linear range. Hence it is suggested for feasibility evaluation purposes, that clutter limits the achievable RF cancellation/isolation performance. The Qualcomm reflection measurements are not useful for outdoor deployment scenario, and such performance would anyway be insufficient for devices with higher transmit power. Lacking further measurement data, RAN4 should clearly indicate in the TR that field measurements of outdoor base station clutter reflections have not been studied, and that the results could limit deployment options. |
| Qualcomm | @Nokia: For clutter impact, we have observed that the clutter is direction specific and thus proper selection of Tx and Rx beam pair can alleviate the clutter impact. We do not believe that clutter would have drastic limitations on the deployment options, but we can further study clutter impact in RAN4. |
| Ericsson | Generally agree that clutter will lead to reflections and interference and that digital interference cancellation can supress to some extent clutter reflections. However, for a large WA array this may come at the cost of significantly increased complexity. Would be good to see some more simulations/results on the IC gain. It may not be possible for the scheduler to always match TX and RX beams considering real traffic patterns. |
| ZTE | For observation 4, this is also aligned with our understanding. Indeed analog filter could be replacement if necessary. For observation 1, this is also fine for us. |
| Samsung | How and whether the clutter impact the SBFD performance is highly dependent on detailed clutter’s placement from gNB, materials and other parameters of the concerned clutter. One way to proceed is we can conclude FR2 first based on QC’s observation. If there is more study on FR1, we can of course capture that in TR, since it can be useful for us to understand more about the limitation on deployment for SBFD gNB. |
| Intel | We agree with all four observations. As proponent of 2, we should clarify, that 1) the finite delays in that complicate the cancellation algorithm are delays in the delay spread profile. 2) When sectors are located at some distance apart, the cabling delay between the transmitters or AAS can complicated the feedback portion of the algorithm. |

#### Issue 2-1-2: Impact of multi-carrier support at BS

[Moderator] Agreement from last RAN4 meeting:

- For the impact of SBFD on the multi-carrier BS:

  - FFS the effect of multi-carrier aspects on many related feasibility aspects such as improved linearization, CFR, filtering, PIM, beam nulling and digital interference cancellation

- Proposals/observations:

  - Observation 1 (Intel): If we assume that the far out noise is always less than the adjacent channel power, then having multiple Tx operating on multi-carrier will have only small impact on the total leaked signal power. (Moderator Note: common to both FR1 and FR2-1)

  - Proposal 1 (Intel): In the case of multi-carrier, we should add analysis to account for the impact of far out noise.  This will limit the effectiveness of potential digital IC since far out noise won’t be cancelled. (Moderator Note: common to both FR1 and FR2-1)

  - Proposal 2 (Qualcomm): To progress the feasibility and coexistence work in RAN4, it is proposed to focus on single carrier case and capture high level information on multi-carrier support in the TR.

  - Proposal 3 (Ericsson): Multi-carrier operation poses different feasibility aspects which need to be properly studied and documented during the duplex evolution SI as feasibility study on single carrier case does not cover the feasibility aspects for multi-carrier capable BS which is the normal mode of operation for BS.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Huawei | We support proposal 2 |
| CMCC | Proposal 2 is preferred. |
| Nokia, Nokia Shanghai Bell | Proposal 1: Agreed Proposal 2: Not agreed, please see below. Proposal 3: Agreed. Multi-carrier operation is basic operation mode for BS and a critical part of feasibility is to study this effect. |
| Qualcomm | We can have general statement on the challenges for Multi-carrier operation but we should focus the scope within Rel-18 on the single carrier case. |
| Ericsson | We should differentiate two types of multi-carrier operation: - One or more carriers configured for static TDD, and one single carrier configured for SBFD (from a gNB perspective) - Zero or more carriers configured for static TDD, two or more carriers configured for SBFD (from a gNB perspective) Case 1 is essential for a useful basestation; BS need to be able to transmit more than one carrier. Fortunately, although case 1 has some implications for SBFD that should be documented, generally it is in line with what we have studied for single carrier. Case 2, with more than 1 SBFD carrier is more complex. So we would propose a slight modification of Qualcomms proposal: Differentiate case 1 and case 2 for multi-carrier above. Describe qualitatively in the TR the implications of both cases and how they relate to what has been studied. |
| ZTE | We are also fine to focus on single carrier firstly, however we could also fully understand that multi-carrier is also one important direction worth consideration. From my understanding, case 1 might be similar as SBFD with large DL bandwidth or UL bandwidth indeed. |
| Samsung | P2 and the above suggestion from QC is okay for us. |
| Kumu | We support Proposal 2. |
| Intel | We support all three proposals |
| CATT | We support proposal 2. |

### Sub-topic 2-2: BS Aspects: Analysis on component capabilities

#### Issue 2-2-1: RF SIC

- Proposals/Observations from Kumu on RF SIC:

  - Observation 1 (Kumu): Using 3GPP_38.901_Uma_LOS channel, we show that having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. This is the case even when beam-nulling is also used in mitigating inter-sector interference. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability.

  - Proposal 1 (Kumu): RF cancellation should be used in SBFD to mitigate self-interference pre Rx LNA in terms of minimizing non-linearity effects and overall self-interference residue.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Proposal 1: RF cancellation is not seen feasible due the immense complexity in mMIMO WA BS. |
| Ericsson | We are not convinced of the feasibility of analogue IC for a WA BS (considering the number of interconnections, insertion loss etc.). However, it would obviously be useful (if it would actually be possible with reasonable complexity and insertion loss etc.) to really deal with the TX power falling into the RX sub-band and avoid saturation. |
| ZTE | Open for further discussion, however the implementation complexity and cost should be also taken into account. |
| Samsung | Based on Kumu’s input, the feasibility of RF cancellation is clearly shown, while we can discussion further on the complexity issue. We can of course identify and recognize the implementation complex for a certain MIMO and frequency/BW configuration, while we don’t agree with above Nokia’s conclusion to directly see RF cancellation infeasible without further scrutinizing the further analysis on complexity, which the proponent company of RF cancellation can further provide to justify the complexity issue we suggest. |
| Kumu | Complexity can be captured in further discussions. Kumu has already shown RF cancellation feasibility in previous contributions with hardware prototypes. We have also done analysis on the added circuit size for mMIMO, which can be included for discussion. The power consumption and circuit size impact of RF cancellation are quite small with integrated circuit implementation of RF cancelers. |
| Intel | We support Proposal 1, RF cancellation seems to be a needed technology to get workable RSIC budget for WA BS. Cancelling a portion of the interference energy before it saturates the LNA is important. |

#### Issue 2-2-2: Desense allocation between TX and RX (based on 1Db desense)

- Proposals:

  - CATT: Desense allocation between TX and RX (based on 1Db desense)

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | We don’t understand the purpose of this issue. Detailed desense allocation is related to implementation and maybe it’s hard to conclude one principle. |
| Qualcomm | It is not clear how this proposal would be considered in the feasibility framework. |
| Ericsson | The RSIC tables capture the desense considering both TX and RX. It does not really seem necessary to agree on some allocation to each part if companies show whether the total desense is 1dB or not. We should bear in mind that the 1dB in the RSIC tables is only for self-interference, so other sources of interference will lead to further desensitization. In real life, if further desensitization cannot be tolerated then equipment would need to be designed to have less than 1dB desense from self-interference alone. However, we propose companies just report the additional desense they see from inter-sector etc., not try to apportion between different interference sources. |
| ZTE |  |
| Samsung | In our understanding, to equally allocating desens allocation may not always be correct since these two factors usually not equally influencing. More importantly, the digital IC afterwards can further tackle the residual interference, which is not considered in CATT’s analysis. We suggest the group shall still stick to the original WF which contains the analysis framework, based on which companies already provide the technical inputs. |
| Kumu | Agree with other companies – allocating desense is hard given every company’s values are implementation specific. |
| Intel | We are not sure this is something that is agreeable given the variety of implementation approaches and views on different parameters. |
| CATT | We didn’t intend to make this as a proposal, we used this separation to do the analysis, and find that it could be assumed to separate the burden. And we totally agree that not all of the implementations have the same separation. But we think both Tx contribution and Rx contribution should be analyzed in the feasibility study. |

### Sub-topic 2-3: Remaining issues for RSI dependency on blocking, AGC and ADC

#### Issue 2-3-1: Assumption for input power metric to LNA

- Proposals:

  - Observation 1 (Intel): since the maximum expected input from RSIC and self-interference leakage is on the order of ~-40dBm, the Rx will always be operating in a weak non-linear region where RMS power is the best way to measure signal power.

  - Proposal 1 (Intel): We should agree to model the Gnb Rx for LNA and AGC with Option 1, RMS power of the input signal.

  - Proposal 2 (Nokia): Gnb receiver saturation, non-linearity, and AGC model is based on RMS power of the input signal.

  - Proposal 3 (Qualcomm): For FR1 BS, other distortions such as ADC quantization noise and distortions were considered in our simulation and measurements, and it was observed that ADC performance is not limiting. Similarly, phase noise and residual sideband are not significant contributors.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Huawei | Agree with proposal 3 and agree that RMS power is used in the model. |
| Nokia, Nokia Shanghai Bell | Proposal 1: Similar as Proposal 2 and therefore agreed. |
| Qualcomm | Proposal 3 from our side was based on our simulation and measurements as stated and looking into the provided cancellation numbers in the feasibility framework from different companies, we observe that several RAN4 had made similar observations on the impact of AGC and ADC quantization. |
| Ericsson | We agree with RMS power. We have also observed that ADC and phase noise are not significant as long as the RX power is below the blocking level; as captured in RSIC proposals. For the RX power, for FR1 WA we see somewhat higher power than -40dBm. |
| ZTE | Similar view as Huawei on proposal 3. |
| Samsung | RMS power used in the model is okay for us. For P3, we already have the following agreement from last meeting: Agreement Analysis on other distortions (Phase noise, ADC quantization noise, Residual sideband, ADC distortions) - For FR1 BS, other distortions such as ADC quantization noise and distortions were considered in our simulation and measurements, and it was observed that ADC performance is not limiting. Similarly, phase noise and residual sideband are not significant contributors. |
| Kumu | RMS power in the model is fine with us. Agree with Proposal 3. |
| Intel | Proposal 1 and 2 are similar, we agree. We also agree with Proposal 3. |
| CATT | We’re not sure we should discuss this in detail. |

#### Issue 2-3-2: Impact on RSI from analogue sub-band filter

- Proposals related to sub-band filtering:

  - Observation 1 (Huawei): Analogue sub-band filter after LNA can provide the needed suppression for the receiver parts after the filter, and the impacts to RX sensitivity due to insertion loss is negligible.

  - Proposal 2 (Huawei): if analogue sub-band filter is adopted in the solution, the blocking performance can be improved at least 10 dB. The following values of A and B can apply to the agreed noise figure model.

  - Observation 1 (Ericsson): The use of filters in the early statges of the RF chain to improve IIP3 would cause a large increase in power consumption, size and weight and a decrease in the level of integration possible. Also the filtering would need to be tuned to a specific carrier and so would need to be operator specific and hardware inflexible.

- Proposals on Noise figure model update (related to Issue 1-5-1):

  - Option 1 (Huawei): If analogue sub-band filter is adopted in the SBFD solution for FR1 macro BS, the blocking performance can be improved ~ 10 dB. The following values of A and B can apply to the agreed noise figure model.

    - The values of A, B, C and D:

A = -33dBm

B = -15dBm

C = 5dB

D = 14dB

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

  - Related to issue 1-5-1, and company shall just provide views in issue 1-5-1.

| Company | Comments |
| --- | --- |
| CMCC | We support to evaluate two sets of value/modeling, one without analog filtering and the other with analog filtering. |
| Qualcomm | We are ok with Huawei proposal to consider typical values not minimum requirements. Our preference to omit the subband filtering aspect and keep it implementation agnostic |
| Ericsson | Although the impact of filters can be speculated, it is not really realistic to consider it for commercial BS. It would potentially need filters tuned to a specific carrier, inflexibility for allocating carriers, de-integration of AAS RX chains, space challenges, increased power and thermal management issues etc. So in our view to assess the operation of a realistic deployment we should not consider filtering around the LNA. (Filtering just before the ADC an be used to protect the ADC if needed and is not problematic, but does not impact RX linearity). |
| ZTE | Okay to consider the analog filter which has been used by WA BS self interference in some contributions. |
| Samsung | We are also okay to include analog filtering which can be used for WA BS. |
| Kumu | Agree with Qualcomm on keeping it implementation agnostic |
| Intel | We agree to support Huawei proposal for analog sub-band filtering as another option |
| CATT | For the analog sub-band filter, agree with Ericsson, it may be an expensive component. And if the performance of the analog sub-band filter is good, the IL may be large. So may have impact to the NF. |

#### Issue 2-3-3: Third-order intermodulation (IM3)

- Proposals:

  - Proposal 1 (Qualcomm): FR1 BS IIP3 model: For FR1 BS IM3 model, RAN4 to adopt a Pin-dependent (average total input power) piecewise linear model as shown in the Figure above to characterizes IIP3. The proposed model captures IM3 contributions and AGC impact on IIP3 which can be utilized by RAN4 to progress the SBFD feasibility work.

Figure: BS IIP3

  - Observation 1 (Qualcomm): FR1 BS IM3: The IM3 contributions are not significant when the total input power signal + jammer is lower than -52 dBm.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | In our view, the BS IM3 is already covered blocking noise figure model. |
| Nokia, Nokia Shanghai Bell | Proposal 1: IIP3 model should not be needed because the NF model already includes the contribution of non-linearities. |
| Qualcomm | Our view that this model provides a relationship between the total input power and the IIP3, while the blocking model considers the relationship between the NF and the total input power. The intention is not to overwrite the already agreed blocking model but to complement it. That being said, we are ok with adopting the blocking model. |
| Ericsson | The figure should not be considered in isolation; if AGC is used to improve IP3 then it will also increase the receiver noise figure and degrade the sensitivity. The sensitivity degradation then needs to be considered in addition. The blocking model accounts for both effects. |
| ZTE | We are also fine with current blocking models. |
| Samsung | We also think the IIP3 model (depending on Pin for total input power) is not necessary and hard to be aligned, but the current agreed analysis framework still open the door for company to study IM3 by adopting more complex IIP3 model rather than a single IIP3 performance value irrelevant to Pin, and company can just provide the input. Based on the analysis framework, the impact of IM3 can also be analyzed and to be captured in TR. |
| Intel | Our view is that we already incorporated the IIP3 effect into the NF model |

### Sub-topic 2-4: RSIC capability overall feasibility analysis

#### Issue 2-4-1: RSIC capability analysis table from companies

- Proposals from Intel:

- Table 1 – Residual Self-Interference Cancellation (RSIC) Analysis Table

| FR-1 |  |  |  | Intel |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Medium Range BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm | 38 dBm | 24 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | Digital SB Filtering and DPD | Digital SB Filtering and DPD | Digital SB Filtering and DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 70 dBc | 60 dBc | 55 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and EM shielding structure | TX/RX panel separation and EM shielding structure | TX/RX panel separation and EM shielding structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc | 10 dBc | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, less 0.5dB | Limited, less 0.5dB | Limited, less 0.5dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -76dBm =①-②-③-④ | -77dBm =①-②-③-④ | -86dBm =①-②-③-④ |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 11 dB | 11 dB | 0 dB |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 10 dB | 10 dB | 0 dB |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |  | No RF IC required |
|  |  | Impacts to RX sensitivity (due to e.g., insertion losses) due to RF IC or other techniques before LNA |  | 0.2 dB | 0.2 dB | 0 dB |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -42 dBm =①-③-④-⑤ | -43 dBm =①-③-④-⑤ | -41 dBm =①-③-④-⑤ |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 60 dB | 60 dB | 60 dB |
|  |  | Frequency isolation techniques |  | Digital filtering after 12bit ADC | Digital filtering after 12bit ADC | Digital filtering after 12bit ADC |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -10 dBm | -10 dBm | -10 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -106 dBm | -109 dBm | -103 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g., ADC noise, phase noise etc.) | +0.5dB NF from SI Pin of TX SB at LNA | 0dB | +1dB NF from SI Pin of TX SB at LNA |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | =-103 | =-104.9 | =-98.9 |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | 5 dBc | 5 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | Limited, ~0dB | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc | 15 dBc | 15 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 151.9dB ①-total distortion in Tx and Rx SB | 142.8dB ①-total distortion in Tx and Rx SB | 122.8dB ①-total distortion in Tx and Rx SB |
| Noise floor ⑩dBm |  |  |  | -95.5 dBm/ 20MHz CBW | -91 dBm/ 20MHz CBW | -87 dBm/ 20MHz CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -101.5 dBm | -97 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 150.5 dBc | 135 dBc | 117 dBc |
| SBFD configuration |  |  |  | DUD | DUD | DUD |
| Guardband assumption (if exist) |  |  |  | 5 PRBs | 5 PRBs | 5 PRBs |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | 100MHz | 100MHz |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

- From CATT on LA BS

  - Observation 1 (CATT): From the analysis in table, the following capabilities are needed,

    - overall RSIC for Tx leakage capability is 118dB

    - overall RSIC for blocking capability is 70dB

    - ACS is 55dBc

    - IIP3 is -16dBm

| Item |  | Parameters and calculations | example |
| --- | --- | --- | --- |
| General assumption |  | BS total Tx power @ DL SB (dBm) ① | 24 |
|  |  | SBFD configuration | DUD |
|  |  | UL subband width (MHz) ② | 20 |
|  |  | DL subband width (MHz) ③ (Note1) | 40 |
|  |  | Guard band assumption |  |
|  |  | BS NF (dB) ④ | 13 |
| Tx leakage contribution |  | Tx Frequency isolation capability (dB) ⑤ | 45 |
|  |  | DL subband RF filter suppression capability (dB) ⑥ | 0 |
|  |  | Antenna isolation capability (dB) ⑦ | 70 |
|  |  | Tx Beam nulling/isolation capability (dB) ⑧ | NA |
|  |  | DL EIRP impact due to beam nulling in TX sub-band | —— |
|  |  | Digital IC for Tx leakage capability (dB) ⑨ | 3 |
|  |  | Overall RSIC for Tx leakage (dB) ⑩ |  |
|  |  | ⑩=⑤+⑥+⑦+⑨ | 118 |
|  |  | Tx leakage contribution to SI (dBm/UL subband) ⑪ |  |
|  |  | ⑪=①-10*log10(③②)-⑩ | -97.01 |
| Rx path noise contribution | Blocking analysis | Rx beam nulling/isolation capability (dB) ⑫ | NA |
|  |  | RX sensitivity degradation caused by RX beam nulling | —— |
|  |  | UL subband RF filter suppression capability (dB) ⑬ | 0 |
|  |  | Overall RSIC for blocking capability (dB) ⑭ |  |
|  |  | ⑭=⑦+⑬ | 70 |
|  |  | Tx SB interference signal level at Rx path (dBm) ⑮ |  |
|  |  | ⑮=①-⑭ | -46 |
|  | Rx IMD | Rx IIP3 capability (dBm) ⑯ | -16 |
|  |  | Rx IM3 contribution (dBm) ⑰ |  |
|  |  | ⑰=⑮-2*(⑯-⑮) | -106 |
|  | Rx ACS | ACS capability (dB) ⑱ | 55 |
|  |  | Rx ACS contribution (dBm) ⑲ |  |
|  |  | ⑲=⑮-⑱ | -101 |
|  |  | Any other RX impacts if significant (dBm) ⑳(Note 2) | -127 |
|  | Rx path overall noise contribution (dBm) ㉑ |  |  |
|  | ㉑=10*log10(10⑰10+10⑲10+10⑳10) |  | -99.80 |
| Total SI contribution (dBm) ㉒ |  |  |  |
| ㉒=10*log10(10⑪10+10㉑10) |  |  | -95.17 |
| Noise floor (dBm/UL subband) ㉓ |  |  |  |
| ㉓=-174+10*log10②*106+④ |  |  | -87.99 |
| Overall REFSENS degradation (dB) ㉔ |  |  |  |
| ㉔=10*log1010㉒10+10㉓10-㉓ |  |  | 0.76 |
| Note 1：③ is single DL subband width |  |  |  |
| Note 2: If ⑳ is very lower than ⑰ and ⑲, the value of it inserted in the table must be at least 10dB lower than ⑰ and ⑲ |  |  |  |

- From Huawei on MR and LA BS:

Table 2.3-1: RSIC Analysis for FR1 MR BS and LA BS

| FR1 |  |  |  |
| --- | --- | --- | --- |
| BS class |  |  |  | Medium Range BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 | 24 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 | 45 |
|  |  | Frequency isolation techniques used |  | DPD | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 | 70 |
|  |  | Spatial isolation techniques used |  | spatial separation between TX/RX panel | spatial separation between TX panel to single RX |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 | N/A |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited | N/A |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -98 | -97 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -47 | -46 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | digital filtering | digital filtering |
|  |  | Frequency isolation techniques |  | digital filtering | digital filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -5 | -5 |
|  |  |  | Rx IM3 contribution (dBm) | -131 | -128 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | negligible | negligible |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | negligible | negligible |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 | N/A |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited | N/A |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 | N/A |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 146 | 121 |
| Noise floor ⑩dBm |  |  |  | -90 dBm/20 MHz | -87 dBm/20 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -96 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 134 | 117 |
| SBFD configuration |  |  |  | [40, 20, 40] | [40, 20, 40] |
| Guardband assumption (if exist) |  |  |  | Existing SU | Existing SU |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |

- From Kumu on WA BS

| FR1 (or FR2-1) |  |  | Kumu Networks, Inc. |  |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (High) | Wide Area BS (Mean) | - |
| BS TX Power dBPTX = ① dBm |  |  | 53 dBm | 53 |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc | 45 dB | 45 |  |
|  |  | Frequency isolation techniques used | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc | 65 dB | 65 |  |
|  |  | Spatial isolation techniques used | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  | 14.39 dB | 16.98 |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. (Note 1) dBPSI_leakage |  | -57 dBm | -57 dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc | 26.44 dB | 27.95 |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc | 29.89 dB | 30.66 |  |
|  |  | RF IC techniques and other tech. (before LNA) | Analog RF IC joint tune with beam-nulling RF SIC Complexity : 192 RF taps for 32 Tx and 32 Rx antennas |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA | 0.2 dB (assuming 15 dB Rx coupler) | 0.2 |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) ①-③-④-⑤ dBPSI_TXSB |  | <-52.83 dBm | -56.93 |  |
|  | Frequency isolation at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc | 0 dB | 0 |  |
|  |  | [Frequency isolation] techniques | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |  |
|  | Rx IMD | Rx IIP3 capability (dBm) | -20 | -20 |  |
|  |  | Rx IM3 contribution (dBm) | -118.49 | -130.79 |  |
|  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | 5 dB (noise figure) | 5 |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized (Note 1, 2) dBPSI_Selectivity |  | -118.49 dBm | -130.79 |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  | 11.75 dB | 16.08 |  |
|  | RX sensitivity degradation caused by RX beam nulling |  | TBD (?) | TBD |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  | 0 dB | 0 |  |
| Overall RSIC capability dBRSIC (Note 1) ②+③+⑧+⑨ |  |  | 151.64 dB | 156.64 |  |
| Noise floor ⑩dBm |  |  | -89 dBm/100MHz | -89 dBm/100MHz |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  | -95 dBm | -95 |  |
| Required RSIC budget (①-⑪dBc) |  |  | 148 dB | 148 |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

- Proposals form Samsung:

- Table 1: FR1 RSIC budget calculation Summary

| FR1 |  |  |  | Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Medium Range BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm | 38 dBm | 24 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD utilized | DPD utilized |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 80 dBc | 80 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | TX/RX panel separation and RF barrier structure | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc | 5 dBc | 5 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | Limited, ~0dB | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -81 dBm Note: provided by ①-②-③-⑨ dBm | -92 dBm Note: provided by ①-②-③-⑨ dBm | -106 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 25 dBc | 25 dBc | 25 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | 0dBc | 0dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering or RF interference cancellation | subband filtering or RF interference cancellation | subband filtering or RF interference cancellation |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | TBA | TBA | TBA |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -61 dBm Note: provided by ①-③-④-⑤dBm | -72 dBm Note: provided by ①-③-④-⑤dBm | -86 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dBc | 25 dBc | 20 dBc |
|  |  | Frequency isolation techniques |  | Filtering | Filtering | N/A |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm | -20dBm | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -143dBm | -176dBm | -218dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | N/A | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -101 dBm Note: provided by ①-③-④-⑤-⑥dBm | -97 dBm Note: provided by ①-③-④-⑤-⑥dBm | -106 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | 5 dBc | 5 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | Limited, ~0dB | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 20 dBc | 20 dBc | 20 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 150.0 dBc | 148.8 dBc | 147.0 dBc |
| Noise floor ⑩dBm |  |  |  | -95dBm/20MHz | -90dBm/20MHz | -87dBm/20MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -101 dBm | -96 dBm | -93dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 150 dBc | 134 dBc | 117 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | DUD(40-20-40MHz) | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB | 5 PRB | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 20MHz | 20MHz |
| Others |  |  |  |  |  |  |

- Proposals from Ericsson (R4-2304544)

| FR1 |  |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |  |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 53 dBm |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |  |  |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 70 dBc |  |  |
|  |  | Spatial isolation techniques used |  | A combination of spatial isolation, chokes, absorption, mushroom EBG. 80dB can be achieved over a bandwidth of 100-200MHz and for certain beam directions. However, since the isolation needs to be achieved across the whole band and considering that depending on beam direction the isolation in fact varies from around 55dB to more than 80dB, 70dB is adopted as an average antenna isolation over a reasonable bandwidth. |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Varies up to a maximum 3-5dB EIRP loss in DL, depending on TX beam direction |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -72 dBm |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc |  |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc |  |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier operation. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -27 dBm |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  |  |
|  |  | Frequency isolation techniques |  | The RX input level is -27 dBm, and hence the receiver is in high non-linearity; no possibility for interference mitigation as part of the digital receive combining algorithms. Increasing IIP3 towards levels above +5dBm would necessitate designs with low levels of integration, increased energy consumption, thermal issues, size etc. |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS) |  |  |
|  |  |  | Rx IM3 contribution (dBm) | Even without ADC overload: -17 dBm (RAN4 minimum receiver) -37 dBm (Realistic) -61 dBm (Optimistic) |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | ADC could be overloaded; this can be mitigated with filtering prior to ADC except for direct conversion architectures. |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | Receiver in high non-linearity |  |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | RX processing does not mitigate analogue non-linearity |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Receiver saturated |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Digital IC not possible due to receiver non-linearity and would anyhow be highly complex due to large number of TX/RX for wide area. |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | Transmitter: 125 dB Receiver: N/A due to receiver saturation |  |  |
| Noise floor ⑩dBm |  |  |  | -96 dBm/CBW |  |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 155 dBc |  |  |
| SBFD configuration |  |  |  | 40-20-40 MHz |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB. |  |  |
| bandwidth over which suppression is achieved |  |  |  | >300 MHz |  |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

| FR1 |  |  |  | Ericsson |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 dBm | 38 dBm | 38 dBm | 35 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 65-70 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc |
|  |  | Spatial isolation techniques used |  | A combination of spatial isolation, chokes, absorption, mushroom EBG. 80dB can be achieved over a bandwidth of 100-200MHz and for certain beam directions. However, since the isolation needs to be achieved across the whole band and considering that depending on beam direction the isolation in fact varies from around 55dB to more than 80dB, 65-70dB is adopted as an average antenna isolation over a reasonable bandwidth. |  |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Varies up to a maximum 3-5dB EIRP loss in DL, depending on TX beam direction |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -87 dBm | -87 dBm | - 87 dBm | -90 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier operation. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -42 dBm | -42 dBm | -42 dBm | -45 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  |  |  |
|  |  | Frequency isolation techniques |  | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. For RX, the 3rd column represents improved receiver linearity in the analogue domain. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. It is assumed that inter-sub band RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -27.6 dBm | -17.6 dBm | -13 dBm | -17.6 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No significant issues for medium range BS power level other than mentioned above. Phase noise reciprocal mixing is not significant for this frequency range and power levels. |  |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10-15 dBc (Transmitter) |  |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 109 dBc | 128 dBc | 135 dBc | 134 dBc |
| Noise floor ⑩dBm |  |  |  | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -96 dBm | -96 dBm | -96 dBm | -96 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 134 dBc | 134 dBc | 134 dBc | 131 dBc |
| SBFD configuration |  |  |  | 40-20-40 |  |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |  |  |  |
| Others |  |  |  | The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed). |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |  |

| FR1 |  |  |  | Ericsson (preliminary) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Local Area BS (3GPP minimum) | Local Area BS (Realistic RX) |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 24 dBm | 24 dBm |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc |  |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 70 dBc | 70 dBc |  |
|  |  | Spatial isolation techniques used |  | Physical distance, isolation structures |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 dBc | 0 dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | TX beam nulling not assumed due to array size |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 dBm | -91 dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | 0 dBc |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | 0 dBc |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed. |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc | 0 dBc |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -46 dBm | -46 dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  |  |
|  |  | Frequency isolation techniques |  | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -24.6 dBm | -14 dBm |  |
|  |  |  | Rx IM3 contribution (dBm) | -88.8 dBm | -110 dBm |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No other significant impacts other than those mentioned above |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -88.8 dBm | -110 dBm |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc on transmitter |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 112 dBc | 124 dBc |  |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW | -87 dBm/CBW |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm | -93 dBm |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117 dBc | 117 dBc |  |
| SBFD configuration |  |  |  | 40-20-40, see annex |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB, see annex |  |  |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |  |  |
| Others |  |  |  | The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed). |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | FR1 WA BS TX power is 53dBm. All the different isolation techniques have serious challenges as explained in R4-2304540. The drawbacks of each selected solution are not described in the table, which leads to false interpretation of the feasibility of each technique. We agree with the tables provided by Ericsson. Alternatively, to the Ericsson’s table, or in addition to it, we propose to capture the explanations and drawbacks into separate clauses in the TR, as suggested in our contribution. |
| Ericsson | One thing we could discuss is whether to capture the reasons for the differences in the analysis, where differences exist. For example, some companies assume RX filter, some analogue IC etc. We may not agree on whether they should be considered or not, but it could help to at least start out by understanding where the differences arise. As well as RSIC tables we could have a summary table of different things that impact the analysis where each company captures their comment why/why not to consider it. |
| Samsung | Our understanding is based on different assumptions for adopting certain techniques and the different interpretation of the effort/cost to achieve the benefits of SBFD operation, it is hard to justify or preclude a certain technique to be utilized. From that perspective, our understanding is we can add subsection to allow companies’ technical input for at least WA BS. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc. |
| Kumu | Agree with Samsung. companies should be allowed to use techniques (including innovative ones) that help with SBFD feasibility. Complexity tradeoffs of techniques may be discussed. Response to Nokia regarding R4-2304540: The TDoc mentions TRX^2 complexity for RF cancellation, but we have shown in multiple previous contributions (R4-2218051, R4-2300061 etc) that circuit complexity of TX+RX can achieve effective RF cancellation. |
| Intel | We agree with Samsung that different companies make different assumptions that ripple through their entire RSIC budget. It would be difficult to eliminate any one cancellation method and have meaningful numbers. Perhaps could work on a combined table with the assumptions everyone can agree on, but that might lead to an overly pessimistic answer. We can’t see there being an ideal way to agree |
| CATT | More discussion is needed on how to capture all of the results, should we align the calculation again? |

#### Issue 2-4-2: Remaining issues for RSIC analysis framework

- Proposals:

  - Proposal 1 (Huawei): It is proposed that the scalling factor should be used in the budget calculation for the asymmetrical SBFD DL/UL configurations.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Qualcomm | Agree with Proposal 1, this also aligns with the modeling we carry out in the adjacent coexistence study. |
| Ericsson | Is this proposal for ACLR ? |
| ZTE | Fine with ACLR, how about ACS, it seems this information is missing in some contribution, not sure the reasons. |
| Samsung | Based on the current analysis table, company can provide the SBFD subband configuration which is used as the assumption/basis to derive other values. |

#### Issue 2-4-3: Excel spreadsheet for RSIC capability analysis

- Proposals:

  - ZTE proposed an excel spreadsheet for RSIC capability analysis, contained in R4-2305400.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Proposal 1: What is the benefit of separate spreadsheet? |
| Qualcomm | Similar question as Nokia. It is not clear to us the added value from having additional spreadsheet compared to the already provided tables. |
| Ericsson | The current spreadsheet does not seem to take into account IP3 etc. We agree with Qualcomm and Nokia it is not needed to develop a spreadsheet, the tables can be provided. |
| ZTE | For some tables proposed, it’s difficult to figure out how to get the values in each row, that’s the reason to add the spread sheet here. |
| Samsung | Given the discussion in Feb meeting, we doubt the discussion on excel spreadsheet to have an aligned calculation is hard to be achieved. |
| Intel | While we like the idea of trying to find better alignment between companies, we feel the spreadsheet will be considerable work but may not have that much value. We prefer to see the table included in the TR. |

#### Issue 2-4-4: Conclusion from RSIC analysis

- Proposals:

  - Proposal 1 (Qualcomm/Kumu/Samsung): RAN4 to confirm the adoption of the impact of self-interference modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB) to study the impact of SBFD operation on the RF requirements.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | We also support proposal 1. Besides, such self-interference modeling could still be used in RAN1. |
| Nokia, Nokia Shanghai Bell | Proposal 1 can be agreed for the coexistence study, but it should not be viewed as an agreement that cancellation of self-interference is feasible to the indicated level. |
| Qualcomm | We should at least agree on the framework and try to further discuss the value of X within the feasibility track. |
| Ericsson | This should be discussed in the thread on co-existence simulations, not this thread. An alternative is for companies to take the isolation they propose and use that. When deriving RF requirements, it is important to use accurate assumptions. |
| Samsung | We agree with P1. |
| Kumu | Agree with Proposal 1 |
| CATT | OK for the co-existence simulation calibration. |

### Sub-topic 2-5: Co-channel co-site gNB-gNB CLI model

#### Issue 2-5-1: Remaining issues for inter-sector interference analysis framework

- Proposals/Observations on desense:

  - Proposal 1 (Samsung): For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors.

      - There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study.

  - Observation 1 (Ericsson): A “1dB criterion” for inter-sector interference would be on top of the self-interference.

  - Proposal 2 (Ericsson): The impact of (i) self-interference only and of (ii) combined self-interference+inter-sector interference should be evaluated. The impact of inter-sector interference could be interpreted as (ii) – (i).

- Proposals/Observations on analysis framework table:

  - Proposal 3 (Ericsson): For the proposed inter-sector table, it may discussed whether the term “RSIC” is the right one.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Proposal 1 is OK for us. there is no need for inter-sector interference criteria. Regarding proposal 2, in co-existence simulation we simulate (ii), the sum of self-interference and inter-sector interference. Don’t know the purpose why we need this difference of (ii)-(i), instead, the sum of self and inter-sector interference is the key. |
| Nokia, Nokia Shanghai Bell | Proposal 1: To limit the UL performance degradation to minimum due interference in SBFD, total sensitivity degradation should be maximum of 1dB. This should include all interference i.e. also inter-sector/inter-site interference and not just self-interference. Proposal 2: Agreed Proposal 3: Agreed, RSIC is most likely not the right term here. |
| Qualcomm | We support proposal 1 as it provides a good methodology to analyze the different interference components within the feasibility study. For proposal 2, the separation is not clear for us. Having an aggregate degradation metric captured via X should be sufficient to consider the different interference components. |
| Ericsson | We are fine with the proposal to report the additional desense from inter-sector interference. Proposal 1 seems to be similar to our proposal 2, the difference being whether we consider desense compared to the noise floor from inter-sector only or all sources. We could report both. As Nokia indicate, it is actually total degradation that is important. |
| ZTE | For the proposal 3 from Ericsson, this has already been used in the coexistence study, not sure why we need to discuss again here. |
| Samsung | As proponent, we support P1. For P2, if the intention is just to clarify the desense XdB, we can translate that into YdB relative to RX noise floor. Of course the interference impact shall be considered by considering the sum of all interference. Specifically, the interference contains: - UL interference from UE in neighboring cells (which exists for non-SBFD system) - Self-interference from SBFD DL sub-band (new for SBFD gNB receiver) - Inter-sector co-channel interference from co-site other sectors’ SBFD DL sub-band (new for SBFD gNB receiver) - gNB-gNB (inter-site) co-channel interference from other gNBs’ SBFD DL sub-band (new for SBFD gNB receiver) - adj-channel from other gNB’s adjacent channel DL transmission Compared with noise floor – 6dB, some of items may be higher (still depending on the spatial isolation for each case), but it should be noted that the interference condition shall also include (1), and the impact on top of that shall be considered, rather than directly compared with noise floor – 6dB. We believe a certain criteria is hard to be agreed, so P1 is provided for WF. |
| Kumu | We support Proposal 1 |
| Intel | We agree with proposal 1 |
| CATT | Ok with proposal 1. |

#### Issue 2-5-2: Beam nulling considered for inter-sector interference analysis

- Proposals/Observations:

  - Observation 1 (Kumu): Considering mitigating inter-sector interference using beam-nulling , we show that the mean case rx sensitivity degradation is 0.6 dB. The degree of freedom remaining after self-interference cancellation (using beam-nulling and RF cancellation) provides up to 10 dB additional isolation when beam-nulling of inter-sector interference cancellation is applied.

  - Proposal 1 (Kumu): Inter-sector interference reporting should consider beam-nulling based interference mitigation for feasibility analysis.

  - Observation 2 (Samsung): Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR1 BS implementation.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Thank Kumu for showing more information about the feasibility of beam-nulling of inter-sector interference cancellation. For observation 2, one question, main beam is the worst case and the dominant contribution for inter-sector interference. If so, do we still need to consider sidelobe case? |
| Nokia, Nokia Shanghai Bell | Proposal 1: TX beam-nulling will have effect in EIRP, which diminishes the gain here. RX beam-nulling is not seen as feasible as the UL performance cannot be compromised any more. |
| Qualcomm | We had similar views as observation 2. Additional 10-15 dB cancellation can be achieved from beam nulling. |
| Ericsson | Question to Kumu… do you think the same isolation between sectors would be achieved without RF IC for the self-interference ? i.e., is the inter-sector beam nulling enabled by being able to do RF IC to gain degrees of freedom for the beamforming ? Of course, the impact of nulling on DL EIRP should also be considered. |
| Samsung | We want to clarify the analysis framework example (from Samsung’s paper) which is given in last meeting’s WF led by Ericsson. When evaluating “spatial isolation”, we just study the antenna element to element S12 parameter which don’t include the benefits of beamforming into account. In the self-interference table, we believe the “beam nulling” means gNB can actively beam steering the TX and RX to avoid conflicting, but for inter-sector case, active beam nulling is hard to achieved. However, even without active beam nulling, considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe. This impact from beamforming shall be included, but it can be calculated as part of “spatial isolation” in our understanding. |
| Kumu | Answer to Ericsson: In our simulations, we saw inter-sector beamforming give 10dB improvement without RFIC, when we ignore self-interference. With self-interference optimization included, just beamnulling is not enough for self-interference itself, so it obviously doesn’t work when trying to optimize both self-interference and inter-sector interference. Adding RFIC for self-interference makes the two objectives (self-nulling and inter-sector nulling) achievable simultaneously. Agree with Qualcomm – 10-15dB beam-nulling is achievable for inter-section interference mitigation. |
| Intel | We agree with Proposal 1 and both observations |

#### Issue 2-5-3: Spatial Isolation

- Proposals/Observations:

  - Proposal 1 (Qualcomm): For co-channel co-site inter-sector inter-gNB CLI, at least similar or improved spatial isolation compared to the self-interference framework (e.g., by means of additional electromagnetic absorbers between the different sectors or radiation mask) should be considered to provide sufficient inter-gNB CLI mitigation.

  - Proposal 1a (Samsung): RAN4 shall confirm RAN1 that additional spatial isolation can be achieved for co-site inter-sector co-channel CLI, with typical value at least 35dB.

  - Observation 1 (Samsung): Installing EM conjugated structure between sectors can provide additional inter-sector spatial isolation at the level of 25dB.

  - Observation 2 (Samsung): There are many other deployment-relevant methods to further improve inter-sector antenna isolation, including:

      - Larger horizontal distance between co-site sectors;

      - Vertical antenna arrangement for co-site sectors;

      - Different boresight angle directions, or different electrical tilts or combination thereof for co-site sectors.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Proposal 1: Use of absorbers may not always be feasible in the field due to weather conditions and wind load. Also, they add to the site costs. Further, compared to a single antenna enclosure, it may not be possible to precisely adjust the spacing of these added materials. If RAN4 is lacking measurement data, it should be clearly stated in the TR that inter-sector spatial isolation is not fully evaluated, and may be limiting the deployment possibilities. Proposal 1a: In TR 38.828, the isolation between co-located base stations (zero grid shift) is 30 dB in FR1 and 50-70 dB in FR2. Therefore, we see that the assumptions made for SBFD are too high. |
| Qualcomm | It is indeed important to have measurements within RAN4 if possible, however, several companies in RAN4 have shown the feasibility of having sufficient spatial isolation for the co-site inter-sector case. It is difficult to compare to findings in 38.828 since no additional consideration for spatial isolation was considered. We share similar views as observation 2. |
| Ericsson | In real deployments, it is not clear how isolation / conjugated structures etc. would be built considering site constraints and other requirements (weight, weather, wind load etc.). It would be good to see some concrete proposals and simulations / measurements on how sites are arranged and give rise to such isolation. Larger horizontal and vertical distance can help a bit, although again this is not feasible for sites that are crowded, compact, restricted etc. If antennas pointing is changed to mitigate inter-sector interference then the pointing direction an no longer be optimized for deployment performance, so the cost to system performance should be analyzed. Based on current analysis, we do not see a basis for RAN4 as a group to endorse some number on isolation from these techniques. To enable some endorsement, it would be good to review some more practical details and simulations/measurements on how it would be achieved. Just quoting a dB number is not really a study of feasibility. |
| Samsung | We support the proposals and observations here. For 38.828’s conclusion, at least the assumption of separate TX and RX panel is not considered so we don’t think the conclusion can be directly applied to SBFD gNB. As we elaborate more in the Proposal and Observation, installing EM conjugated structure and other deployment-relevant methods are the ways to further improve inter-sector isolation, which is obviously not considered in 38.838 either. |
| Intel | We would like to see further discussion and explanation on the feasibility of the ideas in Proposal 1 and Proposal 1a. |
| CATT | Agree with Proposal 1 and Observation 2. |

#### Issue 2-5-4: Digital IC

- Proposals/Observations:

  - Option 1 (Samsung): Digital IC can be applied for co-site inter-sector case.

  - Option 2: No digital IC can be applied for co-site inter-sector case.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 2: it can be treated the same way as self-interference. |
| Nokia, Nokia Shanghai Bell | Agreed with option 2. Digital IC is not feasible here. |
| Ericsson | For WA BS, digital IC would have a very large complexity. Not sure if we need to decide this or it can be added to the suggested listing of reasons why different companies RSIC proposals differ. |
| ZTE | Agree with option 1 |
| Samsung | Similar to self-interference analysis framework, we believe the inter-sector analysis shall still be company input-driven, since a certain technique (e.g., digital IC here) used or not used shall be dependent on company’s input and their own choice. |
| Kumu | Agree with Samsung. Different companies should report results based on their choice of techniques used. |
| Intel | We agree with option 1. Different companies can discuss their different views on what value is feasible |
| CATT | Digital IC may be challenge for WA. |

#### Issue 2-5-5: Inter-sector CLI suppression analysis table from companies

- Proposals from Intel:

  - Observation 1: the inter-sector interference is worse than the self-interference due to the lack of IC from Tx beam nulling.  Methods to further improve the IC may be needed.

  - Observation 2: For MR BS and LA BS, where there is a more relaxed NF, it is easier to meet the required interference cancellation even though MR BS, and LA BS have worse isolation than WA BS.

| - FR-1 |  |  |  | Intel |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Medium Range BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm | 38 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  |  |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 77 dBc | 67 dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Spatial separation and EM shielding structure | Spatial separation and EM shielding structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -73dBm =①-②-③ | -74dBm =①-②-③ |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 11 dB | 11 dB |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 10 dB | 10 dB |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g., insertion losses) due to RF IC or other techniques before LNA |  | 0.2 dB | 0.2 dB |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -39 dBm =①-③-⑤ | -40 dBm =①-③-⑤ |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 60 dB | 60 dB |
|  |  | Frequency isolation techniques |  | Digital filtering after 12bit ADC | Digital filtering after 12bit ADC |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -10 dBm | -10 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -97 dBm | -100 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g., ADC noise, phase noise etc.) | +2dB NF from SI Pin of TX SB at LNA | +1.5dB NF from SI Pin of TX SB at LNA |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | =-94.4 | =-96.4 |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc | 15 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 143.4dB ①-total distortion in Tx and Rx SB | 134.4 dB ①-total distortion in Tx and Rx SB |
| Noise floor ⑩dBm |  |  |  | -94 dBm/ 20MHz CBW | -89.5 dBm/ 20MHz CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -100 dBm | -95.5dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 149 dBc | 133.5 dBc |
| SBFD configuration |  |  |  | DUD | DUD |
| Guardband assumption (if exist) |  |  |  | 5 PRBs | 5 PRBs |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | 100MHz |
| Others |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

- Proposals from Kumu:

  - We further report the inter-sector interference simulation results in the following table:

| FR1 (or FR2-1) |  |  | Kumu Networks, Inc. |  |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (Mean) |  | - |
| BS TX Power dBPTX = ① dBm |  |  | 53 dBm |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc | 45 dB |  |  |
|  |  | Frequency isolation techniques used | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc | 85 dB |  |  |
|  |  | Spatial isolation techniques used | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  | 19.67 dB |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. (Note 1) dBPSI_leakage |  | -57 dBm |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc | 0 dB |  |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc | 0 dB |  |  |
|  |  | RF IC techniques and other tech. (before LNA) | None used for inter-sector interference cancellation |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA | 0.2 dB (assuming 15 dB Rx coupler) |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) ①-③-④-⑤ dBPSI_TXSB |  | <-51.65 dBm |  |  |
|  | Frequency isolation at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc | 0 dB |  |  |
|  |  | [Frequency isolation] techniques | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |  |
|  | Rx IMD | Rx IIP3 capability (dBm) | -20 |  |  |
|  |  | Rx IM3 contribution (dBm) | -115.0 |  |  |
|  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | 5 dB (noise figure) |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized (Note 1, 2) dBPSI_Selectivity |  | -115.0 dBm |  |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  | 16.47 dB |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  | TBD (?) |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  | 0 dB |  |  |
| Overall RSIC capability dBRSIC (Note 1) ②+③+⑧+⑨ |  |  | 146.47 dB |  |  |
| Noise floor ⑩dBm |  |  | -89 dBm/100MHz |  |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  | -95 dBm |  |  |
| Required RSIC budget (①-⑪dBc) |  |  | 148 dB |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

- Proposals from Samsung:

Table 2: Analysis framework for co-channel co-site inter-sector gNB-gNB CLI

| FR1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | [110] dBc if additional [10dB] contributed by BF considered |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 75dB for typical spatial isolation, additional 25dB by installing EM conjugated structure between sectors, and additional [10dB] contributed by different beamforming directions of sectors |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -61dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -143dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -101 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Not used in this case, but technical possible if spatial isolation is lower |
| Overall interference suppression capability dB(Note 1) |  |  |  | 148.8 dBc |
| Noise floor ⑩dBm |  |  |  | -95 dBm/CBW |
| Residual Interference budget with 1 dB desense target (⑪dBm=⑩dBm-6dB) |  |  |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  |  |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | 20MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Proposal from Ericsson:

  - Wide Area BS:

| FR1 (or FR2-1) |  |  |  | Ericsson |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 53 dBm in 2 adjoining sectors |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD, CFR |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 75-90 dBc (Varies depending on scheduled beam directions) |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Typical site layout with around 400mm between sectors |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | Up to 5 dB |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to inter-sector interference (Note 1) |  |  | -64 to -79 dBm (Varies depending on scheduled beam directions) |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to self interference (Note 1) (as captured in self-interference RSIC table) |  |  | -72 dBm |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to both inter-sector and self-interference (Note 1) (as captured in self-interference RSIC table) |  |  | -64 to -71 dBm (Varies depending on scheduled beam directions) |
|  | RF IC and other tech. (before LNA) | Analogue supression in TX sub-band of inter-sector dBαSI-RFIC = ⑤ dBc |  | 0 dBc |
|  |  | Analogue supression in RX sub-band of inter-sector dBαSI-RFIC = ⑧ dBc |  | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -19 to -34 dBm (Varies depending on scheduled beam directions) |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to self interference only (as documented in the self-interference RSIC table) |  |  | -27 dBm |
|  | Total Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -18.4dBm to -26.2 dBm (Varies depending on scheduled beam directions) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 0 dBc |
|  |  | Frequency isolation techniques |  | The RX input level is -19 dBm, and hence the receiver is in high non-linearity; no possibility for interference mitigation as part of the digital receive combining algorithms. Increasing IIP3 towards levels above +5dBm would necessitate designs with low levels of integration, increased energy consumption, thermal issues, size etc. |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS) |
|  |  |  | Rx IM3 contribution due to inter-sector interference only (dBm) | (Varies depending on scheduled beam directions) +7dBm to -38dBm (Minimum for RAN4 requirement) -13dBm to -58dBm (Realistic for AAS) -37dBm to -82dBm (Optimistic for AAS) |
|  |  |  | Rx IM3 contribution due to self-interference only (dBm) | -17 dBm (RAN4 minimum receiver) -37 dBm (Realistic) -61 dBm (Optimistic) |
|  |  |  | Rx IM3 contribution due to total RX power (self-interference + inter-sector) (dBm) | (Varies depending on scheduled beam directions) +8.8dBm to -14.6dBm (Minimum for RAN4 requirement) -11.2dBm to -34.6dBm (Realistic for AAS) -35.2dBm to -58.6 dBm (Optimistic for AAS) |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | ADC could be overloaded; this can be mitigated with filtering prior to ADC except for direct conversion architectures. |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to inter-sector-interference only dBPSI_Selectivity(Note 1, 2) |  |  | Receiver in high non-linearity |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to self-interference only (as quoted in RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | Receiver in high non-linearity |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to total interference (as quoted in self-interference RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | Receiver in high non-linearity |
|  | Digital processing interference supression capability |  |  | Digital IC not possible due to receiver non-linearity and would anyhow be highly complex due to large number of TX/RX for wide area. |
| Total desensitization when considering self-interference only |  |  |  | Receiver in high non-linearity |
| Total desensitization when considering total self- and inter-sector interference |  |  |  | Receiver in high non-linearity |
| Overall RSIC capability dBRSIC (Note 1) when considering self-interference only |  |  |  | Transmitter: 125 dB Receiver: N/A due to receiver saturation |
| Overall RSIC capability dBRSIC (Note 1) when considering self-interference and inter-sector interference |  |  |  | Transmitter: 117-124 dB Receiver: N/A due to receiver saturation |
| Noise floor ⑩dBm |  |  |  | -96 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 155 dBc |
| SBFD configuration |  |  |  | 40-20-40 MHz |
| Guardband assumption (if exist) |  |  |  | 5 PRB. |
| bandwidth over which suppression is achieved |  |  |  | >300 MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

  - Medium Range BS:

| FR1 (or FR2-1) |  |  |  | Ericsson |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Medium range BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 35 dBm in 2 sectors |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD, CFR |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 75-90 dBc (Varies depending on scheduled beam directions) |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Typical site installation with 400mm between sectors |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | Up to 5dB (Depending on beam directions) |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to inter-sector interference (Note 1) |  |  | -82dBm to -97dBm (Varies depending on scheduled beam directions) |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to self interference (Note 1) (as captured in self-interference RSIC table) |  |  | -90 dBm |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to both inter-sector and self-interference (Note 1) (as captured in self-interference RSIC table) |  |  | -82 dBm to -96.3 dBm (Varies depending on scheduled beam directions) |
|  | RF IC and other tech. (before LNA) | Analogue supression in TX sub-band of inter-sector dBαSI-RFIC = ⑤ dBc |  | 0 dBc |
|  |  | Analogue supression in RX sub-band of inter-sector dBαSI-RFIC = ⑧ dBc |  | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -37dBm to -52dBm (Varies depending on scheduled beam directions) |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to self interference (as documented in the self-interference RSIC table) |  |  | -45 dBm |
|  | Total Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -36.6dBm to -44.2 dBm (Varies depending on scheduled beam directions) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 0 dBc |
|  |  | Frequency isolation techniques |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -17.6 dBm |
|  |  |  | Rx IM3 contribution due to inter-sector interference only (dBm) | -75.8 to -120.8dBm (Varies depending on scheduled beam directions) |
|  |  |  | Rx IM3 contribution due to self interference only (dBm) | -100 dBm |
|  |  |  | Rx IM3 contribution due to total RX power (self-interference + inter-sector) (dBm) | -74.6 to -97.4dBm (Varies depending on scheduled beam directions) |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No significant issues for medium range BS power level other than mentioned above. Phase noise reciprocal mixing is not significant for this frequency range and power levels. |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to inter-sector-interference only dBPSI_Selectivity(Note 1, 2) |  |  | -74.8 to -120.8dBm (Varies depending on scheduled beam directions) |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to self-interference only (as quoted in self-interference RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | -100 dBm |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to total interference (as quoted in self-interference RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | -74.6 to -97.4dBm (Varies depending on scheduled beam directions) |
|  | Digital processing interference supression capability |  |  | 10-15dB TX self-interference |
| Total desensitization when considering self-interference only |  |  |  | 0.5 dB |
| Total desensitization when considering total self- and inter-sector interference |  |  |  | 1.5 to 15dB (Depending on beam direction) |
| Overall RSIC capability dBRSIC (Note 1) when considering self-interference only |  |  |  | 134 dB |
| Overall RSIC capability dBRSIC (Note 1) when considering self-interference and inter-sector interference |  |  |  | 110 – 129 dBc (Depending on scheduled beam directions) |
| Noise floor ⑩dBm |  |  |  | -90 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -96 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 131 dBc |
| SBFD configuration |  |  |  | 40-20-40 |
| Guardband assumption (if exist) |  |  |  | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We see that there are over-optimistic isolation values used and absorbers are not feasible, please see Issue 2-5-3. We agree with the values provided by Ericsson. |
| Qualcomm | We would like to understand what is the way forward from those tables in this meeting? It is assumed that companies will provide those numbers as a TP in the relevant section/subsection of the TR. |
| Ericsson | Similar to self-interference, one suggestion is to capture each companies data and then produce a summary table of different techniques (e.g. assuming isolation, assuming RX filtering, assuming beam nulling etc.) that cause the differences between the observations, together with a space for each company to describe why they do/do not make the assumption. |
| Samsung | From our perspective, we agree with Qualcomm that companies will be encouraged to provide the numbers and captured as a TP in the relevant section. From that perspective, our understanding is we can add subsection to allow companies’ technical input. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc. |
| Kumu | Agree with Qualcomm and Samsung. Companies should be encouraged to provide numbers and capture in the relevant sections of the TR. |
| Intel | We support the use of the tables as one step toward finding agreement. We agree with the observation that different companies have different views on isolation values, that should lead to further study. |
| CATT | Decision should be made on how to capture the results. |

#### Issue 2-5-6: Conclusion from Co-channel co-site inter-sector analysis

- Proposals:

  - Proposal 1 (Qualcomm): RAN4 to confirm the adoption of the impact of co-channel co-site inter-sector CLI modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB). The values could be updated if new insights are reached in the feasibility study.

  - Observation 1 (Samsung): Samsung’s input for co-channel co-site inter-sector CLI for FR1 BS is provided in Table-2 in R4-2305203, in which the interference from one co-site sector can be suppressed to the level lower than noise floor by 5dB.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Duplicate from issue 2-4-4 |
| Ericsson | The question what to assume for inter-sector inrterference should be discussed in the simulation assumptions thread, not this one. It is not obvious whether the 1dB is in addition to the self interference ? Alternatively each company could use the assumption on isolation they have derived. |
| Samsung | @Nokia: it is not duplicate since it is for co-channel co-site inter-sector case, while 2-4-4 for self-interference case. |
| Kumu | Support Proposal 1 |
| Intel | We support Proposal 1. In our case, our analysis showed that we couldn’t meet the 6dB below the noise floor value. This results in a desense greater than [1]dB. We couldn’t see a way to include this actual desense value achieved in the table. Perhaps this should be added? |

### Sub-topic 2-6: Co-channel inter-site gNB-gNB CLI model

#### Issue 2-6-1: RX selectivity level

- Proposals/Observations:

  - Observation 1 (CMCC): commercial gNB could at least achieve 50dB Rx selectivity.

  - Proposal 1 (CMCC): it’s suggested to at least use 50dB for inter-site Rx selectivity.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Huawei | Typical value is larger than 60 dB selectivity. And in our analysis in R4-2216239, 62 dB receiver selectivity can be assumed for BS capable of SBFD operation, based on current BS hardware capability. |
| CMCC | 50dB is also much conservation, the value proposed by Huawei is also OK for us. |
| Nokia, Nokia Shanghai Bell | Proposal 1: This might be achievable, but we see it should not be normative requirement. Could be ok only for simulation purposes. This agreement should not limit companies from providing results with the baseline selectivity performance. |
| Qualcomm | No strong preference. We do agree that 46 dB is minimum requirement and typically typical performance is higher than the minimum RAN4 requirement. |
| Ericsson | Whatever we decide, it is important that the IM3 is considered in addition as this is the dominant issue for the receiver. |
| ZTE | We could also achieve high ACS requirement than the specification . usually this could up to link budget if receiver is not blocked yet. |
| Samsung | One way to proceed is to reply RAN1 with a range [50-62]dB based on implementation typical performance (compared with 46dB for min. requirement), from which company can choose to use for their simulation. This range can be used also for adjacent-channel inter-site ACS value, based on our understanding. |
| Intel | We agree with the value given by Huawei |
| CATT | Agree with Ericsson，IM3 is also need to be considered. |

## Companies views’ collection for 1st round

### Open issues

Moderator: views are provided behind each sub-topic/issue.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304543 (Ericsson TP on 10.1 background) | Company A |
|  | Company B |
| R4-2304544 (Ericsson TP on 10.2 WA BS) | Company A |
|  | Company B |
| R4-2304540 (Nokia TP on 10.2 WA BS) | Company A |
|  | Company B |
| R4-2304883 (Qualcomm TP on 10.2 WA BS) | Ericsson: Good to list the different techniques. We should add something about implications, e.g. DL EIRP impact of beam nulling or complexity impact of IC. There are some more techniques to add. It would be good to make a table with each company able to comment on which techniques they did/not consider and why/why not, as it enables the differences in the RSIC tables to be understood better, |
|  | Company B |
| R4-2304431 (CATT TP on 10.4 LA BS) | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

### Sub-topic 2-1: BS aspects: deployment and CA support

|  | Status summary |  |
| --- | --- | --- |
| Sub-topic #2-1 |  | Issue 2-1-1: Assumption on site deployment aspects Discussion status summary: - Companies still have different views on the effect of clutter on achievable RSIC performance. Tentative agreements: - The effect of clutter on achievable RSIC performance: - The following observations are provided by some companies: - Nearby clutter can appear similar to self-interference leakage and also be treated by the interference cancellation algorithm. - For interference cancellation, two effects can both degrade performance: 1) strong reflectors that require the cancellation algorithm to treat finite delays and 2) feedback delay that can occur when sectors are physically located at some distance apart. - For FR2, the measured 28/39GHz path loss between Tx and Rx antennas including clutter reflections is typically approximately 80 dB or better for empty conference room environment. - The residual self-interference including both direct leakage and clutter echo can be cancelled using non-linear digital cancellation algorithm where additional 10dB-20dB of residual self-interference cancellation could be achieved via digital cancellation. Candidate options: N/A Recommendations for 2nd round: - Discussion further on the above tentative agreement. |
|  |  | Issue 2-1-2: Impact of multi-carrier support at BS Discussion status summary: - P2 received most supports while the below tentative agreement is drafted based on which. For other proposals, it is more like how to proceed to study the multi-carrier support at BS in the work item phase (if any). Tentative agreements: - For the impact of multi-carrier support at BS on SBFD operation: - To progress the feasibility and coexistence work in this study item, RAN4 shall focus on single carrier case and capture high level information on multi-carrier support in the TR. Candidate options: N/A Recommendations for 2nd round: - Discussion further on the above tentative agreement. |

### Sub-topic 2-2: BS Aspects: Analysis on component capabilities

| Sub-topic #2-2 | Issue 2-2-1: RF SIC Discussion status summary: - P1 on the feasibility of RF SIC is supported by: Kumu, Samsung and Intel, while opposed by Nokia and Ericsson. The analysis on complexity is suggested to be discussed further. Tentative agreements: - For the implementation feasibility of RF SIC for SBFD operation: - RF cancellation should can be used in SBFD to mitigate self-interference pre-Rx LNA in terms of minimizing non-linearity effects and overall self-interference residue. - FFS on the implementation complexity. Candidate options: N/A Recommendations for 2nd round: - Discussion further on the above tentative agreement. |
| --- | --- |
|  | Issue 2-2-2: Desense allocation between TX and RX (based on 1Db desense) Discussion status summary: - Most of companies believe the desense allocation-based analysis is related to implementation and not necessary and hard to be concluded, especially considering the analysis framework can be used to analyze TX and RX based on companies’ input. Tentative agreements: N/A. Candidate options: N/A Recommendations for 2nd round: - Issue is closed based on majority view. |

### Sub-topic 2-3: Remaining issues for RSI dependency on blocking, AGC and ADC

| Sub-topic #2-3 | Issue 2-3-1: Assumption for input power metric to LNA Discussion status summary: - P1/P2 are agreeable to companies, while P3 has been captured as agreement from last meeting and no need to repeat. Tentative agreements: - gNB receiver saturation, non-linearity, and AGC model is based on RMS power of the input signal. Candidate options: N/A Recommendations for 2nd round: - Confirm the above tentative agreement. - No need further agreement for P3, since there is agreement from last meeting: Agreement (captured in WF R4-2302883) Analysis on other distortions (Phase noise, ADC quantization noise, Residual sideband, ADC distortions) - For FR1 BS, other distortions such as ADC quantization noise and distortions were considered in our simulation and measurements, and it was observed that ADC performance is not limiting. Similarly, phase noise and residual sideband are not significant contributors.. |
| --- | --- |
|  | Issue 2-3-2: Impact on RSI from analogue sub-band filter Discussion status summary: - Same discussion as Issue 1-2-1 and 1-5-1. Tentative agreements: N/A. Candidate options: N/A Recommendations for 2nd round: - Issue 2-3-2 can be closed and discussion shall be on Issue 1-2-1 and 1-5-1. |
|  | Issue 2-3-3: Third-order intermodulation (IM3) Discussion status summary: - A relationship between IIP3 value and the total input power is given. - Companies think the IIP3 effect is incorporated in NF model. Tentative agreements: N/A. Candidate options: N/A Recommendations for 2nd round: - FFS how to proceed on this new IIP3 model. |

### Sub-topic 2-4: RSIC capability overall feasibility analysis

| Sub-topic #2-4 | Issue 2-4-1: RSIC capability analysis table from companies Discussion status summary: - Inputs are provided for RSIC capability analysis tables from companies. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - It is observed that based on different assumptions for adopting certain techniques and the different interpretation of the effort/cost to achieve the benefits of SBFD operation by different companies, it is hard to justify or preclude a certain technique to be utilized. - Moderator suggest we can add subsection (e.g., one subsection per company) to allow companies’ technical input for at least WA BS. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc. - FFS the general principle of how to treat TPs. |
| --- | --- |
|  | Issue 2-4-2: Remaining issues for RSIC analysis framework Discussion status summary: - P1 should be the common understanding, based on the agreement on coexistence study, while companies are question how P1 impact the RSIC analysis framework considering the companies provide the inputs for individual parameters. Tentative agreements: N/A. Candidate options: N/A Recommendations for 2nd round: - FFS how to proceed with P1. |
|  | Issue 2-4-3: Excel spreadsheet for RSIC capability analysis Discussion status summary: - The excel sheet is provided because the proponent company have the concern on “For some tables proposed, it’s difficult to figure out how to get the values in each row, that’s the reason to add the spread sheet here.” - Seems the alignment on an agreeable excel sheet for RSIC capability is very difficult to achieve. Tentative agreements: N/A. Candidate options: N/A Recommendations for 2nd round: - Suggest to stop the discussion on excel spreadsheet. - Suggest to further consider a procedure to address the concern below: - “For some tables proposed, it’s difficult to figure out how to get the values in each row, that’s the reason to add the spread sheet here”. |
|  | Issue 2-4-4: Conclusion from RSIC analysis Discussion status summary: - P1 is supported by Qualcomm/Kumu/Samsung/CMCC. - From Nokia and Ericsson, P1 can be agreed for the coexistence study. Tentative agreements: - For the self-interference modeling used for coexistence study: - RAN4 to confirm the adoption of the impact of self-interference modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB) to study the impact of SBFD operation on the RF requirements. Candidate options: N/A Recommendations for 2nd round: - Double check the tentative agreement with the added main bullet given by moderator. |

### Sub-topic 2-5: Co-channel co-site gNB-gNB CLI model

| Sub-topic #2-5 | Issue 2-5-1: Remaining issues for inter-sector interference analysis framework Discussion status summary: - P1 received the majority companies’ support, which seems cover the intention for P2. - P3 seems reasonable proposal received no objection. Tentative agreements: - Remaining issues for inter-sector interference analysis framework - For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors. - There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study. - For the proposed inter-sector table, the wrongly used term “RSIC” shall be corrected. Candidate options: N/A Recommendations for 2nd round: - Check the tentative agreement. - Discuss how to refine the analysis table based on companies’ input and discussion here. |
| --- | --- |
|  | Issue 2-5-2: Beam nulling considered for inter-sector interference analysis Tentative agreements: - Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR1 BS implementation. Candidate options: N/A Recommendations for 2nd round: - FFS tentative agreement (based on O2) to collect further comments. |
|  | Issue 2-5-3: Spatial Isolation Discussion status summary: - Companies can’t agree on the feasibility and corresponding additional spatial isolation by other methods like EM absorbers. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Discuss how to capture the different observations in the TR: - Similar to self-interference analysis framework, the inter-sector analysis shall still be company input-driven, since a certain technique (e.g., EM absorber, digital IC here) used or not used shall be dependent on company’s input and their own choice. |
|  | Issue 2-5-4: Digital IC Discussion status summary: - Different views on the feasibility of digital IC for co-channel co-site inter-sector interference. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Similar to Issue 2-5-3: Discuss how to capture the different observations in the TR. |
|  | Issue 2-5-5: Inter-sector CLI suppression analysis table from companies Discussion status summary: - Input given based on last meeting’s suggested analysis framework. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Suggest to use WF to further discuss the analysis framework for inter-sector CLI. - Same as RSIC, moderator suggest we can add subsection (e.g., one subsection per company) to allow companies’ technical input for at least WA BS. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc. |
|  | Issue 2-5-6: Conclusion from Co-channel co-site inter-sector analysis Discussion status summary: - P1 seems receive majority support. Tentative agreements: - For the co-channel co-site inter-sector interference modeling used for coexistence study: - RAN4 to confirm the adoption of the impact of co-channel co-site inter-sector CLI modelling that was used in the calibration phase to be further used in the coexistence study (N = noise floor -6 dB). The values could be updated if new insights are reached in the feasibility study. Candidate options: N/A Recommendations for 2nd round: - Double check the tentative agreement with the added main bullet given by moderator. |

### Sub-topic 2-6: Co-channel inter-site gNB-gNB CLI model

| Sub-topic 2-6 | Issue 2-6-1: RX selectivity level Discussion status summary: - Same issue discussed in Issue 1-2-1. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Issue can be closed because the same issue discussed in Issue 1-2-1. |
| --- | --- |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| All TPs (R4-2304543 R4-2304544 R4-2304540 R4-2304883 R4-2304431) | Postponed: Considering this is the 1st meeting for company to submit TPs, it is suggested to postpone all TPs to future meeting, while RAN4 discuss how to treat the TPs, by considering: - For companies’ input based on the analysis framework (for self-interference and co-site inter-sector co-channel interference), discuss how to capture: - [Moderator] suggest to add subsection (e.g., one subsection per company) to allow companies’ technical input for at least WA BS. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc. - For different views on adopting a certain technique: - [Moderator] It is observed that based on different assumptions for adopting certain techniques and the different interpretation of the effort/cost to achieve the benefits of SBFD operation by different companies, it is hard to justify or preclude a certain technique to be utilized. - Others based on companies’ discussion in GTW. |

## Discussion on 2nd round (if applicable)

2nd round discussion is performed on WF directly.

# Topic #3: Implementation Feasibility of SBFD: FR2 BS

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304268 | Intel Corporation | Observation 1: If we assume that the far out noise is always less than the adjacent channel power, then having multiple Tx operating on multi-carrier will have only small impact on the total leaked signal power. Proposal 1: In the case of multi-carrier, we should assume a limit to the effectiveness of potential digital IC, that is of the same magnitude as the difference between Tx adjacent channel power and the out of band noise floor. Proposal 2: Add the Intel RSIC values in Table 1 to TR 38.858. Observation 2: For FR-2, which has better self-interference and co-site, inter-sector interference, simultaneous UL and DL transmission is feasible without degradation in sensitivity. Proposal 3: Add the Intel co-site, inter-sector values in Table 2 to TR 38.858. |
| R4-2304545 | Ericsson | TP to TR 38.858 section 10.5 Feasibility of FR2 BS aspects |
| R4-2305204 | Samsung | Observation 1: Samsung’s input for RSIC budget calculation for FR2-1 BS is provided in Table-1. Proposal 1: According to SIC budget calculation in Table-1, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation, beam nulling and digital IC applied, for FR2-1 BS. Proposal 2: For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors. (Note: The same as the proposal for FR1) - There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study. Observation 2: Samsung’s input for co-channel co-site inter-sector CLI for FR2 BS is provided in Table-2, in which the interference from one co-site sector can be suppressed to the level much lower than noise floor. Observation 3: Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR2 BS implementation. (Note: The same as the proposal for FR1) Observation 4: Digital IC is still technically feasible to cancel the residual co-channel co-site inter-sector interference. (Note: The same as the proposal for FR1) Proposal 3: For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4: o FR2: 100-123dB with 113dB being typical value. |
| R4-2305303 | Huawei, HiSilicon | Proposal 1: It is proposed to change them to ”Beam nulling /isolation in TX sub-band” and ”Beam nulling /isolation in RX sub-band” which are more suitble for FR2 ABF or HBF architectures. Proposal 2: It is proposes to remove ”before LNA” in the table and to evaluate the Self-Interference signal in gNB TX subband at ”RX ant” as that for Self-interference leakage in gNB RX subband. Proposal 3: 100 dB inter-sector isoaltion is achieveable for FR2 |
| R4-2304193 (Wrong AI, and shall be treated in thread [307]) | CMCC | Wrong AI, and shall be treated in thread [307]. |
| R4-2304538 (Wrong AI, and shall be treated in thread [307]) | Nokia, Nokia Shanghai Bell | Wrong AI, and shall be treated in thread [307]. |
| R4-2304541 (copied from Topic #4 ) | Ericsson | Observation 1 The use of filters in the early statges of the RF chain to improve IIP3 would cause a large increase in power consumption, size and weight and a decrease in the level of integration possible. Also the filtering would need to be tuned to a specific carrier and so would need to be operator specific and hardware inflexible. Observation 2 A “1dB criterion” for inter-sector interference would be on top of the self-interference. Observation 3 The receiver related distortion depends on the total input power to the receiver, including both self-interference and inter-sector interference. Observation 4 It is not possible to consider inter-sector receiver interference independently from self-interference. Observation 5 For the proposed inter-sector table, it may discussed whether the term “RSIC” is the right one. Observation 6 During a WI phase, there is a need to discuss whether all of a 1dB sensitivity degradation should be used for self-interference (so then inter-site and sector degradations come on top), or whether some margin of the 1dB should be considered for inter-site and inter-sector interference. Based on the discussion in the previous sections we propose the following: Proposal 1 Capture the range of interference levels/desensitization due to inter-sector interference (together with self-interference) under relevant assumptions. Proposal 2 The impact of (i) self-interference only and of (ii) combined self-interference+inter-sector interference should be evaluated. The impact of inter-sector interference could be interpreted as (ii) – (i). |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Remaining issues for FR2 BS

#### Issue 3-1-1: Interference from co-channel jammer

- Proposals:

  - Proposal 1 (Qualcomm): FR2 BS interference model with co-channel jammer: FR2 BS interference can be modelled as a fixed level of interference 34 dB below the total input power.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Huawei | In our view, for FR2 the IM3 is not the limiting factor, hence new model may not be needed. |
| Ericsson | Also in FR2 the physics of the receiver mean that IM3 and blocking should be considered. We do not think that the interference is just a fixed offset from the total power. It is sufficient each company reports their IP3 assumption and assumption on IM3 in the receiver. As Huawei suggest it is probably not a significant issue like in FR1, but still good to report. Also phase noise should in principle be considered for FR2. Our analysis shows phase noise is not a significant or limiting issue though. |
| ZTE | Maybe Huawei and Ericsson suggestion is more reasonable to report the IP3 and IM3 results directly. |
| Intel | We are not able to understand why a fixed interference level is needed. Further study is needed. |

### Sub-topic 3-2: RSIC capability overall feasibility analysis

#### Issue 3-2-1: RSIC capability analysis table from companies

- Proposals from Intel:

| FR-2 |  |  |  | Intel |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Digital SB Filtering and DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and EM shielding structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, less 0.5dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -83dBm =①-②-③-④ |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dB |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dB |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |
|  |  | Impacts to RX sensitivity (due to e.g., insertion losses) due to RF IC or other techniques before LNA |  | 0 dB |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -55 dBm =①-③-④-⑤ |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dB |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -30 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -105 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g., ADC noise, phase noise etc.) |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | =-97.0 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 127dB ①-total distortion in Tx and Rx SB |
| Noise floor ⑩dBm |  |  |  | -87 dBm/ CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 123 dBc |
| SBFD configuration |  |  |  | DUD |
| Guardband assumption (if exist) |  |  |  | 5 PRBs |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Proposals from Samsung:

- Table-1: FR2-1 RSIC budget calculation Summary

| FR2-1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 87 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -95 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | Not applicable |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | No impact |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -67 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -161dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -91 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 10 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 134.5 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -89 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |
| Guardband assumption (if exist) |  |  |  | 5PRB |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |

- Proposal from Ericsson:

| FR1 (or FR2-1) |  |  |  | Company-A |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | 30dBm TRP | 35dBm TRP |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |  |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 75-98 dBc (depending on beam steering directions) |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Typical site deployment with 400mm between sectors |  |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  |  |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to inter-sector interference (Note 1) |  |  | -70 to -93 dBm (depending on beam steering directions) |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to self interference (Note 1) (as captured in self-interference RSIC table) |  |  | -88 dBm |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to both inter-sector and self-interference (Note 1) (as captured in self-interference RSIC table) |  |  | -70 to -92 dBm |  |
|  | RF IC and other tech. (before LNA) | Analogue supression in TX sub-band of inter-sector dBαSI-RFIC = ⑤ dBc |  | 0 dBc |  |
|  |  | Analogue supression in RX sub-band of inter-sector dBαSI-RFIC = ⑧ dBc |  | 0 dBc |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc |  |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -42 dBm to -65 dBm |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to self interference (as documented in the self-interference RSIC table) |  |  | -60 dBm |  |
|  | Total Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -42 to -59 dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 0 dBc |  |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -35 dBm |  |
|  |  |  | Rx IM3 contribution due to inter-sector interference only (dBm) | -56 dBm to -125 dBm |  |
|  |  |  | Rx IM3 contribution due to self interference only (dBm) | -110 dBm |  |
|  |  |  | Rx IM3 contribution due to total RX power (self-interference + inter-sector) (dBm) | -56 dBm to -107 dBm |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to self-interference only (as quoted in self-interference RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | -110 dBm |  |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to total interference (as quoted in self-interference RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | -56 dBm to -107 dBm |  |
|  | Digital processing interference supression capability |  |  | 10dBc on self-interference TX |  |
| Total desensitization when considering self-interference only |  |  |  | 0.4 dB |  |
| Total desensitization when considering total self- and inter-sector interference |  |  |  | 1.2 to 32 dB (Depending on beam direction) |  |
| Overall RSIC capability dBRSIC (Note 1) when considering self- interference only |  |  |  | -97 dB |  |
| Overall RSIC capability dBRSIC (Note 1) when considering self-interference and inter-sector interference |  |  |  | 85 to 122 dBc |  |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 123 dBc |  |
| SBFD configuration |  |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Ericsson | From results so far, there seems to be a general finding that 30dBm TRP FR2 works with less than 1dB desense. For FR2, we could still, similar to FR1 list different techniques used and each companies view why they do / do not consider it. |
| Samsung | Similar as FR1, different subsection can be set up to accommodate the input from different companies. |
| CATT | More discussion is needed on how to capture all of the results, should we align the calculation again? |

#### Issue 3-2-2: Remaining issues for RSIC analysis framework (Specific to FR2-1 only)

- Proposals:

  - Proposal 1 (Huawei): It is proposed to change them to ”Beam nulling /isolation in TX sub-band” and ”Beam nulling /isolation in RX sub-band” which are more suitble for FR2 ABF or HBF architectures.

  - Proposal 2 (Huawei): It is proposes to remove ”before LNA” in the table and to evaluate the Self-Interference signal in gNB TX subband at ”RX ant” as that for Self-interference leakage in gNB RX subband.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Qualcomm | We are ok with proposal 1. |
| Ericsson | Don’t quite follow the Huawei comment. Beam nulling can (i) direct power in the TX sub-band away from the receiver, thus reducing the input power to the receiver and (ii) direct RX leakage in the RX subband at the transmitter away from the receiver, reducing receiver interference. Actually it is not clear that the beam nulling suppresses both by the same amount, although I think in general we are assuming that. Anyhow, isn’t beam nulling used for both sub-bands ? So really it should not mention either TX or RX ? For “before LNA” and “at RX ant”… it changes the reference point to before the antenna. Then that brings complications how each company accounts for the antenna gain. Our simulations on isolation are from TX panel to RX input after the antenna. Maybe “before LNA” is better because it eliminates variation in how to translate the RX power at an antenna to the impact to the LNA. When quoting the spatial isolation, each company should consider that the isolation is from TX panel to LNA input, whatever architecture is assumed. |
| ZTE | Don’t understand Huawei’s suggestion and more clarifications are needed. |
| Samsung | Why P1 and P2 are specific to FR2 rather than to be applied to both FR1 and FR2-1? |

### Sub-topic 3-3: Co-channel co-site gNB-gNB CLI model

#### Issue 3-3-1: Spatial isolation

- Proposals:

  - Observation 1 (Huawei):

Table 2.2-1: Inter-sector isolation for existing modules

| Test cases | Existing modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 96 | 99 | 103 |
| Case 2 | 120°installation | 85 | 90 | 100 |

Table 2.2-2: Inter-sector isolation for improved modules

| Test cases | Improved modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 101 | 103 | 104 |
| Case 2 | 120°installation | 100 | 101 | 102 |

  - Proposal 1 (Huawei): 100 dB inter-sector isolation is achievable for FR2.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Ericsson | The first table corresponds quite well to our simulation results for 120 degree installation, which showed 75dB (lowest seen isolation.. like an even higher %tage on the CDF) to 98dB. As commented elsewhere, it is difficult to just agree numbers for isolating sectors in the absence of any details on how the sectors would look and how practical issues with site space, installation complexity, site restrictions, weight, load etc. would be overcome. For today’s sites, we have to provide significant filtering to co-locate BS in different bands (with isolation 30dB or sometimes even less), so it seems odd that such isolation improvements can be obtained for SBFD that are not available for current deployments. |

#### Issue 3-3-2: Digital IC

- Proposals/Observations:

  - Option 1 (Samsung): Digital IC can be applied for co-site inter-sector case.

  - Option 2: No digital IC can be applied for co-site inter-sector case.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We do not have strong view, but we think for FR2 digital IC may not be needed. |
| Qualcomm | We are ok with option 1. |
| Ericsson | Digital IC complexity could be very high for a wide area BS. Could this be listed as an issue for companies to give differing opinions on after the RSIC analysis in the TR ? |
| ZTE | For FR2, digitial IC could be also used |
| Intel | We are ok with option 1. Digital IC still can function for FR2, yet we expect lower performance due to wider channel BW that needs to be corrected. |
| CATT | We think it’s challenge for WA. |

#### Issue 3-3-3: Inter-sector CLI suppression analysis table from companies

- Proposals from Intel:

| FR-2 |  |  |  | Intel |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Digital SB Filtering and DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 87 dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Spatial separation and EM shielding structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -85dBm =①-②-③ |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dB |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dB |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |
|  |  | Impacts to RX sensitivity (due to e.g., insertion losses) due to RF IC or other techniques before LNA |  | 0 dB |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -57 dBm =①-③-⑤ |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dB |
|  |  | Frequency isolation techniques |  | Digital filtering after ADC |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -30 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -97 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g., ADC noise, phase noise etc.) |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | =-94.6 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 124.6 dB ①-total distortion in Tx and Rx SB |
| Noise floor ⑩dBm |  |  |  | -87 dBm/ CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 123 dBc |
| SBFD configuration |  |  |  | DUD |
| Guardband assumption (if exist) |  |  |  | 5 PRBs |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Proposal from Huawei:

- Table 2.2-3: Inter-sector interference analysis

| FR2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 35 dBm/200MHz |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dB |
|  |  | Frequency isolation techniques used |  | DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 100 |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Spatial separation |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -99 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc |
|  | Self-Interference signal in gNB TX subband, measured at RX ant. |  |  | -65 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |
|  |  |  | Rx IM3 contribution (dBm) |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | -- |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 134 dBc |
| Noise floor ⑩dBm |  |  |  | -88 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 129 dBc |
| SBFD configuration |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Proposal from Samsung:

  - Samsung’s input for co-channel co-site inter-sector CLI for FR2 BS is provided in Table-2, in which the interference from one co-site sector can be suppressed to the level much lower than noise floor.

Table 2: Analysis framework for co-channel co-site inter-sector gNB-gNB CLI

| FR2-1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 108 dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 88dB for typical spatial isolation, and additional 20dB contributed by different beamforming directions of sectors |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -78dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -194dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -102 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Not used in this case |
| Overall interference suppression capability dB (Note 1) |  |  |  | 130.5 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  |  |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |
| Guardband assumption (if exist) |  |  |  | 5PRB |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

Proposals from Ericsson:

| FR1 (or FR2-1) |  |  |  | Company-A |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | 30dBm TRP | 35dBm TRP |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |  |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 75-98 dBc (depending on beam steering directions) |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Typical site deployment with 400mm between sectors |  |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  |  |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to inter-sector interference (Note 1) |  |  | -70 to -93 dBm (depending on beam steering directions) |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to self interference (Note 1) (as captured in self-interference RSIC table) |  |  | -88 dBm |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage due to both inter-sector and self-interference (Note 1) (as captured in self-interference RSIC table) |  |  | -70 to -92 dBm |  |
|  | RF IC and other tech. (before LNA) | Analogue supression in TX sub-band of inter-sector dBαSI-RFIC = ⑤ dBc |  | 0 dBc |  |
|  |  | Analogue supression in RX sub-band of inter-sector dBαSI-RFIC = ⑧ dBc |  | 0 dBc |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc |  |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -42 dBm to -65 dBm |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to self interference (as documented in the self-interference RSIC table) |  |  | -60 dBm |  |
|  | Total Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -42 to -59 dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 0 dBc |  |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -35 dBm |  |
|  |  |  | Rx IM3 contribution due to inter-sector interference only (dBm) | -56 dBm to -125 dBm |  |
|  |  |  | Rx IM3 contribution due to self interference only (dBm) | -110 dBm |  |
|  |  |  | Rx IM3 contribution due to total RX power (self-interference + inter-sector) (dBm) | -56 dBm to -107 dBm |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to self-interference only (as quoted in self-interference RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | -110 dBm |  |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to total interference (as quoted in self-interference RSIC table) dBPSI_Selectivity(Note 1, 2) |  |  | -56 dBm to -107 dBm |  |
|  | Digital processing interference supression capability |  |  | 10dBc on self-interference TX |  |
| Total desensitization when considering self-interference only |  |  |  | 0.4 dB |  |
| Total desensitization when considering total self- and inter-sector interference |  |  |  | 1.2 to 32 dB (Depending on beam direction) |  |
| Overall RSIC capability dBRSIC (Note 1) when considering self- interference only |  |  |  | -97 dB |  |
| Overall RSIC capability dBRSIC (Note 1) when considering self-interference and inter-sector interference |  |  |  | 85 to 122 dBc |  |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 123 dBc |  |
| SBFD configuration |  |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| Ericsson | Similar comments to FR1; it could be good to capture the tables but then also a summary table of each technique and with a view from each company why they do / do not assume it. |
| Samsung | Ericsson’s comment is reasonable to us. |

## Companies views’ collection for 1st round

### Open issues

Moderator: views are provided behind each sub-topic/issue.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304545 (Ericsson TP to 10.5 FR2 BS) | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

### Sub-topic 3-1: Remaining issues for FR2 BS

|  | Status summary |  |
| --- | --- | --- |
| Sub-topic #3-1 |  | Issue 3-1-1: Interference from co-channel jammer Discussion status summary: - Interference model for FR2-1 is given, as a fixed level of interference 34 dB below the total input power. - Company suggest to report IM3 directly in the analysis framework. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - FFS the model, and FFS if the issue can be closed considering companies are allowed to report IM3 in their own input based on analysis framework. |

### Sub-topic 3-2: RSIC capability overall feasibility analysis

| Sub-topic #3-2 | Issue 3-2-1: RSIC capability analysis table from companies Discussion status summary: - Inputs are provided for RSIC capability analysis tables from companies. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - It is observed that based on different assumptions for adopting certain techniques and the different interpretation of the effort/cost to achieve the benefits of SBFD operation by different companies, it is hard to justify or preclude a certain technique to be utilized. - Moderator suggest we can add subsection (e.g., one subsection per company) to allow companies’ technical input for at least WA BS. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc. - FFS the general principle of how to treat TPs. |
| --- | --- |
|  | Issue 3-2-2: Remaining issues for RSIC analysis framework (Specific to FR2-1 only) Discussion status summary: - Different views received on P1 and P2. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - FFS how to proceed with these two proposals. |

### Sub-topic 3-3: Co-channel co-site gNB-gNB CLI model

| Sub-topic #3-3 | Issue 3-3-1: Spatial isolation Discussion status summary: - Not fully discussed in 1st round. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Discuss how to capture the different observations in the TR. |
| --- | --- |
|  | Issue 3-3-2: Digital IC Discussion status summary: - Different views on the feasibility of digital IC for co-channel co-site inter-sector interference. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Similar to FR1: Discuss how to capture the different observations in the TR. |
|  | Issue 3-3-3: Inter-sector CLI suppression analysis table from companies Discussion status summary: - Input given based on last meeting’s suggested analysis framework. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Suggest to use WF to further discuss the analysis framework for inter-sector CLI (same as FR1). - Same as RSIC, moderator suggest we can add subsection (e.g., one subsection per company) to allow companies’ technical input for at least WA BS. Based on that, the conclusion can be made based on the condition that certain techniques are utilized etc. (same as FR1) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2304545 | Postponed: Considering this is the 1st meeting for company to submit TPs, it is suggested to postpone all TPs to future meeting, while RAN4 discuss how to treat the TPs, by considering the aspects mentioned in Topic#2’s corresponding section. |

## Discussion on 2nd round (if applicable)

2nd round discussion is performed on WF directly.

# Topic #4: Impacts on BS RF requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304400 (Moved to Topic #2) | Kumu Networks |  |
| R4-2304432 | CATT | TP provided. |
| R4-2304537 | Nokia, Nokia Shanghai Bell | Proposal 1: Maximum TRP is needed in the requirement of OTA sensitivity within SBFD time slot that reflects end use case. Proposal 2: It is proposed to introduce OTA ICS requirement. Proposal 3: It is proposed to relax reference sensitivity level requirement by less than 0.5dB during SBFD time slots Proposal 4: RAN4 to discuss tighter interference C/I required in order to verify BS performance to detect wanted signal in the presence of ACL from other gNB. |
| R4-2304541 | Ericsson | Observation 6 During a WI phase, there is a need to discuss whether all of a 1dB sensitivity degradation should be used for self-interference (so then inter-site and sector degradations come on top), or whether some margin of the 1dB should be considered for inter-site and inter-sector interference. Observation 7 RAN4 should consider whether to add an interferer to the SBFD OTA sensitivity test to test inter-sector/inter-site interference suppression. Observation 8 When considering self-interference only, inter-sub-band emissions and selectivity requirements are redundant if an SBFD OTA sensitivity requirement is defined. Observation 9 Inter-sub-band emissions and selectivity requirements are relevant for regulating the impact to SBFD operation from other sites an sectors. Observation 10 Potentially the need for an inter-sub-band selectivity requirement could be mitigated if the (average) power from other sectors/sites would be added in the OTA sensitivity test. |
| R4-2305306 | Huawei, HiSilicon | Proposal 1: the need of in-channel inter sub-band requirement is FFS, which is related the assumption of BS-BS isolation, and adopted interference suppression technology. Proposal 2: Co-location requirements are also pending on RAN4 co-existence study. If sub-band filter per operator is assumed, new co-location requirements are needed. |
| R4-2305402 | ZTE Corporation | Discussion on BS requirement impact. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1: BS Aspects: RF requirement impact

Sub-topic description:

[Moderator] In last meeting, BS RF requirement impact has been initially discussed, with the following agreements in WF [R4-2302969]:

The existing RF requirements with respect to wanted signal as below are still applicable within SBFD time slot: Conducted RF requirement; Radiated RF requirement. BS output power Output power dynamics Transmitted signal quality Occupied bandwidth Dynamic range; Radiated transmit power OTA base station output power OTA output power dynamics OTA transmitted signal quality OTA occupied bandwidth OTA dynamic range. The following requirements are new requirements, - OTA sensitivity within SBFD time slot - Other requirements not precluded The following requirements may be impacted, - Transmitter intermodulation - Out of band blocking The following requirements are not applicable, - Transmit ON/OFF power requirement within SBFD time slot - OTA receiver spurious emissions

And no agreements for:

No agreements for the following: - In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity: - Option 1: No such requirement needed. RAN4 consider the SBFD performance requirement for receiver sensitivity with the simultaneous TX in the SBFD time slot, in which the in-channel adjacent subblock leakage ratio and in-channel adjacent subblock blocking requirements can be guaranteed implicitly. - Option 2: New requirements are needed for In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking + Adjacent subband selectivity. - ACLR, ACS, in-band blocking, intermodulation:

#### Issue 4-1-1: OTA sensitivity within SBFD time slot

- Proposals/Observations:

  - Proposal 1 (Nokia): Maximum TRP is needed in the requirement of OTA sensitivity within SBFD time slot that reflects end use case.

  - Proposal 2 (Nokia): It is proposed to relax reference sensitivity level requirement by less than 0.5dB during SBFD time slots.

  - Observation 1 (Ericsson): During a WI phase, there is a need to discuss whether all of a 1dB sensitivity degradation should be used for self-interference (so then inter-site and sector degradations come on top), or whether some margin of the 1dB should be considered for inter-site and inter-sector interference.

  - Observation 2 (Ericsson): RAN4 should consider whether to add an interferer to the SBFD OTA sensitivity test to test inter-sector/inter-site interference suppression.

  - Proposal 3 (ZTE): PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ 1dB desens target]

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Proposal 3 as the baseline. REFSENSE only consider the self-interference. Regarding inter-site or inter-sector requirements, RAN4 may need to define inter-sub band Tx leakage and Rx selectivity requirements. |
| Nokia, Nokia Shanghai Bell | We feel there needs to be margin for inter-site and inter-sector interference in the 1dB sensitivity degradation and it should not be used all for self-interference. Observation 2 (Ericsson) sounds reasonable to us. |
| Ericsson | It makes sense to allow for a different declaration and test of TRP for SBFD and non-SBFD slots. Also EIRP should be declared and tested separately, as beam nulling may impact EIRP. For the sensitivity degradation, there is a need to discuss whether the whole 1dB should be allocated for self-interference in reality considering that there will be inter-sector etc. That could be accounted for by adding some interference as we suggest or using a lower desense in the requirement as Nokia suggest. |
| ZTE | From our understanding, from per-sector self interference perspective, it should be reasonable to go with Proposal 3. for sure, the degradation value could be further updated if necessary. For inter-sector, if we also agree to do such kind of conformance testing and also requirement definition, yes, this will create an new REFSENS requirement indeed. In short, we could further discuss the following two cases: 1) per-sector, SBFD REFSENS; 2) inter-sector, SBFD REFSENS; |
| Samsung | We are discussing requirement, rather than practical deployment. Again, in practical deployment, a legacy gNB receiver’s interference shall be the UL signals from UE in neighboring cells, which is also not captured in existing BS RF requirement. Then why we need to consider the interferences from co-site inter-sector and inter-site ones to be accounted here? Inter-sub band TX and RX selectivity requirement (if introduced) is just gNB RF requirement, which shall not be regarded as “inter-site” or “inter-sector” requirements. |
| CATT | Agree with Proposal 1 and Proposal 3. |

#### Issue 4-1-2: In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity within SBFD time slot

- Proposals/Observations:

  - Proposal 1 (Nokia): It is proposed to introduce OTA ICS within SBFD time slot requirement.

  - Proposal 2 (Huawei): the need of in-channel inter sub-band requirement is FFS, which is related the assumption of BS-BS isolation, and adopted interference suppression technology.

  - Observation 1 (Ericsson): When considering self-interference only, inter-sub-band emissions and selectivity requirements are redundant if an SBFD OTA sensitivity requirement is defined.

  - Observation 2 (Ericsson): Inter-sub-band emissions and selectivity requirements are relevant for regulating the impact to SBFD operation from other sites an sectors.

  - Observation 3 (Ericsson): Potentially the need for an inter-sub-band selectivity requirement could be mitigated if the (average) power from other sectors/sites would be added in the OTA sensitivity test.

  - Proposal 4 (ZTE): for self interference scenario, inter-band sub-band emission and selectivity requirements are not needed.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

  - The issue can be discussed for the requirement is defined with only self-interference or with self-interference+inter-gNB interferences.

| Company | Comments |
| --- | --- |
| CMCC | Proposal 1 is preferred. Regarding inter-site or inter-sector requirements, RAN4 may need to define inter-sub band Tx leakage and Rx selectivity requirements. |
| Nokia, Nokia Shanghai Bell | A fundamental difference between self-interference and interference from other gNBs is that self-interference is, at least in theory, easier to cancel. Hence the addition of the power from all other gNBs would have to be virtually impossible to cancel, i.e. coming simultaneously from different directions, not having OFDM symbol orthogonality and even including the other gNBs' adjacent channel leakage inside the UL subband. |
| Qualcomm | In general ok with proposal 1 and 2. |
| Ericsson | As we indicated, a selectivity requirement is needed unless interference from other sites is dealt with by adding interference in the OTA sensitivity requirement. |
| ZTE | For self interfernece conformance testing, we think that Ericsson and ZTE’s view are aligned, however for inter-sector conformance testing, at least from the receiver side ICS requirement, by defining new REFSENS requirement, there are no such kind of requirement needed which is implicitly tested by the refense. For transmitter ACLR requirement or OBUE requirement for other sectors, this could be further discussed whether the specific requirement are needed. E.g..by placing co-location reference antenna similar as SBFD antenna. |
| Samsung | We prefer Proposal 4, while to further study that as P2 is also okay for us. |
| CATT | Agree with proposal 1. |

#### Issue 4-1-3: ACLR and ACS

- Proposals/Observations:

  - Proposal 1 (Huawei): Co-location requirements are also pending on RAN4 co-existence study. If sub-band filter per operator is assumed, new co-location requirements are needed.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Current, there is no 0% grid shift in RAN4 simulation. co-location case may only work when sub-band filter is used, otherwise, we are afraid gNB receiver would be blocked. |
| Nokia, Nokia Shanghai Bell | Proposal 1: Sub-band filtering is not seen feasible as explained earlier. |
| Qualcomm | We would prefer to wait for the coexistence study before discussing the ACLR/ACS values. |
| Ericsson | A filter per operator implies operator specific hardware that is inflexible. Also it implies that not just the SBFD operator, but also other operators need to upgrade their AAS basestations by swapping them for BS with filters. If we would create new co-location requirements it would also raise the question is this only for SBFD, why not in general for co-location if such filtering solutions are assumed. |
| ZTE | Co-location requirement could be further discussed and maybe co-site intersector is more appropriate targeted scenario, right? |
| Samsung | Same as QC, since the required ACLR and ACS shall be provided based on the required values obtained from co-existence study. |

#### Issue 4-1-4: Transition ON-OFF power and transition period

[Moderator] Agreement from last meeting: The following requirements are not applicable: Transmit ON/OFF power requirement within SBFD time slot.

- Proposals/Observations:

  - Proposal 1 (ZTE): to further discuss the necessity of transition requirement for DL transmission and SBFD slot switching scenario and SBFD slots switching scenario and its exact requirement if necessary.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Proposal 1 seems reasonable. For the switching from D to X(SBFD slot), there is also the switching from Tx to Rx(U in SBFD slot). Detailed discussion in future WID phase. |
| Qualcomm | We are ok with proposal 1. |
| Ericsson | As well as ON/OFF, there is a need to consider requirements when a slot changes. For example, when a DL slot switches to an SBFD slot then part of the array may switch from transmitting to receiving and there will be some kind of transient time for the switch. Similarly if switching from SBFD to UL then part of the array will switch from transmit to receive. Then of course there may be usual DL-UL switching. We should consider what requirements may be needed for switching of parts of an array. |
| ZTE | We are also fine with proposal and open for detailed RF requirement discussion. |
| Samsung | Okay with P1. |
| CATT | Ok to further discuss. |

#### Issue 4-1-5: Tx intermodulation requirement and co-location out-of-band blocking

- Proposals/Observations:

  - Proposal 1 (Huawei): Co-location requirements are also pending on RAN4 co-existence study. If sub-band filter per operator is assumed, new co-location requirements are needed.

  - Proposal 2 (CATT): For transmitter intermodulation and OTA transmitter intermodulation requirements, 30 dB coupling loss is assumed between two co-location gNBs. For SBFD BS, Rx REFSENS requirements for this scenario needs to be discussed further, i.e. if the same interference level is assumed and if Tx sub-band is active for this test. For the similar reason, co-location out of band blocking requirements also needs to be discussed further.

  - Observation 1 (ZTE): The existing transmitter intermodulation requirement might be not applicable for SBFD BS.

  - Observation 2 (ZTE): For the SBFD slot/symbols, wanted signal power level for co-location blocking requirement could be defined in the following equation similar as that of ACS requirement and IBB requirement.

- Wanted signal power level = REFSENS +10*log10(10^6/10+10^[1dB desens target/10])

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

| Company | Comments |
| --- | --- |
| CMCC | for Tx IMD, legacy assume 30dB isolation. But according to the feasibility study, the spatial isolation is much larger than 30dB. so the interference level may need to be updated accordingly. For the observation 2 from ZTE, legacy assuming 6dB REFSENSE desense for co-location blocking requirements, current SBFD may assume another 1 dB desense, the aggregated impact is 7dB desense which is sum in scale of dB rather than in scale of linearity as in observation 2. |
| Nokia, Nokia Shanghai Bell | Proposal 1: Sub-band filtering is not seen feasible as explained earlier. |
| Ericsson | The TX IM is important to ensure that all emisisons requirements are met even if an interferer temporarily arises. It is not necessary to prove SBFD operation when there is a TX IM interferer, but it should be established that the transmitter withstands the self-interference. |
| ZTE | For Tx intermodualtion, we share similar view as CMCC, the large signal could block the receiver, then it is not necessary to consider anymore. Or other more reasonble co-location requirement could be considered if possible. |
| Samsung | Agree with CMCC. |
| CATT | We agree that the detail requirement needs more study. |

#### Issue 4-1-6: Other new requirements (if needed)

- Proposals/Observations:

  - Proposal 1 (Nokia): RAN4 to discuss tighter interference C/I required in order to verify BS performance to detect wanted signal in the presence of ACL from other gNB.

  - Proposal 2 (ZTE): dynamic range requirement should be further considered to configure both downlink transmission signal and wanted signal, interfering signal of dynamic range within the carrier. Self interference caused by the downlink transmission is much lower than interference signal power level and wanted signal power level, therefore from our understanding, additional desens for dynamic range requirement should be marginal and could be ignored.

  - Proposal 3 (ZTE): For the SBFD slot/symbols, there might be some self interference and additional Rx intermodulation caused by downlink part especially when considering with CW signal or NBB or general Rx intermodulation signals configured during the Rx IMD test, therefore it’s suggested to have further study for it.

  - Proposal 4 (ZTE): please check the detailed proposals in the contribution.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

  - Moderator suggest Nokia to clarify more on which requirement/test should be defined with tighter interference C/I.

| Company | Comments |
| --- | --- |
| CMCC | Regarding Rx dynamic range, legacy use AWGN signal as interference signal. For SBFD case, our initial view is that the environment noise floor may be even larger due to additional gNB Tx which may be assumed as white noise if this interference is much lower and from large amounts of gNB. further discussion is required. |
| Nokia, Nokia Shanghai Bell | For clarification: Testing an SBFD capable gNB when transmitting and receiving simultaneously can verify that the gNB's desensitization due to self-interference is sufficiently low. However, such a test may just indicate that a single gNB still has a sufficient sensitivity, but not that a network with additional inter-sector and inter-site interference between gNBs has a sufficiently low desensitization. Hence a requirement about a gNB's sensitivity in the presence of inter-sector/inter-site interference from other SBFD base stations, explicitly including their adjacent channel leakage acting as co-channel interference in the UL sub-band, should be considered. The co-channel interference due to other SBFD base stations' adjacent channel leakage may be modeled as AWGN (similarly to the dynamic range test). |
| Qualcomm | For proposal 1, does this imply revisiting of the ICS/ACS maximum mean interfering signal values? Would the inband blocking requirement be revisited as well? For testing, it would be challenging to test 1dB desensitization for SBFD capable gNBs. |
| Ericsson | Nokia have a point that the interference floor on which dynamic range is derived could be increased due to a different in-channel interference environment. Further consideration is needed how significant the rise could be compared to the current requirement. ZTE have a point that an external interferer could modulate with the DL part… this needs some more consideration. It may be related to TX IM. |
| ZTE | For dynamic range requirement, Ericsson and Nokia ‘s understanding is correct, there might be some more in-channel noise arising due to the other gNB DL transmission leakage. To Qualcom, we don;t need to revise the ACS and IBB requirement since this is out of carrier. |
| Samsung | For P1, based on Nokia’s clarification here, the next question is why legacy UE shall not be tested with their major interference source present? i.e., the inference from neighboring cells’ UE uplink signals. |

## Companies views’ collection for 1st round

### Open issues

Moderator: views are provided behind each sub-topic/issue.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304432 (TP from CATT) | ZTE: suggest to postpone this TP and merge to big TP for BS RF impacts. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #4-1 | Issue 4-1-1: OTA sensitivity within SBFD time slot Discussion status summary: - Discussion on OTA sensitivity within SBFD time slot is given. - P3 seems the baseline while [1dB] can be further clarify. - Whether or not the presence of inter-sector and inter-site inference is not confirmed. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Further discuss how to derive the agreement based on P3. |
|  | Issue 4-1-2: In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity within SBFD time slot Discussion status summary: - The necessity of In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity within SBFD time slot is further discussed without a clear conclusion yet. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - FFS the necessity of In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity within SBFD time slot |
|  | Issue 4-1-3: ACLR and ACS Discussion status summary: - Company proposed that Co-location requirements are also pending on RAN4 co-existence study, while it is identified that 0% grid shift is not yet studied in RAN4 co-existence study. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - FFS if co-located requirement shall be based on RAN4 co-existence, which currently don’t contain the case of 0% grid shift. |
|  | Issue 4-1-4: Transition ON-OFF power and transition period Discussion status summary: - P1 is okay for most of companies. Tentative agreements: - The potential new requirement on transition ON-OFF power and transition period: - FFS the necessity of transition requirement for DL transmission and SBFD slot switching scenario and SBFD slots switching scenario and its exact requirement if necessary. Candidate options: N/A Recommendations for 2nd round: - Check the above tentative agreement. |
|  | Issue 4-1-5: Tx intermodulation requirement and co-location out-of-band blocking Discussion status summary: - For transmitter intermodulation and OTA transmitter intermodulation requirements, 30 dB coupling loss is assumed between two co-location gNBs right now, but seems it can be further discussed due to the agreed larger spatial isolation range. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - FFS based on P1, P2, and observations from ZTE. - How to draft the agreement/WF is FFS |
|  | Issue 4-1-6: Other new requirements (if needed) Discussion status summary: - Tighter interference condition is discussed, while whether or not the presence of inter-sector and inter-site inference is not confirmed. Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - FFS based on P1, P2, and P3. - How to draft the agreement/WF is FFS |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2304432 (TP from CATT) | Postponed: this TP can be postponed and to be further merged to big TP for BS RF impacts. |

## Discussion on 2nd round (if applicable)

2nd round discussion is performed on WF directly.

# Topic #5: Regulatory survey

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304024 | Spark NZ Ltd | Text proposal given |
| R4-2304539 | Nokia, Nokia Shanghai Bell | Text proposal given |
| R4-2305207 | Samsung | Text proposal given |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 5-1: Text Proposal on regulatory aspects

Sub-topic description:

[Moderator] As one of the objectives in WID as below, RAN4 is tasked to summarize the regulatory aspects for deploying the duplex enhancement in TDD unpaired spectrum:

The detailed objectives are as follows: - ... - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Open issues and candidate options before e-meeting:

#### Issue 5-1-1: Text Proposal on regulatory aspects

- Proposals

  - Option 1 (Nokia, R4-2216204):

Outside Europe, national regulations do not appear to represent a major bottleneck for the deployment of SBFD in the 3400-3800 MHz range. For deployments within CEPT countries, SBFD may require changes to the current regulations. Therefore, it is envisioned that co-existence between SBFD and static TDD is thoroughly investigated in RAN4 so that performance results can be used as a basis for discussions with regulatory bodies defining harmonized standards at least for the 3400-3800MHz band.

Following clauses describe regulatory aspects in ITU Regions 1,2 and 3.

  - Option 2 (Spark and Ericsson, R4-2304024):

13.1	Region 1

13.1.1	Europe

CEPT made coexistence studies with adjacent services assuming a certain DL/UL ratio for IMT TDD bands, e.g. 3.4-3.8GHz band in Europe. The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation and consequently may affect the outcomes of the coexistence studies and, consequently, the regulated license conditions.

To address the cross-border issue and facilitate coordination, ECC recommended the usage of two frame structures in the 3.4-3.8GHz frequency band (ECC Recommendation(20)03).

However, enabling operation with various TDD patterns and removing the need of synchronized networks, CEPT has specified additional baselines for unsynchronized or semi-unsynchronized networks. Nevertheless, those baselines are more stringent, making the BS design more challenging, impacting final cost and possibly product’s volume and weight. As an example, for the 3.4-3.8GHz band, inside the band, ECC specified below and above the block edge a restricted baseline of -34dBm/5 MHz EIRP for non AAS BS or -43dBm/MHz TRP for AAS BS (ECC Decision(11)06).

13.2	Region 2

13.2.1	North America

No TDD pattern has been mandated in US, nor in Canada, but operators are encouraged to coordinate their network deployment and make sure they don’t interfere with each other.

Unsynchronized operation is allowed, more stringent regulation parameters have not been specified for such case but, again, operators would have to work their differences to avoid any claim to FCC/ISED.

When SBFD will be introduced, Regulators might issue some consultations to understand whether there would be some oversight of the feature.

13.3	Region 3

13.3.1	China

In China, spectrum is allocated with clearly stating it for TDD or FDD operation. MIIT has specified a TDD pattern that should be used by the operators when operating adjacent TDD networks, assuming then synchronization between those operators.

There is no SBFD regulatory requirements in China until now. MIIT mainly cares interference between different operators. Necessary interference coordination mechanism and solutions may be proposed by MIIT to avoid interference before any SBFD deployment.

13.3.2	Japan

No TDD pattern has been mandated in Japan, but operators are required to coordinate their network deployment to avoid interference. Operators are allowed to use unsynchronized operation as far as there is no interference with the adjacent network(s), e.g. for indoor usage.

13.3.3		New Zealand

In NZ a TDD pattern has been mandated and in addition the networks must be time synchronised. Operator deployments that do not conform to the synchronisation requirement must not interfere with deployments that are conforming with the described synchronisation requirements , and therefore cannot claim protection from interference. Therefore it will be extremely difficult to introduce SBFD.

13.3.4		Australia

In Australia there is a defined frame structure requirement referred to as ‘fall back’ structures. Operators are not obligated all the time to adopt the defined frame structure (i.e. operator can use different structures if there are no issues) but if there are any interference/coexistence issues that cannot be mutually resolved between mobile operators, the regulator may require network operators are required to implement the ‘fall back’ frame a defined structure.

13.3.5		India

In India no frame structure is mandated. In case operators have incompatible frame structures resulting in interference then the onus of mitigating interference falls amongst the operators.

13.4	Summary

Regulators always pay high attention to any new technology that might create interference to incumbent services operating in or adjacent to the considered spectrum, specifying new conditions to prevent any such interference.

When allocating spectrum to IMT TDD operation, Regulators made coexistence studies with incumbent services assuming a certain TDD pattern. Based on the conclusions of those studies, Regulators have then specified the corresponding specific parameters to enable such deployment.

In most of the countries, operators are expected to synchronize their adjacent TDD networks. Some Regulators have even recommended specific TDD frame structure usage to facilitate this, addressing then cross-border issues between countries (e.g. in Europe).

To enable unsynchronized TDD deployments without creating interference in the adjacent network(s), some Regulators have specified more stringent parameters (e.g. CEPT specified below and above the block edge a restricted baseline of -34dBm/5 MHz EIRP for non AAS BS or -43dBm/MHz TRP for AAS BS), increasing BS design’s complexity significantly.

Regulators might revise existing regulatory rules to allow SBFD operations and/or mandate more stringent requirements.

Nevertheless, when deployed in environments which guarantee and prevent any interference in the adjacent spectrum (like isolated indoor deployment), no specific condition nor recommendation have been specified by the Regulators, allowing any TDD deployment in such environments as long as no interference disturbs adjacent services. For such type of deployments, existing regulation rules should not be impacting when operating SBFD.

  - Option 3: Samsung (R4-2305207):

13.1 Region 1

13.1.1	Europe

Regulators made coexistence studies assuming a certain DL/UL ratio for IMT TDD band 3.4-3.8GHz band in Europe. The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation and consequently may affect TDD synchronisation.

In many CEPT countries, the same frame format is effectively mandated both indoor and outdoor in the 3400-3800 MHz frequency band. Several frame structures for TDD MFCN networks have been recommended by ECC to facilitate synchronisation in the frequency band 3400-3800 MHz at boarder areas. However, unsynchronised or semi-synchronised operation of TDD MFCN networks are not precluded with certain requirements and/or procedures of cross-border coordination between administrations.

However, unsychronised or semi-synchronised operation of TDD MFCN networks are not precluded with certain requirements and/or procedures of cross-border coordination between administrations. It is already possible today to use different TDD frame structure for isolated deployment.

13.2 Region 2

13.2.1	North America

No TDD pattern has been mandated in US, nor in Canada, but operators are encouraged to coordinate their network deployment and make sure they don’t interfere with each other.

Unsynchronized operation is allowed, more stringent regulation parameters have not been specified for such case but, again, operators would have to work their differences to avoid any claim to FCC/ISED.

13.3 Region 3

13.3.1	China

In China, spectrum is allocated with clearly stating it for TDD or FDD operation. There is no SBFD regulatory requirements in China until now. MIIT mainly cares interference between different operators. Necessary interference coordination mechanism and solutions may be proposed by MIIT to avoid interference before any SBFD deployment.

13.3.2	Japan

No TDD pattern has been mandated in Japan but operators are required to coordinate their network deployment to avoid interference. Operators are allowed to use unsynchronized operation as far as there is no interference with the adjacent network(s), e.g. for indoor usage.

13.4	Summary

The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation and consequently may affect TDD synchronisation which will lead to potential interference to incumbent services.

Changes to current regulations may be required to allow the operation of SBFD. Therefore, suggestions to relevant administrative authorities are needed based on the results of co-existence studies between SBFD and legacy TDD system, as well as the consequent performance results defined for the operation of SBFD.

- Recommended WF

  - Discussion based on TP draft directly in 1st round

  - The company responsible for this part of TP shall be assigned for revised TP.

## Companies views’ collection for 1st round

### Open issues

Moderator:  Discussion in below table to collect views on TPs directly.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304024 (TP from Spark and Ericsson) | Spark: There was topic leader ( Cable Labs) that was appointed in the Athens meeting. The TPs for regulatory text should be collated by the topic leader. |
|  | ZTE: How to understand the following wording? avoid any claim to FCC/ISED. Is that possible to confirm with reach regions? E.g co-signed by operators from each region. |
| R4-2304539 (TP from Nokia) | Ericsson: The conclusion outside Europe would need further clarifications, it assumes SBFD will not create any additional interference to adjacent network or services, comparing to a synchronized TDD network. Also, the text is focused on 3400-3800MHz, which is only one band considered in this WI. |
| R4-2305207 (TP from Samsung) | Ericsson: For Europe, it should be clarified that the requirements for unsynchronized operations are much more stringent. We could merge the summary with ours. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #5-1 | Tentative agreements: N/A Candidate options: N/A Recommendations for 2nd round: - Moderator suggest to ask CableLabs (the company which is responsible for this regulatory chapter based on TR work splitting) to lead a WF, which capture the different views on concerns issues to be listed in WF. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2304024 R4-2304539 R4-2305207 | Postponed and further merged to big TP to regulatory chapter: based on WF (to be led by CableLabs) to further discuss the different points among three versions of TPs firstly in this meeting. |

## Discussion on 2nd round (if applicable)

2nd round discussion is performed on WF directly.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on implementation feasibility of SBFD: Self-interference | Samsung |  |
|  | WF on implementation feasibility of SBFD: Co-channel Co-site/inter-site interference | Ericsson |  |
|  | WF on BS RF requirement impact for SBFD | CATT |  |
|  | WF on regulatory aspects for SBFD | CableLabs |  |
|  | Reply LS on interference modelling for duplex evolution | Samsung | To: RAN_1 Reply to R1-2302087 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| All TPs |  |  |  | Postponed |  |
| All other Tdocs |  |  |  | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

## 2nd round

| New Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2305916 | WF on implementation feasibility of SBFD: Self-interference | Samsung | Recommend to be treated in final round GTW. | Based on v08 version, Different views on the Issue 2-4-4 |
| R4-2305917 | WF on implementation feasibility of SBFD: Co-channel Co-site/inter-site interference | Ericsson | Agreeable | Based on v07 version |
| R4-2305918 | WF on BS RF requirement impact for SBFD | CATT | Agreeable | Based on v06 version |
| R4-2305919 | WF on regulatory aspects for SBFD | CableLabs | Agreeable | Based on v05 version |
| R4-2305920 | Reply LS on interference modelling for duplex evolution | Samsung | Recommend to be treated in final round GTW. | Based on v09 version Different views on three parts: (1) spatial isolation value/range for co-site adjacent channel interference diverged views (2) the bullet “In current RAN4 adjacent-channel co-existence simulation, grid shift uses 100% as baseline and no evaluation for the case with 0% grid shift has been performed yet.” different views on this (3) the NF model 10dB improvement for FR1 WA BS, and the NF models for other FR1 BS classes and FR2-1 BS. need GTW discussion |