3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2305995

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			6.8.7.1

Source:	Moderator (MediaTek)

Title:	Topic summary for [106bis-e][326] IoT_NTN_Demod_Part1

Document for:	Information

Introduction

This summary covers the contributions submitted under the agenda 6.8.7.1 for IoT-NTN UE demodulation requirements.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information for companies

| Company | Contact | Email address |
| --- | --- | --- |
| MediaTek | Licheng Lin | licehng.lin@mediatek.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Qualcomm | Dhananjaya Ponukumati | dsarma@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Demodulation requirements for IoT-NTN

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304881 | Qualcomm Incorporated | Proposal 1: NBIoT: Proposed SNR numbers in Table 2.1 for alignment purpose Proposal 2: For NBIoT test 2, propose NPDCCH repetition factor be 256 Proposal 3: eMTC: Proposed SNR numbers in Table 3.1 for alignment purpose Proposal4: Propose to use AT commands and no GNSS receiver for UE location information |
| R4-2304917 | MediaTek | Draft CR to TS36.102, PDSCH requirements for NB-IoT over NTN |
| R4-2304918 | MediaTek | Proposal 1: Add “Annex E: propagation conditions” in TS36.102. |
| R4-2305175 | Ericsson | Proposal 1: For Cat-M1 PDSCH test 1, add the following note in FRC: The downlink subframes are scheduled at the 8th and 9th subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe). Proposal 2: For Cat-M1 PDSCH test 2, add the following note in FRC: PDSCH subframes are scheduled at the 10th to 17th subframes every period=32 ms. Information bit payload is available at the 10th to 17th subframes with repetition. (Starting from the 0th subframe) Proposal 3: For Cat-M1 PDSCH test 3, add the following note in FRC: PDSCH subframes are scheduled at the 66th to 129th subframes every period=160 ms. Information bit payload is available at the 66th to 129th subframes with repetition. (Starting from the 0th subframe) |
| R4-2305176 | Ericsson | draft CR: Introduction of eMTC UE demodulation requirements for NTN |
| R4-2305488 | Huawei, HiSilicon | Draft CR on IOT NTN demodulation performance requirements (TS36.102, Rel-18) |
| R4-2305489 | Huawei, HiSilicon | Simulation results |

Open issues summary

Issue 1: Specification skeleton

- Proposals

  - Option 1 (MediaTek): Add “Annex E: propagation conditions” in TS36.102.

- Recommended WF

  - Agree Option 1.

- Conclusion from the offline GTW discussion on April 18th:

  - Agree Option 1.

Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements

- Agreement in RAN4#106: - dropping/puncturing MPDCCH/PDSCH when MPDCCH/PDSCH subframe collides with PSS/SSS/PBCH/SI subframe - The following DL scheduling pattern are agreed to be the baseline and companies are encouraged to bring simulation results based on the following DL scheduling pattern. - Other scheduling patterns are not precluded. - For Cat-M1 PDSCH test 1: - For Cat-M1 PDSCH test 2: - For Cat-M1 PDSCH test 3:

- Proposals

  - Option 1 (Ericsson):

    - For Cat-M1 PDSCH test 1:

      - The downlink subframes are scheduled at the 8th and 9th subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe).

    - For Cat-M1 PDSCH test 2:

      - PDSCH subframes are scheduled at the 10th to 17th subframes every period=32 ms. Information bit payload is available at the 10th to 17th subframes with repetition. (Starting from the 0th subframe)

    - For Cat-M1 PDSCH test 3:

      - PDSCH subframes are scheduled at the 66th to 129th subframes every period=160 ms. Information bit payload is available at the 66th to 129th subframes with repetition. (Starting from the 0th subframe)

- Recommended WF

  - In RAN4#106, RAN4 agreed to drop/puncture MPDCCH/PDSCH when MPDCCH/PDSCH subframe collides with PSS/SSS/PBCH/SI subframe.

  - Although the scheduling of SIBs (except SIB1-SR) is up to eNB implementation, we can still follow the previous agreement to define PDSCH requirements if the scheduling for SIB in previous is feasible.

  - Can we follow the previous agreements in RAN4#106?

- Conclusion from the offline GTW discussion on April 18th:

  - Agree with Cat-M1 PDSCH test 1 from Ericsson’s proposal. (schedule two PDSCH subframes in one radio frame)

  - Agree with Cat-M1 PDSCH test 2 and test 3 from Huawei’s proposal.

  - Add a note in TS36.102: MPDCCH and PDSCH are dropped when colliding with SIB2/SIB3.

Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements

- Proposals

  - Option 1 (Qualcomm): 256

- Recommended WF

  - Agree Option 1.

- Offline GTW discussion on April 18th:

  - Huawei: more simulation about NPDCCH to check

  - Ericsson: same view as Huawei

- Conclusion from the offline GTW discussion on April 18th:

  - Continue the discussion

Issue 4: UE location information

- Proposals

  - Option 1 (Qualcomm): Use AT commands and no GNSS receiver for UE location information

- Recommended WF

  - Collect inputs.

- Offline GTW discussion on April 18th:

  - Huawei: No need for UE location information in demodulation, only assume 0.1 ppm frequency error.

  - Ericsson: Do not need to consider AT command. Follow the same principle in NR-NTN.

  - QC: Not for simulation results but for demodulation conformance test.

- Conclusion from the offline GTW discussion on April 18th:

  - Continue the discussion

Issue 5: Simulation results alignment

- The simulation results are captured in the following link. Companies are encouraged to check the values and fill in or update the results.

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_106bis-e/Inbox/Drafts/%5B106bis-e%5D%5B326%5D%20IoT_NTN_Demod_Part1/simulation%20results/draft%20R4-2304919%20Summary%20of%20simulation%20results%20for%20IoT-NTN%20UE%20demodulation%20requirements.xlsx

Companies views’ collection for 1st round

Open issues

| Company |  | Comments |
| --- | --- | --- |
| Huawei |  | Issue 1: Specification skeleton Agree with the recommended WF. Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements We think the transmission for other SIBs such as SIB2/3 is needed for this case, it is RAN5 default configuration for the test setup. The difference for the NTN case comparing to the legacy case is that only 1.4MHz bandwidth should be considered, the collision cannot be avoided. So we prefer to keep previous agreement. Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements Further simulation is needed to see whether current NPDCCH BLER performance is enough at the 70% NPDSCH maximum throughput point. Currently we prefer to keep previous agreement and do further simulation until next meeting to do final decision. Issue 4: UE location information From our understanding, there is no needed to explicitly model UE location information for demodulation requirements. Only maximum residual frequency error that is 0.1 ppm defined in RF session should be explicitly modelled. |
| Ericsson |  | Issue 1: Specification skeleton We support the recommended WF. Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements For Test 1, we prefer to schedule 2 DL within 1 radio frame, rather than 1 DL, which can shorten the test time. For Test 3, we are generally fine with the previous agreements, but we are wondering if we need to specify SIB2/SIB3 in the scheduling. However we are ok to capture the following note in the FRC table like: - MPDCCH/PDSCH is dropped in the subframes SIB2/SIB3 is scheduled’. We suggest to review the FRC in the draft CR. Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements We prefer to keep the current assumption, 128. We have developed the method to increase SNR level during the NPDCCH transmission to avoid larger repetition level in TS36.101. We think Rep=128 should be ok if we reuse the same methodology. However if Qualcomm have concern for Rep=128 for NPDCCH, we are ok to study. Issue 4: UE location information We are not sure it is the scope of RAN4 demodulation requirements. We would like to understand the reason to program the UE location manually in IoT-NTN UE demodulation tests. |
| MediaTek |  | Issue 1: Specification skeleton Support the recommended WF. Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements We understand the intention from Ericsson to shorten the test time for case 1. However, we are not sure how to schedule SIB2/SIB3 in the proposed method. Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements We agree with Ericsson that there are already some methods to increase the SNR of NPDCCH to prevent the performance of NPDCCH is the bottleneck for PDSCH requirements. (Note 1 and Note 2 in TS36.101 Table 8.12.1.1.2-1) As far as I remember, we did not discuss about the PDCCH repetition in the previous meetings. According to TS36.101 Table 8.12.1.1.2-1, the NPDDCH repetition number is 256 for test number 2 which is used in our simulation. I think all companies share the same view to leverage the configuration of test cases in TS36.101 as much as possible. Therefore, in the draft CR R4-2304917, we use the same NPDSCCH repetition 256 as in TS36.01 for the test case 2. Issue 4: UE location information We are also wondering if it is within the scope of RAN4 demodulation requirements. Maybe we do not need to consider this in UE demodulation requirements. |
| Ericsson2 |  | Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements According to the offline GTW, we propose the following FRC for eMTC tests, based on the TS36.101 R.79/80/81 FDD. Parameter; Unit; Values. Reference channel; R.xx FDD; R.yy FDD; R.zz FDD. Channel bandwidth; MHz; 10; 10; 10. Allocated resource blocks; Note 3; 6; 6. Allocated PDSCH subframes (Note 7); Note 4; Note 5; Note 6. Modulation; 16QAM; QPSK; QPSK. Target Coding Rate; 1/2; 1/3; 1/10. Information Bit Payload. For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; 744; 504; 152. Number of Code Blocks. For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Code blocks; 1; 1; 1. Binary Channel Bits. For Sub-Frames 0,1,2,3,4,5,6,7,8,9; Bits; 1656; 1656; 1656. Max. Throughput averaged over one period; Mbps; 0.149; 15.75; 0.950. UE DL Category; M1; M1; M1. Note 1: 2 symbols allocated to control channel region. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: Allocated PRB positions for PDSCH are {3, 4, 5} within the assigned narrowband. Allocated PRB positions for MPDCCH are {0, 1} within the assigned narrowband. Note 4: The downlink subframes are scheduled at the 8th and 9th subframes every period (10ms). Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe). Note 5: PDSCH subframes are scheduled at the 10th to 17th subframes every period (32ms). Information bit payload is available from the 10th to 17th subframes with repetition. (Starting from the 0th subframe) Note 6: PDSCH subframes are scheduled at the 96th to 159th subframes every period (160ms). Information bit payload is available at the 96th to 159th subframes with repetition. (Starting from the 0th subframe) Note 7: MPDCCH and PDSCH are dropped when colliding with SIB2/SIB3.. Note 1: Max Tput over one period is derived as follows: - Test 1: 744bits / 10ms - Test 2: 504bits / 32ms - Test 3: 152 bits / 160ms |
| Qualcomm | Issue 1: Specification skeleton Support the recommended WF. Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements Our view is that scheduling pattern be ensured so that there is no UL ACK/NACK collision with SF4 or any SIB2/3. If this happens, is ACK/NACK given higher priority over DL SIB/MIB read? Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements Thanks, Ericsson, for commenting on the flexibility to change PDCCH SNR with N_oc1 or N_oc2 parameters. In our simulation, both values were the same, and hence the earlier proposal to increase the MPDCCH repetition. In the light of this information, we are OK to either choice of MPDCCH repetition number. Issue 4: UE location information Our view is that location of UE is required for timing requirements apart from DL frequency error precompensation. We would like the testing to be independent of GNSS receiver. We request companies to consider agreeing that GNSS receiver is not mandatory/essential while testing UE demod performance requirements. |  |
| Rohde & Schwarz | Issue 4: UE location information RAN5 is currently discussing how to handle the required location information for NR NTN testing. In our view, for IoT NTN we should follow the same approach that RAN5 agrees for NR NTN. From Test system viewpoint we would like to avoid two different implementations for these 2 Use Cases and follow a common approach. |  |
| Ericsson3 | Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements For Qualcomm, according to TS36.211 Table 6.4.1-1, SIB1-BR is scheduled in SF#4 every 20ms. For test 1, SF#14, 34, 54, … do not transmit SIB1-BR and these subframes can be used for HARQ-ACK transmission. For SIB2/SIB3 it is up to RAN5, but UE need to transmit HARQ-ACK on the subframe where SIB2/SIB3 is scheduled, HACK-ACK should be prioritized according to RAN1 spec, however we don’t need to capture it in the FRC. Issue 4: UE location information Thank you R&S for your information. We agree RAN4 should apply same approach that RAN5 agrees for NR NTN. We propose RAN4 come back this issue when RAN5 reach conclusion. |  |
| MediaTek2 | Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements We are also OK with PDCCH repetition 128 to reduce the testing time. Issue 4: UE location information We do not know how to capture this in RAN4 specification if we agree to use AT commands for UE location information, and we think this can be determined in RAN5 alone. Therefore, we suggest not to discuss this issue in RAN4 and leave RAN5 to make the decision. |  |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304917 (MediaTek) | Title: Draft CR to TS36.102, PDSCH requirements for NB-IoT over NTN |
|  | Ericsson: See our comments in ‘R4-2304917 Draft CR to TS36.102, PDSCH requirements for NB-IoT over NTN_v01_Ericsson.docx’ in ‘draft CRs’. |
|  | MediaTek: Thanks for comments. Basically, we agree with the modifications. The unit of k_offset will be modified to “ms" following the definition in TS36.331 as below. k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.. However, the OCNG and NPDCCH reference channel should be captured in Annex A prepared by Qualcomm. |
| R4-2305176 (Ericsson) | Title: draft CR: Introduction of eMTC UE demodulation requirements for NTN |
|  | MediaTek2: We consider the dedicated spectrum for Cat-M1 over NTN. Therefore, we do not need the field “Number of OFDM symbols for PDCCH per component carrier” in Table 8.2.1.1-1. |
|  | Ericsson5: We agree with the comment by MediaTek5. We will revise it. |
| R4-2305488 (Huawei) | Title: Draft CR on IOT NTN demodulation performance requirements (TS36.102, Rel-18) |
|  | Ericsson: See our comments in ‘R4-2305488 Draft CR on IOT NTN demodulation performance requirements_v01_Ericsson.docx’ in ‘draft CRs’. |
|  | MediaTek: We have agreements in R4-2220278 as below and we think these should be captured in TS36.102. - Adopt the following applicability rule for Cat-M1 UE for NTN. ntn-Connectivity-EPC-r17; ntn-ScenarioSupport-r17; Applicability. Supported; GSO only; UE needs to pass TS36.101 Cat-M1 requirements only according to the UE capability. Supported; NGSO only; UE needs to pass TS36.101 Cat-M1 requirements and the additional NGSO test in TS36.102 according to the UE capability. Supported; Not specified (supporting both GSO and NGSO); UE needs to pass TS36.101 Cat-M1 requirements and the additional NGSO test in TS36.102 according to the UE capability. Not supported; -; UE needs to pass TS36.101 Cat-M1 requirements only according to the UE capability. - Adopt the following applicability rule for Cat-NB1/NB2 UE for NTN. ntn-Connectivity-EPC-r17; ntn-ScenarioSupport-r17; Applicability. Supported; GSO only; UE needs to pass TS36.101 Cat-NB1/NB2 requirements only according to the UE capability. Supported; NGSO only; UE needs to pass TS36.101 Cat-NB1/NB2 requirements and the additional NGSO test in TS36.102 according to the UE capability. Supported; Not specified (supporting both GSO and NGSO); UE needs to pass TS36.101 Cat-NB1/NB2 requirements and the additional NGSO test in TS36.102 according to the UE capability. Not supported; -; UE needs to pass TS36.101 Cat-NB1/NB2 requirements only according to the UE capability.. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1: Specification skeleton | Agreements: - Add “Annex E: propagation conditions” in TS36.102 Recommendation for 2nd round: - No discussion |
| Issue 2: DL scheduling for Cat-M1 PDSCH demodulation requirements | Agreements: - Agree with Cat-M1 PDSCH test 1 from Ericsson’s proposal. (schedule two PDSCH subframes in one radio frame) - Agree with Cat-M1 PDSCH test 2 and test 3 from Huawei’s proposal. - Add a note in TS36.102: MPDCCH and PDSCH are dropped when colliding with SIB2/SIB3. Recommendation for 2nd round: - No discussion |
| Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements | Tentative agreements: - For NB-IoT PDSCH demodulation requirements, set NPDCCH repetition factor to be 128 in test 2. Recommendation for 2nd round: - Discuss if the tentative agreements are accepted. |
| Issue 4: UE location information | Tentative agreements: - Leave RAN5 to discuss this issue. If the decision of RAN5 impacts the UE demodulation requirements, RAN4 can come back for discussion. Recommendation for 2nd round: - Discuss if the tentative agreements are accepted. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2304917 (MediaTek) | To be revised |
| R4-2305176 (Ericsson) | To be revised |
| R4-2305488 (Huawei, HiSilicon) | To be revised |

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on IoT-NTN UE demodulation requirements | MediaTek |  |
|  | Big CR to TS36.102: Introduction of IoT-NTN UE demodulation requirements | MediaTek | For email approval |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2304917 |  | Draft CR to TS36.102, PDSCH requirements for NB-IoT over NTN | MediaTek | Revised |  |
| R4-2305176 |  | draft CR: Introduction of eMTC UE demodulation requirements for NTN | Ericsson | Revised |  |
| R4-2305488 |  | Draft CR on IOT NTN demodulation performance requirements (TS36.102, Rel-18) | Huawei, HiSilicon | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2305879 |  | Draft CR to TS36.102, measurement channels for IoT-NTN | Qualcomm | Agreeable |  |
| R4-2305960 |  | WF on IoT-NTN UE demodulation requirements | MediaTek | Agreeable |  |
| R4-2305961 |  | Big CR to TS36.102: Introduction of IoT-NTN UE demodulation requirements | MediaTek | Agreeable | Discuss under post-meeting email process |
| R4-2305962 |  | Draft CR to TS36.102, PDSCH requirements for NB-IoT over NTN | MediaTek | Agreeable |  |
| R4-2305963 |  | draft CR: Introduction of eMTC UE demodulation requirements for NTN | Ericsson | Agreeable |  |
| R4-2305964 |  | Draft CR on IOT NTN demodulation performance requirements (TS36.102, Rel-18) | Huawei, HiSilicon | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents