3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2306000

Electronic Meeting, Apr 17th ‒ Apr 26th, 2023

Title: 	WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

Agenda Item:	5.8.4

Source: 	Qualcomm

Document for:	Approval

Moderator’s note: For all the open Issues listed in the WF below, other options are not precluded unless explicitly specified.

# Topic #1: General Aspects for FR2 Multi-Rx Demod

Issue 1-1-1: On  new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements.

<Way forward >

- Proposals:

- Option 1:

  - Use RH approach to generate OTA chamber internal H matrix for M-Rx 2 layer LLS simulations defined as:
RH= 1ɳ1*ɳ1|ɳ1|2ɳ2*1ɳ1ɳ2*ɳ1ɳ2ɳ2*ɳ11ɳ1*|ɳ2|2ɳ2ɳ2*1 ⊙RH, where ⊙ is the elementwise product

  - Define requirements using the following configurations of AoA in the OTA chamber for 2-layer (1+1) case, with corresponding relative sidelobe antenna gains of:

- AoA30: ɳ_1= ɳ_2= 0.1

- AoA90: ɳ_1= ɳ_2= 0.01

- AoA150: ɳ_1= ɳ_2= 0.03

- Option 2:

    - Reuse the current MIMO correlation matrix calculation methods defined in RAN4 spec to generate correlation matrix for each TRP-to-RX MIMO channel matrix Hi,j. Channel matrices from different TRPs to a Rx chain, i.e., Hi,1 vs Hi,2, are assumed to be statistically independent.

  - MIMO channel matrices from a TRP to two Rx chains, i.e., Hi,1 vs Hi,2, can be correlated, and the correlation coefficient αB between two Rx chains can be categorized as low (independent), medium and high which may depend on the AoA offset between two beam pairs. FFS: exact value of αB.

Issue 1-1-2: whether to assume correlation matrix approach for simulation and requirements derivation only.

- Agreement:

- Assume correlation matrix approach for simulation and requirements derivation only.

Issue 1-1-3: Whether to place the test probes at offset associated with the assumed correlation for OTA setup.

- Agreement:

- Place the test probes at offset associated with the assumed correlation for OTA setup.

Issue 1-1-4: Whether to Prioritize definition of new correlation matrix for demodulation requirement with multi-RX in FR2for OTA setup.

- Agreement:

- Prioritize definition of new correlation matrix for demodulation requirement with multi-RX in FR2for OTA setup

Issue 1-1-5: Best beam pair selection for demodulation performance requirements.

- Agreement:

- Wait for agreement in RF and/or RRM session on metric for best beam pair selection and discuss if applicable and feasible for demod setup.

- UE must maintain the beam from each panel fixed during each multi-Rx performance test

Issue 1-1-6: Whether to study cross-talk between TRPs after Rx beamforming is achieved.

- Agreement:

- Consider cross-talk between TRPs captured via the correlation matrix as being discussed under Issue 1-1-1

Issue 1-1-7: Assumption on UE Antenna module.

- Agreement:

- Don’t explicitly include antenna module placement in demod discussion; can be considered through correlation modelling under Issue 1-1-1 (if needed).

Issue 1-1-8: Whether to study applicability rule for UE panel deployment.

- Agreement:

- Don’t consider applicability rule for UE panel deployment.

Issue 1-1-9: Whether to consider MCS reduction for IFF method (if agreed for demodulation requirement).

- Agreement:

- Discuss later once achievable MCS is agreed.

Issue 1-1-10: Delta AoA assumption for single DCI

- Agreement:

- Further evaluate the performance with the full set of AoA offset from RF session, down-selection on the values for performance requirements introduction can be carried based on evaluation results and analysis from companies.

Issue 1-1-11: Delta AoA assumption for multi- DCI fully overlapping.

- Agreement:

- Further evaluate the performance with the full set of AoA offset from RF session, down-selection on the values for performance requirements introduction can be carried based on evaluation results and analysis from companies.

Issue 1-1-12: Whether to introduce requirements for both single-DCI and multi-DCI scenarios.

- Agreement:

- Keep the current agreement of transmissions schemes and layer combinations for evaluation for companies to provide initial simulation results. Consider both sDCI with SDM and mDCI with fully overlapping resources, prioritize mDCI with fully overlapping.

Issue 1-1-13: Performance requirement for multi- DCI non-overlapping.

- Agreement:

  - Define at least one set of performance requirements for multi-DCI non-overlapping (if agreed to introduce requirements).

Issue 1-1-14: Receiver assumption.

<Way forward >

- Proposals

  - MMSE-IRC receiver

    - Option 1: 2x2 Per TRP

    - Option 2: Jointly processing as 4x4

    - Companies are encouraged to further evaluate the performance, UE processing complexity impact on above options.

Issue 1-1-15: Whether to study isolation level between two parallel TRP-to-RX links for 2x2 independent UE processing.

- Agreement:

- No need to consider isolation level separately. The isolation level between two parallel TRP-to-Rx links are inherently considered as part of the correlation model discussed under Issue 1-1-1.

Issue 1-1-16: PN model and PN compensation.

- Agreement:

- Leave PN impact and modelling to companies’ implementation, the impact can be considered into results submitted by companies and the impact on test parameters i.e., MCS selection subject to companies’ results.

Issue 1-1-17: Assumption on FFT window.

- Agreement:

  - Consider single FFT window as baseline assumption for FR2 multi-Rx demodulation requirements definition.

# PDSCH Demodulation Requirements

### Sub-topic 2-1: Test scope for PDSCH demodulation requirements

Issue 2-1-1: AoA assumption

- Agreement:

    - Do not define demodulation requirements for all AoA offsets considered in RF requirements.

    - Define demodulation requirements for 1 or 2 correlation types based on feasibility and performance evaluation.

Issue 2-1-2: Power imbalance between TRPs

- Agreement:

    - Do not introduce power imbalance between the TRPs for FR2 multi-Rx demod requirements.

### Sub-topic 2-2: Simulation assumptions for PDSCH performance evaluation

Issue 2-2-1: Channel model

<Way forward >

- Proposals

  - Option 1: Consider including TDLA30-300 for 120kHz/200MHz requirements, if introduced

  - Option 2: Consider TDLA30-300 for 100 MHz/120 kHz.

  - Option 3: Consider TDLA30-75 for 100MHz/120KHz

Issue 2-2-2: SCS/BW

- Agreement:

  - Define 120kHz/100MHz for SCS/Bandwidth as first priority.

Issue 2-2-3: MIMO layers per TRP

- Agreement:

  - Consider the following for initial evaluation-

    - Single-DCI based SDM scheme using 1+1- and 2+2-layer combination

    - Multi-DCI based fully-overlapping resource allocation using 2+2-layer combination

    - Multi-DCI with non-overlapping resource allocation using 2+2-layer combination.

Issue 2-2-4: MCS

- Agreement:

  - Consider the following MCS for initial evaluation-

    - Use MCS 13 or lower for defining rank 2+2, higher inter-TRP interference cases with SDM/fully-overlapping scheduling.

    - Use MCS 17 for rank 1+1 cases or lower inter-TRP interference cases with SDM or non-overlapping case.

Issue 2-1-5: PTRS Port

<Way forward >

- Proposals

  - Option 1: One PTRS port per TRP for both sDCI and mDCI schemes

  - Option 2: One port PTRS for s-DCI SDM scheme

Issue 2-1-6: PTRS Pattern

- Agreement:

  - For PTRS pattern, consider K=2, L=1 as the frequency and time density, respectively.

Issue 2-2-7: Time offset between TRPs

<Way forward >

- Proposals

  - Option 1: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}.

  - Option 2: Use time offset of TRP1 with respect to TRP2 as {0.2us, -0.05us} for different test cases.

  - Option 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI and mDCI based SDM scheme with timing offset between TRxPs as follow

    - Timing offset of the second TRxP from the first TRxP [us] as -0.0625 or 0.25.

  - Option 4: Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases.

|  |  | Timing offset[us] |
| --- | --- | --- |
| Multi-DCI based multi-TRP | non-overlapping | -0.0625 |
|  | full-overlapping | -0.0625 |
| Single-DCI based multi-TRP | SDM | 0.25 |

Issue 2-2-8: Frequency offset between TRPs

<Way forward >

- Proposals

- Option 1: Select 3000Hz frequency offset for all demodulation cases as starting point.

- Option 2: 600Hz

- Option 3: 400Hz

Issue 2-2-9: TDD slot pattern

- Agreement:

  - Define the FR2 multi-Rx UE demodulation requirements for PDSCH using FR2.120-1 TDD UL-DL pattern, given by DDDSU and S: 10D+2G+2U.

Issue 2-2-10: Test cases and simulation parameters for sDCI SDM

- Agreement:

  - Further discuss after relevant open issues are finalized.

Issue 2-2-11: Test cases and simulation parameters for mDCI fully overlapping

- Agreement:

  - Further discuss after relevant open issues are finalized.

Issue 2-2-12: Test cases and simulation parameters for mDCI non-overlapping

- Agreement:

  - Further discuss after relevant open issues are finalized.

Issue 2-2-13: Performance metric

- Agreement:

  - Define the FR2 multi-Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

Issue 2-2-14: Whether to consider additional margin for 47GHz carrier frequency

- Agreement:

  - Discuss later when requirements are defined.

# CSI Reporting Requirements

### Sub-topic 3-1: Test scope for CSI reporting requirement for FR2 multi-Rx

Issue 3-1-1: Whether to evaluate feasibility of introducing PMI reporting requirement for sDCI SDM scheme.

- Agreement:

  - Evaluate feasibility of introducing PMI reporting requirement for sDCI SDM scheme.

Issue 3-1-2: Whether to introduce PMI reporting requirement for mDCI scheme.

- Agreement:

  - Don’t introduce PMI reporting requirement for mDCI scheme.

### Sub-topic 3-2: Test setup for CSI reporting requirement

Issue 3-2-1: Whether to evaluate performance after defining correlation matrix to model spatial aspects.

- Agreement:

  - Evaluate performance after defining correlation matrix to model the spatial aspects.

### Sub-topic 3-3: Simulation assumptions

Issue 3-3-1: MIMO layer

- Agreement:

  - Consider 1 MIMO layer per TRP.

Issue 3-3-2: Number of CSI-RS ports

<Way forward >

- Proposals

    - Option 1: 8-port per TRP.

    - Option 2: Define CSI requirements with 4-port per TRP as a minimum. In addition to requirements with 4-port per TRP, requirements with 8-port per TRP can be included.

    - Option 3: 4-port per TRP.

    - Option 4: 2-port per TRP.

Issue 3-3-3: MCS

- Agreement:

  - Consider MCS13 for initial evaluation.

Issue 3-3-4: Whether to assume LI in report quantity for multi-Rx FR2

<Way forward >

- Proposals

    - Option 1: Include LI in the report quantity to facilitate PTRS port mapping.

    - Option 2: No

Issue 3-3-5: Whether to assume time and frequency offsets.

- Agreement:

  - Don’t consider time and frequency offsets for CSI reporting requirements.

Issue 3-3-6: Simulation parameters for PMI reporting requirement for sDCI SDM scheme

- Agreement:

  - Further discuss after relevant open issues are finalized including the TDD slot pattern.