3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2306174

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			5.32.4

Source:	Moderator (Ericsson)

Title:	Topic summary for [106bis-e][226] NR_redcap_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

RRM contributions from agenda item 5.32.4, related to Rel-18 eRedCap WI, are discussed here.

# Topic #1: Extended eDRX for RRC INACTIVE

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304153 | Apple | Proposal 1: For both serving cell and neighbor cell requirements in RRC Inactive state for eDRX > 10.24, - the long eDRX cycle (>10.24 s) value range is from 20.48sec to 10485.76sec - 1.28s ≤ PTW length ≤ 40.96s, with 1.28s granularity Proposal 2: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements, the requirement shall be specified based on T like in R17 Inactive mode requirement with eDRX, and T can be referred to RAN2 definition in TS38.304. Proposal 3: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements, the detection/measurement/evaluation delay requirement shall be specified in the step of: - INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec - DRX cycle, if eDRX_Inactive 20.48sec Proposal 4: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, UE shall perform measurements/evaluation within single PTW(s) for both IDLE eDRX and INACTIVE eDRX. Proposal 5: new CG-SDT requirement shall be specified for Rel-18 eRedCap UE with PTW. Details are FFS. Proposal 6: RAN4 to discuss the UE behavior and corresponding requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec). |
| R4-2304169 | ZTE Corporation | Proposal 1: Idle state eDRX requirements for eDRX cycle > 10.24 sec are used as baseline for Inactive state. |
| R4-2304415 | CATT | Proposal 1: For serving cell and neighbour cell requirements in RRC Inactive state for eDRX > 10.24, Idle state eDRX requirements for eDRX cycle > 10.24 sec are used as baseline. - Maximum INACTIVE (RAN paging) eDRX length = 10485.76s - 1.28s ≤ PTW length ≤ IDLE state PTW length, with 1.28s granularity Proposal 2: Before RAN4 specify requirements, the eDRX cycle and PTW length should first be determined by RAN2/CT/SA. Proposal 3: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, measurement requirements on serving cell wait for RAN2 conclusion. Proposal 4: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, UE performs intra-frequency/inter-frequency measurement according to a fixed period. Proposal 5: New CG-SDT requirement shall be specified for Rel-18 eRedCap UE with PTW. - Prefer CG-SDT SDT requirement is only used within PTW. |
| R4-2304515 | vivo | Proposal 1: Regarding serving cell/neighbor cell inactive requirement, it is ok to wait for further input before staring the specification work. Proposal 2: When configured with both IDLE and INACTIVE eDRX configurations for serving cell and neighbour cell measurements, ok to discuss whether requirements are based T. Proposal 3: When to measure when configured with both IDLE and INACTIVE eDRX configurations for serving and neighbour cell measurements, UE shall perform measurements within PTW(s) for both idle eDRX and inactive eDRX, i.e., option 1. |
| R4-2304598 | MediaTek inc. | Proposal 1: The IDLE mode eDRX requirements for eDRX cycle > 10.24s can be used as baseline for serving and neighbouring cells requirements in INACTIVE mode. Proposal 2: RAN4 to wait for RAN2 conclusion for the scenario when UE is configured with both IDLE and INACTIVE eDRX configurations for serving and neighbouring cell measurements. Proposal 3: RAN4 to discuss the necessity to define CG-SDT requirements with PTW before defining the requirements. |
| R4-2304784 | Xiaomi | Proposal 1: RAN4 to take the assumptions on measurements on eDRX with PTW agreed for IDLE eDRX as baseline for INACTIVE eDRX: - the number of samples needed for Nserv of serving cell measurement (measured in DRX cycles) must be contained in a single PTW length, - the number of samples needed for Tmeasure,NR /Tevaluate,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) must be contained in a single PTW length, - the number of samples needed for Tdetect,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) could be splitted into different PTWs. Proposal 2: RAN4 to take the requirements for IDLE eDRX > 10.24s as baseline for enhanced INACTIVE eDRX: - For the UE in RRC_INACTIVE configured with both IDLE and INACTIVE eDRX cycle longer than 10.24s, UE performs serving cell measurement based on T and corresponding PTW determined in RAN2; - For the UE in RRC_INACTIVE configured with both IDLE and INACTIVE eDRX cycle longer than 10.24s, UE performs intra-frequency /inter-frequency measurement based on RAN paging and corresponding RAN PTW. Proposal 3: RAN4 to define SDT requirements for Rel-18 eRedCap based on RAN2 progress. |
| R4-2305286 | Huawei, HiSilicon | Proposal 1: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, UE shall NOT perform measurements outside PTW(s) for idle eDRX and inactive eDRX. Proposal 2: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, measurement requirements on serving cell wait for RAN2 conclusion. Proposal 3: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, define intra-frequency/inter-frequency measurement requirements according to a fixed period where the fixed period can be further studied. |
| R4-2305569 | Nokia, Nokia Shanghai Bell | - Idle state eDRX requirements for eDRX > 10.24 sec are used as baseline for serving cell requirements and neighboring cell requirements in RRC Inactive state. For Enhanced INACTIVE eDRX, the RRM core requirements should be updated in subclauses 5.1B.2.2, 5.1B.2.3 and 5.1B.2.4 of TS 38.133. |
| R4-2305732 | Ericsson | - For eDRX cycles > 10.24 seconds, the measurement rules in Table 2 for serving cell and neighbour cell measurements from Rel-17 RedCap are reused. - For R18 RedCap UE in INACTIVE state with both IDLE and INACTIVE eDRX configurations larger than 10.24s with coinciding PTWs, UE shall perform measurements within PTW(s) for both idle eDRX and inactive eDRX. RAN4 to clarify the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding. RSRP2 time range in FR1 with eDRX cycle > 10.24 sec is defined as (T2 –640ms) ≤ T2’ ≤ T2. |
| R4-2305802 | Qualcomm Incorporated | Proposal 1: RAN4 to specify cell-reselection requirements to support eDRX >10.24s in the RRC_INACTIVE mode with - PTW length value range of 1.28s ≤ PTW length ≤ 40.96s, with 1.28s granularity. - Long eDRX cycle (>10.24 s) value range of enhanced INACTIVE eDRX is same as IDLE eDRX from 20.48s to 10485.76s Proposal 2: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, measurement requirements on serving cell wait for RAN2 conclusion. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Extended RRC INACTIVE eDRX cycle

Sub-topic description:

Under this sub-topic the requirements for eDRX cycle > 10.24s with PTW for Rel-18 RedCap UE in RRC INACTIVE state is discussed.

Open issues and candidate options before meeting:

Issue 1-1: Extended eDRX range and PTW

- Proposals

  - Option 1 (Apple, CATT, QC): For both serving cell and neighbor cell requirements in RRC Inactive state for eDRX > 10.24,

      - the long eDRX cycle (>10.24 s) value range is from 20.48sec to 10485.76sec

      - 1.28s ≤ PTW length ≤ 40.96s, with 1.28s granularity

- Recommended WF

  - RAN2 has already agreed on following [RAN2#121]:

- “PTW length value range of enhanced INACTIVE eDRX is same as IDLE eDRX, i.e. from 1.28s to 40.96s in the step of 1.28s.

- Long eDRX cycle (>10.24 s) value range of enhanced INACTIVE eDRX is same as IDLE eDRX from 20.48s to 10485.76s, i.e., hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024”

  - RAN4 can proceed with the detailed requirements based on this agreement. No need to further discuss option 1.

Issue 1-2: Extended eDRX range requirements in RRC INACTIVE

- Proposals

  - Option 1 (ZTE, MTK, Xiaomi, Nokia, Ericsson): Idle state eDRX requirements for eDRX cycle > 10.24 sec are used as baseline for Inactive state.

- Recommended WF

  - Option 1 is recommended to be agreed.

Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements

- Proposals

  - Option 1 (Apple, vivo, Xiaomi, Ericsson): The requirement shall be specified based on T like in R17 Inactive mode requirement with eDRX, and T can be referred to RAN2 definition in TS38.304.

  - Option 2 (CATT, MTK, HW, QC): Wait for RAN2 conclusion.

    - Option 2a (CATT): Before RAN4 specify requirements, the eDRX cycle and PTW length should first be determined by RAN2/CT/SA.

- Recommended WF

  - Discuss the options.

Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements

- Proposals

  - Option 1 (CATT, Apple, Xiaomi, HW): UE performs intra-frequency/inter-frequency measurement according to a fixed period.

  - Option 1a (Apple): The detection/measurement/evaluation delay requirement shall be specified in the step of:

    - INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec

    - DRX cycle, if eDRX_Inactive  20.48sec

  - Option 2 (MTK): Wait for RAN2 conclusion.

- Recommended WF

  - Check if option 1 can be agreed.

Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements

- Proposals

  - Option 1 (Apple, vivo, Xiaomi): UE shall perform measurements/evaluation within single PTW(s) for both IDLE eDRX and INACTIVE eDRX.

  - Option 2 (HW): UE shall NOT perform measurements outside PTW(s) for idle eDRX and inactive eDRX.

  - Option 3 (Ericsson): UE shall perform measurements within PTW(s) for both idle eDRX and inactive eDRX.

- Recommended WF

  - Options are similar, but difference in the wording. Check if option 1 can be agreed.

Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements

- Proposals

  - Option 1 (Xiaomi): The number of samples needed for Tdetect,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) could be splitted into different PTWs.

- Recommended WF

  - Discuss the option.

Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide

- Proposals

  - Option 1 (Ericsson): RAN4 to clarify the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding.

- Recommended WF

  - Discuss the option.

Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX

- Proposals

  - Option 1 (Nokia): For Enhanced INACTIVE eDRX, the RRM core requirements should be updated in subclauses 5.1B.2.2, 5.1B.2.3 and 5.1B.2.4 of TS 38.133.

- Recommended WF

  - In this meeting focus on resolving issues 1-7, the impacted clauses can be discussed based on that.

Issue 1-9: Transition requirements

- Proposals

  - Option 1 (Apple): RAN4 to discuss the UE behavior and corresponding requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec).

- Recommended WF

  - Discuss the option.

Issue 1-10: CG-SDT requirements with PTW

- Proposals

  - Option 1 (Apple, CATT): New CG-SDT requirement shall be specified for Rel-18 eRedCap UE with PTW. Details are FFS.

    - Option 1a (CATT): Prefer CG-SDT SDT requirement is only used within PTW.

  - Option 2 (MTK, Xiaomi, Ericsson): RAN4 to discuss the necessity to define CG-SDT requirements with PTW before defining the requirements.

- Recommended WF

  - Discuss the options.

Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range

- Proposals

  - Option 1 (Ericsson): RSRP2 time range in FR1 with eDRX cycle > 10.24 sec is defined as (T2 –640ms) ≤ T2’ ≤ T2.

- Recommended WF

  - Discuss the option.

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Apple | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE fine with option 1. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements support option 1, same principle shall be used as for R17 RedCap. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements Support option 1 and 1-a. In R17 the measurement/detection/evaluation delay is defined in the step of DRX cycle or eDRX cycle depending on whether the eDRX cycle length is above 10.24s or not, e.g., as shown in the note3 of table 4.2B.2.3-2. However, in Inactive mode, UE shall follow the Inactive eDRX configuration from RAN rather than IDLE eDRX configuration from CN. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements Support option 1. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements Fine with option 1 since it’s same assumption as for R17 IDLE eDRX. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Need to check such scenario with RAN2 and FFS on UE behavior. Our understanding is UE shall follow Inactive eDRX if UE is in Inactive mode. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Agree with recommended WF to address the technical parts first. Issue 1-9: Transition requirements Support option 1. Issue 1-10: CG-SDT requirements with PTW Support option 1, details shall be FFS. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range Need to conclude on issue 1-10 first. It can be FFS. |
| vivo | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE OK with option 1. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements Support option 1, ok to wait for further RAN2 input. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements OK with option 1 and 1-a. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements Support option 1. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements OK with option 1. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide FFS Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Agree with recommended WF Issue 1-9: Transition requirements Open for discussion Issue 1-10: CG-SDT requirements with PTW Prefer option 2. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range FFS |
| Huawei | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE Option 1 is not clear to us. We are wondering whether inactive_eDRX is configured here? If yes, then there are two PTWs, we don’t know how to use the requirements of “Idle state eDRX requirements for eDRX cycle > 10.24 sec” as a baseline. If this issue focus on “No inactive-eDRX but with idle_eDRX”, then we think option 1 is reasonable. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements Support option 2. In R17 RedCap, the RRM requirements of serving cell are specified based on T (paging DRX). In R18 RedCap, when two PTW exist, there are multiple kinds of T. The principle of how to determine T when two PTW are overlapped shall wait for RAN2. Which T UE shall use to perform serving cell measurement needs further consideration. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements Support option 1. Although in RAN2 T is still under discussion, we think one principle can be first settled down. As in R17 RedCap, the measurement period of intra-frequency/inter-frequency measurement depends on inactive eDRX cycles. The reason why T is not used for neighbour cell measurement is that paging DRX T is different within idle_PTW and outside idle_PTW. Measurements as per changed T would complicate UE implementation. We think the same principle can be reused for two PTWs case for R18 RedCap. How to determine the fixed period is FFS. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements We think option 1 and option 2/3 are talking about different issues. We read papers from companies, we think option1 means that measurement and evaluation period must be constrained in one PTW window. In other words, to obtain one measurement result, across PTW windows are not expected, as the next valid samples may be far away. We are fine with the principle, however the wording need to be more accurate, for example: “Measurements/evaluation period shall be constraint to within single PTW window for both IDLE eDRX and INACTIVE eDRX”. Option2/3 has the similar view. It means that UE shall perform measurement within PTW, outside PTW measurement is not expected. UE can perform measurement - within PTW(s) of inactive-eDRX (as inactive_eDRX is always not less than idle_eDRX, so in time domain, PTW of idle_eDRX would be coinciding with PTW of Inactive_eDRX), or - within PTW of idle-eDRX. Therefore we prefer to use option 2. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements Fine with option 1, just like existing equation eDRX_cycle_length×23PTW/DRX_cycle_length. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide The issue identified in option 1 is valid. We are open to further discuss. In our understanding, as IDLE eDRX is not less than inactive eDRX, so in time domain, all PTW of idle_eDRX would be coinciding with PTW of Inactive_eDRX. So option 1 mainly focus the case that the PTWs of INACTIVE_Edrx are not coinciding with PTWs of idle_eDRX. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Not against option 1 but we share the same view as Apple, we shall address the technical parts first. Issue 1-9: Transition requirements Option 1 is fine. Issue 1-10: CG-SDT requirements with PTW Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range |
| Ericsson | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE We support the recommended WF. To Huawei, our understanding is that whether UE is always configured with two PTWs when configured with INACTIVE eDRX cycle is not yet agreed in RAN2. Reusing the current IDLE mode requirements as baseline means in this case the delay requirements (e.g. in terms of number of DRX cycles) needed to perform for example the serving cell evaluation or neighbour cell detections and measurements from corresponding IDLE mode eDRX cycle requirements can be reused. But the case when UE is configured with both IDLE and INACTIVE eDRX cycles needs to be discussed when RAN2 has agreed that case. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements We support option 1 which is aligned with how the Rel-17 INACIVE eDRX requirements were specified. To our understanding, it is not yet agreed in RAN2 whether the UE is also configured with IDLE eDRX when configured with INACTIVE eDRX cycles. This is true for Rel-17 RedCap where INACTIVE eDRX cycle is ≤ 10.24s. But the serving cell measurements can still be based on T following the Rel-17 approach, we don’t think this part is impacted since T is defined in RAN2 and RAN4 specification simply refers to that definition. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements We support option 1 and 1a which is aligned with how the Rel-17 INACIVE eDRX requirements were specified. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements In Rel-18 RedCap, there will be PTW in INACTIVE eDRX and in this case we think the measurements shall still be performed within any of the PTWs of IDLE eDRX or INACTIVE eDRX. Our understanding is that similar issues are being discussed in RAN2 and therefore RAN4 shall wait for RAN2 progress. This will avoid any mis alignments between the WGs. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements We are fine with option 1. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Our understanding is that similar issues are being discussed in RAN2 and therefore RAN4 shall wait for RAN2 progress. This will avoid any mis alignments between the WGs. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Issue 1-9: Transition requirements Transition requirements can be discussed after RAN4 has identified the exact requirements needed. Therefore we are fine to keep transition requirements as FFS. Issue 1-10: CG-SDT requirements with PTW CG-SDT and eDRX are independent features and can be configured independently. CG-SDT transmission can occur anytime and the existing TA validation requirement shall apply. We agree that some modification is needed to the existing TA validation requirements in FR1 and FR2 with eDRX which is being discussed in issue 1-11. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range We support option 1. RSRP2 range in current TA validation requirements for FR1 with eDRX, the MIN function does not make sense for the case when eDRX cycle is > 10.24. Therefore it can be corrected as shown in proposal 1. |
| Nokia | Sub topic 1-1: We are fine with the recommended WF. Issue 1-2: Extended eDRX range requirements in RRC INACTIVE We are fine with the recommended WF. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements We support option 1. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements We support options 1 and 1a. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements We support option 1. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements We support option 1. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide This transition scenarios between RRC_IDLE and RRC_INACTIVE and vice versa should be discussed in RAN2 first. Hence the UE behavior is FFS. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX We support option 1 but we are fine with the recommended WF. Issue 1-9: Transition requirements Same as for issue 1-7. It should be discussed in RAN2 first. Issue 1-10: CG-SDT requirements with PTW We support option 1. In our view, CG-SDT requirements for long INACTIVE eDRX need to be specified, since PTW needs to be taken into account. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range First, issue 1-10 needs to be decided. Thus, it can be FFS. |
| Qualcomm | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE Fine with option 1. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements Support Option 2. We don’t disagree with option 1 but we prefer waiting for RAN2 conclusion before we define the requirements. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements Fine with option 1 and 1-a. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements We share the same views as Huawei. Option 1 says that the measurement and evaluation periods shall be confined to PTW duration and no filtering across PTWs. We are fine with option 1 which was also the principle during Rel-17 RedCap. Option 2/3, on the other hand, mention about not performing any measurements outside PTW and we are okay with that too. But which PTW shall be used is still unclear and we prefer waiting for conclusions on other issues before deciding this. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements Fine with option 1 which was also the assumption during Rel-17 RedCap Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Suggest waiting for RAN2 conclusions. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Agree with recommended WF Issue 1-9: Transition requirements We can discuss it after requirements are finalized Issue 1-10: CG-SDT requirements with PTW Prefer option 2. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range FFS |
| MediaTek | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE Support recommended WF. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements In current TS 38.304 there is no T definition to handle the new INACTIVE mode eDRX cycle longer than 10.24s with PTW. RAN2 shall define the definition first and identify the impact of the long eDRX for both INACTIVE and IDLE modes, then we can discuss this issue. Therefore, we support option 2 to postpone this issue. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements Fine with option 1 and 1a. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements Given that this is for serving cell, then the measurement has to be done in a single PTW. Also, there is no measurement outside the PTW. Yet, we are fine to wait for RAN2 progress as in E/// suggestion. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements In general, we are fine to reuse existing principles from IDLE mode requirements. We support option 1 for Tdetect. Also, we suggest that to agree on Tmeasure and Tevalaute to be measured in a single PTW or they can handled with serving cell measurements in issue 1-5. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Wait for RAN2 and keep FFS in RAN4. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Support recommended WF. Issue 1-9: Transition requirements FFS. Issue 1-10: CG-SDT requirements with PTW We need more time to check the necessity of this. So far we don’t believe there is a need to define such combination but we are open to keep it FFS to check this further. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range We need more time to check this. |
| CATT | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE Ok with option 1. For the concern of Huawei, we have the same understanding with Ericsson. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements We support option 1. The serving cell measurement requirements in Rel-17 RedCap are specified based on T (paging DRX), where T is different within and outside idle_PTW. When there are two PTWs in Rel-18 eRedCap, the value of T will be more complex when the two PTWs overlap. The principle for determining T should be wait for RAN2 inclusion. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements Fine with option 1 and 1a. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements We have the same understanding with Huawei. Proposal 2 and proposal 3 are the same. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements Option 1. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Wait for RAN2. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Support recommended WF. Issue 1-9: Transition requirements FFS. Issue 1-10: CG-SDT requirements with PTW We support option 1 but also can compromise to option 2. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range FFS |
| Xiaomi | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE OK with option 1. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements Support option 1, also fine to wait for further RAN2 input. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements Fine with option 1 and 1a. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements Support option 1. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements Support option 1. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Based on out current understanding, the general measurement rule discussed in previous issues are also applicable to the case when IDLE and INACTIVE eDRX PTW do not coincide, as RAN2 also need to decide T and corresponding PTW for this case. We are open to further check. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Agree with recommended WF Issue 1-9: Transition requirements Open for discussion Issue 1-10: CG-SDT requirements with PTW Support option 2. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range FFS |
| ZTE | Sub topic 1-1: Issue 1-2: Extended eDRX range requirements in RRC INACTIVE OK with option 1. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements Support option 1. Referring to Rel-17 RedCap, the measurement requirements for serving cell are specified based on T. And the principle for T should be to wait for further RAN2 input. Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements OK with option 1 and 1a. Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements Support option 1. Meanwhile, we are fine to wait for RAN2 progress. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements OK with option 1. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Wait for RAN2. Issue 1-8: Impacted clauses in specification due to extended INACTIVE eDRX Support recommended WF Issue 1-9: Transition requirements FFS Issue 1-10: CG-SDT requirements with PTW Prefer option 2. Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range FFS |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic #1 | Issue 1-2: Extended eDRX range requirements in RRC INACTIVE - Option 1 (QC, Apple, vivo, ZTE, MTK, Xiaomi, Nokia, Ericsson, CATT): Idle state eDRX requirements for eDRX cycle > 10.24 sec are used as baseline for Inactive state. Recommendations for 2nd round: Check whether following can be agreed: Following Idle state requirements for eDRX cycle > 10.24 sec are used as baseline for Inactive state for eDRX cycle > 10.24 sec: - the delay requirements (e.g. in terms of number of DRX cycles) needed to perform for example the serving cell evaluation or neighbour cell detections and measurements from corresponding IDLE mode eDRX cycle requirements can be reused. Issue 1-3: When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements - Proposals - Option 1 (Apple, vivo, Xiaomi, Ericsson, Nokia, CATT, ZTE): The requirement shall be specified based on T like in R17 Inactive mode requirement with eDRX, and T can be referred to RAN2 definition in TS38.304. - Option 2 (CATT, MTK, HW, QC, vivo, Xiaomi): Wait for RAN2 conclusion. - Option 2a (CATT): Before RAN4 specify requirements, the eDRX cycle and PTW length should first be determined by RAN2/CT/SA. Recommendation for 2nd round: Given that T is defined in RAN2 and RAN4 specification simply refers to that definition, can option 1 be agreed? Issue 1-4: When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements - Proposals - Option 1 (CATT, Apple, Xiaomi, HW, vivo, Nokia, QC, MTK, ZTE): UE performs intra-frequency/inter-frequency measurement according to a fixed period. - Option 1a (Apple, vivo, HW, Nokia, QC, MTK, CATT, Xiaomi, ZTE): The detection/measurement/evaluation delay requirement shall be specified in the step of: - INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec - DRX cycle, if eDRX_Inactive 20.48sec - Option 2 (MTK): Wait for RAN2 conclusion. Tentative agreement: UE performs intra-frequency/inter-frequency measurement according to a fixed period, how to determine the fixed period is FFS. The detection/measurement/evaluation delay requirement shall be specified in the step of: - INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec - DRX cycle, if eDRX_Inactive 20.48sec Issue 1-5: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements Tentative agreement: - Measurements/evaluation period shall be constraint to within single PTW window for both IDLE eDRX and INACTIVE eDRX”. When configured with both IDLE and INACTIVE PTWs, regarding when to measure (within PTW(s) or not to measure outside PTW(s), RAN4 to wait for RAN2 progress. Issue 1-6: When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements Tentative agreement: The number of samples needed for Tdetect,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) could be splitted into different PTWs. Issue 1-7: When to measure when IDLE and INACTIVE eDRX PTW do not coincide Tentative agreement: RAN4 to wait for RAN2 progress to discuss following: - Whether to clarify the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding. Issue 1-9: Transition requirements - Proposals - Option 1 (Apple): RAN4 to discuss the UE behavior and corresponding requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec). Tentative agreement: RAN4 to discuss the need for transition requirements after the INATIVE eDRX with PTW requirements are finalized. Issue 1-10: CG-SDT requirements with PTW - Option 1 (Apple, CATT, Nokia): New CG-SDT requirement shall be specified for Rel-18 eRedCap UE with PTW. Details are FFS. - Option 1a (CATT): Prefer CG-SDT SDT requirement is only used within PTW. - Option 2 (MTK, Xiaomi, Ericsson, vivo, QC, CATT, ZTE): RAN4 to discuss the necessity to define CG-SDT requirements with PTW before defining the requirements. Recommendation for 2nd round: Discuss the following: - What type of new requirements are needed or why the legacy requirements can’t be reused? - If eDRX and CG-SDT are independent features and can be configured independently, is there a need to define new requirements? Issue 1-11: CG-SDT requirements with PTW: RSRP2 time range Tentative agreement: Following option is to be discussed after RAN4 has identified whether new requirements are to be specified for Rel-18 eRedCap with PTW: - RSRP2 time range in FR1 with eDRX cycle > 10.24 sec is defined as (T2 –640ms) ≤ T2’ ≤ T2. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: Baseband BW reduction impact

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304169 | ZTE Corporation | Proposal 2: RAN4 to await progress in RAN1 to identify impact to RRM core requirements. |
| R4-2304515 | vivo | Proposal 4: For impact of THARQ, there is not any impact on THARQ. |
| R4-2304598 | MediaTek inc. | Proposal 4: RAN4 shall reuse existing requirements for SIB1 reading. Proposal 6: RAN4 needs to identify what is the impact from THARQ on RRM requirements before agreeing to change the exiting requirements. |
| R4-2304784 | Xiaomi | Proposal 4: For CGI reading requirements, the SIB1 decoding delay for Rel-18 eRedCap needed to be extended compared with Rel-17 RedCap. |
| R4-2305286 | Huawei, HiSilicon | Proposal 4: No RRM impacts are foreseen due to UE BB bandwidth reduction. |
| R4-2305569 | Nokia, Nokia Shanghai Bell | - RAN4 should discuss whether an extended CGI reporting period is needed. |
| R4-2305732 | Ericsson | No impact on CGI reading requirements due to baseband BW reduction of PDSCH. No impact on paging reception requirements due to baseband BW reduction. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 Baseband BW reduction impact

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1: Impact on CGI reading requirements

- Proposals

  - Option 1 (MTK, HW, Ericsson): RAN4 shall reuse existing requirements for SIB1 reading.

  - Option 2 (Xiaomi): The SIB1 decoding delay for Rel-18 eRedCap needed to be extended compared with Rel-17 RedCap.

  - Option 3 (ZTE): RAN4 to await progress in RAN1 to identify impact to RRM core requirements.

- Recommended WF

  - RAN1 had reached following agreement [R4-2301885]:

SIB1 bandwidth Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for SIB1 (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of SIB1 to be within 5 MHz - Option 2: Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: Replace the agreement on SIB1(PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: UE post-FFT buffering “assumption” (replaced by later agreement)

  - Taking into consideration the above agreement, discuss whether the existing requirements (option 1) can be reused.

Issue 2-2: Impact on paging requirements

- Proposals

  - Option 1 (Ericsson, HW): No impact on paging reception requirements due to baseband BW reduction.

  - Option 2 (ZTE): RAN4 to await progress in RAN1 to identify impact to RRM core requirements.

- Recommended WF

  - RAN1 had reached following agreement in RAN1#112 [R1-2301885]:

Paging bandwidth Agreement: Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

  - Taking into consideration the above agreement in RAN1, discuss whether the existing requirements (option 1) can be reused.

Issue 2-3: Impact on THARQ

- Proposals

  - Option 1 (vivo, HW): There is not any impact on THARQ.

  - Option 2 (MTK): RAN4 needs to identify what is the impact from THARQ on RRM requirements before agreeing to change the exiting requirements.

- Recommended WF

  - Discuss if any RRM requirements are impacted, and if so be more specific and refer to the type of requirements in TS 38.133.

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Apple | Issue 2-1: Impact on CGI reading requirements fine with option 1. Issue 2-2: Impact on paging requirements Fine with option 1. Issue 2-3: Impact on THARQ Option 1. |
| vivo | Issue 2-1: Impact on CGI reading requirements Support option 1. Issue 2-2: Impact on paging requirements OK with option 1. Issue 2-3: Impact on THARQ Support option 1. |
| Huawei | Issue 2-1: Impact on CGI reading requirements Fine with option 1. Issue 2-2: Impact on paging requirements Fine with option 1. Issue 2-3: Impact on THARQ Can go with Option 1, if RAN1 has modification on Harq, RAN4 can revisit. |
| Ericsson | Issue 2-1: Impact on CGI reading requirements We support the recommended WF. Issue 2-2: Impact on paging requirements We support the recommended WF. Issue 2-3: Impact on THARQ According to our understanding, RAN1 did not discuss the PDSCH processing time due to the eRedCap baseband BW reduction. We don’t see any impact on T_HARQ, and therefore we don’t need to discuss more. |
| Nokia | Issue 2-1: Impact on CGI reading requirements We support option 1 based on latest RAN1 status. Issue 2-2: Impact on paging requirements We support option 1 based on latest RAN1 status. Issue 2-3: Impact on THARQ We support option 1. |
| Qualcomm | Issue 2-1: Impact on CGI reading requirements Option 1. Issue 2-2: Impact on paging requirements Option 1. Issue 2-3: Impact on THARQ Option 1, also support the recommended WF |
| MediaTek | Issue 2-1: Impact on CGI reading requirements Support recommended WF. Issue 2-2: Impact on paging requirements Support recommended WF. Issue 2-3: Impact on THARQ We have similar understanding as E/// and Huawei that there is no current impact on THARQ. We support option 1 and if later RAN1 reaches a different agreement that impact THARQ then RAN4 can revisit. |
| CATT | Issue 2-1: Impact on CGI reading requirements Support recommended WF. Issue 2-2: Impact on paging requirements Support recommended WF. Issue 2-3: Impact on THARQ Option 1 and also support the recommended WF. |
| Xiaomi | Issue 2-1: Impact on CGI reading requirements For option 1, the exiting requirement is for R17 RedCap which has up to 20MHz BB BW. We still have concern on it. In our understanding, if Rel-18 eRedCap UE is requested to report CGI, the existing requirement cannot always applicable as the SIB1 transmission is up to NW implementation and eRedCap UE needs more time to process SIB1 due to 5MHz BB BW. Issue 2-2: Impact on paging requirements Agree with option 1. Issue 2-3: Impact on THARQ Agree with option 1. |
| ZTE | Issue 2-1: Impact on CGI reading requirements Support recommended WF. Issue 2-2: Impact on paging requirements Support recommended WF. Issue 2-3: Impact on THARQ Option 1 and also support the recommended WF. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic#2 | Issue 2-1: Impact on CGI reading requirements Recommendations for 2nd round: - Option 1 (Apple, vivo, MTK, HW, Ericsson, Nokia, QC, CATT, ZTE): RAN4 shall reuse existing requirements for SIB1 reading. - Option 2 (Xiaomi): The SIB1 decoding delay for Rel-18 eRedCap needed to be extended compared with Rel-17 RedCap. Moderator’s view: The RAN1 agreement clearly states that the SIB1 can be scheduled with larger than 5 MHz as in legacy operation for Rel-18 bandwidth reduced RedCap UE. According to the comments in the 1st round, most companies think the Rel-18 RedCap UE shall be able to acquire SIB1 within the legacy delay specified in Rel-17. CGI requirements does not contain anything on processing time. Based on this information, check if option 1 can be agreed. Issue 2-2: Impact on paging requirements Tentative agreement: No impact on paging reception requirements due to baseband BW reduction. Issue 2-3: Impact on THARQ Tentative agreements: There is not any impact on THARQ due to baseband BW reduction. |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: Relaxed RLM/BFD

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304153 | Apple | Proposal 7: it shall be decided in RAN plenary whether RAN4 needs to specify the RLM/BFD relaxations for RedCap UEs as a part of eRedCap WI. |
| R4-2304598 | MediaTek inc. | Proposal 7: RAN4 shall define the Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap. |
| R4-2305569 | Nokia, Nokia Shanghai Bell | RAN4 to define the RLM/BFD relaxation requirements for both 1 Rx and 2 Rx RedCap UEs in eRedCap WI. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1 Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap

- Proposals

  - Option 1 (Apple): it shall be decided in RAN plenary whether RAN4 needs to specify the RLM/BFD relaxations for RedCap UEs as a part of eRedCap WI.

  - Option 2 (MTK, Nokia): RAN4 shall define the Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap.

- Recommended WF

  - Discuss the options.

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 3-1

| Company | Comments |
| --- | --- |
| Apple | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap Option 1. The scope related issue shall be decided in RAN plenary. This issue has been discussed in last RANP meeting but no conclusion was achieved. |
| vivo | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap Prefer option 2. |
| Huawei | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap Similar view as Apple. The scope related issue shall be decided in RAN plenary. |
| Nokia | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap We support option 2. We don’t think that RAN plenary needs to be involved in this technical detail, it can be rather included in the RRM WF. It is clear that RLM/BFD relaxation in connected mode, introduced for UE power savings in Rel-17, will equally contribute to RedCap UE power savings. |
| Qualcomm | Although we support option 2, but we agree with Apple that the scope shall be decided in RAN plenary. |
| MediaTek | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap Our preference is option 2. However, it is not clear whether it is supported in the WID, therefore, we can volunteer to an LS to RAN plenary and ask for their feedback. |
| CATT | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap Prefer option 2. And also agree to send an LS to RAN plenary for confirmation. |
| Xiaomi | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap Ok with option 2, but also agree that the scope issue need to be clarified first. |
| ZTE | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap Prefer option 2, but the scope should be decided in RAN plenary. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic#3 | Issue 3-1: Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap - Proposals - Option 1 (Apple): it shall be decided in RAN plenary whether RAN4 needs to specify the RLM/BFD relaxations for RedCap UEs as a part of eRedCap WI. - Option 2 (MTK, Nokia): RAN4 shall define the Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap. Tentative agreement: Since relaxation of relaxed RLM/BFD does not belong to any of the Rel-18 RedCap objectives, and there is no consensus to develop those within Rel-18 WI scope, this issue is closed, and no more discussions needed. |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on R18 RedCap RRM requirements | Ericsson | To capture the agreements and open issues |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-23xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-23xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents