3GPP TSG-RAN WG4 Meeting # 106bis-e		R4-2306178

# Online, April 17 – April 26, 2023

Agenda item:			6.8.8

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [106bis-e][230] LTE_NBIOT_eMTC_NTN_req

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for NB-IoT/eMTC core & perf. requirements for NTN, including the following topics covered

- Topic#1: RRM core requirements (AI 6.8.5)

- Topic#2: RRM perf requirements (AI 6.8.6)

List of candidate target of discussions for this topic.

- 1st round: Decide on the scope, priority, options and tentative agreement to be discussed in the 2nd round. Conclude issues with strict consensus, if any.

- 2nd round: Conclude the issues identified in the 1st round.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| MediaTek | Hsuanli Lin | Hsuanli.Lin@mediatek.com |
| Qualcomm | CH Park | chparkqc@qti.qualcomm.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| CMCC | Shiyuan Wang | wangshiyuan@chinamobile.com |
| Nokia | Rafhael Amorim | rafhael.medeiros_de_amorim@nokia.com |
| Ericsson | Santhan Thangarasa | Santhan.thangarasa@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: RRM core requirements (AI 6.8.5)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304503 | Nokia, Nokia Shanghai Bell | Proposal 1: Neighbor cell measurements requirements defined in TS 38.133 apply for Rel-17 NTN UEs, regardless of availability of neighbor cell ephemeris information, as long as both serving and neighbor cell are associated to a GEO satellite. Observation 1: The mobility sections are currently not part of the “release independency” applicability rules defined in TS 36.307 Proposal 2: As the current release independency applicability rules do not cover the IDLE Mode mobility section in TS 36.133, and the core part for this WI was already completed, the discussion on enhancements of serving cell measurements should also be postponed to the Rel-18 WI (IoT_NTN_enh). Observation 2: The main reason to execute serving cell measurements is to trigger neighbor cell measurements. Observation 3: In Rel-17, and there are no requirements for neighbor cell measurements if the criterion S is not met. Observation 4: RAN2 is currently discussing whether to provide additional information regarding the availability of target cells Observation 5: Collecting serving cell measurements before t-service will not lead to any change of action by the UE regardless whether the measurements satisfy or not the criterion S, because RAN4 agreed UE starts neighbor cell measurements solely based on t-service. Those measurements will only lead to additional expenditure of energy by the UE. Observation 6: Stage 2 requirements and RAN4 agreements already indicate UE starts neighbor cell measurements based on the indication of t-service. Proposal 3: Do not introduce additional conditions for the measurement relaxation of the serving cell in the current WI. Observation 7: Paging requirements only apply for UEs camped in a detected cell. Proposal 4: Do not create additional requirements after t-serviceStart regarding UE paging interruption. Proposal 5: The definition of “in-coverage” for measurement continuation/resuming shall not depend on UE executing previous measurements to define the “in-coverage status”. Proposal 6: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by SystemInformationBlockType32. Observation 8: The additional energy consumption introduced by this change does not compensate the gains for the UE. Observation 9: RAN2 is discussing Rel-18 enhancements for mobility aspects in relation to t-service. Proposal 7: Do not apply additional requirements for UE configured with eDRX cycle before t-service for this WI. Proposal 8: In order to align with the definitions in TS36.304 and the design of t-service, in TS 36.133, Section 4.6A,  replace “If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area.”  by If SystemInformationBlockType32 [2] has been received and the UE has determined it will be out of coverage after t-service, the UE is not required to perform neighbor cell measurements during the last [k] SI modification periods, before t-Service [2] |
| R4-2304648 | CMCC | Proposal 1: When no satellite assistance information is provided, neighbor cell measurement requirements are applicable only for GEO case. Proposal 2: Reuse the legacy mechanism of serving cell measurement relaxation factor N. Proposal 3: Apply Option 2 with the prerequisite of UE will not go into discontinuous coverage after ‘t-Service’. Proposal 4: Support to replace the original wording to “If the UE has a received and stored a valid version for SystemInformationBlockType32 [2], and the UE has determined it is out of coverage after t-service, the UE is not required to perform cell measurements after t-service until the UE has regained coverage”. Proposal 5: Option 1 can be applied with the prerequisite of UE will not go into discontinuous coverage after ‘t-Service’. |
| R4-2304819 | MediaTek inc. | Proposal 1: For GEO, when the satellite assistance information of neighbour cells in system information is not provided, intra-frequency neighbor cell measurements requirements are still applicable while the following requirement are NOT applicable: - 4.6A.2.5 Measurements of inter-frequency NB cells for UE category NB1 in normal coverage - 4.6A.2.6 Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage - 4.7A.2.1.3 Measurements of inter-frequency cells for UE category M1 in normal coverage - 4.7A.2.2.3 Measurements of inter-frequency cells for UE category M1 in enhanced coverage - 5.5A E-UTRAN Handover for Cat-M1 UEs for Satellite Access - 8.13A.2.2 E-UTRAN inter frequency measurements by UE category M1 with CE mode A - 8.13A.3.2 E-UTRAN inter frequency measurements by UE category M1 with CE Mode B Proposal 2: For GEO, when the satellite assistance information of neighbour cells in system information is not provided, requirements can apply for intra-frequency RRC Re-establishment. Proposal 3: the existing tables of relaxation factor N can be reused and no need to introduce new tables. Proposal 4: No need to introduce new requirement to apply non-DRX requirements upon ‘t-Service-r17’. It can remove ”FFS: If the UE is configured with ‘t-Service-r17’ [2] then after t-Service-r17, radio link monitoring requirements defined in this clause shall not apply.” |
| R4-2305259 | Huawei, HiSilicon | Proposal 1: When no satellite assistance information is provided, neighbor cell measurement requirements are not applicable for both NGSO and GSO. Observation 1: Legacy relaxation criteria will lead to very long delay for cell re-selection after t-service. Proposal 2: For serving cell: In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN: - The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle. For neighbor cell: UE shall exit neighbor cell measurement relaxation as defined in 36.304 when time span to T-service is shorter than T, where T = Tdetect,NB_Intra_NC. And RAN4 to discuss whether to capture it in Rel-17 IoT NTN or in Rel-18 IoT NTN enh. Proposal 3: Do not revise paging requirements upon t-serviceStart-r17. Proposal 4: Revise the wording for discontinuous coverage as follows: If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is will be out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after during the last one SI modification period before t-Service [2]. when the serving cell stop serving the area. |
| R4-2305734 | Ericsson | - For UEs capable of performing intra-frequency neighbour cell measurements without satellite assistance information, RAN4 to define neighbour cell measurement requirements. - The values in tables used for deriving the serving cell measurement relaxation factors are reduced by factor 2 for NGSO for eMTC and NB-IoT in IDLE mode. - In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [4] DRX cycles before ‘t-Service-r17’ if not configured with eDRX_IDLE cycle. - In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [1] DRX cycles before ‘t-Service-r17’ if configured with eDRX_IDLE cycle. - Whether to consider following option for relaxed neighbour cell measurement requirements depends on whether neighbour cell measurements are supported for GSO in first release of IoT NTN WI: - The eMTC and NB-IoT UE is allowed to meet the relaxed neighbour cell requirements provided that it has found more than [1] satellite including the serving satellite. - When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search until UE has acquired the system information carrying the Ephemeris. - Paging requirements apply when UE is in coverage, which is the time when UE has acquired the SI of the target cell and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario. - In-coverage is the time that UE has acquired the SI of the target cell, and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario. - If the UE is configured with DRX cycle ≥ [1.28] sec, then the UE shall meet the serving cell evaluation requirements defined for DRX cycle of [640] ms starting from at least 2 configured DRX cycles before ‘t-Service-r17’. - If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’. - If the UE is configured with eDRX_IDLE cycle then the UE shall meet the requirements defined for eDRX_IDLE cycle of 5.12 s in Table 4.7A.2.1.1-2 during 2 eDRX_IDLE cycles immediately after ‘t-ServiceStart-r17’ [2] regardless of the configured eDRX_IDLE cycle. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Applicability of neighbor cell measurement requirements

#### Issue 1: Requirements applicability when no satellite assistance information is provided

Background

Agreements (from RAN4#106)

- When no satellite assistance information is provided

  - For NGSO case the neighbor cell measurements requirements are not applicable

  - FFS on neighbor cell measurements requirements for GSO case

- FFS whether to apply intra-frequency neighbor cell measurements requirements for GEO case

Agreements (from RAN4#104-e)

- Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.The following Tentative Agreement can be FFS in next meeting

RAN2 LS: R2-2213018 Reply LS on information for neighbor/target cell in IoT NTN

- In case of handover to an NTN cell, the SIB31 of target NTN cell (which contains its satellite assistance information) is provided in RRCConnectionReconfiguration.

- For neighbor cell measurements, RAN2 has agreed not to introduce the satellite assistance information of neighbour cells in system information in Rel-17 IoT NTN, but this will be supported in Rel-18.

Proposals

- Option 1: When no satellite assistance information is provided, neighbor cell measurement requirements are not applicable for both NGSO and GSO. (Huawei)

- Option 2:  Neighbor cell measurements requirements defined in TS 38.133 apply for Rel-17 NTN UEs, regardless of availability of neighbor cell ephemeris information, as long as both serving and neighbor cell are associated to a GEO satellite. (Nokia, CMCC)

- Option 3: For GEO, when the satellite assistance information of neighbour cells in system information is not provided, intra-frequency neighbor cell measurements requirements are still applicable. (MTK)

  - requirements can also apply for intra-frequency RRC Re-establishment

  - the following requirement are NOT applicable:

    - 4.6A.2.5 Measurements of inter-frequency NB cells for UE category NB1 in normal coverage

    - 4.6A.2.6 Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage

    - 4.7A.2.1.3 Measurements of inter-frequency cells for UE category M1 in normal coverage

    - 4.7A.2.2.3 Measurements of inter-frequency cells for UE category M1 in enhanced coverage

    - 5.5A E-UTRAN Handover for Cat-M1 UEs for Satellite Access

    - 8.13A.2.2 E-UTRAN inter frequency measurements by UE category M1 with CE mode A

    - 8.13A.3.2 E-UTRAN inter frequency measurements by UE category M1 with CE Mode B

- Option 3a: For UEs capable of performing intra-frequency neighbour cell measurements without satellite assistance information, RAN4 to define neighbour cell measurement requirements. (Ericsson)

Recommended WF

  - Collect comments in the 1st round.

  - Also provide views on whether to include IDLE mode (4.6A/4.7A) and HO for Cat-M1 (5.5A) in 36.307 (release independent) in R17?

Offline Discussion:

- Nokia: both option 2 /3 are ok

- Ericsson: option 3 is fine to include serving cell

Tentative agreement in offline session:

- For GEO, when the satellite assistance information of neighbour cells in system information is not provided, intra-frequency neighbor cell measurements requirements are still applicable.

- Note: for serving cell evaluation, the requirement is still applicable regardless satellite type.

- Remove HO for Cat-M1 (5.5A) in Rel-17 36.307 (release independent).

| Company | Comments |
| --- | --- |
| Qualcomm | Okay with both Option 1 and Option 3. |
| Huawei | Support option 1 or option 3. |
| MTK | We prefer Option 3, as it covers more. |
| CMCC | We are fine with Option 3. Since the requirements couldn’t be applied, we prefer not to include IDLE mode (4.6A/4.7A) and HO for Cat-M1 (5.5A) in 36.307, release independent from R17. We have another question about these non-applicable requirements, will these requirements be captured in TS 36.133hx0? |
| Nokia | Options 2 and 3 are ok for us. For the other question: LEO, HO requirements cannot be applicable (there are no UE measurement requirements). The only discussion is whether to include serving cell measurements in this WI. |
| Ericsson | We support option 2 because it will improve the mobility performance of the UE compared to option 1 and 3 where support for neighbour cell measurements is limited. For example, for the case when the serving and the target cells are being served by the GEO satellite under same carrier, then if the UE can use assistance information of the serving cell for measuring on the neighbour cell it would be beneficial. |

### IDLE state mobility requirements - measurement relaxation

#### Issue 2-1: NGSO, serving cell measurement

Proposals

- Option 1: Postpone the discussion on enhancements of serving cell measurements to the Rel-18 WI (IoT_NTN_enh). (Nokia)

- Option 1a: Do not introduce additional conditions for the measurement relaxation of the serving cell in the current WI. (Nokia)

- Option 2b: (Huawei)

  - For serving cell: In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN:

    - The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

  - For neighbor cell: UE shall exit neighbor cell measurement relaxation as defined in 36.304 when time span to T-service is shorter than T, where T = Tdetect,NB_Intra_NC.

  - RAN4 to discuss whether to capture it in Rel-17 IoT NTN or in Rel-18 IoT NTN enh.

- Option 2a: (Ericsson)

  - In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [4] DRX cycles before ‘t-Service-r17’ if not configured with eDRX_IDLE cycle.

  - In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [1] DRX cycles before ‘t-Service-r17’ if configured with eDRX_IDLE cycle.

- Option 2b: Apply Option 2 with the prerequisite of UE will not go into discontinuous coverage after ‘t-Service’. (CMCC)

Recommended WF

  - Regarding the enhancements of serving cell measurement relaxation, could we move this discussion in Rel-18 WI?

| Company | Comments |
| --- | --- |
| Qualcomm | Okay with the moderator’s recommendation. |
| Huawei | Suggest to discuss serving cell measurement relaxation in this WI and discuss the neighbor cell in Rel-18 Iot NTN enh. The reason is that without any clarifications for current spec, legacy serving cell measurement relaxation is allowed already. In NTN, UE is most likely entering relaxed mode and can hardly exit the relaxed mode. The cell selection/reselection is finally based on serving cell measurement result. Even UE can measure neighbor cell before t-Service, the cell selection/reselection is also unnecessarily delayed by serving cell relaxation for N DRX cycles. |
| Nokia | We are ok with moderator recommendation. The serving cell measurements seem to be applicable for triggering neighbor cell measurements and the reselection procedure (by ranking the serving cell together with the measured neighbor cells). Therefore, we have a slight preference to discuss serving cell measurements, together with other mobility features in the other WI. But we might compromise to discuss it in this WI.. The main problem for us is the unnecessary use of UE power to collect additional measurements of the serving cell before “t-service” when they will become obsolete. The UE already has neighbor cell measurements (based on the Ttrigger rule, as we discuss in our contribution). Nothing prevents the UE to perform cell reselection, except when the conditions of the serving cell are good. The problem is that before t-service the serving cell signal will likely be very good (there’s no graceful switch, just a hard switch at t-service). Collecting more samples form the serving cell will waste UE power, and have almost no gain in return. Maybe a better solution is to allow UE to perform cell reselection at t-service considering the serving cell is “gone”, i.e., rank it for last. |
| Ericsson | We have similar view as Huawei and suggest to discuss the relaxation of serving cell under this WI. We are fine to discuss the neighbour cell relaxation in the enhancement WI. Since support for serving cell measurement relaxation is introduced in this WI, it is also reasonable to discuss and define the requirements associated with those. Options 2a and 2b are quite similar, we are fine to compromise to Huawei’s option 2b. |

#### Issue 2-2: NGSO, serving cell measurement - relaxation factor N

Proposals

- Option 1: The values in tables used for deriving the serving cell measurement relaxation factors are reduced by factor 2 for NGSO for eMTC and NB-IoT in IDLE mode. (Ericsson)

- Option 2: Reuse the legacy mechanism of serving cell measurement relaxation factor N. (CMCC, MTK)

Recommended WF

  - Discuss proposals

| Company | Comments |
| --- | --- |
| Qualcomm | Option with Option 2. |
| Huawei | Option 2. |
| MTK | Option 2. |
| CMCC | Option 2 |
| Nokia | Option 2. |
| Ericsson | Following agreement was reached in RAN4#105-meeting [R4-2220362]: #### sIssue 2-2-1: For NGSO, whether to revise the relaxed measurement requirements in IDLE mode For NGSO, for RRM measurement relaxation, UE is allowed to follow the relaxed measurement requirements, where the relaxation is less compared to corresponding GSO relaxed requirements. How to define the NGSO relaxed requirements are FFS.. First of all, RAN4 shall maintain the current agreement, especially given that this WI is already closed. The above agreement states that RRM measurement relaxation is allowed for NGSO and the level of relaxation is less compared to corresponding GSO relaxed requirements. The open issue is to agree on how much less NGSO requirements are relaxed compared to GSO relaxed requirements. Option 2 does not reflect the above agreement. Option 2 will have NW impact since with this approach the NW needs to keep track of the type of satellite and based on that adjust the relaxation factor. For IoT NTN, the existing implementations from TN are expected to be reused and changes requiring new or modified implementation shall be avoided. Option 1 has less impact on the existing implementation since the values of the relaxation factors are changed which means the existing implementations can be reused. Therefore we support option 1. |

#### Issue 2-3: GSO, neighbour cell measurement

Proposals

- Option 1: Whether to consider following option for relaxed neighbour cell measurement requirements depends on whether neighbour cell measurements are supported for GSO in first release of IoT NTN WI: The eMTC and NB-IoT UE is allowed to meet the relaxed neighbour cell requirements provided that it has found more than [1] satellite including the serving satellite. (Ericsson)

Recommended WF

  - Discuss proposals

| Company | Comments |
| --- | --- |
| Qualcomm | Do not support Option 1. UE not the edge of serving satellite’s beam footprint may not see any other GSO satellite. |
| Huawei | Same views as Qualcomm. |
| MTK | Not support Option 1. |
| CMCC | Share similar view as QC |
| Nokia | Same view as QC. Do not support option 1. |

### IDLE state mobility requirements – others

#### Issue 3-1: Discontinuous Coverage - clarification

Background

Agreements (from RAN4#105)

When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Definition of in coverage is FFS

Excerpt from TS36.304 regarding SIB32

4	General description of Idle mode

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in coverage the UE shall perform all idle mode tasks.

Proposals

- Option 1: (Nokia)

  - The definition of “in-coverage” for measurement continuation/resuming shall not depend on UE executing previous measurements to define the “in-coverage status”

  - RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by SystemInformationBlockType32

- Option 2: (Ericsson)

  - In-coverage is the time that UE has acquired the SI of the target cell, and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario.

  - When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search until UE has acquired the system information carrying the Ephemeris.

Recommended WF

  - Discuss which text is more preferrable?

| Company | Comments |
| --- | --- |
| Huawei | We support neither of them. As quoted by moderator, it is already defined by RAN2 which is up to UE implementation. |
| MTK | We are open to also capture it in the 133 and we slightly prefer to the wording in Option 1. |
| Nokia | We are proponents of Option 1 and we support it. We have a slight preference to capture in 133 the point in time where the UE is expected to continue its measurements. We do not support option 2: it is limited to earth-fixed scenario, whereas SIB32 also exists for earth moving scenarios. And it create s a requirement that can be bypassed by the UE: it depends on the time UE has acquired SI. So as long as the UE doesn’t acquire the SI, no need to resume the measurements. |
| Ericsson | Following agreement was reached in earlier RAN4 meeting where the definition of the in-coverage is FFS [R4-2220362]: “When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Definition of in coverage is FFS.” We therefore suggest that RAN4 defines the in-coverage. We support option 2 because it is more concrete as it also takes into account the case when the UE is provided with t-serviceStart-r17. This case is not addressed in option 1. |

#### Issue 3-2: Discontinuous Coverage - wording alignment

Background

Agreements (from RAN4#106)

- FFS whether to

- replace “If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area.”

- by “If the UE has a received and stored a valid version for SystemInformationBlockType32 [2], and the UE has determined it is out of coverage after t-service, the UE is not required to perform cell measurements after t-service until the UE has regained coverage”.

Proposals

  - Proposal 1 (revised from the previous meeting): to align with the definitions in TS36.304 and the design of t-service, in TS 36.133, Section 4.6A, revise the wording  “If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area.” as follows:

- Proposal 1-0: “If the UE has a received and stored a valid version for SystemInformationBlockType32 [2], and the UE has determined it is out of coverage after t-service, the UE is not required to perform cell measurements after t-service until the UE has regained coverage” (CMCC)

- Proposal 1-1: “If SystemInformationBlockType32 [2] has been received and the UE has determined it will be out of coverage after t-service, the UE is not required to perform neighbor cell measurements during the last [k] SI modification periods, before t-Service [2]” (Nokia)

- Proposal 1-2: If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is will be out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after during the last one SI modification period before t-Service [2]. when the serving cell stop serving the area. (Huawei)

Recommended WF

  - Proposals are similar. Discuss based on Proposal 1-1 and the value of k.

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 1-1 and 1-2, where K = 1. |
| CMCC | We confuse about the SI modification period, from UE perspective, how does UE know whether SI will be modified or not? |
| Nokia | We agree with Huawei. |
| Ericsson | Proposals are similar, we are fine both 1-1 and 1-2, the value of K can be 1. |

#### Issue 3-3: Discontinuous Coverage - revision on paging requirement

Proposals

- Option 1: Do not create additional requirements after t-serviceStart regarding UE paging interruption. (Nokia, Huawei)

- Option 2: Paging requirements apply when UE is in coverage, which is the time when UE has acquired the SI of the target cell and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario. (Ericsson)

Recommended WF: Discuss proposals.

| Company | Comments |
| --- | --- |
| Huawei | Option 1. |
| Nokia | Option 1. |
| Ericsson | We support option 2 because the UE cannot receive the paging before it has received the SI of the serving cell after being out of coverage. For example, UE would need some time to acquire the SIB1 and read the SI after regaining the coverage (as indicated by t-ServiceService), and only after that paging reception is possible. |

#### Issue 3-4: NGSO, serving cell measurement upon t-Service-r17

Background

Agreements (from RAN4#105)

- If the UE is configured with ‘t-Service-r17’ in the serving cell and eDRX_cycle, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s starting from at least [K] before ‘t-Service-r17’.

Proposals

- Proposal 1: (Ericsson)

  - If the UE is configured with DRX cycle ≥ [1.28] sec, then the UE shall meet the serving cell evaluation requirements defined for DRX cycle of [640] ms starting from at least 2 configured DRX cycles before ‘t-Service-r17’.

  - If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’.

  - If the UE is configured with eDRX_IDLE cycle then the UE shall meet the requirements defined for eDRX_IDLE cycle of 5.12 s in Table 4.7A.2.1.1-2 during 2 eDRX_IDLE cycles immediately after ‘t-ServiceStart-r17’ [2] regardless of the configured eDRX_IDLE cycle.

- Proposal 2: Postpone the discussion on enhancements of serving cell measurements to the Rel-18 WI (IoT_NTN_enh).

Recommended WF:

- Discuss proposals

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 2. |
| Nokia | Option 2. It is our understanding that RAN2 is discussing to provide UE with the information about the availability of target cell also in non-discontinuous coverage (R2-2303652). This will only impact Rel-18 UEs and shall be discussed in Rel-18 WI. |
| Ericsson | This issue is quite related to issue 2-1. Both issues are related to serving cell measurements before reaching t-Service. We therefore propose to treat them together in same WI. By evaluating the serving cell according to shorter DRX cycles before discontinuous coverage for NGSO, the UE would have time to complete the evaluation before satellite is gone. We are fine to focus on the first two bullets related to measurements before ‘t-Service’. It is noted that this issue is unique for IoT NTN compared to NR NTN since measurement delays are considerably longer for IoT NTN and IoT NTN supports operation with eDRX which may require much longer time to complete an evaluation/measurement before the serving cell coverage is impacted. |

### RLM

#### Issue 4: For Cat-M1 UE in NGSO and eDRX, whether to revise the RLM requirement upon t-service

Background

Agreements (from RAN4#106)

- Option 1:

  - If the eMTC UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, then at least during [1] eDRX cycle before t-Service-r17, the UE shall meet the non-DRX requirements.

- Option 2:

  - No need to introduce new requirement to apply non-DRX requirements upon ‘t-Service-r17’

  - Remove ”FFS: If the UE is configured with ‘t-Service-r17’ [2] then after t-Service-r17, radio link monitoring requirements defined in this clause shall not apply.”

Proposals

- Option 1: If the the eMTC UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, and the UE will not go into discontinuous coverage after ‘t-Service’, then at least during [1] eDRX cycle before t-Service-r17, the UE shall meet the non-DRX requirements (CMCC)

- Option 2: Do not apply additional requirements for UE configured with eDRX cycle before t-service for this WI. (Nokia, MTK)

  - Option 2a: It can remove ”FFS: If the UE is configured with ‘t-Service-r17’ [2] then after t-Service-r17, radio link monitoring requirements defined in this clause shall not apply.” (MTK)

Recommended WF

  - Discuss proposals

| Company | Comments |
| --- | --- |
| MTK | Option 2. |
| CMCC | We are also fine with Option 2 |
| Ericsson | We support option 1. It would be beneficial for the UE to speed up the RLM evaluations before reaching t-Service. |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304476 Sony | Moderator’s Note: on TS 36.307 (R17) |
|  | Huawei: Some requirements needs to be updated based on conclusion in 1.2.1. e.g. Reestablishment, neighbor cell measurement, HO. |
|  | Ericsson: Need to be revised based on the outcome of the open issues. |
| R4-2304477 Sony | Moderator’s Note: on TS 36.307 (R18) |
|  | Huawei: Huawei: Some requirements needs to be updated based on conclusion in 1.2.1. e.g. Reestablishment, neighbor cell measurement, HO. |
| R4-2305260 Huawei, HiSilicon | Moderator’s Note: 4.6A, NB, IDLE |
|  | Ericsson: Need to be revised based on the outcome of the open issues. |
| R4-2305738 Ericsson | Moderator’s Note: 4.7A, M1, IDLE |
|  | Huawei: Depends on open issues. |
| R4-2305652 MediaTek inc | Moderator’s Note: 1) 6.5A/6.7A, NB/M1, Re-establishment and 2) 7.19A M1 RLM |
|  | Huawei: GEO parts depends on pending issues. |
|  | Ericsson: Need to be revised based on the outcome of the open issues. |

## Summary for 1st round

### Open issues

#### Issue 1: Requirements applicability when no satellite assistance information is provided

Tentative agreement in offline session:

- For GEO, when the satellite assistance information of neighbour cells in system information is not provided, intra-frequency neighbor cell measurements requirements are still applicable.

- Note: for serving cell evaluation, the requirement is still applicable regardless satellite type.

- Remove HO for Cat-M1 (5.5A) in Rel-17 36.307 (release independent).

Recommendations for 2nd round: Issue closed.

#### Issue 2-1: NGSO, serving cell measurement

Status Summary: No consensus to enhance serving cell measurement for NGSO.

Recommendations for 2nd round: Continue discuss in the 2nd round on the following options:

  - Option 1: Postpone the discussion on enhancements of serving cell measurements to the Rel-18 WI

  - Option 2: For serving cell: In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN:

    - The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

#### Issue 2-2: NGSO, serving cell measurement - relaxation factor N

Status Summary: No consensus to reduce relaxation factor N

Proposals

- Option 1: The values in tables used for deriving the serving cell measurement relaxation factors are reduced by factor 2 for NGSO for eMTC and NB-IoT in IDLE mode. (Ericsson)

- Option 2: Reuse the legacy mechanism of serving cell measurement relaxation factor N. (CMCC, MTK)

Recommendations for 2nd round: Continue discuss in the 2nd round.

#### Issue 2-3: GSO, neighbour cell measurement

Status Summary: Option 1 is not supported by companies in the 1st round.

Recommendations for 2nd round: No need to discuss in the 2nd round.

#### Issue 3-1: Discontinuous Coverage - clarification

Status Summary: No consensus

Proposals

- Option 1: (Nokia)

  - The definition of “in-coverage” for measurement continuation/resuming shall not depend on UE executing previous measurements to define the “in-coverage status”

  - RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by SystemInformationBlockType32

- Option 2: (Ericsson)

  - In-coverage is the time that UE has acquired the SI of the target cell, and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario.

  - When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search until UE has acquired the system information carrying the Ephemeris.

Recommendations for 2nd round: Continue discuss in the 2nd round.

#### Issue 3-2: Discontinuous Coverage - wording alignment

Status Summary:  proposal 1-1 has majority support, but one questions raised.

Proposals

  - Proposal 1 (revised from the previous meeting): to align with the definitions in TS36.304 and the design of t-service, in TS 36.133, Section 4.6A, revise the wording  “If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area.” as follows:

- Proposal 1-1: “If SystemInformationBlockType32 [2] has been received and the UE has determined it will be out of coverage after t-service, the UE is not required to perform neighbor cell measurements during the last [k] SI modification periods, before t-Service [2]” (Nokia)

Recommendations for 2nd round: Continue discuss in the 2nd round based on Proposal 1-1, regarding this questions:

“how does UE know whether SI will be modified or not?”

#### Issue 3-3: Discontinuous Coverage - revision on paging requirement

Status Summary: No consensus to revise paging requirement.

Proposals

- Option 1: Do not create additional requirements after t-serviceStart regarding UE paging interruption. (Nokia, Huawei)

- Option 2: Paging requirements apply when UE is in coverage, which is the time when UE has acquired the SI of the target cell and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario. (Ericsson)

Recommendations for 2nd round: Continue discuss in the 2nd round. Is it necessary to clarify the condition regarding “acquire the SIB1 and read the SI after regaining the coverage”?

#### Issue 3-4: NGSO, serving cell measurement upon t-Service-r17

Status Summary: No consensus on enhancements of serving cell measurements

Recommendations for 2nd round: Continue discuss in the 2nd round on the following options:. Please take the comments in 1st round into account.

Proposals

- Option 1:

  - If the UE is configured with DRX cycle ≥ [1.28] sec, then the UE shall meet the serving cell evaluation requirements defined for DRX cycle of [640] ms starting from at least 2 configured DRX cycles before ‘t-Service-r17’.

  - If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’.

- Option 2: Postpone the discussion on enhancements of serving cell measurements to the Rel-18 WI (IoT_NTN_enh).

#### Issue 4: For Cat-M1 UE in NGSO and eDRX, whether to revise the RLM requirement upon t-service

Status Summary: No consensus to revise RLM requirement.

Proposals

- Option 1: If the the eMTC UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, and the UE will not go into discontinuous coverage after ‘t-Service’, then at least during [1] eDRX cycle before t-Service-r17, the UE shall meet the non-DRX requirements (CMCC)

- Option 2: Do not apply additional requirements for UE configured with eDRX cycle before t-service for this WI. (Nokia, MTK)

Recommendations for 2nd round: Continue discuss in the 2nd round.

## Discussion on 2nd round (if applicable)

# Topic #2: RRM perf requirements (AI 6.8.6)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304375 | Qualcomm Korea | Observation 1: For stationary IoT NTN UE, on-chip GNSS receiver will significantly increase device cost and reduce device lifespan. Observation 2: Based on UE capability spec, stationary IoT NTN UEs installed at a specific location are not required to have GNSS receivers attached to the devices once the location information is obtained. Proposal 1: RAN4 defines GNSS simulator-based test approach only for UE UL transmission timing accuracy requirements and adopts AT command-based test approach for the rest of the test cases. The test case for UE UL transmission timing accuracy requirements is defined such that a stationary UE is allowed to use a detachable external peripheral module to be used to receive GNSS signal generated by GNSS simulator and program the acquired the UE position from the GNSS simulator into the UE transceiver before starting the test. The mechanism and interface for UE location programming between the peripheral module and the UE transceiver are UE vendor/OEM-specific and shall not be defined by the conformance test spec. |
| R4-2304504 | Nokia, Nokia Shanghai Bell | Observation 1: For the transmit timing test, the usage of the AT command to convey UE position does not fully test the new functionality introduced in NTN. Proposal 1: For all UE timing test cases, the UE location is acquired by GNSS positioning, and the test parameter for GNSS simulated signal power levels are defined for the test setup. Observation 2: It is preferred for RRM tests, specially the transmit timing test case, that that the elevation angle between the UE and the serving cell is not set close to 90 degrees for NGSO scenarios, as there will be almost no variation on the UE transmit timing. Proposal 2: Create reference SIB31 configuration for serving and neighbor cells for the NTN test cases. Proposal 3: In the configuration for NGSO test cases, the satellite information shall be initiated at a position where the elevation angle (between UE and satellite of the serving cell) measured by the UE at the chosen GNSS position used for the test is below [35] degrees. The serving satellite is set to be moving away from the UE whereas the neighbor satellite is set to be approaching the UE position during the tests. a. The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test. b. If RAN4 identifies some test cases where a higher elevation angle is more meaningful, RAN4 might specify a second configuration for such test cases. c. FFS whether to define that the UE should be in the orbital plane Proposal 4: FFS if neighbor and serving satellites need to be necessarily configured on the same orbital plane. Proposal 5: The common delay parameters provided in SIB31 for the RRM test cases can be obtained as follows: for each satellite motion trajectory configuration vector, define one GNSS location on Earth to represent the uplink time synchronization reference point. The common delay is then calculated from the distance between the satellite position defined by the satellite motion trajectory and the time synchronization reference point Proposal 6: Use the same PHR table for GSO and NGSO operations (Option 1). Proposal 7: For test cases defined for M1 UEs, define the channel bandwidth equal to [10] MHz Proposal 8: For UEs in CE mode, when the number of repetitions is greater than 1, RAN4 to discuss the segment duration (TxDuration) to be used in the reference configuration. |
| R4-2304649 | CMCC | Proposal 1: If the neighbor cell measurement requirements are agreed to applied for GEO in core sepc, then the test cases should be also introduced for GEO. Otherwise, no need to introduce neighbor cell measurement test cases. Proposal 2: Define UE initial transmission timing test for UL segmented transmission, the test parameter in A.7.1.18 Test 1 can be the starting point. Proposal 3: In test case, set Nsegmentprecompensation (npusch-TxDuration) as 32ms, and Ngapprecompensation (ntn-SegmentedPrecompensationGaps) as 2 slots. Proposal 4: For the test requirement of UL segmented transmission, - The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in core requirements until the UE transmit timing offset is withinNTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN respective to the first detected path (in time) of DL SSB. The adjustment will be only performed at the start of a transmission segment boundary. - For the first transmission in each segment, the test system shall verify that the UE transmit timing offset stays within NTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN of the first detected path of DL SSB. Proposal 5: Follow the same approach as NR NTN for GNSS Position Acquisition Method for UE transmit timing test cases. |
| R4-2304820 | MediaTek inc. | Proposal 1: For NB-IoT LEO, legacy PHR table can be reused for IoT NTN. Proposal 2: The legacy RSRP/RSRP/NRSRP/NRSRQ measurement report mapping can be reused for IoT NTN. Proposal 3: Introduce the following clauses for regarding the measurement accuracy for IoT NTN: - 9.1.21A Measurement accuracy for UE category M1 for satellite access - 9.1.22A Measurement accuracy for UE Category NB1 for satellite access - 9.1.23A Power Headroom for UE Category NB1 for Satellite Access Proposal 4: Introduce the following test cases of intra-frequency measurements for GEO, for NB-IoT and M1, respectively: - For NB-IoT: RRC_IDLE state; Cell Re-Selection; HD – FDD Intra frequency case for UE Category NB1 in normal coverage; NB 1-1. HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation; NB 1-3. HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX; NB 1-4. RRC Connection Control; RRC Re-establishment; HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage; NB 2-2. - For M1: RRC_IDLE state; Cell Re-Selection; E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage; M1 1-1. E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage; M1 1-2. E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation; M1 1-5. E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation; M1 1-6. RRC Connection Control; RRC Re-establishment; E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA; M1 3-1. E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA; M1 3-2. |
| R4-2305261 | Huawei, HiSilicon | Proposal 1: Do not introduce test cases involving neighbor cell measurement for GSO. Observation 1: For transmit timing requirements (Te for the first transmission), it does not make much difference compared with repetition cases in legacy TN. Proposal 2: Do not define dedicated UE initial transmission timing test cases for UL segmented transmission. Observation 2: For PHR table selection, what matters is the coverage level selection during random access regardless of the type of satellites. Proposal 3: For NB-IoT and eMTC, legacy PHR table can be reused for IoT NTN. |
| R4-2305735 | Ericsson | Observation 1: The supported frequency bands for eMTC and NB-IoT over NTN are different from frequency bands of eMTC and NB-IoT over TN. - RAN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands. - RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands. - RAN4 to follow the same approach as NR NTN for GNSS Position Acquisition Method for UE transmit timing test cases. Observation 2: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 1 200 km above the earth. - For UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 4. - For UE in normal coverage managed by LEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 5. - For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 6. - For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 7. - For UE category M1 operation with GSO and NGSO, the legacy UE category M1 PHR reporting tables are reused. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

#### Issue 1-1: Measurement accuracy

Proposals

- Proposal 1: (Ericsson)

  - RANN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

  - RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- Proposal 2: Introduce the following clauses for regarding the measurement accuracy for IoT NTN: (MTK)

  - 9.1.21A        Measurement accuracy for UE category M1 for satellite access

  - 9.1.22A        Measurement accuracy for UE Category NB1 for satellite access

  - 9.1.23A        Power Headroom for UE Category NB1 for Satellite Access

Recommended WF

  - RANN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

  - RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

  - RAN4 to define Power Headroom for UE Category NB1 for Satellite Access in separate clause

Tentative agreement in offline session:

- RAN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to define Power Headroom for UE Category NB1 and M1 for Satellite Access in separate clause

| Company | Comments |
| --- | --- |
| Huawei | Fine with fist two bullets. For PHR, why only NB1 is considered? |
| MTK | On the 3rd bullet, it can be revised to include eMTC - RAN4 to define Power Headroom for UE Category NB1 “and M1” for Satellite Access in separate clauses |
| CMCC | Fine with the recommended WF and further revision from MTK on 3rd bullet |
| Nokia | Fine with both proposals. |
| Ericsson | Agree to the WF, but PHR for cat-M should also be in separate clause although the requirements are identical. But we can use references to point to legacy requirements. |

#### Issue 1-2: Measurement Reporting Table

Proposals

- Proposal 1: The legacy RSRP/RSRP/NRSRP/NRSRQ measurement report mapping can be reused for IoT NTN. (MTK)

Recommended WF

- Is Proposal 1 agreeable?

Tentative agreement in offline session:

- Agree Proposal 1.

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 1. |
| MTK | proposal 1. |
| Nokia | Fine with proposal 1. |
| Ericsson | Proposal 1 is agreeable. |

#### Issue 2: PHR reporting for NB-IoT

Background

Agreements (from RAN4#105)

  - For NB-IoT operation with GSO, the legacy NB-IoT PHR reporting tables are used as baseline.

  - For NB-IoT operation with NGSO, FFS need to adjust the reporting values compared to the legacy reporting values.

  - For UE category M1 operation with GSO and NGSO, the legacy UE category M1 PHR reporting tables are used as baseline.

Agreements (from RAN4#106)

- Option 1: For NB-IoT and eMTC, legacy PHR table can be reused for IoT NTN.

- Option 2: For NB-IoT operation with NGSO, the RAN4 to discuss the need to implement a different table in the specification for NGSO.

Proposals

- Proposal 1: For NB-IoT, legacy PHR tables can be reused for IoT NTN. (Huawei, MTK)

- Proposal 1a: Use the same PHR table for GSO and NGSO operations (Nokia)

- Proposal 2: For NB-IoT LEO (Ericsson)

- Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by LEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

- Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by LEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- Proposal 3: For NB-IoT GEO, modify the values in exiting tables to include more negative PHR reporting values. (Ericsson)

- Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -30 |
| POWER_HEADROOM_1 | -30 PH -20 |
| POWER_HEADROOM_2 | -20 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

Recommended WF:

- No need to discuss PHR reporting tables for M1, as it has been agreed to reuse the legacy tables.

- For NB-IoT LEO, legacy PHR reporting values can be reused for IoT NTN.

  - Note: moderator’s understanding is that the values in Proposal 2 are the same as the legacy

- For NB-IoT GEO, discuss if proposal 3 is agreeable or the legacy table is sufficient.

Discussion in offline:

- Ericsson: GEO need higher power. Only 4 fields in NB-IoT. Need to modify the values for lower ranges.

- Huawei: depends on CE-level, not necessary, should keep positive values also.

- Nokia: agree with Huawei. Maximum power most of time.

- Qualcomm: what’s the benefit expected?

  - Ericsson: benefit for repetition

  - Qualcomm: can it be dynamic for GEO IoT? Need to take accuracy into account.

Tentative agreement in offline session:

- No need to discuss PHR reporting tables for M1, as it has been agreed to reuse the legacy tables.

- For NB-IoT LEO, legacy PHR reporting values can be reused for IoT NTN.

- For NB-IoT GEO, continue discussion.

| Company | Comments |
| --- | --- |
| Qualcomm | We share the same understanding as Moderator for Proposal 2. We agree that the range of PHR for GEO may need to be adjusted as proposed by Ericsson (Proposal 3). It would be appreciated if Ericsson could provide more details about, e.g. Tx power, directivity gain, etc. It would help us better understand where the values came from. |
| Huawei | Support proposal 1/1a. The PHR is finally decided by the CE level selected during RACH. GEO does not necessarily mean larger Pathloss. |
| MTK | Regarding GEO PHR, because the power is limited in GEO and likely the POWER_HEADROOM_0 will be used in most cases, we tend to think the benefit on optimizing the ranges (Proposal 3) is limited. |
| Nokia | We agree with Huawei. The tables are dependent on the CE level, not in the satellite class. As such, we prefer to just re-use the legacy tables. Support 1/1a. |
| Ericsson | For cat-M1, we agree that the legacy PHR reporting values can be reused. For NB-IoT GEO, we support proposal 3. But for NB-IoT GEO, it is noteworthy that the GEO satellites operate around 36 000 km above the earth while LEO satellites operate around 1 200 km above the earth. The UE uplink transmit power is related to the operating altitude of the satellites, e.g. the UE may therefore need to use more uplink transmit power for reaching a satellite at 36 00 km above the earth compared to reaching a satellite above 16 000 km above the earth. TS 38.821 contains free space pathloss analysis for GEO and different configurations of LEO, where it is shown that the free space pathloss is higher for GEO compared to LEO, see the results in Table 6.1.3.3-1 for the scenarios defined in Table 6.1.1.1-9. This means, for a UE with only 4 reportable values, it is reasonable to assume that more negative values are needed to report for GEO compared to LEO since the UE is expected to operate at higher power for GEO compared to LEO. |

#### Issue 3: GNSS Position Acquisition Method

Proposals

- Proposal 1: RAN4 to follow the same approach as NR NTN for GNSS Position Acquisition Method for UE transmit timing test cases. (CMCC, Ericsson)

- Proposal 1a:  For all UE timing test cases, the UE location is acquired by GNSS positioning, and the test parameter for GNSS simulated signal power levels are defined for the test setup. (Nokia)

- Proposal 2: RAN4 defines GNSS simulator-based test approach only for UE UL transmission timing accuracy requirements and adopts AT command-based test approach for the rest of the test cases. The test case for UE UL transmission timing accuracy requirements is defined such that a stationary UE is allowed to use a detachable external peripheral module to be used to receive GNSS signal generated by GNSS simulator and program the acquired the UE position from the GNSS simulator into the UE transceiver before starting the test. The mechanism and interface for UE location programming between the peripheral module and the UE transceiver are UE vendor/OEM-specific and shall not be defined by the conformance test spec. (Qualcomm)

Recommended WF

- RAN4 defines GNSS simulator-based test approach for UE UL transmission timing accuracy requirements and adopts AT command-based test approach for the rest of the test cases.

- Test parameter for GNSS signal power levels defined in [B.6.1] is reused.

- Discuss regarding this proposal

  - The test case for UE UL transmission timing accuracy requirements is defined such that a stationary UE is allowed to use a detachable external peripheral module to be used to receive GNSS signal generated by GNSS simulator and program the acquired the UE position from the GNSS simulator into the UE transceiver before starting the test. The mechanism and interface for UE location programming between the peripheral module and the UE transceiver are UE vendor/OEM-specific and shall not be defined by the conformance test spec.

Tentative agreement in offline session:

- [GNSS or GNSS signal emulation] test approach for UE UL transmission timing accuracy requirements and adopts AT command-based test approach for the rest of the test cases.

- Test parameter for GNSS signal power levels defined in [B.6.1] is reused.

- Discuss whether to send LS to RANP on the highlighted part. The above bullets can be revised based on RANP feedback if any.

| Company | Comments |
| --- | --- |
| Qualcomm | Okay with Recommended WF. Regarding whether IoT NTN UE shall have an on-chip GNSS receiver, we do not think it is desirable from use-case perspective because IoT UEs, particularly NB-IoT, are mostly for metering devices installed at specific location, hence, no support of mobility. With this use case, if such IoT UEs are forced to have on-chip GNSS receivers, it will significantly increase the cost of the overall device and reduce the lifespan of the device. The idea of allowing UE to have a detachable external peripheral module that reads GNSS signal from GNSS simulator and programs the location to the UE in Proposal 2 is proposed in order to make the test procedure compliant with IoT NTN UE capability description (TS 38.306) and satisfy the group’s desire to have at least one test case that should be tested in a fully integrated manner. |
| Huawei | Fine with the recommended WF. But for the highlighted part, we are wondering what is the spec impact. Does RAN4 have sufficient authority to make such conclusion? |
| CMCC | Fine with the first two bullets in the recommended WF. For the third bullets, we think this can be used as a test method for IOT device without on-chip GNSS, however, whether such method should be specified in TS should be discussed. In our view, there is no need to specify it into TS36.133. |
| Nokia | We do not oppose to it on at a first sight. However, this is not in line with the Stage 2 requirements on TS 36.300 (clause 23.21.1): “In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.” So, it seems that the impact of the highlighted proposal may go beyond of RAN4 scope and power. We support to send a LS to other RAN groups (e.g. RAN2, RAN1, RAN-P). Once the requirement for GNSS is revised, we may support the test case proposal. |

#### Issue 4: Test for Segmented UL Transmissions

Background

Agreements (from RAN4#106)

  - RAN4 will not re-define UE behaviour during the gap between segment transmissions

  - There is no need to define additional gap applicability rules in RAN4

  - Tx samples within gap between segment transmissions should not be taken into account in performance measurement at TE.

  - FFS: whether to define UE initial transmission timing test for UL segmented transmission

Proposals

- Proposal 1: Do not define dedicated UE initial transmission timing test cases for UL segmented transmission. (Huawei)

- Proposal 2: Define UE initial transmission timing test for UL segmented transmission, the test parameter in A.7.1.18 Test 1 can be the starting point. (CMCC)

  - In test case, set Nsegmentprecompensation  (npusch-TxDuration) as 32ms, and Ngapprecompensation (ntn-SegmentedPrecompensationGaps) as 2 slots.

  - For the test requirement of UL segmented transmission,

    - The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in core requirements until the UE transmit timing offset is withinNTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN respective to the first detected path (in time) of DL SSB. The adjustment will be only performed at the start of a transmission segment boundary.

    - For the first transmission in each segment, the test system shall verify that the UE transmit timing offset stays within NTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN of the first detected path of DL SSB.

Recommended WF

- Discuss proposals.

Offline Discussion:

- CMCC: test for new feature. Test configuration can be FFS.  To test timing on each segment.

- Huawei: what’s the test purpose. On the 1st transmission. Is necessary?

- Qualcomm: same as Huawei. test on 1st transmission or transmission during segments?

- Huawei: seems the same as legacy.

- CMCC: on 1st transmission of each segments.

Tentative agreement in offline session:

- Continue discussion.

| Company | Comments |
| --- | --- |
| Qualcomm | Okay with Proposal 1. |
| Huawei | Support proposal 1. UL segment does not bring extra efforts on Te but just allow UE to adjust timing within repetition. |
| CMCC | Proposal 2. We prefer to add a test case for segmented transmission, since it is a new feature. Although Te_ntn requirements are not changed, the UE behavior is different from legacy. |

#### Issue 5-1: Test Cases Configuration – M1 channel bandwidth

Background

Agreements (from RAN4#105), including

  - RAN4 to introduce new side conditions for IDLE and CONNECTED mode measurements for both NB-IoT and eMTC NTN.

  - RAN4 to introduce new RMC assuming the channel bandwidth of 1.4MHz for eMTC NTN.

  - Reuse the test case environment and setup of NR NTN as baseline.

Proposals

- Proposal 1: For test cases defined for M1 UEs, define the channel bandwidth equal to [10] MHz (Nokia)

Moderator’s Note:

- According to 5.3A, Channel bandwidth for category M1 is 1.4MHz. And it has agreed to introduce new RMC assuming the channel bandwidth of 1.4MHz for eMTC NTN.

Recommended WF

- No need to discuss

#### Issue 5-2: Test Cases Configuration – segment duration

Proposals

- Proposal 1: For UEs in CE mode, when the number of repetitions is greater than 1, RAN4 to discuss the segment duration (TxDuration) to be used in the reference configuration. (Nokia)

Recommended WF: discuss the proposal

| Company | Comments |
| --- | --- |
| Huawei | Depends on issue 4. |

#### Issue 6-1: Configuration of Satellite parameters

Background

Agreements (from RAN4#106)

Issue 8-1: Test setup and Satellite assistance information

- RAN4 to reuse the Rel-17 NR NTN test framework for IoT NTN tests which includes predefining configurations for serving cell’s satellite.

- Create reference SIB31 configuration

- Not to consider the configuration of the common delay parameters, as discussed in NR NTN in R4-2220711

Issue 8-3: Satellite trajectory and TE reference propagator model

- Follow the same reference model as NR NTN for satellite trajectory and TE reference propagator model

Proposals

- Proposal 1: Create reference SIB31 configuration for serving and neighbor cells for the NTN test cases. (Nokia)

- Proposal 2: The common delay parameters provided in SIB31 for the RRM test cases can be obtained as follows: for each satellite motion trajectory configuration vector, define one GNSS location on Earth to represent the uplink time synchronization reference point. The common delay is then calculated from the distance between the satellite position defined by the satellite motion trajectory and the time synchronization reference point (Nokia)

Recommended WF

- Could we re-use the parameter settings (if applicable) for SIB19 as specified in A.3.36.4 in 38.133?

Discussion:

- Qualcomm: impact on K_offset?

Tentative agreement in offline session:

- re-use the parameter settings (if applicable) for SIB19 as specified in A.3.36.4 in 38.133.

  - FFS how to set common delay for LEO.

| Company | Comments |
| --- | --- |
| Qualcomm | Okay with Recommended WF only for the applicable information. |
| Huawei | Okay with Recommended WF only for the applicable information. |
| MTK | We can reuse the applicable parameter setting for SIB19. Example on SIB31 is given by CR R4-2305653. |
| CMCC | Fine with the recommended WF |
| Nokia | Ok with the WF. However, the reference configuration in NR neglects common delay (sets it to zero). Are we willing to go with the same assumption? |

#### Issue 6-2: High level test procedure for SAN RRM tests

- Proposal 1: In the configuration for NGSO test cases, the satellite information shall be initiated at a position where the elevation angle (between UE and satellite of the serving cell) measured by the UE at the chosen GNSS position used for the test is below [35] degrees. The serving satellite is set to be moving away from the UE whereas the neighbor satellite is set to be approaching the UE position during the tests. (Nokia)

  - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test.

  - If RAN4 identifies some test cases where a higher elevation angle is more meaningful, RAN4 might specify a second configuration for such test cases.

  - FFS whether to define that the UE should be in the orbital plane

- Proposal 2: FFS if neighbor and serving satellites need to be necessarily configured on the same orbital plane. (Nokia)

Recommended WF

- Could we re-use the “High level test procedure for SAN RRM tests” as specified in B.5 in 38.133?

Tentative agreement in offline session:

- re-use the applicable parts in “High level test procedure for SAN RRM tests” as specified in B.5 in 38.133

| Company | Comments |
| --- | --- |
| Qualcomm | Okay with Recommended WF only for the applicable parts. |
| Huawei | Okay with Recommended WF only for the applicable parts. |
| MTK | We can take B.5 in 38.133 as baseline unless necessary adjustment is identified. |
| CMCC | Fine with the recommended WF |
| Ericsson | Fine with the recommended WF. |

#### Issue 7: Tests involving neighbor cell measurement

Background

Agreements (from RAN4#106)

  - Do not introduce the following test cases in R17 IOT-NTN for NGSO case

    - Cell-reselection

      - Intra-frequency measurement

      - Inter-frequency measurement

    - RRC Re-establishment

    - RRC Connection Release with Redirection

    - E-UTRAN inter frequency and intra frequency measurements

    - Measurement accuracy

      - Intra-frequency measurement accuracy requirements

      - Inter-frequency measurement accuracy requirements

    - HO requirements

  - FFS on test cases for GSO case

Proposals

- Proposal 1: Do not introduce test cases involving neighbor cell measurement for GSO (Huawei)

- Proposal 2: If the neighbor cell measurement requirements are agreed to applied for GEO in core spec, then the test cases should be also introduced for GEO. Otherwise, no need to introduce neighbor cell measurement test cases. (CMCC)

- Proposal 3: Introduce the following test cases of intra-frequency measurements for GEO, for NB-IoT and M1, respectively: (MTK)

| RRC_IDLE state | Cell Re-Selection | HD – FDD Intra frequency case for UE Category NB1 in normal coverage | NB 1-1 |
| --- | --- | --- | --- |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation | NB 1-3 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX | NB 1-4 |
| RRC Connection Control | RRC Re-establishment | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | NB 2-2 |

| RRC_IDLE state | Cell Re-Selection | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-2 |
|  |  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-5 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-6 |
| RRC Connection Control | RRC Re-establishment | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-1 |
|  |  | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-2 |

Recommended WF

- Discuss the core part (Issue 1) in the 1st round.

## Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2305060 Nokia, Nokia Shanghai Bell | Moderator’s Note: TC on A.13.3.2, NB, RA |
|  | Company: |
| R4-2305262 Huawei, HiSilicon | Moderator’s Note: TC on A.13.4.1&2, NB, timing |
|  | Company: |
| R4-2305653 MediaTek inc. | Moderator’s Note: 1) SIB31 in A.3.28, 2) TC on A.13.4.3, NB, RLM |
|  | Nokia:We don’t have a strong objection to this CR, but can we discuss as a group whether we think we should use NPDCCH reference channel with 2 transmitter antennas or with 1? My initial assessment is that 1 transmitter antenna is more fair for NTN. |
|  | MTK: Thanks Nokia for the comment. We also think 1Tx is more reasonable for NTN and we also notice that Demod session has the agreement as follows (WF R4-2220278) Issue 1-2-3: Antenna configuration Agreement: - Define requirements with 1Tx1Rx for both Cat-M1 and NB-IoT over NTN. Thus we can revise this CR for 1 TX as follows: NPDCCH parameters defined in A.3.1.6.3; R.30 HD-FDD. Antenna Configuration; 1x1. We can have 1 issue to discuss it in the 2nd round. |
| R4-2304388 Qualcomm | Moderator’s Note: TC on A.13.6.2, NB, CQR |
|  | Nokia:We don’t have a strong objection to this CR, but can we discuss as a group whether we think we should use NPDCCH reference channel with 2 transmitter antennas or with 1? My initial assessment is that 1 transmitter antenna is more fair for NTN. |
|  | MTK: Similar comment as in R4-2305653 |
| R4-2305346 Huawei, HiSilicon | Moderator’s Note: TC on A.14.3.2, M1, RA |
|  | Nokia:This CR sets the Channel Bandwidth to 10 MHz. But as stated in Issue 5-1, channel bandwidth should be set to 1.4 MHz for M1. |
| R4-2305061 Nokia, Nokia Shanghai Bell | Moderator’s Note: TC on A.14.4.1&2, M1, timing |
|  | Huawei: Suggest to discuss whether we need to consider additional margin in the Te TC. |
| R4-2305736 Ericsson | Moderator’s Note: TC on A.14.4.3, M1, RLM, no DRX |
|  | Nokia:This CR sets the Channel Bandwidth to 10 MHz. But as stated in Issue 5-1, channel bandwidth should be set to 1.4 MHz for M1. |
| R4-2304650 CMCC | Moderator’s Note: TC on A.14.4.3, M1, RLM, DRX |
|  | Nokia:This CR sets the Channel Bandwidth to 10 MHz. But as stated in Issue 5-1, channel bandwidth should be set to 1.4 MHz for M1. |
| R4-2305737 Ericsson | Moderator’s Note: TC on A.14.6.2, M1, CQR |
|  | Nokia: This CR sets the Channel Bandwidth to 10 MHz. But as stated in Issue 5-1, channel bandwidth should be set to 1.4 MHz for M1. |
| R4-2305798 Ericsson | Moderator’s Note: M1, conditions in annex B |
|  | Nokia: The inter-frequency measurement conditions should be captured in Rel 18 IoT_NTN_enh WI. |
| R4-2305799 Ericsson | Moderator’s Note: NB, conditions in annex B |
|  | Huawei: Are these conditions for intra-/inter-f neighbor cell for NB connected mode? If so, we think they should captured in Rel-18 Iot NTN enh. |
| R4-2305654 MediaTek inc. | Moderator’s Note: Measurement accuracy and PHR (9.1.21A/22A/23A) |
|  | Company: |

## Summary for 1st round

### Open issues

#### Issue 1-1: Measurement accuracy

Tentative agreement in offline session:

- RAN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to define Power Headroom for UE Category NB1 and M1 for Satellite Access in separate clause

Recommendations for 2nd round: Issue closed.

#### Issue 1-2: Measurement Reporting Table

Tentative agreement in offline session:

- Agree Proposal 1.

Recommendations for 2nd round: Issue closed.

#### Issue 2: PHR reporting for NB-IoT

Tentative agreement in offline session:

- No need to discuss PHR reporting tables for M1, as it has been agreed to reuse the legacy tables.

- For NB-IoT LEO, legacy PHR reporting values can be reused for IoT NTN.

- For NB-IoT GEO, continue discussion.

Recommendations for 2nd round: Continue discussion on for NB-IoT GEO:

- Option 1: reuse legacy PHR reporting values for IoT NTN

- Option 2: modify the values in exiting tables to include more negative PHR reporting values

- Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -30 |
| POWER_HEADROOM_1 | -30 PH -20 |
| POWER_HEADROOM_2 | -20 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

#### Issue 3: GNSS Position Acquisition Method

Tentative agreement in offline session:

- [GNSS or GNSS signal emulation] test approach for UE UL transmission timing accuracy requirements and adopts AT command-based test approach for the rest of the test cases.

- Test parameter for GNSS signal power levels defined in [B.6.1] is reused.

- Discuss whether to send LS to RANP on the highlighted part. The above bullets can be revised based on RANP feedback if any.

Recommendations for 2nd round: Discuss LS in email sub-tread

#### Issue 4: Test for Segmented UL Transmissions

No consensus to define UE initial transmission timing test cases for UL segmented transmission.

Proposals

- Proposal 1: Do not define dedicated UE initial transmission timing test cases for UL segmented transmission. (Huawei)

- Proposal 2: Define UE initial transmission timing test for UL segmented transmission, the test parameter in A.7.1.18 Test 1 can be the starting point. (CMCC)

  - In test case, set Nsegmentprecompensation  (npusch-TxDuration) as 32ms, and Ngapprecompensation (ntn-SegmentedPrecompensationGaps) as 2 slots.

  - For the test requirement of UL segmented transmission,

    - The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in core requirements until the UE transmit timing offset is withinNTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN respective to the first detected path (in time) of DL SSB. The adjustment will be only performed at the start of a transmission segment boundary.

    - For the first transmission in each segment, the test system shall verify that the UE transmit timing offset stays within NTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN of the first detected path of DL SSB.

Recommendations for 2nd round: Continue discussion.

#### Issue 5-2: Test Cases Configuration – segment duration

Status Summary: related to Issue 4

Recommendations for 2nd round: Merge to Issue 4.

#### Issue 6-1: Configuration of Satellite parameters

Tentative agreement in offline session:

- re-use the parameter settings (if applicable) for SIB19 as specified in A.3.36.4 in 38.133.

  - FFS how to set common delay for LEO.

Recommendations for 2nd round: continue discussion how to set common delay for LEO.

#### Issue 6-2: High level test procedure for SAN RRM tests

Tentative agreement in offline session:

- re-use the applicable parts in “High level test procedure for SAN RRM tests” as specified in B.5 in 38.133

Recommendations for 2nd round: Issue closed.

#### Issue 7: Tests involving neighbor cell measurement

No discussion in the 1st round

Proposals

- Proposal 1: Do not introduce test cases involving neighbor cell measurement for GSO (Huawei)

- Proposal 2: If the neighbor cell measurement requirements are agreed to applied for GEO in core spec, then the test cases should be also introduced for GEO. Otherwise, no need to introduce neighbor cell measurement test cases. (CMCC)

- Proposal 3: Introduce the following test cases of intra-frequency measurements for GEO, for NB-IoT and M1, respectively: (MTK)

| RRC_IDLE state | Cell Re-Selection | HD – FDD Intra frequency case for UE Category NB1 in normal coverage | NB 1-1 |
| --- | --- | --- | --- |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation | NB 1-3 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX | NB 1-4 |
| RRC Connection Control | RRC Re-establishment | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | NB 2-2 |

| RRC_IDLE state | Cell Re-Selection | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-2 |
|  |  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-5 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-6 |
| RRC Connection Control | RRC Re-establishment | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-1 |
|  |  | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-2 |

Recommendations for 2nd round: to discuss the specific test list in the 2nd round.

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NB-IoT/eMTC RRM requirements for NTN | MediaTek |  |
|  | LS on GNSS position acquisition in timing test | Qualcomm | To: RAN5; |

Existing tdocs

| Tdoc number |  | Revised to |  | Title |  | Source |  | Recommendation |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-2304476 |  |  |  | CR to TS 36.307 (Rel-17): release independence RRM requirements for IoT NTN |  | Sony |  | Revised |  | Revise for 1st round conclusion on Issue 1, e.g. remove HO for Cat-M1 (5.5A) in Rel-17 36.307 and add necessary notes |  |
| R4-2304477 |  |  |  | CR to TS 36.307 (Rel-18): release independence RRM requirements for IoT NTN |  | Sony |  | return to |  | Cat A |  |
| R4-2305260 |  |  |  | DraftCR on maintenance of NB-IoT for IoT NTN |  | Huawei, HiSilicon |  | Revised |  | Core |  |
| R4-2305738 |  |  |  | Correction to IDLE mode cat-M IoT NTN requirements |  | Ericsson |  | Revised |  | Core |  |
| R4-2305652 |  |  |  | CR on RRM core requirements maintenance for LTE NB-IoT/eMTC over NTN Re-establishment |  | MediaTek inc. |  | Revised |  | Core |  |
| R4-2305060 |  |  |  | DraftCR on TS 36.133 Random Access Test Cases for NB-IoT over NTN |  | Nokia, Nokia Shanghai Bell |  | Agreeable |  | Perf |  |
| R4-2305262 |  |  |  | DraftCR on timing test cases of NB-IoT for IoT NTN |  | Huawei, HiSilicon |  | Agreeable |  | Perf |  |
| R4-2305653 |  |  |  | Introduction of test cases of NB-IoT Radio Link Monitoring and reference configurations for satellite access |  | MediaTek inc. |  | Revised |  | Perf |  |
| R4-2304388 |  |  |  | draft Cat-B CR CQI for NB-IoT NTN |  | Qualcomm Korea |  | Revised |  | Perf |  |
| R4-2305346 |  | draftCR on TCs for random access for eMTC over NTN | Huawei, HiSilicon | Revised | Perf |
| R4-2305061 |  | DraftCR on TS 36.133 Timing Test Cases for eMTC over NTN | Nokia, Nokia Shanghai Bell | Revised | Perf |
| R4-2305736 |  | RLM test cases for eMTC over NTN | Ericsson | Revised | Perf |
| R4-2304650 |  | draft CR on E-UTRAN RLM test for Cat-M1 UE in CEMode A in DRX | CMCC | Revised | Perf |
| R4-2305737 |  | Channel quality reporting accuracy test cases for eMTC over NTN | Ericsson | Revised | Perf |
| R4-2305798 |  | DraftCR on conditions for UE Cat-M1 bands in annex B | Ericsson | Revised | Perf |
| R4-2305799 |  | Draft CR on conditions for NB-IoT bands in annex B | Ericsson | Revised | Perf |
| R4-2305654 |  | Introduction of requirement for measurement accuracy and power headroom for Satellite Access | MediaTek inc. | Revised | Perf |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents