3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2306197

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			6.4.5

Source:	Moderator (Qualcomm Incorporated)

Title:	Topic summary for [106-bis-e][117] LTE_NR_US_900MHz

Document for:	Information

# Introduction

This document is a summary of the contributions submitted under agenda items 6.4.2 (Band definition and co-existence requirements), 6.4.3 (UE RF requirements), and 6.4.4 (BS RF requirements) of the RAN4 #106-bis-e electronic meeting.

# Topic #1: UE RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2305121 | ZTE Corporation | draft CR to TS 36.133: Introduction of 900 MHz LTE Band in the US Add band 106 to the relevant band group in the band groups table. Because the reference sensitivity of band 106 reuse the value of band 8, the band group of band 106 should be consistent with that of band 8. |
| R4-2305355 | ZTE Corporation | draft CR to TS36.101: the introduction of 900 MHz LTE new band To introduce the UE RF requirements of 900 MHz LTE new band into the spec. |
| R4-2305821 | Qualcomm Incorporated | Proposal: EARFCN for 3 MHz channels other than 899 MHz shall be disallowed in the specification. Proposal: An emission limit of -30 dBm/MHz is specified for UE coexistence to protect the downlinks of Band 5 and Band 26 when the 3 MHz LTE channel is centered at 899 MHz. No A-MPR is allowed. |

## Open issues summary

### Sub-topic 1-1

Issue 1-1: Channel numbering

- Proposals

  - Option 1: All channel numbers across the band are specified for 3 MHz channels (R4-2305355, ZTE)

  - Option 2: Only 3 MHz channel centered at 899 MHz is specified (R4-2305821, Qualcomm)

- Recommended WF

  - Discuss any channel number restrictions for 3 MHz LTE.

  - Also, comments are welcomed for channel number restrictions (if any) for 1.4 MHz LTE as well as 3.75 kHz and 15 kHz NB1 and NB2.

### Sub-topic 1-2

Issue 1-2: UE coexistence

- Proposals

  - Option 1:  No UE coexistence is specified, either from or to Band 106 (R4-2305355, ZTE)

  - Option 2:  An emission limit of -30 dBm/MHz is specified for UE coexistence to protect the downlinks of Band 5 and Band 26 when the 3 MHz LTE channel is centered at 899 MHz.  No A-MPR is allowed.  (R4-2305821, Qualcomm)

  - Option 3:  Other, please specify

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Channel numbering Sub topic 1-2: UE coexistence |
| Nokia | Sub topic 1-1: Option 1 Sub topic 1-2: Option 3: We will provide A-MPR simulation results against multiple protection levels in next RAN4 meeting, hence we want to postpone decision by one meeting, |
| Qualcomm | Sub topic 1-1: Channel numbering Option 2. The FCC only envisioned 3 MHz channels centered at 899 MHz and also reflected in the WF of R4-2303486. In our view, the question is not whether the channel should be restricted but how to implement the restriction in the specification. Channel numbering maybe by note “only EARFCN XXX is applicable to 3 MHz channels” is the most direct way to do this. Sub topic 1-2: UE coexistence We support option 2 based on our measurements, but we welcome results from other companies in the next meeting. |
| Murata | Sub topic 1-2: UE coexistence Option 2 is reasonable, and supporting data can be provided next meeting to support this option. 3rd order emissions do not come into play when CBW is restricted to 3MHz operation per the FCC requirement. ΔFOOB does extend into band5/26, and the suggested compromised limit to [-30] dBm/MHz from 891.5 to 894MHz is sufficient to cover the composite CIM5 and ACLR2 effect with no additional back-off or AMPR. |
| ZTE | Sub topic 1-1: Channel numbering Option 1. All channel numbers across the band should be specified. To clarify only 3 MHz channel centered at 899 MHz is specified, maybe we can note like Qualcomn comments. Sub topic 1-2: UE coexistence Option 2 and 3 are ok for us. |
| Anterix | Sub topic 1-1: Channel numbering Option 2 or 1 with the following exceptions. Due to current FCC allocations of narrowband communications in the adjacent bands, the 3 MHz center channel is by default 899 MHz, it currently cannot be any other value. In the future, if further clearing happens from 896-875.5 MHz and 900.5-901 MHz then the 3 MHz center could be placed within the 896-901 MHz allocation. We are OK with option 2 with the ability to select other EARFCN when/if the band gets cleared. Due to channel clearing inconsistencies across the United States markets there needs to be channel number flexibility for a 1.4 MHz LTE or NB1 /NB2 center channel placements within the 3 MHz allocation. Sub topic 1-2: UE coexistence Option2. We have performed initial coexistence testing and the results are similar to what is reported in R4-2305821 by Qualcomm and Murata. We agree with the proposed WF of an emission limit of -30 dBm/MHz stated in Option 2 is the best way forward. |
| AT&T | Sub topic 1-1: Channel numbering Option 2. This was the agreement in the WF in R4-2303486. We agree with QC’s proposed way to handle the restriction in the specification. Sub topic 1-2: UE coexistence Option 3. Thanks to QC for their measurements at this meeting. We prefer to allow additional companies to present their results at the next meeting as already confirmed by Nokia. |
| Ericsson | Sub topic 1-1: Channel numbering Option 1 for the wideband channels in case further clearing happens, EARFCN cannot be added afterwards for a band number. Sub topic 1-2: UE coexistence The -30 dBm/MHz should be considered further, the FCC limit of -13 dBm/MHz (RR Cat A spurious emissions) used as an argument is not intended for ensuring intra-system robust operations, if it were for this limit all UE-UE tables could be removed from the RAN4 specifications. The R&O states "With this Report and Order, we provide a variety of 900 MHz users a new opportunity to leverage broadband capacity for more robust communication networks, especially for industries that provide crucial services to the American public." Applications with less risk of UE-UE interference? |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2305121 draft CR to TS 36.133 | Nokia: OK |
|  | Company B |
| R4-2305355 draft CR to TS36.101 | Nokia: We should comeback in next meeting due to sub topic 1-2. |
|  | Qualcomm: The CR is incomplete. |
|  | AT&T: Agree with Nokia that we should comeback to this at the next meeting. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Sub topic 1-1: Channel numbering All EARFCN channel numbers are added to the specification. A note is also added to indicate a restriction to 899 MHz for 3 MHz channel bandwidth. Sub topic 1-2: UE coexistence -30 dBm/MHz protection with no A-MPR for the 3 MHz channel at 899 MHz is one option, but companies requested more time to conduct further studies. Candidate options: Recommendations for 2nd round: Sub topic 1-1: Channel numbering Work on a note to restrict the 3 MHz channel placement to 899 MHz. One suggestion is “In the present version of the specification, only EARFCN XXX is applicable for 3 MHz channels”. Comments and modifications are welcomed. Is it the common understanding that this same note will be implemented in both the BS and UE specifications? Sub topic 1-2: UE coexistence Wait for further studies to be presented at the next meeting. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2305121 draft CR to TS 36.133 | Agreeable |
| R4-2305355 draft CR to TS36.101 | To be noted, return to at the next meeting. |

## Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Channel numbering note |

# Topic #2: BS RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304222 | Huawei, Hisilicon | Not available. Withdrawn? |
| R4-2305356 | ZTE Corporation | CR to TS36.141 the introduction of 900 MHz LTE new band Not available. Replaced by R4-2305837 |
| R4-2305357 | ZTE Corporation | CR to TS37.141 the introduction of 900 MHz LTE new band Not available. Replaced by R4-2305838 |
| R4-2305358 | ZTE Corporation | CR to TS37.145-1 the introduction of 900 MHz LTE new band Not available. Replaced by R4-2305839 |
| R4-2305359 | ZTE Corporation | CR to TS37.145-2 the introduction of 900 MHz LTE new band Not available. Replaced by R4-2305840 |
| R4-2305360 | ZTE Corporation | CR to TS38.141-1 the introduction of 900 MHz LTE new band Not available. Replaced by R4-2305841 |
| R4-2305837 | ZTE Corporation | CR to TS36.141: the introduction of 900 MHz LTE new band |
| R4-2305838 | ZTE Corporation | CR to TS36.141: the introduction of 900 MHz LTE new band Moderator note: Title of the CR should be “CR to TS37.141” |
| R4-2305839 | ZTE Corporation | CR to TS37.145-1: the introduction of 900 MHz LTE new band |
| R4-2305840 | ZTE Corporation | CR to TS37.145-2: the introduction of 900 MHz LTE new band |
| R4-2305841 | ZTE Corporation | CR to TS38.141-1: the introduction of 900 MHz LTE new band |
| R4-2305578 | Nokia, Nokia Shanghai Bell | draftCR to 36.104 on introduction of Band 106 |
| R4-2305579 | Nokia, Nokia Shanghai Bell | draftCR to 36.141 on introduction of Band 106 |
| R4-2305580 | Nokia, Nokia Shanghai Bell | draftCR to 37.104 on introduction of Band 106 |
| R4-2305581 | Nokia, Nokia Shanghai Bell | draftCR to 37.141 on introduction of Band 106 |
| R4-2305582 | Nokia, Nokia Shanghai Bell | draftCR to 38.104 on introduction of Band 106 |
| R4-2305583 | Nokia, Nokia Shanghai Bell | draftCR to 38.141-1 on introduction of Band 106 |
| R4-2305584 | Nokia, Nokia Shanghai Bell | draftCR to 38.141-2 on introduction of Band 106 |

## Open issues summary

### Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2305837 CR to TS36.141 | Nokia: NB-IoT support changes are missing |
|  | Qualcomm: We think EARFCN for 3 MHz should be restricted to 899 MHz only. |
|  | Anterix: Add note for 3 MHz channel support at 899 MHz, if spectrum opens up, allows flexibility. |
|  | AT&T: Agree with QC that we need to add the restriction as agreed in the WF in R4-2303486. |
| R4-2305838 CR to TS37.141 | Nokia: NB-IoT support changes are missing, n106 should not be added at this time. “This requirement does not apply to E-UTRA BS operating in band 106.” to be changed to “This requirement does not apply to BS operating in band 106.” |
|  | Company B |
| R4-2305839 CR to TS37.145-1 | Nokia: “This requirement does not apply to E-UTRA BS operating in band 106.” to be changed to “This requirement does not apply to BS operating in band 106.” |
|  | Company B |
| R4-2305840 CR to TS37.145-2 | Nokia: “This requirement does not apply to E-UTRA BS operating in band 106.” to be changed to “This requirement does not apply to BS operating in band 106.” |
|  | Company B |
| R4-2305841 CR to TS38.141-1 | Nokia: for both changes the last column should not have any text since it is NR specification |
|  | Company B |
| R4-2305578 draftCR to 36.104 | Qualcomm: We think EARFCN for 3 MHz should be restricted to 899 MHz only. |
|  | Anterix: Add note for 3 MHz channel support at 899 MHz, if spectrum opens up, allows flexibility. |
|  | AT&T: Agree with QC that we need to add the restriction as agreed in the WF in R4-2303486. |
| R4-2305579 draftCR to 36.141 | Company A |
|  | Company B |
| R4-2305580 draftCR to 37.104 | Nokia: see comment to 37.104, revision is needed |
|  | Company B |
| R4-2305581 draftCR to 37.141 | Qualcomm: Does Band n5 and n26 need an exemption? |
|  | Nokia: there is the following text in draftCR: “The requirement does not apply to E-UTRA BS operating in Band 5 or 26.” However, it needs to be corrected to “The requirement does not apply to BS operating in Band 5 or 26.” |
| R4-2305582 draftCR to 38.104 | Nokia: revision is needed to add “The requirement does not apply to NR BS operating in Band 5 or 26.” |
|  | Company B |
| R4-2305583 draftCR to 38.141-1 | Nokia: revision is needed to add “The requirement does not apply to NR BS operating in Band 5 or 26.” |
|  | Company B |
| R4-2305584 draftCR to 38.141-2 | Nokia: revision is needed to add “The requirement does not apply to NR BS operating in Band 5 or 26.” |
|  | Company B |

## Summary for 1st round

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2305837 | Noted. To be merged with revision of R4-2305579. |
| R4-2305838 | To be revised, merged with R4-2305581. Should this be a draftCR instead of formal CR for this meeting? |
| R4-2305839 | To be revised. Should this be a draftCR instead of formal CR for this meeting? |
| R4-2305840 | To be revised. Should this be a draftCR instead of formal CR for this meeting? |
| R4-2305841 | Noted. To be merged with revision of R4-2305583 |
| R4-2305578 | To be revised |
| R4-2305579 | To be revised, merged with R4-2305837 |
| R4-2305580 | To be revised |
| R4-2305581 | Noted, to be merged with revision of R4-2305838 |
| R4-2305582 | To be revised |
| R4-2305583 | To be revised, merged with R4-2305841 |
| R4-2305584 | To be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2305838 CR to TS37.141 |  |
| Revision of R4-2305839 CR to TS37.145-1 |  |
| Revision of R4-2305840 CR to TS37.145-2 |  |
| Revision of R4-2305578 draftCR to 36.104 |  |
| Revision of R4-2305579 draftCR to 36.141 |  |
| Revision of R4-2305580 draftCR to 37.104 |  |
| Revision of R4-2305582 draftCR to 38.104 |  |
| Revision of R4-2305583 draftCR to 38.141-1 |  |
| Revison of R4-2305584 draftCR to 38.141-2 |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on UE RF requirements of US 900 MHz band | Anterix |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2304222 |  | BS RF requirements for US 900 MHz LTE | Huawei, Hisilicon | Withdrawn |  |
| R4-2305121 |  | draft CR to TS 36.133: Introduction of 900 MHz LTE Band in the US | ZTE Corporation | Agreeable |  |
| R4-2305355 |  | draft CR to TS36.101 the introduction of 900 MHz LTE new band | ZTE Corporation | Noted |  |
| R4-2305356 |  | CR to TS36.141 the introduction of 900 MHz LTE new band | ZTE Corporation | Withdrawn |  |
| R4-2305357 |  | CR to TS37.141 the introduction of 900 MHz LTE new band | ZTE Corporation | Withdrawn |  |
| R4-2305358 |  | CR to TS37.145-1 the introduction of 900 MHz LTE new band | ZTE Corporation | Withdrawn |  |
| R4-2305359 |  | CR to TS37.145-2 the introduction of 900 MHz LTE new band | ZTE Corporation | Withdrawn |  |
| R4-2305360 |  | CR to TS38.141-1 the introduction of 900 MHz LTE new band | ZTE Corporation | Withdrawn |  |
| R4-2305578 |  | draftCR to 36.104 on introduction of Band 106 | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2305579 |  | draftCR to 36.141 on introduction of Band 106 | Nokia, Nokia Shanghai Bell | Revised | Merged with R4-2305837 |
| R4-2305580 |  | draftCR to 37.104 on introduction of Band 106 | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2305581 |  | draftCR to 37.141 on introduction of Band 106 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2305582 |  | draftCR to 38.104 on introduction of Band 106 | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2305583 |  | draftCR to 38.141-1 on introduction of Band 106 | Nokia, Nokia Shanghai Bell | Revised | Merged with R4-2305841 |
| R4-2305584 |  | draftCR to 38.141-2 on introduction of Band 106 | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2305821 |  | 900 MHz band coexistence with Band 5/26 | Qualcomm Incorporated | Noted |  |
| R4-2305837 |  | CR to TS36.141: the introduction of 900 MHz LTE new band | ZTE Corporation | Noted |  |
| R4-2305838 |  | CR to TS36.141: the introduction of 900 MHz LTE new band | ZTE Corporation | Revised | Merged with R4-2305581. Should this be a draftCR instead of formal CR for this meeting? |
| R4-2305839 |  | CR to TS37.145-1: the introduction of 900 MHz LTE new band | ZTE Corporation | Revised | Should this be a draftCR instead of formal CR for this meeting? |
| R4-2305840 |  | CR to TS37.145-2: the introduction of 900 MHz LTE new band | ZTE Corporation | Revised | Should this be a draftCR instead of formal CR for this meeting? |
| R4-2305841 |  | CR to TS38.141-1: the introduction of 900 MHz LTE new band | ZTE Corporation | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-23xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-23xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents