3GPP TSG-RAN WG4 Meeting # 106bis-e                                                       R4-2306199

Online, April 17 – April 26, 2023

Agenda item:			6.6.6

Source:	Moderator (MediaTek Inc.)

Title:	Email discussion summary for [106-bis-e][119] IoT_NTN_FDD_LS_band

Document for:	Information

Introduction

This scope of the email thread [119] discussion is agenda items 6.6.1, 6.6.2, 6.6.3, 6.6.4, 6.6.5 for making progress on WI IoT_NTN_FDD_LS_band with topics below.

- Topic#1 General and Work Plan

- Topic#2 System parameters

- Topic#3 UE RF requirements

The aim for the 1st round is to maximise agreement and common understanding on the above aspects.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Jin Wang | jinwang@huawei.com |
| Qualcomm Incorporated | Gene Fong | gfong@qti.qualcomm.com |
| MediaTek Inc. | Daniel Hsieh | Daniel.hsieh@mediatek.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: General & workplan

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304496 | Mediatek India Technology Pvt. | - RAN4#106bis (April 2023, electronic) - General aspects. - Identification of the system and RF parameters that need discussions. - To study whether some system and RF parameters could be inherited from the existing NTN and/or terrestrial bands. 2) RAN4#107 (May 2023, F2F) - If applicable, make initial agreements on system and RF parameters. - Further discussions on the system and RF aspects that need further technical analysis. - Endorsement of draft running CRs capturing agreements made during the meeting. 3) RAN4#108 (August 2023, F2F) - Further discussions on the system and RF aspects aiming at reaching final conclusions. - Approval of the CRs for the core specifications especially for UE RF 4) RAN4#108bis (October 2023, electronic) - Finalisation of the core system and RF aspects. - Approval of the CRs for the rest core specifications. 5) RAN4#109 (November 2023, F2F) - Finalisation of remaining issues if any. Proposal: Approve the proposed workplan for the NTN L-/S-band spectrum WI. |
| R4-2304497 | Mediatek India Technology Pvt. | Draft of TR Skeleton for IoT NTN bands |

Open issues summary and company views collection

Sub-topic 1-1 Work plan

Sub-topic description: The WI work plan for fulfilling target deadline of the core part in the December TSG RAN meeting

Issue 1-1: Work plan for WI IoT_NTN_FDD_LS_band core part

- Proposals

  - Option 1: Approve the work plan about core part

- RAN4#106bis (April 2023, electronic) - General aspects. - Identification of the system and RF parameters that need discussions. - To study whether some system and RF parameters could be inherited from the existing NTN and/or terrestrial bands. 2) RAN4#107 (May 2023, F2F) - If applicable, make initial agreements on system and RF parameters. - Further discussions on the system and RF aspects that need further technical analysis. - Endorsement of draft running CRs capturing agreements made during the meeting. 3) RAN4#108 (August 2023, F2F) - Further discussions on the system and RF aspects aiming at reaching final conclusions. - Approval of the CRs for the core specifications especially for UE RF 4) RAN4#108bis (October 2023, electronic) - Finalisation of the core system and RF aspects. - Approval of the CRs for the rest core specifications. 5) RAN4#109 (November 2023, F2F) - Finalisation of remaining issues if any.

  - Option 2: Other

- Recommended WF

  - Agree Option 1.

Sub-topic 1-2 Draft TR for IoT NTN bands

Sub-topic description: Draft TR Skeleton for IoT NTN bands

Issue 1-2: Draft TR for IoT NTN bands

- Proposals:

  - Option 1: Consider the draft TR skeleton in R4-2304497 for IoT NTN bands

  - Option 2: Other

- Recommended WF

  - Agree Option 1, and comments invited.

Companies views’ collection for 1st round

Open issues

Sub-topic 1-1

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 1-1: RAN4 #108bis in October has been converted from e-meeting to face-to-face in Xiamen. |

Sub-topic 1-2

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2: |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Issue 1-1: Work plan for WI IoT_NTN_FDD_LS_band core part Received comments: RAN4 #108bis in October has been converted from e-meeting to face-to-face in Xiamen. Tentative agreements: Revision of R4-2304496 is needed. Candidate options: To revise R4-2304496 with following modification. 4) RAN4#108bis (October 2023, F2F) Recommendations for 2nd round: The revised R4-2304496 based on the 1st-round comments should be provided. To agree the revised R4-2304496. |
| Sub-topic#1-2 | Issue 1-2: Draft TR for IoT NTN bands - Proposals: - Option 1: Consider the draft TR skeleton in R4-2304497 for IoT NTN bands - Option 2: Other - Recommended WF - Agree Option 1, and comments invited. Tentative agreements: To endorse R4-2304497. Candidate options: None. Recommendations for 2nd round: None. Issue 1-2 can be closed. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Open Issues

Issue 1-1: Work plan for WI IoT_NTN_FDD_LS_band core part

Candidate options:

  - Option 1: Approve revised R4-2304496 with following modification:

4)	RAN4#108bis (October 2023, F2F)

  - Option 2: Other

Recommended WF:

- Agree Option 1

Companies views’ collection for 2nd round

Companies are encouraged to comment on the proposed Way Forwards by the Moderator.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary for 2nd round

Topic #2: System parameters

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2305698 | MediaTek Inc. | Proposal 1: Take band number 254 for the L/S band for IoT NTN operation. Proposal 2: The addition of Band 254 does not have any impact on the channel bandwidth sub-clauses for category M1 or category NB1/NB2. Proposal 3: Channel spacing sub-clauses keep untouched with the addition of Band 254 for category M1 and category NB1/NB2. Proposal 4: Set offset number NOffs-UL for Band 254 to 261503, and NOffs-DL to 228735 for the channel numbers. Proposal 5: RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz, and further study variable duplex and asymmetric UL/DL operation. |
| R4-2304802 | ZTE | Proposal 1: The new FDD frequency bands for NB-IoT/eMTC NTN operation should be numbered as Table 2.1-1. Proposal 2: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, channel bandwidth and transmission bandwidth configuration for eMTC NTN operation should be defined as Table 2.2-1. Proposal 3: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should be defined as Table 2.2-2. Proposal 4: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the channel raster, carrier frequency and EARFCN can be defined as Table 2.3-1. Proposal 5: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the TX–RX frequency separation should be defined as Table 2.4-1. |

Open issues summary and company views collection

Sub-topic 2-1 Operating band

Sub-topic description: operating band.

Issue 2-1: Operating band

- Proposals

  - Option 1: Take band number 254 for the L/S band for IoT NTN operation

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 254 | 1610 MHz | – | 1626.5 MHz | 2483.5 MHz | – | 2500 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

  - Option 2: Other

- Recommended WF

  - Agree Option 1.

Sub-topic 2-2 Channel bandwidth

Sub-topic description: channel bandwidth for Cat-M1 and category NB1 and NB2.

Issue 2-2: Channel bandwidth

- Proposals

  - Option 1: The addition of Band 254 does not have any impact on the channel bandwidth sub-clauses for category M1 or category NB1/NB2.

  - Option 2: Other

- Recommended WF

  - Agree Option 1.

Sub-topic 2-3 Channel arrangement

Sub-topic description: channel spacing, channel raster and Tx-Rx frequency separation.

Issue 2-3-1: Channel spacing

- Proposals

  - Option 1: Channel spacing sub-clauses keep untouched with the addition of Band 254 for category M1 and category NB1/NB2.

  - Option 2: Other

- Recommended WF

  - TBA

Issue 2-3-2: Channel raster

- Proposals

  - Option 1: Set offset number NOffs-UL for Band 254 to 261503, and NOffs-DL to 228735 for the channel numbers.

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 254 | 100 | 2483.5 | 228735 | 228571 –<1>- 228735 | 1610 | 261053 | 260889 –<1>- 261053 |

  - Option 2:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 254 | 100 | 2483.5 | 228571 | 228571-<1>-228735 | 1610 | 261339 | 261339-<1>-261503 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

- Recommended WF

  - TBA

Issue 2-3-3: Tx-Rx frequency separation

- Proposals

  - Option 1: RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz, and further study variable duplex and asymmetric UL/DL operation

  - Option 2: Other

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub-topic 2-1

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 2-1: No objection to allocating Band 254 as it's the obvious choice, but we should keep the same band numbering as the NR NTN band. Since the NR NTN band is still [n254] (I think), perhaps we should keep this one also in square bracket? |

Sub-topic 2-2

| Company | Comments |
| --- | --- |
| XXX | Issue 2-2: |

Sub-topic 2-3

| Company | Comments |
| --- | --- |
| XXX | Issue 2-3-1: Issue 2-3-2: Issue 2-3-3: |
| Huawei | Issue 2-3-2: Option 2 is aligned with the formula for deriving F_DL/F_UL. Can the proponent clarify option 1? For other issues, fine with the recommended WF. |
| Qualcomm | Issue 2-3-2: Are the uplink values correct for option 1? The text reads “Set offset number NOffs-UL for Band 254 to 261503”, but the values in the table are 261053. Issue 2-3-3: For LS band, there was a request for asymmetric UL/DL and variable duplex to allow for flexibility in deployment. I’ve asked for more details and clarification in that thread, but if there is added flexibility needed for NTN deployments (depending on the response), then would it be needed here also? |
| MediaTek | Issue 2-3-2: A typo for 261053 in the table. It should be 261503. And the lower end of the UL should be 261339, not 260889. And we used the higher end as the offset by mistake, it should be the lower end. Therefore, after correction, we got the same values as Option 2. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304803 (ZTE) | Company A |
|  | Company B |
| R4-2304805 (ZTE) | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1: Operating band - Proposals - Option 1: Take band number 254 for the L/S band for IoT NTN operation E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 254; 1610 MHz; –; 1626.5 MHz; 2483.5 MHz; –; 2500 MHz; FDD. NOTE: Satellite bands are numbered in descending order from 256. Received comments: NR NTN band is still [n254], perhaps we should keep this one also in square bracket? Tentative agreements: Revision of R4-2304803 and R4-2304805 is needed. Candidate options: Band 254 revised to be Band [254] Recommendations for 2nd round: Draftr CR R4-2304803 and R4-2304805 shoud be revised. |
| Sub-topic#2-2 | Issue 2-2: Channel bandwidth - Proposals - Option 1: The addition of Band [254] does not have any impact on the channel bandwidth sub-clauses for category M1 or category NB1/NB2. Tentative agreements: To agree Option1. Candidate options: None. Recommendations for 2nd round: None. This Issue 2-2 can be closed. |
| Sub-topic#2-3 | Issue 2-3-1: Channel spacing - Proposals - Option 1: Channel spacing sub-clauses keep untouched with the addition of Band [254] for category M1 and category NB1/NB2. Tentative agreements: To agree Option1. Candidate options: None. Recommendations for 2nd round: None. This Issue 2-3-1 can be closed. Issue 2-3-2: Channel raster - Proposals - Option 2: E-UTRA Operating Band; ΔFRaster (kHz); Downlink; Uplink. FDL_low (MHz); NOffs-DL; Range of NDL (First – <Step size> – Last); FUL_low (MHz); NOffs-UL; Range of NUL (First – <Step size> – Last). [254]; 100; 2483.5; 228571; 228571-<1>-228735; 1610; 261339; 261339-<1>-261503. NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz.. Received comments: Option 2 Tentative agreements: To agree Option2. Candidate options: None. Recommendations for 2nd round: None. This Issue 2-3-2 can be closed. Issue 2-3-3: Tx-Rx frequency separation - Proposals - Option 1: RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz, and further study variable duplex and asymmetric UL/DL operation Received comments: For LS band, the variable duplex is also discussed in NR NTN LS-BAND email thread. Tentative agreements: To agree “RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz”. Candidate options: Proposal 1: RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz Proposal 2: To discuss variable duplex and asymmetric UL/DL operation next RAN4#107 meeting if there are more inputs. Recommendations for 2nd round: To agree Proposals 1 and 2. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2304803 | Band 254 revised to Band [254] Recommendation on draft CR: “to be revised” |
| R4-2304805 | Band 254 revised to Band [254] Recommendation on draft CR: “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Open issues

Issue 2-1: Operating band

Candidate options:

- Proposal 1: Band 254 revised to Band [254]

  - To revise Draft CR R4-2304803 and R4-2304805

Recommended WF:

- Agree Proposal 1.

Issue 2-3-3: Tx-Rx frequency separation

Candidate options:

- Proposal 1: RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz

- Proposal 2: To discuss variable duplex and asymmetric UL/DL operation next RAN4#107 meeting if there are more inputs.

Recommended WF:

- Agree Proposals 1 and 2.

Companies views’ collection for 2nd round

Companies are encouraged to comment on the proposed Way Forwards by the Moderator.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary for 2nd round

Topic #3: UE RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304802 | ZTE | Proposal 6: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Table 2.5-1 and 2.5-2, respectively. |
| R4-2304491 | Mediatek India Technology Pvt. | Observation 1: Regarding the new IoT NTN FDD band (L+S band) UE RF requirements, to distinguish band-agnostic UE RF requirements from TS 36.102 is provided in Table 2.1-1. Proposal 1: To agree the proposal in the Table 2.1-1 if there is no specific concern, or at least use them as the starting point for further discussions on new IoT NTN FDD band (L+S band) UE RF requirements. |

Open issues summary and company views collection

Sub-topic 3-1 TX requirements

Sub-topic description: UE maximum output power and MPR.

Issue 3-1-1: UE maximum output power

- Proposals

  - Option 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Tables below, respectively.

Table: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 254 | 23 | +/-2 | 20 | +/-2 |

  - Option 2: Other

- Recommended WF

  - Agree Option 1.

Issue 3-1-2: UE maximum output power reduction

- Proposals

  - Option 1: Reuse the 36.102 MPR requirements for category M1 and NB1/NB2

  - Option 2: Other

- Recommended WF

  - Agree Option 1.

Sub-topic 3-2 RX requirements

Sub-topic description: UE RX in-band blocking, narrow-band blocking and spurious emission requirements.

Issue 3-2-1: In-band blocking

- Proposals:

  - Option 1: Reuse the 36.102 in-band blocking requirements

  - Option 2: Other

- Recommended WF

  - TBA

Issue 3-2-2: Narrow-band blocking

- Proposals:

  - Option 1: Reuse the 36.102 narrow-band blocking requirements

  - Option 2: Other

- Recommended WF

  - Agree Option 1.

Issue 3-2-3: RX spurious emission

- Proposals:

  - Option 1: Reuse the 36.102 spurious emission requirements

  - Option 2: Other

- Recommended WF

  - Agree Option 1.

Companies views’ collection for 1st round

Open issues

Sub-topic 3-1

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: Issue 3-1-2: |
| Qualcomm | Issue 3-1-1: We don’t think PC5 makes much sense for satellite uplink, even if it’s in the WID. Does the proponent of PC5 have a link budget to show that there is sufficient power with 20 dBm, especially if there is MPR? Issue 3-1-2: Given the challenging uplink link budget, should we have an opportunity to see if the MPR can be improved instead of copying it over? |
| Huawei | For the list issues, we’re fine with the recommended WF. PC5 has been supported for other IoT-NTN bands. We don’t see why it cannot be supported for this new band. |
| MediaTek | Issue 3-1-1: Option 1. We observed that PC3 is used in NR NTN for CBW 5MHz~20MHz. NTN IoT is considering NB1/NB2 1RB(200kHz) and Cat-M1 6RB(CBW 1.4MHz, signal BW 6*180kHz). NTN IoT UL power density level is much higher compared to NR_NTN. Issue 3-1-2: Option 1. We think that reusing the 36.102 MPR requirements for category M1 and NB1/NB2 can be as the starting point. |

Sub-topic 3-2

| Company | Comments |
| --- | --- |
| XXX | Issue 3-2-1: Issue 3-2-2: Issue 3-2-3: |
| Huawei | Fine with the recommended WF. |
| MediaTek | Issue 3-2-1: Option 1 Issue 3-2-2: Option 1 Issue 3-2-3: Option 1 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2304803 (ZTE) | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1-1: UE maximum output power - Proposals - Option 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Tables below, respectively. Table: UE Power Class for category M1 EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). [254]; 23; +/-2; 20; +/-2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance.. Table: UE Power Class for category NB1 and NB2 EUTRA band; Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). [254]; 23; +/-2; 20; +/-2. Tentative agreements: To agree Option 1 Candidate options: Option 1 Recommendations for 2nd round: To agree Option 1 Issue 3-1-2: UE maximum output power reduction - Proposals - Option 1: Reuse the 36.102 MPR requirements for category M1 and NB1/NB2 Tentative agreements: To agree Option 1 with considering it as starting point. Candidate options: Option 1: Reuse the 36.102 MPR requirements for category M1 and NB1/NB2 as the starting point Recommendations for 2nd round: To agree updated Option 1 |
| Sub-topic#3-2 | Issue 3-2-1: In-band blocking - Proposals: - Option 1: Reuse the 36.102 in-band blocking requirements Issue 3-2-2: Narrow-band blocking - Proposals: - Option 1: Reuse the 36.102 narrow-band blocking requirements Issue 3-2-3: RX spurious emission - Proposals: - Option 1: Reuse the 36.102 spurious emission requirements Tentative agreements: To agree Option 1 in Issue 3-2-1 To agree Option 1 in Issue 3-2-2 To agree Option 1 in Issue 3-2-3 Candidate options: None. Recommendations for 2nd round: None. Issues 3-2-1, 3-2-2 and 3-2-3 can be closed. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Open issues

Issue 3-1-1: UE maximum output power

Candidate options:

Option 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Tables below, respectively.

Table: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [254] |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| [254] | 23 | +/-2 | 20 | +/-2 |

Recommended WF:

- Agree Option 1, and comments invited.

Issue 3-1-2: UE maximum output power reduction

Candidate options:

- Option 1: Reuse the 36.102 MPR requirements for category M1 and NB1/NB2 as the starting point

Recommended WF:

- Agree Option 1, and comments invited.

Companies views’ collection for 2nd round

Companies are encouraged to comment on the proposed Way Forwards by the Moderator.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary for 2nd round

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-230xxxx | WF on IoT_NTN_FDD_LS_band | MediaTek Inc. |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2304803 | R4-230xxxx | Draft CR to TS36.102 Introduction of a new FDD band (L+S band) for IoT NTN operation | ZTE | Revised |  |
| R4-2304496 | R4-230xxxx | Workplan for IoT NTN FDD LS band | Mediatek India Technology Pvt. | Revised |  |
| R4-2304497 |  | Draft of TR Skeleton for IoT NTN bands | Mediatek India Technology Pvt. | Endorsed |  |
| R4-2304805 | R4-23xxxxx | Draft CR to TS36.108 Introduction of a new FDD band (L+S band) for IoT NTN operation | ZTE | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-23xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents