3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-23XXXXX

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			5.5.5

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Topic summary for [106-bis-e][125] FR1_enh2_part3

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

This email thread discusses FR1 8Rx UE RF requirements for FWA/CPE/Vehicle/Industrial devices.

Topic#1 discusses ΔRIB for 8Rx. In this meeting, moderator includes discussion for FDD band considering the remaining RAN4 meeting.

Topic#2 discusses ΔTRxSRS where ΔTRxSRS for n79 and indication of ΔTRxSRS is discussed.Topic#3 discusses other issues.

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2304186 | On power imbalance indication due to ?TRxSRS | Nokia, Nokia Shanghai Bell |
| R4-2304452 | 8RX UE RF requirements | Qualcomm Finland RFFE Oy |
| R4-2304510 | Discussion on FR1 8RX UE RF requirements | Mediatek India Technology Pvt. |
| R4-2305082 | Discussion on 8Rx UE RF requirements | vivo |
| R4-2305214 | Further view on 8Rx for Rel-18 RF FR1 enhancements | NTT DOCOMO INC. |
| R4-2305294 | Discussion on 8Rx on for CPE FWA vehicle industrial devices | Xiaomi |
| R4-2305603 | On FR1 8Rx UE RF requirements | Huawei, HiSilicon |
| R4-2305604 | draft CR for 38.101-1 removal of 3dB relaxation to PCMAX_H,f,c for PC2 capable UE with TxD | Huawei, HiSilicon |
| R4-2305833 | Further discussion on 8Rx UE RF requirements in FR1 | Ericsson Limited |
| R4-2305846 | On UE Behavior with Delta TRxSRS Power Relaxations | Lenovo |

Topic #1: ΔRIB for 8Rx

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304452 | Qualcomm Incorporated | Proposal 1: Option 1/PDCCH aggregation level =8 applies to 8Rx Proposal 2: Inform RAN5 that 8RX REFSENS requirements are specified under assumption of PDCCH aggregation level=8 Proposal 3: Specify ΔRIB,8R=4.5dB together with assumption of PDCCH AL=8 for 8RX REFSENS |
| R4-2304510 | MediaTek Inc. | Observation 1: In RAN4#106 meeting, some companies suggested that it is probably not needed to strictly specify the requirement of aggregation level in TS 38.101-1. One suggested way in RAN4#106 was to assume PDCCH AL=8 and leave the final decision of PDCCH AL for 8RX values to RAN5. Observation 2: In TS 38.211 Table 7.3.2.1-1 for NR UE, PDCCH aggregation level range is from 1 to 16. Proposal 1: To assume PDCCH aggregation level =8 can be applicable to 8RX. Proposal 2: Let RAN5 to confirm or consider whether PDCCH AL=8 for 8RX is applicable or not. Observation 3: RX REFSENS was typically defined as minimum requirement and should not preclude REFSENS performance higher than minimum requirement. Proposal 3: To use -4.0dB ΔRIB for 8RX if PDCCH aggregation level is not changed to AL8. Proposal 5: Value of ΔRIB for 8Rx for FDD band n7 can be decided after having conclusion of 8RX ΔRIB values for TDD bands (i.e., n41, n77/n78 and n79). |
| R4-2305082 | vivo | Proposal 1(Re-submit): Setting the delta_Rib according to the PDCCH aggregation level. - If PDCCH aggregation kept then the same -4.0dB with LTE is expected. - If PDCCH aggregation level = 8, the requirement could be further improved by 0~0.5dB. |
| R4-2305214 | NTT DOCOMO, INC | Proposal 1: For ΔRIB for 8Rx for FWA/CPE/vehicle/industrial devices, - It is concluded that RAN4 has no common understanding on AL. - Inform to RAN5 that RAN4 suggests applying AL=8 for test purpose. - Specify -4.5dB for ΔRIB for 8Rx for n41/n77/n78/n79. Proposal 4: For ΔRIB for 8Rx for n7, apply option 1A: - Option 1A: The difference of n7 and TDD bands n77/n78/n79 could be 0.5 for ΔRIB,8R. |
| R4-2305294 | Xiaomi | Observation 1: the different AL depending on SCS/CBW specified in option 2 is to avoid having tighter ACS requirements for PDCCH/DCI than PDSCH, which is different from what we faced in 8Rx on PDCCH aggregation level. Proposal 1: Apply -4.3 dB for n41/n77/n78/n79 with an assumption of AL=8 |
| R4-2305603 | Huawei, HiSilicon | Proposal 1: Adopt the following table towards PDCCH aggregation level configuration for 8Rx REFSENS test and inform RAN5. - For a specific combination of CBW and SCS, e.g., CBW=10MHz when SCS=60kHz with AL=1, if the PDCCH demodulation would become the bottleneck, then the corresponding PDSCH REFSENS test shall be dropped. Parameter; Value; Comment. Aggregation level; 1; CBW=10MHz when SCS=60kHz. 2; CBW=15MHz when SCS=60kHz. 4; CBW=5MHz when SCS=15kHz CBW=10,15MHz when SCS=30kHz CBW=20,25,30MHz when SCS=60kHz. 8; CBW>=10 when SCS=15kHz CBW>=20 when SCS=30kHz CBW>=40 when SCS=60kHz. Proposal 2: Reuse -4dB as the NR 8Rx delta Rib for both TDD and FDD bands. |
| R4-2305833 | Ericsson | Proposal 1: There is no need to assume a specific value for AL for 8Rx REFSENS specification since PDCCH decoding is not a bottleneck during REFSENS conformance tests. Proposal 2: Once the issue on the AL value configured for REFSENS conformance tests is settled, the REFSENS requirement shall be specified independently from the AL being assumed or not (and to which value if yes), since with the proper AL it is ensured that PDCCH is not a bottleneck for the REFSENS conformance tests. Proposal 3: Adopt ΔRIB,8R = -4.7dB for bands n7 and n41, and ΔRIB,8R = -4.2dB for bands n77, n78 and n79. If one value is preferred, adopt ΔRIB,8R = -4.5dB for all bands. Proposal 10: Adopt ΔRIB,8R=-4.7dB for band n7. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 PDCCH aggregation level (AL)

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1-1: Value of PDCCH AL in RAN4 assumption

- Proposals

  - Option 1: AL=8 (Qualcomm, MediaTek, Xiaomi)

  - Option 2: Use The following table (Huawei)

    - For a specific combination of CBW and SCS, e.g., CBW=10MHz when SCS=60kHz with AL=1, if the PDCCH demodulation would become the bottleneck, then the corresponding PDSCH REFSENS test shall be dropped.

| Parameter | Value | Comment |
| --- | --- | --- |
| Aggregation level | 1 | CBW=10MHz when SCS=60kHz |
|  | 2 | CBW=15MHz when SCS=60kHz |
|  | 4 | CBW=5MHz when SCS=15kHz CBW=10,15MHz when SCS=30kHz CBW=20,25,30MHz when SCS=60kHz |
|  | 8 | CBW>=10 when SCS=15kHz CBW>=20 when SCS=30kHz CBW>=40 when SCS=60kHz |

  - Option 3: No conclusion in RAN4 (DOCOMO)

  - Option 4: No assumption (Ericsson)

- Recommended WF

GTW Discussions:

Ericsson: in our view, no LS is needed to RAN5. If the LS is sent out, we need clarify that there is no consensus. If no consensus, we cannot send such suggestion.

OPPO: generally we are OK with the recommended WF. Anyway the PDCCH aggregation level will be specified in RAN4. It may impact sensitivity discussion.

DOCOMO: to Ericsson, the intention is that at least for RAN4 deriving requirement there is no consensus. But anyway RAN5 needs the level.

Samsung: we are fine with the first bullet. For the second, we also share the same concern as Ericsson. We do not need to send LS to RAN5.

Qualcomm: we would be OK with the recommendation. We disagree with Huawei subbullet.

Agreement:

- No consensus on PDCCH aggregation level for 8Rx in RAN4, and RAN4 does not specify PDCCH aggregation level in RAN4 specification.

Issue 1-1-2: Information to RAN5

- Proposals

  - Option 1: Inform RAN5 that 8RX REFSENS requirements are specified under assumption of PDCCH aggregation level=8 (Qualcomm)

  - Option 2: Let RAN5 to confirm or consider whether PDCCH AL=8 for 8RX is applicable or not. (MediaTek)

  - Option 3: Inform to RAN5 that RAN4 suggests applying AL=8 for test purpose (DOCOMO).

  - Option 4: No LS is sent to RAN5.

- Recommended WF

  - As we agreed no consensus on AL in RAN4, option 1 can be omitted.  Further discuss the following options.

    - Option 2: Let RAN5 to confirm or consider whether PDCCH AL=8 for 8RX is applicable or not. (MediaTek)

    - Option 3: Inform to RAN5 that RAN4 suggests applying AL=8 for test purpose (DOCOMO).

    - Option 4: No LS is sent to RAN5.

Sub-topic 1-2 ΔRIB for TDD bands

Sub-topic description

Open issues and candidate options before meeting:

Issue 1-2-1: ΔRIB for TDD bands

- Proposals

  - Option 1: -4.7dB for n41, -4.2dB for n77/n78/n79 (or single value of -4.5dB) (Ericsson)

  - Option 2: -4.5dB with AL=8 (Qualcomm, [vivo])

  - Option 3: -4.5dB with no conclusion on AL (DOCOMO)

  - Option 4: -4.3dB with AL=8 (Xiaomi, [vivo])

  - Option 5: -4.0dB if AL=8 is not assumed (MediaTek, vivo)

  - Option 6: -4.0dB with option 2 in issue 1-1-1 (Huawei)

- Recommended WF

  - -[4.3-4.5dB] for n41/n77/n78/n79 if AL=8 is used for conformance testing.

  - -4.0dB for n41/n77/n78/n79 if AL=8 is not used for conformance testing.

Sub-topic 1-3 ΔRIB for FDD band

Sub-topic description

Open issues and candidate options before meeting:

Issue 1-3-1: ΔRIB for FDD band n7

- Proposals

  - Option 1: Discuss after conclusion of TDD bands. (MediaTek)

  - Option 2: The difference of n7 and TDD bands n77/n78/n79 could be 0.5 for ΔRIB,8R (DOCOMO).

  - Option 3: 4.0dB, single value for TDD and FDD is preferred (Huawei)

  - Option 4: -4.7dB for n7 (Ericsson)

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-1-1 In general we are OK with the recommendation from Moderator. However, as we have pointed out, AL=8 may not be configurable for some CBW&SCS configurations, so the principle for verification which could be the sub-bullet in Option 2 shall also be captured. Issue 1-1-2 Suggest to discuss the above issue first. |

| DOCOMO | Issue 1-1-1 Although it is not our preference, we can go with recommended WF. For Huawei’s, comment, one way is RAN4 suggests AL=8 as baseline, but RAN5 can further discuss configurable AL for some CBW&SRS configuration. |
| --- | --- |

| Sony | Issue 1-1-1: Recommended WF Issue 1-1-2: Recommended WF |
| --- | --- |

| Meta | Issue 1-1-1 We are fine with recommend WF with some clarification on the CBW and SCS relationship. Issue 1-1-2 We are fine with recommend WF |
| --- | --- |

| Xiaomi | Issue 1-1-1: Ok with recommended WF Issue 1-1-2: Same as above |
| --- | --- |
| ZTE | Issues 1-1-1/1-1-2: Fine with recommended WF |
| OPPO | Issue 1-1-1: Value of PDCCH AL in RAN4 assumption OK with WF, PDCCH aggregation level is not defined in RAN4 but inform RAN5 to be used for testing. Issue 1-1-2: Information to RAN5 Ok with WF. |
| MediaTek | Issue 1-1-2: Information to RAN5 We are okay with option 3. Not sure information sharing and LS from RAN4 to RAN5 should be precluded. If option 3 is not acceptable after further discussion, then we choose option 2. |
| Samsung | Issue 1-1-2: Information to RAN5 Prefer Option4. If company do feel send an LS would be helpful, at least it should be mentioned in LS RAN4 has no assumed PDCCH AL when derive the REFSENS requirement. |
| Qualcomm | Issue 1-1-2: We think that the situation is now very unfortunate as there is no consensus in Ran4 on AL. Hence, any LS into RAN5 would be very weak in the sense that it would not include any Ran4 assumption on AL. |
| Apple | Issue 1-1-1 The AL should ensure PDCCH is not the bottleneck of REFSENS. |
| Ericsson | Issue 1-1-1: Value of PDCCH AL in RAN4 assumption We are fine with the GTW agreement. Issue 1-1-2: Information to RAN5 Option 4: no LS is sent to RAN5. If an LS is needed to be sent, the compromised solution (for the sake of progress) could be to inform RAN5 that there is no common understanding in RAN4 on whether specific AL should be suggested for REFSENS conformance test. |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |
| Sony | Issue 1-2-1: Option 1 |
| Meta | Issue 1-2-1 We are fine with recommend WF |
| Xiaomi | Ok with recommended WF |
| OPPO | Issue 1-2-1: ΔRIB for TDD bands Ok with WF. |
| Samsung | Prefer [4.3-4.5] with no conclusion on AL |
| Qualcomm | Issue 1-2-1: As there is no consensus in RAN4, the number(s) must be set to suit for either AL4 or AL8. Our proposal for n77/n78/n79 is 3.8dB with no conclusion on AL. |
| MediaTek | Issue 1-2-1: We can agree Qualcomm’s proposal as well. |
| vivo | Agree that since no consensus in RAN4, the number should be suitable for both AL4 or 8. The starting point can be -4.0dB as most companies proposed, and some further adjust may be considered. |
| Apple | Issue 1-2-1 -4.0dB for n41/n77/n78/n79 provided AL for PDCCH would not be the bottleneck of REFSENS |
| AT&T | Issue 1-2-1: We support Samsung proposal of [4.3-4.5]. Two options above recommended 4.5 or better with no conclusion on AL. We also need a compromise to account for the case that RAN5 may decide to test with PDCCH AL of 8. |
| Ericsson | Issue 1-2-1: ΔRIB for TDD bands Option 1. It is important to note that the decision on whether to suggest RAN5 to which value to specify the AL for REFSENS conformance test shall not impact the decision on ΔRIB. As a reminder, in Sub-topic 1-1 we are discussing only the potential issue with PDCCH decoding (“PDCCH bottleneck issue”) during REFSENS conformance testing since the AL is fixed during the test. In the field, the network has a freedom to set the appropriate AL depending on the channel conditions. AL is not specified in the core RAN4 specification and it serves to protect the PDCCH only, and once the PDCCH is successfully decoded, AL does not impact the probability of successful decoding of PDSCH, i.e. the reference sensitivity does not depend on the AL. REFSENS is a noise-factor test as measured on the PDSCH with the assumption that PDSCH decoding is not impacted (significantly) by PDCCH misdetection. So, even if “PDCCH bottleneck” issue is identified during the REFSENS conformance test, it is not ΔRIB which should be increased (and thus become less stringent), it is the AL which should be increased. Hence, ΔRIB should be defined in RAN4 under the assumption that the PDCCH is decoded successfully, and the AL issue shall be discussed separately. In any case, it is ultimately up to RAN5 to decide whether to specify AL=8 or some other AL for the REFSENS conformance test to ensure that there is no “PDCCH bottleneck” issue during the test (based on agreed ΔRIB in RAN4 and the estimated operating SNR at the receiver). |
| CHTTL | Support -4.5, Samsung proposal is also acceptable. Can not agree value worse than LTE. |

Sub topic 1-3

| Company | Comments |
| --- | --- |
| DOCOMO | Issue 1-3-1 Option 2 as proponent. It is the same delta we have for 4Rx RIB. |
| Sony | Issue 1-3-1: Option 4. We think n7 should have same ΔRIB as n41. (Option 2 may end up in the same value). |
| Meta | Issue 1-3-1: Option 4. |
| OPPO | Issue 1-3-1: ΔRIB for FDD band n7 Option 2 is ok to us. |
| MediaTek | Issue 1-3-1 Option 1 as proponent. We are okay with Option 3 as well. |
| Samsung | Option 2, similar view Docomo |
| Qualcomm | Issue 1-3-1 Option 3 |
| Apple | Issue 1-3-1 Option 3 |
| AT&T | Option 2. |
| Ericsson | Issue 1-3-1: ΔRIB for FDD band n7 Option 4, but also OK with Option 2 as well. |
| CHTTL | Option 2 |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: Value of PDCCH AL in RAN4 assumption Agreement in GTW: - No consensus on PDCCH aggregation level for 8Rx in RAN4, and RAN4 does not specify PDCCH aggregation level in RAN4 specification. Issue 1-1-2: Information to RAN5 6 companies prefer or are fine with Option 3. 2 companies prefer Option 4. 1 company suggests that consider option 2 if option 3 is not agreed. Tentative agreements: Nothing Candidate options: Option 2: Let RAN5 to confirm or consider whether PDCCH AL=8 for 8RX is applicable or not. Option 3: Inform to RAN5 that RAN4 suggests applying AL=8 for test purpose Option 4: No LS is sent to RAN5. Recommendations for 2nd round: See issue 1-2-1 |
| Sub-topic #1-2 | Issue 1-2-1: ΔRIB for TDD bands Companies’ views are different. Option 1 (3 companies) Option 5 (1 company) Option 7 (3 companies) Option 8 (3 companies) Option 9 (2 companies) Option 10 (1 company) Tentative agreements: Nothing Candidate options: Option 1: -4.7dB for n41, -4.2dB for n77/n78/n79 (or single value of -4.5dB) (Ericsson) Option 5: -4.0dB if AL=8 is not assumed (MediaTek, vivo) Option 7: - -[4.3-4.5dB] for n41/n77/n78/n79 if AL=8 is used for conformance testing. - -4.0dB for n41/n77/n78/n79 if AL=8 is not used for conformance testing. Option 8: -[4.3-4.5] with no conclusion on AL Option 9: -3.8dB with no conclusion on AL Option 10: -4.0dB for n41/n77/n78/n79 provided AL for PDCCH would not be the bottleneck of REFSENS Recommendations for 2nd round: Moderator’s view: - Situation: One party thinks AL is not a bottleneck. And even if it is a bottleneck, the issue is for conformance testing and it shall not impact the decision on ΔRIB. Another party thinks it is a bottleneck. If lower AL is used for conformance testing, it hard for UE to pass REFSENS. - Moderator understands the points from both sides. But several companies have concerns on the AL issue, if RAN4 does not address the AL issue, then these companies cannot agree a better value such as smaller than -4.0dB. considering the above, there are two directions: - Direction 1: RAN4 tries to address the AL issue for conformance testing (Option 2, 3 or other in issue 1-1-2). If it is solved, RAN4 will discuss ΔRIB and would agree –[4.3-4.5] dB. - Direction 2: RAN4 does not address the AL issue and does defer it to RAN5. ΔRIB would be –[4.0]dB. - Moderator suggests further discussion on the above two directions in WF in 2nd round. |
| Sub-topic #1-3 | Issue 1-3-1: ΔRIB for FDD band n7 Option 1 (1 company) Option 2 (6 companies) Option 3 (3 companies) Option 4 (4 companies) Tentative agreements: Nothing Candidate options: Option 1: Discuss after conclusion of TDD bands. (MediaTek) Option 2: The difference of n7 and TDD bands n77/n78/n79 could be 0.5 for ΔRIB,8R (DOCOMO). Option 3: 4.0dB, single value for TDD and FDD is preferred (Huawei) Option 4: -4.7dB for n7 (Ericsson) Recommendations for 2nd round: As most companies supporting option 4 also support option 2. Further discuss the following two options: Option 2: The difference of n7 and TDD bands n77/n78/n79 could be 0.5 for ΔRIB,8R Option 3: 4.0dB, single value for TDD and FDD is preferred |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: ΔTRxSRS

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304186 | Nokia, Nokia Shanghai Bell | Observation 1: Fixed power imbalance approach (maintain power imbalance at respective antenna ports depending on the indicated actual ∆TRxSRS, p) works only when the UE output power at reference antenna port(s) which delivers the highest power across the ports is at maximum. Observation 2: TS38.213 expects the same output power across antenna ports (hereafter, this is called no power imbalance approach) in an SRS resource set. Hence, if UE is compliant to no power imbalance approach, indicated ∆TRxSRS shall not be used as error correction factor at gNB. Observation 3: Contrary to Observation 2, though TS38.213 expects no power imbalance across ports within the same SRS resource set, at least it may not be practical to perform it when the output power at the antenna port to deliver the highest maximum output power exceeds power class – max {∆TRxSRS,p ¦ p=0, 1, …m} Observation 4: Even the power at antenna port to deliver the highest power is lower or equal to Power Class – max {∆TRxSRS,p ¦ p=0, 1, …m}, the UE may not be able to maintain the same power across ports due to short GP. Observation 5: From Observation 1 – 4, TS38.213 seems expect no power imbalance across ports and that is not fit to a solution by indication of actual ∆TRxSRS, while not all the UE may not be able to achieve no power imbalance approach. With the current specification, applicability of indication of ∆TRxSRS is extremely limited. Proposal 1: No introduction of indication of ∆TRxSRS unless clear UE behaviour in terms of power per port as well as associated with requirements are defined. Observation 6: Type 3 PHR approach cannot mitigate power imbalance due to ∆TRxSRS due to the restriction that power control is per SRS resource set. Observation 7: Report channel (gNB to UE) measurement by UE would work, but it requires a new measurement by UE and signalling scheme of it from UE to gNB. The gain may be affected by degree of quickness and accuracy including granularity of the signalled information. Proposal 2: Should not introduce indication of ∆TRxSRS unless the behaviours in terms of power are clearly defined together. All the options need to be further studied in the future releases (given that RAN1 has two meetings after April meeting) |
| R4-2304452 | Qualcomm Incorporated | Proposal 4: ∆TRxSRS is 6dB for 1T8R and 2T8R, and 6.5dB for 1T8R/2T8R for bands beyond n77. Apply the same “+3dB”-principle as already agreed for bands up to n77 between different power classes/TX Diversity Proposal 5: RAN4 can wait RAN1 reply on indication of ΔTRxSRS |
| R4-2305082 | vivo | Observation 1: Performance loss due to SRS IL imbalance is limited for 1T4R and 1T8R, while there is almost no performance loss for 1T2R. Observation 2: Due to non-ideal UE reporting and gNB compensation, including IL measurement error and IL reporting quantization error, performance gain of gNB compensation is marginal. Proposal 2: Considering the various limitations and in accuracies, no SRS IL reporting seems needed at least for 4Rx / 2Rx case. Observation 3: Almost no performance loss is caused by IL imbalance for 1T8R/1T4R/1T2R, when considering better condition of the IL imbalance between UL and DL together. Proposal 3: Considering the overall UL/DL, the need of enhancement is further reduced for UE IL imbalance issue from PDSCH performance perspective. |
| R4-2305214 | NTT DOCOMO, INC | Proposal 2: For ΔTRxSRS for 8Rx for n79 for PC3, apply 5.0dB, 5.5dB, and 6.0dB for 1T8R, 2T8R, and 1T8R/2T8R, respectively. |
| R4-2305603 | Huawei, HiSilicon | Proposal 3: Adopt 7dB for 8Rx ΔTRxSRS for band n79. |
| R4-2305833 | Ericsson | Proposal 4: For n79 for PC3 adopt ΔTRxSRS=5.5dB for ‘t1r8’ and ‘t2r8’ and ΔTRxSRS=6.0dB for ‘t1r8-t2r8’ SRS-TxSwitch capabilities. Observation 1: It could be advantageous if RAN4 group can agree as soon as possible on whether it is beneficial to indicate an information related to IL imbalance for 2Rx and 4Rx cases so RAN1 can potentially work on a general solution applicable to any number of Rx branches (if agreed) and hence avoiding a risk of having the same mechanism being specified in different releases for different number of Rx branches. Proposal 5: The reporting of ΔTRxSRS related information per Rx branch can be applied to 2Rx and 4Rx cases and such applicability should be the design criterion of the reporting mechanism. Proposal 6: Since the current WI preclude the handheld devices, RAN4 to confirm that there are no identified issues with reporting of ΔTRxSRS related information and handheld devices for 2Rx and 4Rx cases. Observation 2: Power control equations for SRS transmission occasions are defined at the antenna connector with the parameters set per SRS resource set which do not include any compensation for the additional insertion loss. Observation 3: The downlink path loss (PL) estimate is also defined at the antenna connector and thus does not capture the additional IL for the given Rx branch. Proposal 7: SRS reporting should also consider the configured maximum power per SRS resource and the PH for each SRS resource. |
| R4-2305846 | Lenovo, Motorola Mobility | Observation 1: Unless the UE is calibrated such that it knows the value ∆TRxSRSA and compensates for this loss as in Figure 1, the SRS output power will lag the power control setting PSRS,b,f,c(i,qs,l) by ∆TRxSRSA. Furthermore, the maximum output power will be reduced to PCMAX,f,c(i) – ∆TRxSRSA. Observation 2: Unless the SRS implementation loss ∆TRxSRSA is compensated by the UE, the total reduction in maximum configured power will be 2∙∆TRxSRSA. Proposal 1: Clarify the required behavior of SRS output vs. power control when SRS power relaxations are allowed. Proposal 2: Require the UE to be calibrated and to compensate the actual SRS power relaxations ∆TRxSRSA as indicated in Figure 2. Proposal 3: Require the UE to signal the actual SRS power relaxations to the gNB so that the gNB can scale the SRS-based channel estimates appropriately when PSRS,b,f,c(i,qs,l) is greater than PCMAX,f,c(i) for at least one SRS port. Proposal 4: Require the UE to support measurements similar to those in [4] so that the SRS power relaxations can be estimated. Proposal 5: Require the UE to compensate the estimated SRS power relaxations as in Figure 1. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 Values of ΔTRxSRS for n79

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1-1: Values of ΔTRxSRS for n79

- Proposals

| Company | 1T8R | 2T8R | 1T8R/2T8R |
| --- | --- | --- | --- |
| Qualcomm | 6.0 | 6.0 | 6.5 |
| DOCOMO | 5.0 | 5.5 | 6.0 |
| Huawei | 7.0 | 7.0 | 7.0 |
| Ericsson | 5.5 | 5.5 | 6.0 |

- Recommended WF

  - TBA

Issue 2-1-2: General principle for ΔTRxSRS for n79

Moderator’s note: Agreement in last meeting for a reference:

<AH Agreement>

- The same value with PC3 ∆TRxSRS applies

  - when the device is capable of power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16

- The value 3dB larger than PC3 ∆TRxSRS applies

  - during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16.

- Proposals

  - Proposal 1: Apply the same “+3dB”-principle as already agreed for bands up to n77 between different power classes/TX Diversity (Qualcomm)

- Recommended WF

  - Moderator’s understanding is that the previous agreement applies to n79 (beyond n77) as well because the agreement and the issues does not mention specific bands.

  - Agree proposal 1 if necessary.

Sub-topic 2-2 Indication of ΔTRxSRS

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-2-1: Further analysis for indication of ΔTRxSRS

- Proposals

  - Proposal 1: Should not introduce indication of ∆TRxSRS unless the behaviours in terms of power are clearly defined together. All the options need to be further studied in the future releases (given that RAN1 has two meetings after April meeting) (Nokia)

  - Proposal 2: RAN4 can wait RAN1 reply on indication of ΔTRxSRS (Qualcomm)

  - Proposal 3: Considering the overall UL/DL, the need of enhancement is further reduced for UE IL imbalance issue from PDSCH performance perspective (vivo)

  - Proposal 4: SRS reporting should also consider the configured maximum power per SRS resource and the PH for each SRS resource. (Ericsson)

  - Proposal 5:	Clarify the required behavior of SRS output vs. power control when SRS power relaxations are allowed. (Lenovo)

  - Proposal 6:  	Require the UE to be calibrated and to compensate the actual SRS power relaxations ∆TRxSRSA as indicated in Figure 2. (Lenovo)

  - Proposal 7:	Require the UE to signal the actual SRS power relaxations to the gNB so that the gNB can scale the SRS-based channel estimates appropriately when PSRS,b,f,c(i,qs,l) is greater than PCMAX,f,c(i) for at least one SRS port. (Lenovo)

  - Proposal 8:	Require the UE to support measurements similar to those in [4] so that the SRS power relaxations can be estimated. (Lenovo)

  - Proposal 9:	Require the UE to compensate the estimated SRS power relaxations as in Figure 1. (Lenovo)

- Recommended WF

  - Collect companies’ view for each proposal.

Issue 2-2-2: Applicability for 2Rx/4Rx

- Proposals

  - Proposal 1: Considering the various limitations and in accuracies, no SRS IL reporting seems needed at least for 4Rx / 2Rx case (vivo).

  - Proposal 2: The reporting of ΔTRxSRS related information per Rx branch can be applied to 2Rx and 4Rx cases and such applicability should be the design criterion of the reporting mechanism. (Ericsson)

  - Proposal 3: Since the current WI preclude the handheld devices, RAN4 to confirm that there are no identified issues with reporting of ΔTRxSRS related information and handheld devices for 2Rx and 4Rx cases. (Ericsson)

- Recommended WF

  - Collect companies’ view for each proposal.

Companies views’ collection for 1st round

Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Huawei | Issue 2-1-2 OK with the recommendation from Moderator. |

| DOCOMO | Issue 2-1-1 According to the current specification, the difference of values of ΔTRxSRS between below and above n79 is 1.5dB.. If we follow the same approach, ΔTRxSRS of 1T8R, 2T8R, 1T8R/2T8R are 5.5 dB, 5.5 dB, and 6.0 dB, respectively. However, for 1T8R, among the contributions, some contributions provided the numerical analysis with the values of insertion loss of switching and cable loss. One contribution proposed 5.0dB and another proposed 4.3dB for 1T8R ΔTRxSRS. So, 5.5dB seems to be not needed. Thus, we would like to propose 5.0dB for 1T8R. |
| --- | --- |

| Meta | Issue 2-1-1 Support QC proposals for ΔTRxSRS for n79 with 6dB for 1T8R, 6dB for 2T8R and 6.5dB for 1T8R/2T8R Issue 2-1-2 Support the recommended WF |
| --- | --- |
| OPPO | Issue 2-1-1: Values of ΔTRxSRS for n79 Averaged value from the inputs is ok to us. And DCM proposal to use the 1.5dB delta from the bands below 79 is also acceptable to us, i.e. 5.5dB, 5.5dB, 6dB for 1T8R, 2T8R, 1T8R/2T8R respectively. Issue 2-1-2: General principle for ΔTRxSRS for n79 Ok with WF and proposal 1. |
| Qualcomm | Issue 2-1-1: Need to think further, yet recognizing that it should be possible to reach consensus in this meeting. With 8RX things are more challenging with higher frequencies (n79), so hence our proposal is to appy 2dB difference to n77 instead of 1.5dB which was used in 4RX. Issue 2-1-2: OK |
| Ericsson | Issue 2-1-1: Values of ΔTRxSRS for n79 To be consistent with SRS-TxSwitch capabilities with up to 4Rx the difference between ΔTRxSRS for bands n77/n78 and n79 should be equal to 1.5dB, i.e. 5.5dB for 1T8R, 5.5dB for 2T8R and 6.0dB for 1T8R/2T8R capability. We are also OK with 5.0dB for 1T8R as proposed by DOCOMO. Issue 2-1-2: General principle for ΔTRxSRS for n79 OK with the recommended WF. |
| Spreadtrum | Issue 2-1-1: Values of ΔTRxSRS for n79 According to the current spec, we agree with the proposal from DOCOMO, ΔTRxSRS of 1T8R, 2T8R, 1T8R/2T8R are 5.5 dB, 5.5 dB, and 6.0 dB, respectively. Issue 2-1-2: General principle for ΔTRxSRS for n79 OK with the recommended WF. |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| Huawei | Issue 2-2-1 We think RAN4 shall wait for RAN1 decision as to this issue. Issue 2-2-2 SRS IL reporting can be applied to 2Rx/4Rx case as well, at least we don’t observe any drawback for it. |
| Meta | Issue 2-2-1 We fine with proposal 2 as follow “RAN4 can wait RAN1 reply on indication of ΔTRxSRS” Issue 2-2-2 Apply same ΔTRxSRS regardless of 2Rx/4Rx device. |
| OPPO | Issue 2-2-1: Further analysis for indication of ΔTRxSRS Proposal 2, wait for RAN1 is ok to us. Issue 2-2-2: Applicability for 2Rx/4Rx Proposal 1 is ok to us, and wait for RAN1 conclusion on 8Rx is also a good choice since RAN1 is still discussing this enhancement. |
| MediaTek | Issue 2-2-1: RAN1 is working on RAN4 LS on this topic. We think RAN4 can wait RAN1 reply. |
| Nokia | Issue 2-2-1: Further analysis for indication of ΔTRxSRS For Proposal 4, does Ericsson say that PHR per SRS port? If per resource is taken, e.g., the reported power per SRS resource for 2TxR becomes the sum of the two ports. How can ΔTRxSRS be utilized with this condition? For Proposal 6, our comment is not exactly about this proposal. In Lenovo’s contribution, double counting ΔTRxSRS is mentioned. We don’t think it happens since lower range of PCMAX, f, c has already counted ΔTRxSRS. So, when UE determines reported PCMAX, f, c, the UE needs to take into account the power compensation due to ΔTRxSRS as achievable output power at antenna port. If the UE considers that it is not possible to compensate for power to be lost due to ΔTRxSRS, then, the UE should report lower value accordingly. For proposal 1, in the end, final decision is made after receiving RAN1 LS, but we should keep technical discussion if we want to introduce the reporting mechanisms. It is not a problem to postpone the technical discussion, but our position is clear, we don’t agree with the introduction of reporting ΔTRxSRS. Issue 2-2-2: Applicability for 2Rx/4Rx No need to discuss this. If, however, reporting was introduced, there would be no reason to exclude applying it to 2Rx/4Rx. |
| Samsung | Issue 2-2-1: Further analysis for indication of ΔTRxSRS On one hand wait for RAN1 reply, and should be postponed to future releases if there is no conclusion in RAN1. On the other hand, RAN4 could also continue the technical discussion and do the comparison among all the options. |
| Qualcomm | Issue 2-2-1: We think it is pretty premature to make conclusions until we hear back form RAN1. Issue 2-2-2: This aspect is also easier to discuss after RAN1 reply. Note that we prefer to apply indication at least to 4RX. |
| Lenovo | Issue 2-2-1: Further analysis for indication of ΔTRxSRS For us, the question is the following. When the PA is switched from one SRS port to another, does the PA adjust power to compensate for the different ΔTRxSRS values as below? If not, then the same SRS power control setting produces different output power at the different SRS ports at all power control settings as in the figure below. There should only be a power difference when the SRS power control setting exceeds Pcmax for the SRS port with the smallest Pcmax value, not at all power control settings as above. In order to compensate for ΔTRxSRS, the UE must be calibrated to determine the actual (not maximum) values of these losses. Is it the common understanding that for the same SRS power control setting, the PA output power is adjusted to compensate for these different implementation losses ΔTRxSRS? |
| vivo | Issue 2-2-1: Further analysis for indication of ΔTRxSRS We have performed extensive simulation in our paper, to check the potential gain on this indication. Indeed, 8Rx has shown gain about 1dB in throughput, and 4Rx also has shown similar though a bit smaller gain under ideal comparison condition. However, if we consider some non-ideal UE reporting and gNB compensation, including IL measurement error and IL reporting quantization error, performance gain of gNB compensation would be marginal, even for 8Rx case. In addition, it is the joint effect of Tx-Rx imbalance that would matter, and the deltaTRxsrs itself may not be that important. Issue 2-2-2: Applicability for 2Rx/4Rx As shown in our simulation, the proposed reporting for 2Rx would not have any gain even in most ideal condition. For 4Rx, some performance gain can be perceived in ideal condition, but the gain considering the imperfections would also be marginal. |
| Nokia | To Lenovo We think that our views on this issue are very close. In our understanding, if UE follows RAN1 spec, the UE should compensate for the power lost due to additional loss since power control setting is per SRS resource set where multiple ports are included with exactly the same parameters. But we wonder if what RAN1 spec forces UE to do is feasible for all the UEs. What would be almost super clear is that the UE cannot follow RAN1 spec if the power from the port which can deliver the highest power approaches the PC – max ΔTRxSRS E.g., PC2, t1r4, where e.g., ΔTRxSRS, p is ., ΔTRxSRS, 0 = 0 dB (reference), ΔTRxSRS, 1 = 3 dB, ΔTRxSRS, 2 = 3 dB, ΔTRxSRS, 3 = 4 dB, if the output power exceeds 26 – 4 = 22 dBm, then, compensation is not possible or very challenging unless PA boosts its power more, since to deliver 22 dBm at port 3, the PA needs to transmit 29 dBm at PA (assuming post PA loss is 3 dB), then, 29 dBm – 3 dB (post PA loss common to all the ports) – 4 dB (additional routing loss) = 22 dBm. And boosting PA power may need extra cost in terms of performance. Hence, our view is that if we utilize reported ΔTRxSRS, p, then, we need to define a mode that UE keeps the power imbalance depending on the reported ΔTRxSRS, p values. Given that the current UE’s behaviour is not clear, we also may need to define another mode that the UE shall keep the power across the ports until PC – max ΔTRxSRS. Network doesn’t apply ΔTRxSRS to UE in the latter mode, while the network shall apply reported ΔTRxSRS as correction factor. To vivo If Rx has loss imbalances, the impact of imbalance in Tx would be limited in case the imbalance in Tx and that in Rx have high correlation. Would you share some specific implementation data in the future meeting? But if there are large Rx loss imbalances across ports, gap between 2Rx performance and 8Rx performance in terms of REFSNSE may be smaller than the requirement to be specified. |
| Apple | Issue 2-2-1 Proposal 2: RAN4 can wait RAN1 reply on indication of ΔTRxSRS |
| Ericsson | Issue 2-2-1: Further analysis for indication of ΔTRxSRS Proposal 4 but fine with Proposal 2 for the moment. To Nokia, in our view per SRS resource is also fine since the output power per SRS resource is split equally across the SRS ports configured, two in 2TxR case. The maximum allowance ΔTRxSRS therefore represents the additional loss for both SRS ports of the resource. Issue 2-2-2: Applicability for 2Rx/4Rx Both proposal 2 and proposal 3. We don’t see an issue to apply the reporting mechanism to 2Rx and 4Rx cases. This would be most beneficial for existing deployments. |
| Nokia | To Ericsson Thank you for sharing your feedback. If power is equally split over antenna ports (at antenna connectors), then, there is no need to report actual ΔTRxSRS. In your comment, we see "The maximum allowance ΔTRxSRS“, but we have thought we have been talking about if the reported actual ΔTRxSRS is meaningful or not, or how to utilize it. Perhaps, we may misunderstand something. |
| Spreadtrum | Issue 2-2-1: Further analysis for indication of ΔTRxSRS Proposal 2, wait for RAN1 is ok to us. At the same time, we agree with Lenovo’s question, so we suggest reporting the maximum output power difference between the diversity branch and main branch instead of reporting the srs IL balance. Specific details need further discussion Issue 2-2-2: Applicability for 2Rx/4Rx At least applied to 4RX |

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: Values of ΔTRxSRS for n79 Option 1 (1 company) Option 2 (3 companies) Option 3 (2 companies) Tentative agreements: Values are very close to each other. Check if the following compromised option is agreeable. 5.5dB, 6.0dB, 6.0dB for 1T8R, 2T8R, 1T8R/2T8R respectively Candidate options: Option 1: 5.0dB, 5.5dB, 6.0dB for 1T8R, 2T8R, 1T8R/2T8R respectively Option 2: 5.5dB, 5.5dB, 6.0dB for 1T8R, 2T8R, 1T8R/2T8R respectively Option 3: 6.0dB, 6.0dB, 6.5dB for 1T8R, 2T8R, 1T8R/2T8R respectively Recommendations for 2nd round: Values are very close to each other. Check if the following compromised option is agreeable. 5.5dB, 6.0dB, 6.0dB for 1T8R, 2T8R, 1T8R/2T8R respectively Issue 2-1-2: General principle for ΔTRxSRS for n79 Tentative agreements: Apply the same “+3dB”-principle as already agreed for bands up to n77 between different power classes/TX Diversity <AH Agreement in last meeting> - The same value with PC3 ∆TRxSRS applies - when the device is capable of power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16 - The value 3dB larger than PC3 ∆TRxSRS applies during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16 Candidate options: Recommendations for 2nd round: Approve the tentative agreements. |
| Sub-topic#2-2 | Issue 2-2-1: Further analysis for indication of ΔTRxSRS 8 companies think RAN4 should/can wait for RAN1 input. 4 companies think RAN4 can discuss technical aspect before receiving RAN1 LS, and some companies already provides technical comments. Tentative agreements: Nothing Candidate options: Recommendations for 2nd round: Considering the feedback in 1st round, moderator suggests: No decision and agreements are made in RAN4#106-bis while technical discussion is allowed during 2nd round in WF if any. Issue 2-2-2: Applicability for 2Rx/4Rx 5 companies think it can apply to 2Rx/4Rx, 2 companies do not think. 3 companies think it would be better to wait for RAN1 feedback. Tentative agreements: Nothing Candidate options: Recommendations for 2nd round: No decision and agreements are made in RAN4#106-bis while technical discussion is allowed during 2nd round in WF if any. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: Others

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304452 | Qualcomm Incorporated | Proposal 6: Proponents and objectors of removing the ΔPPowerClass for SRS antenna switching to PCMAX_H,f,c should provide comments during RAN4#106bis which are captured in WF so that exact concerns on both sides are understood Proposal 7: Option 2/Do not remove the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage Proposal 8: Specify 8RX release independent from Rel-16 Proposal 9: Specify only REFSENS for 8RX, and do not specify other RF requirements for 8RX |
| R4-2304510 | MediaTek Inc. | Observation 4: Regarding release independence for 8RX, there is no Rel-18 8RX definition in the newest RAN4 specification (i.e., TS 38.307). Proposal 4: To specify 8RX release independent from Rel-18 and allow adding 8RX definition into Rel-18 TS 38.307 once Rel-18 version of TS 38.307 is introduced Proposal 6: To specify only REFSENS requirement for 8RX if there is no specific concern observed. |
| R4-2305214 | NTT DOCOMO, INC | Observation 1: pure 8Rx feature, i.e., UE implements 8Rx antennas and meet REFSENS requirements considering ΔRIB can be release independent from Rel-15. Observation 2: Some 8Rx related enhanced features introduced from Rel-Z (Z>15) such as srs-AntennaSwitchingBeyond4RX-r17 can be release-independent from Rel-Z. Proposal 3: 8Rx should be release independent from Rel-15. - NOTE: 8Rx related enhanced features introduced from Rel-Z (Z>15) such as srs-AntennaSwitchingBeyond4RX-r17 should be release-independent from Rel-Z, accordingly. Observation 3: UE demod session seems to wait for defining CA band combination(s) for 8Rx in RF session. However, there are no CA specific RF requirements are expected for 8Rx feature. Proposal 5: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations. Proposal 6: Clarify the relationship between MSD and ΔRIB, 8R. by adding the following sentence in section 7.3A.1. - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0. - NOTE: Wording can be further refined when drafting a CR if necessary. |
| R4-2305294 | Xiaomi | Observation 2: Based on the discussion so far, it seems there are two completely different interpretation why this PCMAX_H,f,c improvement is needed. What the real reason for removing 3dBΔPPowerClass for SRS antenna switching for PCMAX_H,f,c should be first clarified. |
| R4-2305603 | Huawei, HiSilicon | Proposal 4: Draft CR in R4-2305604 for capturing the following enhancement: - For a PC2 capable UE with the support of TxD, if it further indicates the support of 1TxR AS-SRS, the ΔPPowerClass=3dB applied for PCMAX_H,f,c should be removed. |
| R4-2305604 | Huawei, HiSilicon | draft CR for 38.101-1 removal of 3dB relaxation to PCMAX_H,f,c for PC2 capable UE with TxD |
| R4-2305833 | Ericsson | Proposal 8: Do not remove ΔPPowerClass for SRS antenna switching to PCMAX_H,f,c for PC2 capable UE with txDiversity-r16 and xT8R capabilities. Proposal 9: Remove the requirement on the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1 ΔPpowerclass for PCMAX_H,f,c

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c

- Proposals

  - Proposal 1: Remove

    - For a PC2 capable UE with the support of TxD, if it further indicates the support of 1TxR AS-SRS, the ΔPPowerClass=3dB applied for PCMAX_H,f,c should be removed. (Huawei)

  - Proposal 2: Proponents and objectors of removing the ΔPPowerClass for SRS antenna switching to PCMAX_H,f,c should provide comments during RAN4#106bis which are captured in WF so that exact concerns on both sides are understood (Qualcomm)

  - Proposal 3: Do not remove ΔPPowerClass for PCMAX_H,f,c (Ericsson)

- Recommended WF

  - TBA

- Moderator’s understanding in the reason of proposal and objection for reference for discussion:

- Background

  - In current spec, ΔPPowerClass = 3dB applies to PCMAX_H,f,c for PC2 UE supporting TxD for 1TxR cases.

- The reason of proposal:

  - If UE supports PC2 with 23+26dBm or 26+26dBm PA configuration, UE can transmit with full power PA for 1TxR. So, applying ΔPPowerClass to PCMAX_H,f,c is not necessary for such Ues.

- The reason of objection:

  - If UE supports PC2 with 23+23dBm PA configuration, UE may do antenna virtualization and transmit 23+23dBm while it shouldn’t. To prevent this antenna virtualization, applying  ΔPPowerClass to PCMAX_H,f,c is necessary.

GTW discussion

Ericsson: in our view, delta P_power class should not be removed. There is uncertainty of 3dB in any case.

Samsung: in last meeting, we asked what the applicable scenario to remove delta P_power class. It is true to differentiate. We are fine not to remove. At least PC1.5 should not be included.

Huawei: we do not see any mis-alignment.

Tentative agreement:

- Do not remove ΔPPowerClass for PCMAX_H,f,c.

Sub-topic 3-2 Whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

Sub-topic description

Open issues and candidate options before meeting:

Issue 3-2-1: whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

- Proposals

  - Option 1: Do not remove (Qualcomm)

  - Option 2: Remove (Ericsson)

- Recommended WF

  - TBA

Sub-topic 3-3 Release independent

Sub-topic description

Open issues and candidate options before meeting:

Issue 3-3-1: Which release 8Rx can be release independent from.

- Proposals

  - Option 1: Rel-15 (DOCOMO)

  - Option 2: Rel-16 (Qualcomm)

  - Option 3: Rel-18 (MediaTek)

- Recommended WF

  - TBA

Sub-topic 3-4 Which RF requirements to specify for 8Rx

Sub-topic description

Open issues and candidate options before meeting:

Issue 3-4-1: Which RF requirements to specify for 8Rx

- Proposals

  - Proposal 1: Specify only REFSENS for 8RX, and do not specify other RF requirements for 8RX (Qualcomm, MediaTek)

  - Proposal 2: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations. (DOCOMO)

  - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1. (DOCOMO)

    - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

    - NOTE: Wording can be further refined when drafting a CR if necessary.

- Recommended WF

  - Based on the discussion in 18, April GTW, moderator make further clarification in each proposal.

  - Further discuss whether the following proposals are agreeable or not:

    - Proposal 1: Specify only REFSENS for 8RX, and do not specify other RF requirements for 8RX (Qualcomm, MediaTek)

      - UE will be considered to support 8Rx if it meets the REFSENS requirement. No other Rx requirements such as ACS, IBB, and so on will be specified.

    - Proposal 2: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC. (DOCOMO)

    - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1. (DOCOMO)

      - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

      - NOTE: Wording can be further refined when drafting a CR if necessary.

GTW Discussions:

Nokia: does proposal 1 means that we only specify REFSENS requirements for 8Rx. All the other requirements will be treated by replacing REFSENS value or UE will be considered to support 8Rx if it meets the REFSENS requirement.

Qualcomm: REFSENS can be specified. No other requirement will be specified for 8Rx.

AT&T: we share the understanding as Nokia. We should replace the REFSENS for other equirements.

Samsung: we share the similar concern from Nokia for the wording.

Apple: do we consider single band or CA band combination requirements? Do we have consider MSD under 8Rx?

DOCOMO: to Apple, it is related to proposal 2. We propose to apply 8Rx to band combination. We need update MSD according delta RIB.

Mediatek: Regarding Apple comment, we think that regarding CA MSD although it is not in the WID scope we can further discuss it.

Samsung: to Proposal 2 and 3, we support to try them in current WID.

DOCOMO: regarding WID, my understanding is that we do not mention single carrier or CA.

CHTTL: support consider proposal 2 and 3. EN-DC case should be considered.

Qualcomm: to AT&T, in LTE all the requirements for 8Rx is REFSENS. How do you handle the doubled tests?

Companies views’ collection for 1st round

Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 3-1

| Company | Comments |
| --- | --- |
| Meta | Need further discussion on the reason to remove the delta power class |
| Xiaomi | What the real reason of removing 3dBΔPPowerClass for SRS antenna switching for PCMAX_H,f,c should be clarified as there seems two different interpretations on that in the previous meetings. |
| OPPO | Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c If understand correctly on the intention of removing the delta Ppowerclass for PcmaxH is that the TxD impact on the SRS Tx power is only applied for the Pcmax,L and allow UE to transmit higher power when it is possible. From this perspective, we are open to this idea and ok with Proposal 1. |
| Samsung | Fine with tentative agreement |
| Nokia | Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c Support Option 3/tentative agreement unless we find a way to clarify UE performance/behaiour. |
| Ericsson | Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c OK with the tentative agreement from GTW: ΔPpowerclass shall not be removed for PCMAX_H,f,c. |
| Huawei | Not OK with tentative agreement, but we can come back at the next meeting. |

Sub topic 3-2

| Company | Comments |
| --- | --- |
| Meta | Need further discussion on the reason to remove the delta power class |
| Qualcomm | Issue 3-2-1: This is a requirement in RAN1 specification, which RAN4 refers to in 38.101. The guard period cannot be removed as it would have adverse effects on UE side. Furthermore, this requirements as been discussed both in RAN1 and RAN4 earlier in other context than 8RX, and no changes were done so the requirement is very stable. |
| Apple | Issue 3-2-1 Option 1: Do not remove the guard period between two SRS resources |
| Ericsson | Issue 3-2-1: whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage Option 2: remove the guard period. 1T8R is the most critical scenario where eight SRS resources are spread in time with a guard period between each pair of consecutive SRS resources. That could lead to a “channel aging” problem where the estimated downlink CSI based on uplink sounding may not be sufficiently accurate, especially for a vehicle type of devices (where it is assumed that the channel varies rapidly) which is under the scope of this WI. The SRS transmission of up to xT8R with GP would also affect the efficiency in the UL for other UEs in the cell since these cannot transmit during these SRS transmissions. |
| Mediatek | Issue 3-2-1 We share similar view as Qualcomm. We support Option 1. |

Sub topic 3-3

| Company | Comments |
| --- | --- |
| Huawei | Issue 3-3-1 We think 8Rx can be release independent from Rel-17 since 8Rx AS-SRS pattern has been defined in Rel-17 by RAN1. |

| DOCOMO | Issue 3-3-1: Option 1. Although we agree that 8Rx SRS-AS was introduced from Rel-17, but the point is SRS-AS is an optional feature, so 8Rx without SRS-AS can be supported from previous release such as Rel-15. So, possible agreement is 8Rx without SRS AS can be release independent from Rel-15, and 8Rx with SRS AS can be release independent from Rel-X depending SRS AS patterns (4R SRS AS is introduced in Rel-16, and 8R SRS AS is in Rel-17). |
| --- | --- |

| Meta | Issue 3-3-1 Based on Huawei comment on the AS-SRS Pattern in RAN1, we are fine tosupport from Rel-17. |
| --- | --- |
| ZTE | Issue 3-3-1 Either from R16(option 2) or from R17 are fine to us. |
| OPPO | Issue 3-3-1: Which release 8Rx can be release independent from. Ok with from Rel-17 considering Rel-16 UEs are already on the market, and need to minimize the impact to legacy UE. |
| MediaTek | Issue 3-3-1 For moving forward, we are fine to support from Rel-17. |
| Nokia | Issue 3-3-1: It depends on how we define 8Rx. If 8Rx is just 8layer without any specific features like SRS AS pertinent to 8Rx, it can be from Rel15. Better to discuss how 8Rx here being discussed is defined if Rel15 is targeted at. |
| Samsung | Thanks for the further clarification from Docomo, but we wonder how to describe it in 38.307? Introduce two rows in table 5.4-1? One for 8Rx with SRS-AS and the other for without SRS-AS? |
| Qualcomm | Issue 3-3-1 We are ok with either proposal 1 or 2. Please note that from RAN1 or RAN2 perspective, we are not constrained into e.g Rel-17 because 8RX can be implemented with e.g AS-SRS patterns specified in Rel-15 or Rel-16. 8RX does not have always need to have 8RX AS-SRS. |
| AT&T | Issue 3-3-1: We prefer proposal 1. We could accept proposal 2. |
| CHTTL | support Docomo’s view. |

Sub topic 3-4

| Company | Comments |
| --- | --- |
| Huawei | Issue 3-4-1 Both three proposals can be agreeable. |
| ZTE | Issue 3-4-1 Poposal 1. Proposal 2 and 3 are for band combination and it is the similar way as 4Rx. However, in terms of the WID, it seems only single carrier is included and the requirements of single carrier are ongoing discussed. No example band combinations included in the WID. Therefore, it should discuss whether to include band combination for 8Rx discussion first. |
| OPPO | Issue 3-4-1: Which RF requirements to specify for 8Rx Ok with proposal 1, 2, and 3. |
| Qualcomm | Issue 3-4-1: Let me clarify proposal 1. Intention is to Specify only 8RX UE RF requirement for 8RX, and no other RX requirements (which are in 38-101-1 Ch7 for 2RX/4RX). In practice this could be done in 38.101 Chapter 7.2: “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.” Proposal 2 and 3 are in principle OK, however before agreeing we want to discuss Proposal 1. |
| MediaTek | Issue 3-4-1: Regarding proposals 2 and 3, it is necessary to know/clarify the additional MSD requirements/scope and the number of RX operated during intra and inter CA scenarios. We think that proposal 2 and 3 are additional work and should be discussed/decided in following RAN-Plenary meeting. |
| Apple | Issue 3-4-1: With regard to proposals 2 and 3, we have one question for clarification, for a band supporting 8Rx in single band operation, is the band mandated to support 8Rx in a band combination? Also the 8Rx objectives in the current WID do not include the feature under band combinations. Clarifications and/or WID revisions may be needed before Proposals 2 and 3 can be agreed. |
| AT&T | Issue 3-4-1: Thanks, Qualcomm for the clarification on Proposal 1. We would prefer that a UE with eight Rx antenna ports is verified with eight Rx antenna ports. We think that this would follow similar guidelines as 4Rx. We have a suggested update to 38.101-1 clause 7.2 below based on your proposal which does not seem to add any additional test burden. “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.” We support Option 2 since this is the common understanding for CA testing. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2305604 (Huawei, HiSilicon) | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c Several companies have question on the reason of the proposal. 3 companies are OK with not removing. 1 company is OK with removing, and 1 company prefer to remove. Although option 2 (do not remove) is a tentation agreement in 18 April GTW, there is a concern on the tentative agreement. Tentative agreements: Nothing Candidate options: Option 1: Remove For a PC2 capable UE with the support of TxD, if it further indicates the support of 1TxR AS-SRS, the ΔPPowerClass=3dB applied for PCMAX_H,f,c should be removed. Option 2: Do not remove Recommendations for 2nd round: Since several companies have questions on the reason of this proposal and objection, moderator would like to check if the following understanding is aligned with the group. - Background - In current spec, ΔPPowerClass = 3dB applies to PCMAX_H,f,c for PC2 UE supporting TxD for 1TxR cases. - The reason of proposal: - If UE supports PC2 with 23+26dBm or 26+26dBm PA configuration, UE can transmit with full power PA for 1TxR. So, applying ΔPPowerClass to PCMAX_H,f,c is not necessary for such Ues. - The reason of objection: - If UE supports PC2 and indicates TxD capability, UE may do antenna virtualization and transmit 23+23dBm while it shouldn’t (no matter the PA configuration). To prevent this antenna virtualization and avoid 3dB uncertainty, applying ΔPPowerClass to PCMAX_H,f,c is necessary. |
| Sub-topic#3-2 | Issue 3-2-1: whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage 3 companies prefer not removing. 1 company prefer removing. Tentative agreements: Nothing Candidate options: Option 1: Do not remove Option 2: Remove Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#3-3 | Issue 3-3-1: Which release 8Rx can be release independent from. Companies view are different. There are several feedbacks: - 8Rx SRS-AS is introduced from Rel-17. - Some SRS-AS patterns are introduced from Rel-15 or Rel-16. - Which release depends on how RAN4 define “8Rx feature”, i.e., with or without SRS-AS. - How to describe it in TS 38.307 should be clarified. Tentative agreements: Nothing Candidate options: Option 1: Rel-15 Option 2: Rel-16 Option 3: Rel-17 Option 4: Rel-18 Recommendations for 2nd round: Based on 1st round discussion, moderator would like to check if the following recommended WF is agreeable: - 8Rx without SRS-AS can be release independent from Rel-15. - 8Rx with SRS-AS can be release independent from Rel-X. - Further discuss “Rel-X” by clarifying which SRS-AS patterns were introduced from which release. - Further discuss how to describe it in TS 38.307. |
| Sub-topic#3-4 | Issue 3-4-1: Which RF requirements to specify for 8Rx For proposal 1, several companies wanted clarification. The proponent answered. One alternative was provided. For proposal 2 and 3, while there is no clear objection, 2 companies think clarification or/and revision on WID may be needed. There are questions on whether 8Rx is mandatory for UE supporting 8Rx in a band within the band combination, and on additional MSD requirement. Tentative agreements: Nothing Candidate options: Recommendations for 2nd round: Further discuss in 2nd round in the WF. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-xxxxx | WF on UE RF for 8Rx | NTT DOCOMO, INC. |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2305604 |  | draft CR for 38.101-1 removal of 3dB relaxation to PCMAX_H,f,c for PC2 capable UE with TxD | Huawei, HiSilicon | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-23xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-23xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents