3GPP TSG-RAN WG4 Meeting #106-Bis-e												R4-2306209

Electronic Meeting, Apr. 17 – 26, 2023

Agenda item:			5.8.5

Source:	Moderator (Apple)

Title:	Topic summary for [106bis-e][130] FR2_multiRx_UERF_part2

Document for:	Information

# Introduction

This email thread treats the following topic:

- System parameter assumption and UE architecture (5.8.2.1)

On UE behavior assumptions, as contributions belonging to email thread [106bis-e][129] FR2_multiRx_UERF_part1 or [106bis-e][130] FR2_multiRx_UERF_part2 contain proposals, moderator has coordinated with moderator of thread [106bis-e][129] FR2_multiRx_UERF_part1 to treat all relevant proposals in this thread.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Murata | Hidefumi Ohira | hidefumi.ohira@murata.com |
| vivo | Hao DU | Duhao.txyjy@vivo.com |
| Huawei | Gao Xiang | gaoxiang74@huawei.com |
| ZTE | Wei Lin | lin.wei125@zte.com.cn |
| Nokia | Yi Tan | Tan.yi@nokia.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |
| Xiaomi | Zhang Juan | zhangjuan8@xiaomi.com |
| Ericsson | Christian Bergljung | Christian.Bergljung@ericsson.com |
| OPPO | Qifei Liu | liuqifei@oppo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: System parameter assumption, UE architecture and conditions of UE RF requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304120 | Nokia, Nokia Shanghai Bell | Observation 1: Single fixed AoA offset will not cover all the relevant practical use cases. Observation 2: It was agreed in RAN4#105 meeting that the scenario where a single antenna module is used to receive 2 AoAs simultaneously should not be excluded. Observation 3: Module-splitting will be required to receive data-streams on 2 different AoAs particularly in the small AoA offset scenario (e.g., 30∘ or 60∘). Observation 4: The definition of antenna panel is already agreed in R4-2220533 which states that: ‘Panel’ is defined as a group of antenna element that controls beam independently and has the following attributes - Within a panel, one beam can be selected and used for DL reception. - Across different panels, multiple beams (each selected per panel) may be used for DL reception. - ‘Beam’ is assumed to mean spatial filter associated with reception. Observation 5: The interference from the TRP2 signals arriving from the AoA2 direction becomes significant when the angular separation between AoA1 and AoA2 is small. Observation 6: The SIR values corresponding to the 50th percentile of the CCDF curves of Figure 7 is shown in Table 3. Beam Index (Array 2/Array 3); SIR at 50-percentile of CCDF (dB). Beam 7; 10. Beam 6; 17. Beam 5; 21. Beam 4; 24. Beam 3; 24. Beam 2; 26. Beam 1; 26. Table 3: SIR at 50-percentile of CCDF of Figure 7 Proposal 1: Consider multiple fixed AoA offset values for defining the core requirements. Proposal 2: To ensure that spherical coverage is met for the scenario when only one AoA link is active, select Option 1. Proposal 3: Agree to the following proposals for 2 TRP UE behavior assumptions: - Option 1 for module selection. However, the use of maximum RSRP or maximum SINR for module selection is FFS. - Option 1 for beam selection. However, the use of maximum RSRP or maximum SINR for beam selection is FFS. - Option 2 for module-splitting leaving it to UE implementation. Proposal 4: Change ‘module selection’ concept in 2 TRP UE behavior assumptions to the concept of ‘panel selection’ as proposed below: ### 2TRP UE behavior assumptions - Proposal on module panel selection - Option 1: UE assigns ‘first’ module panel to track TRP that yields highest RSRP among all combinations of modules and TRPs. The subsequent module panel is assigned to track the other TRP. - SINR maximization instead of RSRP maximization not precluded. - Option 2: Other - … Agreement: - FFS.. |
| R4-2304129 | Apple | Observation 1: Without clear specification on UE beam reporting criterion, it is very hard to restrict how a UE selects its antenna module or beam for the two TRPs. Proposal 1: Given the previous RAN4 agreement on ‘antenna module’ and ‘panel,’ there is no need to specifically consider antenna module-splitting in defining RF requirement. Proposal 2: It remains to be seen if it is necessary to specify minimum network benefit for enhanced UE, given RAN4 is going to specify some spherical coverage requirement. |
| R4-2304163 | Murata | Observation 1: If AoA offset become larger, CDF seems become better. Observation 2: From the viewpoint of antenna panel, the angle between TRP1 and TRP2 can be both larger than and smaller than AoA offset. Observation 3: The effect of MMSE-IRC is different among measurement grids. Observation 4: For example, SINR become 0.9 or 5.1 dB better in this T-doc by MMSE-IRC. Proposal 1: We support using MMSE-IRC for the case the simulation result of AoA offset = 30 degrees much worse than results of other AoA offset cases. Observation 5: If we change parameters from legacy, CDF can be better without using multi-Rx. (ex. better resolution of phase shifter will improve CDF of legacy UE.) Proposal 2: When we compare the specification of multi-Rx with that of legacy, we should use the same parameters with legacy. And if we use other parameters, we should compare the specification of multi-Rx with that of one panel simulation using the same assumption. Observation 6: Some legacy smartphones have antenna on side. Proposal 3: Add plastic to side cover assumption. Item; Simulation assumption; Note. Front cover (Plastic, Glass, Ceramic, Metal); Glass; This information is meaningful only if it’s the same with the material which covers antennas.. Back cover (Plastic, Glass, Ceramic, Metal); Glass. Side cover / Frame (Plastic, Glass, Ceramic, Metal); Metal, Plastic. Display panel – Full (Y) or Partial (N); Y. Bezel Margin; 1.5mm; Module can’t be placed outer edge of UE to secure mechanical reliability. |
| R4-2305097 | vivo | Observation 1: The multi-Rx UE may show different performance under legacy requirement verification if multiple antenna modules are activated simultaneously. Proposal 1: The test configuration should ensure that only one antenna module is activated for multi-Rx UE during legacy requirement verification. Proposal 2: Same requirement can be applied to the UE support overlapPDSCHsInTimePartiallyFreq-r16 and overlapPDSCHsFullyFreqTime-r16. Proposal 3: No need to discuss and define the RF requirement for the UE only support multiDCI-MultiTRP-r16. |
| R4-2305611 | OPPO | Proposal 1: Considering the UE optimized performance guarantee and the affordable test work load, two AoA separations is proposed as the number of AoA separations that UE must meet the requirement. Proposal 2: it is proposed that the two AoA separations are determined based on UE’s declaration. Proposal 3: The two AoA separations are picked up from the following subgroups respectively. - AoAsep1 is picked up from {[30°], 45°, 60°, 75°, 90°} - AoAsep2 is picked up from {105°, 120°,135°, 150°, 165°, [180°] |

Other related contributions discussing UE behavior assumptions

| R4-2305750 | Sony, Ericsson | Proposal 2: RAN4 shall further discuss the simulation assumption for UE behavior when the UE selects the two beams for the two DL signals. |
| --- | --- | --- |
| R4-2304603 | Qualcomm | Proposal 2: Choose option 1 [1.2.14, 7] on beam selection as baseline for simulation: UE selects beam for each module so RSRP of RS from assigned TRP is maximized. Proposal 3: Choose option 1 [1.2.14, 7] on module selection per TRP for PC3 as baseline for simulation: UE assigns ‘first’ module to track TRP that yields highest RSRP among all combinations of modules and TRPs. The best of the other modules is assigned to track the other TRP. Proposal 4: Choose option 1 [1.2.14, 7] on module splitting as baseline for simulation: no module splitting behavior is assumed for PC3. |
| R4-2304824 | Samsung | Proposal 3: adopt RSRP criteria rather than SINR criteria as beam selection assumption for simulation and requirement derivation. Observation 2: the impact of beam selection prioritization between TRPs is tiny. |

## Open issues summary

### Beam selection

Option 1: Based on RSRP

Option 2: Based on SINR, such as max[log(1+SINR_AoA1) + log(1+SINR_AoA2)]

Option 3: Up to UE implementation. No need to reach an agreement.

| Company | Comments |
| --- | --- |
| Murata | We prefer selecting option 1 or 2 in discussion. |
| vivo | Option 1, which align with RAN1 beam reporting. Option 3 is not acceptable, because this detail will impact the results of simulation, and no agreement will put a great hindrance on simulation campaign between companies. |
| Huawei | From our understanding, Option 1 is currently supported by specification so we prefer to use it at least for evaluation purpose. Further enhancement like Option 2 can be pursued in RRM session. (seems that there is parallel discussion regarding this issue in RRM) |
| ZTE | We support option 1 and the beam selection is based on L1-RSRP in the current specification. |
| Nokia | Option 1 is OK for the simulation. |
| Xiaomi | Option 1 |
| Sony | Fine with option 1 |
| Samsung | Option 1 |
| Ericsson | Option 1 |
| MediaTek | Option 1 is Ok for the simulation. |
| LGE | Option 1 |
| OPPO | Option 1 |

### TRP to module pairing

Option 1: UE assigns ‘first’ module to track TRP that yields highest RSRP among all combinations of modules and TRPs. The best of the other modules is assigned to track the other TRP

Option 2: Use max[log(1+SINR_AoA1) + log(1+SINR_AoA2)] to select the best module/beam for each TRP

Option 3: Up to UE implementation. No need to reach an agreement.

| Company | Comments |
| --- | --- |
| Murata | We prefer selecting option 1 or 2 in discussion. |
| Vivo | Option 1, similar reason in 1.2.1 |
| Huawei | Option 1. |
| Nokia | Option 1 is OK for simulation. |
| Xiaomi | Option 1 is OK in simulation |
| Sony | Fine with option 1 |
| Samsung | Option 1 |
| MediaTek | Option 1 is Ok for the simulation. |
| OPPO | Option 1 |

GTW Agreement:

- Take Option 1 as the baseline assumption for simulation.

  - Companies are expected to clarify the assumptions whether RSRP or SINR is used for simulation.

### On module splitting

Option 1: Module splitting needs to be explicitly considered.

Option 2: Up to UE implementation. No need to further discuss it given the RAN4 agreement on “antenna module” and “panle”

| Company | Comments |
| --- | --- |
| Murata | We support option 2. |
| Vivo | Option 2 |
| Huawei | Option 2. |
| ZTE | Option 2. |
| Nokia | Option 2 |
| Apple | Option 2 |
| Xiaomi | Option 2 |
| Sony | Option 2 |
| Samsung | Option 2 |
| Ericsson | Option 2 |
| MediaTek | Option 2 |
| LGE | Option 2 |
| OPPO | Option 2 |

### Minimum network benefit

Option 1: No need to further discuss given the 2AoA spherical coverage/probability requirement is taking shape and being agreeable.

Option 2: Others

| Company | Comments |
| --- | --- |
| Murata | We support option 1. |
| Vivo | Option 1 is OK for us. |
| Huawei | Option 1. |
| ZTE | We support option 1. |
| Nokia | Option 1 is OK |
| Apple | Option 1. |
| Xiaomi | Option is OK |
| Samsung | Option 1 |
| Ericsson | Option 1 |
| Qualcomm | We are ok to not discuss it, but we may need to bring it back for perspective if there are corner-case implementations in the averaging pool. For now, option 1 is ok. |
| MediaTek | Option 1 is OK |
| LGE | Option 1 |
| OPPO | Option 1 |

### UE capability for mDCI

Given the RAN4 agreement “To have the unified requirement concept for UEs supporting multi-DCIs as for UEs supporting single DCI, RAN4 can focus on fully overlapping in time and in frequency, supported by UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16.”” Do we need to further agree on the following two proposals?

Proposal 2: Same requirement can be applied to the UE support overlapPDSCHsInTimePartiallyFreq-r16 and overlapPDSCHsFullyFreqTime-r16.

Proposal 3: No need to discuss and define the RF requirement for the UE only support multiDCI-MultiTRP-r16.

| Company | Comments |
| --- | --- |
| vivo | As proponent, we can further explain our intention for the proposals above. For multi-DCI based multi-Rx, serval optional capability exists: - multiDCI-MultiTRP-r16 - overlapPDSCHsFullyFreqTime-r16 - overlapPDSCHsInTimePartiallyFreq-r16 Previous agreement only solves the problem that what is the baseline for requirement design, but we don’t know what should we do if UE report multiDCI-MultiTRP-r16 and overlapPDSCHsInTimePartiallyFreq-r16 or only support multiDCI-MultiTRP-r16. |
| Huawei | If discussion outcome from Demod session could be SINR threshold other than -1dB is more appropriate to be considered for different UE capabilities, then we need to distinguish them. Otherwise we can choose Option 3. |
| Nokia | We suggest (new) option 4: There is no interference in non-overlap case. So, same legacy requirements will apply for AoA1 and AoA2 individually. |
| Apple | We support Proposal 3 because RAN4 agreed earlier that we “• Strive to define single set of requirements for both sDCI and mDCI.” Proposal 2 is OK, but things should be fine without it. |
| Xiaomi | We support Proposal 2, since if UE only reports multiDCI-MultiTRP-r16, it means the UE supports multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency. That is, the resources of two PDSCHs are overlapping in time and non-overlapping in frequency, but two PDSCHs are still allocated in the same active BWP, and they can be allocated interlacedly in frequency, the interaction between two PDSCHs still need consider. |
| Qualcomm | We are ok to support option 3, if the intent is to not create unique requirements for one variant of UE ‘UE only support multiDCI-MultiTRP-r16’. The requirements will get very fragmented if choose partial overlap cases, these can be considered for future enhancement. |

### Impact on singel AoA performance

Proposal: The test configuration should ensure that only one antenna module is activated for multi-Rx UE during legacy requirement verification.

| Company | Comments |
| --- | --- |
| Murata | We support proposal. |
| Vivo | As proponent, our concern here is that if multiple modules are still activated simultaneously, the UE performance may be degraded during legacy requirement verification. So we think it is needed to ensure multiRx UE follow legacy behavior under legacy requirement verification. |
| Huawei | Whether Rel-18 UE will get benefits or punishments from the legacy test can be further discussed. But one follow-up question is how to reflect such limitation in the specification? |
| Nokia | It is OK. |
| Apple | We would like to understand what caused the performance degradation in case two antenna modules are activated. Furthermore, it is unclear if we need to reach any agreement on this because in the testing, legacy (single AoA) requirement is verified separately from the two AoA requirement. |
| Xiaomi | We are OK for the proposal, I think it’s just to confirm the test condition for the objective in the WI: - The legacy spherical coverage requirement for reception from a single direction will be kept |
| Sony | It is unclear to us the implication of the proposal. It seems this is only about legacy single AoA test and it is UE implementation issue rather than a test issue in our view. |
| Samsung | Share similar understanding as Sony, maybe it is not necessary to add additional limitation to legacy test, and there seems no way to ensure such limitation either. |
| Ericsson | In our view the multi-RX test is a functional test for UEs indicating the [MIMO] capability, unclear why this test should ensure that the UE does not fail another test. |
| Qualcomm | We agree with vivo’s concern, but this is up to the UE to manage properly. It still must meet legacy requirements when legacy connectivity conditions are presented (single TCI state etc). |
| LGE | For clarification, does UE supporting the multiRx simultaneous have two tests? Legacy test with single antenna module and multiRx test. |

### On MMSE-IRC

Should MMSE-IRC be used for small AoA offset such as 30 degrees for better performance?

| Company | Comments |
| --- | --- |
| Murata | If one AoA offset result is much worse than others, we should use MMSE-IRC to that condition or we should remove that AoA offset condition from evaluation list. (The result may relate to [129] 1.2.5 and connection method, bias and so on.) |
| vivo | The proposal is reasonable but is an implementation issue, and UE can use preferred implementation only if the requirement can be satisfied. We think no need make agreement on this implementation specific issue. |
| Nokia | It is OK. |
| Apple | We also think it can be left to UE implementation and hence no agreement is needed. |
| Sony | We are open for companies to bring in the analysis for more advanced receiver design for core requirement discussion. However, we don’t think this needs to be mandated. |
| Samsung | It is up to implementation. |
| Qualcomm | A more pointed question perhaps is should the requirements discussion assume advanced receivers – our view is no. Perhaps in the future this can be an avenue of enhancement. |

## Companies views’ collection for 1st round

### Open issues

See Section 1.2.

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1.2.1 and 1.2.2 | GTW Agreement: - Take Option 1 as the baseline assumption for simulation. - Companies are expected to clarify the assumptions whether RSRP or SINR is used for simulation. Recommendations for 2nd round: No further discussion |
| Sub-topic #1.2.3 | Option 2 is agreed. Option 2: Up to UE implementation. No need to further discuss it given the RAN4 agreement on “antenna module” and “panle” Recommendations for 2nd round: No further discussion |
| Sub-topic #1.2.4 | Option 1 is agreed. Option 1: No need to further discuss given the 2AoA spherical coverage/probability requirement is taking shape and being agreeable. Recommendations for 2nd round: No further discussion |
| Sub-topic #1.2.5 | To confirm the following tentative agreements in the 2nd round: The same requirement shall be applied to the UEs supporting either of the following two capability combinations: - UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16”. - UE capability “multiDCI-MultiTRP-r16” and “overlapPDSCHsInTimePartiallyFreq-r16”. No need to discuss and define the RF requirement for the UEs only supporting “multiDCI-MultiTRP-r16”. |
| Sub-topic #1.2.6 | To discuss if the following proposal is agreeable in the 2nd round: It is up to UE implementation how many panels are activated for multi-Rx UEs during legacy (R15) requirement verification. |
| Sub-topic #1.2.7 | To discuss if the following proposal is agreeable in the 2nd round: It is up to UE implementation what receiver is used meeting the RF requirement, noting MMSE-IRC receiver is the baseline receiver for NR. |

### CRs/TPs

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2304120 |  |  | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2304129 |  |  | Apple | Noted |  |
| R4-2304163 |  |  | Murata | Noted |  |
| R4-2305097 |  |  | vivo | Noted |  |
| R4-2305611 |  |  | OPPO | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents