3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2306279

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			6.3.5

Source:	Moderator (Qualcomm Incorporated)

Title:	Topic summary for [106-bis-e][116] LTE_terr_bcast_bands_UERF

Document for:	Information

# Introduction

This document is a summary of the contributions submitted under agenda items 6.3.2 and 6.3.3 of the RAN4 #106-bis-e electronic meeting.

# Topic #1: Band plan and UE filter architecture

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304461 | SWR, EBU | 1. Provide a full band (low pass) filter, derived by existing components (as the one reported as reference in [3]) in an appropriate overlapping scheme with existing band pass SAW filter commercially available on the market for NR UEs supporting 600 MHz bands (e.g. NR band n105 or n71), as well as band pass SAW filters derived from the aforementioned commercially available filter. The above filter’s subsystem architecture is proposed to be standardized, according to the requirements defined in [5]. 2. Define a full broadcast band which is overlapping the existing NR 600 MHz bands, that would allow to reuse most of the NR UE hardware design, hence facilitating 5G Broadcast development. 3. Determine the number of additional filters to help reduce the full band filter’s insertion loss in the upper part of sub-700 MHz band while providing the necessary rejection for the adjacent IMT uplink transmissions above 703 MHz. |
| R4-2304878 | Nokia, Nokia Shanghai Bell | Observation 1: Coexistence with band 28 is crucial for 5G broadcast band. Observation 2: At least upper part of the full UHF band needs to be covered by a sub-band filter that can ensure the coexistence with band 28. Proposal 1: For full UHF band, it is proposed to assume multiple UE Rx filters. |
| R4-2305351 | ZTE Corporation | Proposal 1: To define the band for LTE based broadcast as in Table 2.1 and NR-ARFCN as in Table 2.2. Proposal 2: The reduced blocking is feasible and can be defined as -25 dBm. |
| R4-2305823 | Qualcomm Incorporated | Observation: To support UE blocking at 450 MHz, 600 MHz, 700 MHz, and 800 MHz frequency ranges will likely require a multi-filter implementation in the UE. Although it is possible for 3GPP to assume such a reference architecture in specifying requirements, its implementation may not be suitable to current handset design trends. Observation: Specifications for dual connectivity between 5G broadcast and a cellular RAT is out of scope of the present work item. Proposal: If a multi-filter UE architecture is to be assumed to specify requirements for a full UHF band, it should be specified as a separate band. The work item can be closed as long as the requirements for at least one band are agreed. |
| R4-2305824 | Qualcomm Incorporated | Draft CR: Introduction of 5G broadcast UHF bands – Clauses 3, 4, and 5 |

## Open issues summary

On the band plan, there was an agreement at RAN4 #106 [R4-2303489] as follows

Agreement from online session:

- Define the band plan based on available data for BS and UE.

  - Full UHF band, i.e., 470 – 698MHz,

    - with reduced UE blocking

  - Narrow band, i.e., 612 – 652 MHz, which is the same as the downlink operating band of n105.

  - If data for other frequency ranges becomes available before the conclusion of the work item, it can be considered either as a separate band or as a modification to initially defined bands.

Way Forward

- Further clarification and study is needed to elaborate on and quantify requirements with respect to reduced UE blocking for the full UHF band.

- All technical requirements for these bands are still to be defined.

There were no proposals received for any other band plan (frequency range) at this meeting.

However, there were a number of papers on a multi-filter architecture to support the full band with improved blocking, but no concrete proposal on the exact number of filters, filter ranges, etc.  The moderator invites proponents to provide these details for evaluation.

On the proposal for NR-ARFCN from R4-2305351, the moderator understands this work item is for the introduction of E-UTRA (LTE) bands so EARFCN format is needed rather than NR-ARFCN.

A draft CR for system parameters is provided in R4-2305824.  The moderator requests companies to provide comments in section 1.3.2 for this draft CR.

### Sub-topic 1-1

Issue 1-1: Band definition

- Proposals

  - Option 1: Keep the bands according to WF in R4-2303489

  - Option 2: Define a new full band (470 – 698 MHz) in addition to those in R4-2303489 with requirements derived based on a multiple filter reference architecture

  - Option 3: Replace the full band from R4-2303489 with one that has requirements derived based on a multiple filter reference architecture

- Recommended WF

  - TBA

### Sub-topic 1-2

Issue 1-2: Multiple filter UE architecture

- The concept of a UE architecture consisting of multiple filters was proposed by many companies, but the details are scant.

- Moderator suggestion:  At least the following details are needed to derive requirements

  - The insertion losses and rejection as a function of frequency offset over temperature and production

  - The number of filters and their frequency ranges

  - What is the OEM view on the maximum number of filters supportable on a handset design?

- Recommended WF

  - The moderator invites companies to comment and provide details.  Can filter vendors and OEM’s provide details?

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Band definition Sub topic 1-2: Multiple filter UE architecture |
| Nokia | Sub topic 1-1: Band definition We support option 3. We assume full UHF band may be available to 5G broadcast service and may need to be supported by single UE. With 8 MHz overlap, frequency range 470-620 MHz can be specified with reduced blocking (and possibly relaxed REFSENS) with one filter (like in DVB-H). 612-652MHz and 644-698MHz can be based on a similar out-of-band blocking and insertion loss performance as n105 downlink. So total 3 filters could be assumed. Some exemptions about the UE coexistence with band n28 can be considered for 652-698MHz. |
| Huawei | Sub topic 1-1: Band definition Question for clarification: what’s difference between a new full band and the full band agreed in WF R4-2303489? Have we agreed any filters assumption in WF R4-2303489? Sub topic 1-2: Multiple filter UE architecture From UE implementation view, not only it’s very challenge to implement a lot of filters for the low frequency, but also it’s very difficult to support 470MHz antenna. |
| Ericsson | Sub topic 1-1: Band definition We support Option1, those bands could be specified based on existing filters. We could accept option2 but only if it’s based on concrete proposals and filters datasheet or simulations. In the past RAN4 meetings, our bands proposals were not accepted because we were not able to provide such concrete figures, RAN4 shall be consistent. |
| Qualcomm | Sub topic 1-1: Band definition We agree with the comment from Ericsson. |
| Sony | Sub topic 1-1: Band definition Option 1 |
| SWR | Sub topic 1-1; Band definition We support Option 2 as described in R4-2304461 (which also should address Ericsson’s comment). However, the proposal from Nokia just above, e.g. three bands 470-620, 612-652 and 644-698 could be considered as well. More information on this would be welcome. Sub topic 1-2: Multiple filter UE architecture Regarding the comment from Huawei, we think that antenna aspects in the lower part of the UHF band are not within the scope of the current discussion. |
| R&S | Subtopic 1-1: We should at least have the bands we agreed on in the last meeting. If we can reach consensus on adding an additional band based on multi-filter architecture like in Option 2, this can be added. Also we are not opposed to the proposal from Nokia to add additional bands, as this is already captured in the WF from last meeting. |
| ZTE | Sub topic 1-1: Band definition Option 1 |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2305824 Draft CR on clauses 3, 4, and 5 | nokia: Use of 10 MHz channel bandwidth in 5.6H may indicate that SIB1 is broadcast as 52 PRBs. However, we understand 10 MHz is UE implementation issue and is not needed in the spec. SIB1 BW is 30, 35 or 45 PRBs. We think it is only necessary that the requirement allows the use of 10 MHz channel filter but it is not necessary to explicitly written in the spec. It is unclear what it means by “(not drawn to scale)” in figure Figure 5.6H-1. Can you elaborate on the intention of this phrase? |
|  | huawei: Share the similar view with Nokia. It’s very strange that we put 10MHz and 6/7/8MHz together into one table. The channel bandwidth is declared as 10MHz, but the channel spacing is 6/7/8MHz. It’s very hard to understand how this can be implemented. |
|  | Qualcomm: Adding 10 MHz channel bandwidth is beneficial because from the UE side, we are not defining new channel bandwidths. |
|  | MediaTek: If 10 MHz channel bandwidth is removed, UE needs explicitly to design 6/7/8 MHz hardware. |
|  | SWR: We support the draft CR which is in line with the WF of the Athens meeting, however, the sections depend on the definition of the band(s), meaning the sections would need to be aligned corresponding to the decision taken. |
|  | R&S: Thank you Qualcomm for the draft CR. It seems to be aligned with previous discussions. Editorially I think it is necessary to add the newly added terms like Broadcast bandwidth or BWbroadcast to chapter 3. |
|  | We agree with MediaTek, removing 10MHz channel bandwidth is not Ok as it will mean that the corresponding hardware for 6/7/8MHz have to be designed. Furthermore, Table 5.6H and sub-clause 5.6H.1 are a bit confusing. The UE channel is always 10MHz, and the 6,7,8MHz is just how many RBs are scheduled. In other words, we do not have to specify that only a sub-set of RBs is scheduled. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Sub-topic 1-1: Band definition The majority of companies (5 of them) preferred Option 1, while 1 company preferred option 3 and 1 company preferred option 2. Sub-topic 1-2: Multiple filter UE architecture Two alternatives were provided – one in R4-2304461 and another with 3 bands (470-620, 612-652 and 644-698). Candidate options: Recommendations for 2nd round: Sub-topic 1-1: Work to proceed according to the WF in R4-2303489 (i.e., full band with single filter and partial band 612 – 652 MHz). The work can be discussed under Topic 2 on UE RF requirements. Sub-topic 1-2: At the same time, companies are not precluded from continued discussion on other multiple filter architectures, sharing details of filter performance, and specs. Sub-topic 1-3: Comments on the draft CR R4-2305824 can be further discussed. In particular, there was discussion on whether 10 MHz channel bandwidth should be included in the CR and how to capture the 6, 7, and 8 MHz broadcast channels. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2305824 Draft CR on clauses 3, 4, and 5 | Recommend a revision if there is agreement on sub-topic 1-3 in the second round. |

## Discussion on 2nd round (if applicable)

Any continued discussion or further details on the multiple filter full band alternatives can be captured here.

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-2: Multiple filter UE architecture Sub-topic 1-3: 10 MHz channel bandwidth and 6, 7, and 8 MHz broadcast channels in the specification |
| Qualcomm | Sub-topic 1-3: We think it’s important to include the 10 MHz channel bandwidth in the specification for reasons expressed in the first round; however, to minimize the confusion between “channel bandwidth” and “5G terrestrial broadcast bandwidth” another option is to keep the 10 MHz channel bandwidth and specify 5G bandwidth as PMCH_bandwidth in units of RB’s. So instead of 6, 7, and 8 MHz, the spec could be written as 30, 35, and 40 RB’s. This notation would be more aligned with 36.331. In response to Nokia’s question from the first round, the “not drawn to scale” is because the number of RB’s in the diagram doesn’t correspond to 30, 35, or 40 RB’s. I couldn’t fit that many RB’s into the figure. |
| Moderator | The comments below are copied here from the draft WF on UE RF requirements. |
| Qualcomm (proposal in draft WF) | Channel bandwidth of 10 MHz for the UE has been assumed, but there are differing views on whether this detail needs to be included in the specification. - Option 1: Include 10 MHz channel bandwidth and 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth (a la R4-2305824) - Option 2: Do not include 10 MHz channel bandwidth as this is an implementation detail. The specification shall only mention 6, 7, and 8 MHz 5G terrestrial broadcast bandwidths - Option 3: Include 10 MHz channel bandwidth, but instead of referring to 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth, refer to PMCH-bandwidth of 30, 35, and 40 RB’s as specified in 36.331. Way forward: Option 1 |
| Huawei | In the WID, we only have the following assumption, but we don’t have the objective to specify 10MHz channel bandwidth. A lot of details need to be discussed on how to implement the UE requirements for 6/7/8 MHz. These can be left to further discuss in next meeting. |
| Apple | On the one hand, the effective channel bandwidth of the broadcast signal is indeed 6, 7, 8MHz, but on the other hand, a UE is always configured with the 10MHz channel. From the UE implementation perspective, it is always the 10MHz channel, in which some RBs are not scheduled by the network. Our view is that we proceed with something similar to Option 1 or 3. |
| Nokia | We’d need further discussion. For 10 MHz (50 PRBs), 7 MHz (35 PRBs) cannot be centred in the channel. Shouldn’t we support 7 MHz at 100 kHz raster? |
| SWR | Option 2 5G Broadcast channel bandwidths of 6, 7 and 8 MHz for PMCH have been specified by RAN1. For all three bandwidths the PDSCH within the CAS is always 5MHz. So, it does not make sense to consider 10 MHz here. |

## Summary for 2nd round

The following three options were discussed with no agreement.

Channel bandwidth of 10 MHz for the UE has been assumed, but there are differing views on whether this detail needs to be included in the specification.

-	Option 1: Include 10 MHz channel bandwidth and 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth (a la R4-2305824)

-	Option 2: Do not include 10 MHz channel bandwidth as this is an implementation detail.  The specification shall only mention 6, 7, and 8 MHz 5G terrestrial broadcast bandwidths

-	Option 3: Include 10 MHz channel bandwidth, but instead of referring to 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth, refer to PMCH-bandwidth of 30, 35, and 40 RB’s as specified in 36.331.

# Topic #2: UE RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304511 | Mediatek India Technology Pvt. | Proposal 1: Regarding REFSENS, to consider whether 10MHz CBW for noise integration would be applicable. Proposal 2: Regarding the desired 6 MHz channel, if the adjacent 5 MHz ACS interferer could not be outside the channel filter due to requested broadcast scenario, to consider defining ACS case 1 as -16 dB and 5 MHz adjacent channel interferer with frequency offset at 5.5125 MHz. |
| R4-2304879 | Nokia, Nokia Shanghai Bell | Proposal 1: For all receiver requirement, all the channel bandwidths, 6, 7 and 8 MHz, are specified with downlink only PMCH. Proposal 2: For REFSENS evaluation, noise integration bandwidth shall be the same as the channel bandwidth. Proposal 3: If we specify lower ACS than 33 dB for 6 MHz, it is proposed to specify better ACS for 7 and 8 MHz. |
| R4-2305382 | Huawei, HiSilicon | Proposal 1: In order to make progress, it’s recommended to consider the following compromise: For full UHF band, i.e., 470 – 698MHz, a newly defined 6, 7, and 8 MHz PMCH should be defined for REFSENS without 10 MHz PDSCH considering the demands from broadcast operators. For Narrow band, i.e., 612 – 652 MHz, only the existing 10 MHz PDSCH should be defined for REFSENS considering the IMT UE industry eco-system and to reduce the test efforts. Proposal 2: it’s recommended for companies to provide the some measurement datas to derive the maximum input level for UE. Otherwise, RAN4 can just further trade off the values from -15dBm to -22dBm based on the assumption on broadcast transmission power. Observation 1: UE ACS is a major requirement to guarantee the system coexistence between two adjacent channels. RAN4 can’t specify this requirement only based on UE filter implementation. Observation 2: based on the analysis above, UE ACS requirement will have a big impact on system DL ACIR coexistence performance. The system DL ACIR coexistence performance for 25dB UE ACS is much worse than 33dB UE ACS, not to mention 16dB UE ACS. Proposal 3: Option 3 (-33 dB with frequency offset at 6, 7, and 8 MHz) can be specified for the coordinated network deployment. |
| R4-2305822 | Qualcomm Incorporated | Channelization; Power in transmission bandwidth configuration; PInterferer; ACS (updated result). 10 MHz (baseline); REFSENS + 14 dB; REFSENS + 45.5; 33 dB. 6 MHz; REFSENS + 14 dB; REFSENS + 41.5; 29 dB. 7 MHz; REFSENS + 14 dB; REFSENS + 43.0; 30.5 dB. 8 MHz; REFSENS + 14 dB; REFSENS + 44.0; 31.5 dB. |

## Open issues summary

### Sub-topic 2-1

Issue 2-1: Reference sensitivity

- Proposals

  - Option 1: Use 10 MHz noise integration for all 6, 7, and 8 MHz channels.  RMC would be 10 MHz PDSCH based.

  - Option 2: Define 6, 7, and 8 MHz PMCH RMC’s for refsens using 6, 7, and 8 MHz bandwidth for noise integration

  - Option 3:  Apply option 1 for 612 – 652 MHz band, and option 2 for 470 – 698 MHz band

- Recommended WF

  - TBA

### Sub-topic 2-2

Issue 2-2: ACS

- Proposals for 6 MHz channel

  - Option 1: 16 dB (MTK)

  - Option 2: 33 dB (Huawei, Nokia)

  - Option 3: 29 dB (Qualcomm)

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Refsens Sub topic 2-2: ACS |
| Nokia | Sub topic 2-1: Refsens We support Option 2. We do not understand if Option 3 can save test effort significantly if blocking is tested with 6/7/8 MHz. Sub topic 2-2: ACS We support Option 2 and 3. If lower ACS than 29 dB is specified, it is proposed to specify ACS for each channel bandwidth. It may be also useful to specify next adjacent ACS as 33 dB. |
| Huawei | Sub topic 2-1: Refsens Option 3 can be considered as a compromise. Sub topic 2-2: ACS Option 2 and 3 are very close. Seems we can make some compromise to reach the consensus. |
| Qualcomm | Sub topic 2-1: Refsens Option 2 makes the most technical sense. Sub topic 2-2: ACS Option 3. The value of 16 dB from last meeting was not correctly computed since it only considers the edge RB. The value of 33 dB may not be achievable, but we are open to technical evaluation of feasibility from other companies. |
| MediaTek | Sub topic 2-1: Refsens Option 3 as a compromise. Sub topic 2-2: ACS We can understand that high ACS RF requirement may help. Additionally, the ACS interferer is still inside 10MHz-BW filter which means FFT hardware/cost need to cover both desired signal and ACS interferer. If 33dB ACS for 6MHz desired is assumed, it seems half of adjacent power(30dB higher than signal) is inside 10MHz-BW filter. At the beginning, FFT implementation does not need to handle ACS interferer since ACS interferer should be knocked down by digital filters. Even with 16dB ACS, the FFT complexity is higher than the LTE and DVB receiver design which preclude fully reusing original LTE implementation. In other words, one possible way is that LTE UE should consider whether to add 6/7/8MHz Modes back for knocking down the adjacent interferer before add high-dynamic-range FFT or to use DVB receiver. We prefer option1 and take option 3 as a compromise. |
| Sony | Sub topic 2-1: REFSENS Option 2 |
| SWR | Sub topic 2-1: Refsens We support Option 2. Sub topic 2-2: ACS We support Option 2. The target should be 33 dB ACS at channel offsets of 6, 7 and 8 MHz. If this cannot be met, network planning and deployment (based on existing HPHT broadcast network infrastructure) may be a challenge. Moreover, it seems in QCs contributions R4-2302704 and R4-2305822, an interferer of 5 MHz bandwidth is used in order to derive the ACS values. However, in deployments the interferers will have 6, 7 or 8 MHz bandwidth. Therefore, we would appreciate some clarification. |
| R&S | Sub topic 2-1: Refsens Option 2 seems to be the most straight forward one. |
| ZTE | ## Sub topic 2-1: Refsens Option 2 is preferred. |
| Apple | ## Issue 2-1: Reference sensitivity ## Option 1: This is the existing baseline and is according to the WI objectives as it relies on the existing RMC channel and REFSENS testing procedures. Option 2 is a completely new RMC with the testing procedure, which might need further feedback from RAN5. At this point we are not ready to accept Option 2 without understanding all issues that we might face in RAN5 with regards to introduction of a new RMC and the testing procedure. Option 3 is quite strange. ## As a summary, we can take Option 1 as the most straightforward baseline. ## Issue 2-2: ACS ## Firstly, why is it only for the 6MHz channel? The discussion which we had in Athens concluded that if we capture degraded ACS, then it should be done for 6, 7, 8MHz channels. ## Option 2 is clearly out of question as it assumes that a dedicated filter will be used, which is even against WI objectives. ## Option 3 with 29dB looks like a theoretical estimation of potential degradation. With the 6MHz transmission bandwidth, there could be 2MHz blocker either at the left or the right edge of the 10MHz filter resulting in 10log(2/5)=-3.9dB degradation. The actual degradation is a more complex function that depends on the wanted and desired signal strength, as can be seen from the figure below showing SNR degradation as the function of the blocker strength. The graph shows that when the blocker signal is 32dB higher than the wanted signal, then the 6MHz SNR can drop from 40dB to -5dB. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Sub topic 2-1: Refsens Most companies preferred option 2 (6 companies), while 2 companies for option 3 and 1 company for option 1. Sub topic 2-2: ACS Option 2 or option 3 received the most support. One company preferred option 1 but could accept option 3. One company questioned how option 3 was obtained but did not object to it and did not provide an alternate proposal. Candidate options: Recommendations for 2nd round: Sub topic 2-1: Refsens Possible agreement that at least 470 – 698 MHz band is specified with 6, 7, and 8 MHz PMCH (this is common between both Option 2 and Option 3), but 612 – 652 MHz band is further discussed whether 6, 7, 8 MHz PMCH or 10 MHz PDSCH should be used. A possible LS to RAN5 can also be discussed if companies feel feedback is needed on aspects related to a new RMC. Sub topic 2-2: ACS Continued discussion on concrete proposals with technical justification is welcomed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Refsens Sub topic 2-2: ACS |
| Qualcomm | Sub-topic 2-1: We are ok with specifying refsens with 6, 7, and 8 MHz noise integration bandwidth for the full band. But we also prefer to specify it the same way for the partial band. It would be odd to specify it differently. And if in the future there are more bands defined, how would their refsens be specified? We don’t think adding a new RMC is a foreign concept to RAN5 so we don’t see the need for an LS. However, we are open to reviewing a draft if it is made available to understand exactly what questions would need RAN5 response to enable development of RAN4 specifications. Sub-topic 2-2: In response to a comment from Apple in the first round, the results we provided were more “theoretical” in nature which we view to be valid technically. We would also welcome measurements from other companies to confirm these results. We suggest taking the 29 dB ACS as baseline while waiting for additional analysis and measurements from other companies. |
| Moderator | The comments below are copied here from the draft WF on UE RF requirements. |
| Qualcomm (from draft WF) | Refsens - Agreement: The 470 – 698 MHz band is specified with 6, 7, and 8 MHz PMCH for reference sensitivity - The 612 – 652 MHz band is specified with Option 1. 6, 7, and 8 MHz PMCH for reference sensitivity Option 2. 10 MHz PDSCH for reference sensitivity Way forward: Option 1 ACS Way forward: ACS 29 dB (R4-2305822) as the baseline. Companies are encouraged to provide further studies and/or measurements at the next meeting. |
| Apple | On refsens Firstly, it seems that our feedback during the first round was not clear. It is quite strange to assume that different bands will use different channels for reference sensitivity. WID objectives mention clearly that we must existing specifications as much as possible, and Option 2 just leverages existing framework for the references sensitivity. Thus, we can proceed with Option 2 as a baseline. Option 1 with PMCH can be considered but needs further checking on the specification impact as it involves new reference channel. |
| Nokia | On refsens We support the original text by Qualcomm. |
| Apple | On ACS The values from the table can be considered as a baseline as 29dB looks too optimistic. Since the ACS is tested through the actual measurements of the reference channel, we would like to see actual throughput values instead of simulations. At least our results indicate quite severe degradation of SINR. The following values can be considered as a baseline as 29dB looks too optimistic Channelization; Power in transmission bandwidth configuration; PInterferer; ACS. 10 MHz (baseline); REFSENS + 14 dB; REFSENS + 45.5; 33 dB. 6 MHz; REFSENS + 14 dB; REFSENS + 28.5; [16] dB. 7 MHz; REFSENS + 14 dB; REFSENS + 30.0; [17.5] dB. 8 MHz; REFSENS + 14 dB; REFSENS + 35.0; [22.5] dB. |
| Nokia | On ACS We support the original text by Qualcomm. |
| SWR | On ACS Since the summary on 1st round made by the moderator indicates that both options 2 and 3 received the most support, we suggest keeping both options as a reference and continue with further studies accompanied by clear proposals and well detailed technical justifications (for both options). Moreover, we are puzzled by the figure from Apple above. They seem to be similar to the not correct figures from last meeting. We thought QC had corrected the figures furing this meeting |

## Summary for 2nd round

The following options were discussed with no agreement

-	For 470 – 698 MHz band and/or 612 – 652 MHz band

Option 1.  6, 7, and 8 MHz PMCH for reference sensitivity

Option 2.  10 MHz PDSCH for reference sensitivity

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on UE RF requirements for 5G broadcast | Qualcomm |  |
|  | LS on the impact of PMCH RMC’s for 5G broadcast | Apple | To: RAN5 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2304461 |  | UHF band allocation for 5G Broadcast: some proposals based on existing UE filter’s technology | SWR | Noted |  |
| R4-2304511 |  | Discussion on RF requirements for 5G Broadcast | Mediatek India Technology Pvt. | Noted |  |
| R4-2304878 |  | Discussion on Band definition | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2304879 |  | LTE based 5G UE RF open issues | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2305351 |  | Discussion on band definition for LTE based broadcast | ZTE Corporation | Noted |  |
| R4-2305382 |  | Discussion on UE RF requirements | Huawei, HiSilicon | Noted |  |
| R4-2305822 |  | An update on in-channel ACS for 5G broadcast | Qualcomm Incorporated | Noted |  |
| R4-2305823 |  | Considerations for multi-filter UE full-band UHF support | Qualcomm Incorporated | Noted |  |
| R4-2305824 |  | Introduction of 5G broadcast UHF bands – Clauses 3, 4, and 5 | Qualcomm Incorporated | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-23xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-23xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-23xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2306568 |  | Introduction of 5G broadcast UHF bands – Clauses 3, 4, and 5 | Qualcomm Incorporated | Withdrawn |  |
| R4-2306566 |  | WF on UE RF requirements for 5G broadcast | Qualcomm Incorporated | Agreeable | Everything is FFS |
| R4-2306567 |  | LS on the impact of PMCH RMC’s for 5G broadcast | Apple | Withdrawn |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents