3GPP TSG-RAN WG4 Meeting # 106bis-e												R4-2306314

Online, April 17 – April 26, 2023

Agenda item:			8.3

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [106bis-e][152] RAN_task_UERF

Document for:	Information

Introduction

This discussion summary will cover two agendas:

8.1 RedCap HPUE

8.2 RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL

According to the contributions submitted, this discussion summary will focus on the following topics:

- Topic#1: [8.1] RedCap HPUE

  - Sub-topic 1-1: Feasibility for RedCap HPUE

  - Sub-topic 1-2: Specification impact for RedCap HPUE

- Topic#2: [8.2] RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL.

  - Sub-topic 2-1: Assess specification and UE implementation impacts when UE is configured with two serving cells, each with SUL.

Note that the tables for collecting comments for sub-topic issues are arranged just below the corresponding sub-topic/issue.

Topic #1: [8.1] RedCap HPUE

Companies’ contributions summary

| T-doc number | Proposals / Observations/Abstracts | Company |
| --- | --- | --- |
| R4-2304404 | Proposal 1: From the RAN4 specification perspective, it is feasible to enable HPUE including PC2 for RedCap in the band agnostic way, with two alternatives depending on whether only PC2 is targeted in Rel-18: - Alternative 1: In clause 6.2I.1, remove the restriction on the applicable power class for RedCap UE. - Alternative 2: In clause 6.2I.1, add one sentence stating that PC2 also applies for RedCap UE. Proposal 2: To reflect the applicable form factor for PC2 Redcap, the following alternatives can be considered: - Alternative 1: Not restrict the applicable form factor for PC2 Redcap from the specification perspective, and whether PC2 is supported or not can be reported by each UE. - Alternative 2: RAN4 to clearly add the restriction on the applicable form factor for PC2 Redcap in the specification. | China Telecom |
| R4-2304661 | Proposal 1: non-RedCap PC2 RF requirements should apply to RedCap UE and do not specify operating bands for RedCap PC2 UE. Proposal 2: Do not restrict the PC2 support for different form factor of RedCap UE. Proposal 3: Use CR to enable PC2 for RedCap in a generic manner in TEI17 | CMCC |
| R4-2305039 | Draft CR for enabling PC2 for Redcap UE | China Unicom, Huawei, HiSilicon |
| R4-2305109 | Observation 1: 3dB antenna loss is only assumed for RedCap wearable devices. Observation 2: Even with the antenna efficiency loss, lower cell edge target data rate can be acceptable for wearables (e.g. smart watch) compared with smart phones (1Mbps cell edge target data rate assumed in RAN1 studies). Observation 3: The justification on UL coverage issue for RedCap UEs is not clear to the group. Observation 4: For sensor and camera form factor, there is no 3dB antenna loss assumption. Observation 5: The OTA test method for NR RedCap TRP measurement is under development in Rel-18 TRP TRS WI. Observation 6: RedCap is a UE feature in RAN4 spec, the HPUE introduction should be band agnostic way. Proposal 1: Following RAN guidance, RAN4 should focus on the applicable form factor discussion on sensor and camera. Proposal 2: More clarification on the justification of increase Reduced-Capability UEs output power is needed. Proposal 3: RedCap HPUE introduction would be a band agnostic way, if necessity and applicable form factor are confirmed. | vivo |
| R4-2305127 | Observation 1. In Rel-17, PC1.5 FR1 RedCap UE is excluded and PC2 FR1 RedCap UE is based on operator request. Observation 2. PC3 RedCap Tx requirements are defined as band agnostic, and reused from the PC3 Tx requirements (up to 20MHz). Observation 3. Three use cases, i.e. Industry sensor,video surveillance, wearables are applied for PC3 RedCap UE in Rel-17. Observation 4. No additional Tx and Rx requirements are foreseen in the case of PC2 RedCap UE introduced. Proposal. To align with the previous agreements, the supported PC2 RedCap UE bands should be based on operator’s request. | ZTE Corporation |
| R4-2305391 | Proposal 1: 1Tx architecture with 26 dBm PA is assumed for PC2 RedCap UE. Proposal 2: The PC2 requirements in Section 6 can be reused for PC2 RedCap UE. Proposal 3: The Reference Sensitivity Degradation from PC3 to PC2 for FDD bands in table 7.3.2-1c can be reused for PC2 RedCap UE. | Huawei, HiSilicon |
| R4-2305502 | In this contribution RAN task of enabling HPUE for RedCap in band-agnostic manner was discussed. Following observations were made. Observation 1: Both A-MPR and RSD are operating band specific. Observation 2: Enabling HPUE only for HD-FDD operation would still require A-MPR analysis. Observation 3: Band specific work could be avoided by giving sufficient A-MPR and RSD allowance to the UE. Observation 4: Re-using RSD from regular NR UE may not be possible as RedCap UE may not have diversity Rx and may suffer from worse RSD. Observation 5: Re-using A-MPR from regular NR UE requires further discussion on applicability of 1Tx vs. 2Tx for PC2 RedCap Observation 6: HPUE could be enabled in band-agnostic manner for TDD bands which do not have additional emission requirements. | Qualcomm Inc. |
| R4-2305711 | In this contribution we have the following proposals for standardization of RedCap PC2: Proposal 1: RAN4 to make necessary improvements on RedCap PC3 specification in the current RAN4 specs. Proposal 2: To enable PC2 support for RedCap, RAN4 to at least revisit 5.2I, 6.1I, 6.2I.1 and 7.3I.2 in the RAN4 specs. Proposal 3: RAN4 to focus on RedCap PC2 for form-factors of non-wearables. | MediaTek Inc. |
| R4-2305843 | In this contribution, we provided our view on the possibility to specify 1Tx PC2 for RedCap in a generic way and make the following proposal. Proposal on PC2 support for RedCap: - PC2 1Tx is supported by RedCap and 1Tx PC2 MPR applies with PC2 signaling without support for 2Tx signaling - PC2 1Tx is supported in a generic way for TDD band and HD-FDD with the corresponding 1Rx/2Rx PC3 REFSENS - For PC2 FD-FDD, a generic, equation-based RSD, is specified and derived from PC3 REFSENS de-sense level that can apply to both 2Rx and 1Rx FD-FDD cases. In order to be on the safe side additional margin can be added. | Skyworks Solutions Inc. |

Open issues summary

Sub-topic 1-1: Feasibility for RedCap HPUE

Sub-topic description:

Issue 1-1-1: Enable PC2 for RedCap HPUE

- Proposals

  - Option1: it is feasible to enable PC2 for RedCap HPUE

  - Option2: it is not feasible to enable PC2 for RedCap HPUE

- Recommended WF

  - Option1

| Company | Comments on Issue 1-1-1: Enable PC2 for RedCap HPUE |
| --- | --- |
| China Telecom | Option 1. |
| MediaTek | Option 1. |
| CMCC | Option 1 |
| Qualcomm | As long as A-MPR and reference sensitivity degradation are handled appropriately, PC2 for RedCap is feasible. |
| Meta | We think the PC2 for RedCap HUE is feasible based on the chip/Device/Component vendors’ contributions. |
| vivo | First, we are not clear about the description of this general issue. Is the feasibility from RAN4 spec perspective or from product implementation perspective? Secondly, from RAN task, “To study the applicable form factor, e.g., sensor, camera” means that only partial UE form factor might be applicable to HPUE RedCap, more discussion is needed. Additionally, even from requirement perspective, based on the analysis from companies’ paper, some of the requirement can not be reused directly. In summary, without clear clarification of applicable form factor, and discussions on applicable requirements and how to introduce requirements, by now, we think this issue should be option 2. Option 2. |
| Huawei | Option 1. |
| OPPO | Option 1. |
| China Unicom | Option 1. |
| CHTTL | Option 1? |
| Skyworks | Option 1: not impact to requirements for HD-FDD but for FD FDD the PC2 FDD request and RSD should be completed before PC2 is applicable to RedCap and 1Tx FD-FDD RSD would be needed: new requirement for RedCap. For R18 may be only HD-FDD and TDD PC2 should be targeted for RedCap. But we are also willing to develop our idea on equation based RSD is there is time for this. 1Tx PC2 should be assumed |
| Nokia2 | It is technically feasible to have PC2 for RedCap UEs. However, it has to be kept in mind that work is still ongoing to define Rel18 RedCap requirements. There are some additional challenges in Rel18 RedCap because of restricted number of PRBs due to which Rel17 requirements do not directly apply in Rel18. |
| Apple | Feasible for TDD bands and half-duplex FDD bands. For PC2 full-duplex FDD bands, we have to consider the REFSENS impact. For devices with small form factor, such as wearables, the antenna efficiency could be substantially worse than normal handheld UE. As a result, the device may frequently operate at PCMAX and REFSENS level which is most vulnerable to Tx self-interference. For 1Rx UE, the situation could be even worse as there is no less-Tx-impacted diversity path to salvage the REFSENS. |
| ZTE | Option 1. |
| Sony | Option 1 |

Issue 1-1-2: Form factor for PC2 RedCap HPUE

- Proposals

  - Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE

  - Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

- Recommended WF

  - TBA

| Company | Comments on Issue 1-1-2: Form factor for PC2 RedCap HPUE |
| --- | --- |
| China Telecom | Either option 1 or option 2 is ok to us. With option 1, for some of the RedCap device form factors, if it is difficult to support PC2, the UE has the freedom to report not support PC2. |
| MediaTek | Option 2, in particular, RedCap HPUE does not apply to wearables. |
| CMCC | Since PC2 is optional, there is no need to restrict applicable form factor in spec. |
| Qualcomm | We do not see the need to restrict this in specification as UE can always be designed for PC3 even if PC2 is allowed by the specification. |
| Meta | This is up to vendor’s form factor analysis results to support PC2 RedCap HUE. Based on the analysis from vendor, RAN4 can choose wither way between two options. |
| Vivo | Option 2. Similar view with MediaTek, in RAN task, RedCap HPUE applicability discussion is focused on non-wearable. In addition, as we discussed in our paper, for sensor and camera we do not see the coverage issue. |
| Huawei | Share the similar view with China Telecom. Either option 1 or option 2 is ok to us. |
| OPPO | Option 1. It depends on UE implementation. |
| CHTTL | Option 1, we think it can be depend on UE implementation, since PC2 support is optional, we don’t need to specifically restrict the form factor in the spec. And in our understanding, wearable also covers a lot of different type of devices, there might be coverage issue in some of the use cases. |
| Skyworks | Option 1: 1Tx PC2 Pas are not necessarily bigger than PC3 Pas and battery size may not be impacted as the higher power will result in shorter transmissions and avoid repetitions. |
| Nokia2 | Option 1. We do not see any particular need to restrict form factor applicability in the specifications. |
| Sony | Option 1 |

Sub-topic 1-2: Specification impact for RedCap HPUE

Issue 1-2-1: How to specify HPUE including PC2 for RedCap

- Proposals

  - Option 1: Specify the PC2 RedCap HPUE requirements in the band agnostic way

    - Observation 1: Alternative 1: In clause 6.2I.1, remove the restriction on the applicable power class for RedCap UE. Alternative 2: In clause 6.2I.1, add one sentence stating that PC2 also applies for RedCap UE. (CTC, R4-2304404)

    - Observation 2:non-RedCap PC2 RF requirements should apply to RedCap UE and do not specify operating bands for RedCap PC2 UE. (CMCC, R4-2304661)

    - Observation 3: Redcap UEs share the same RF requirements with handheld UEs of the same Tx/Rx numbers. This draft CR proposes to add relevant descriptions in TS38.101-1 to enable PC2 for RedCap. (CUC, R4-2305039)

    - Observation 4: RedCap HPUE introduction would be a band agnostic way, if necessity and applicable form factor are confirmed. (vivo, R4-2305109)

    - Observation 5: PC3 RedCap Tx requirements are defined as band agnostic, and reused from the PC3 Tx requirements (up to 20MHz). (ZTE, R4-2305127)

    - Observation 6: Proposal 2: The PC2 requirements in Section 6 can be reused for PC2 RedCap UE. Proposal 3: The Reference Sensitivity Degradation from PC3 to PC2 for FDD bands in table 7.3.2-1c can be reused for PC2 RedCap UE. (Huawei, R4-2305391)

    - Observation 7: HPUE could be enabled in band-agnostic manner for TDD bands which do not have additional emission requirements.(Qualcomm, R4-2305502)

  - Other Options:

    - Observation 1: Both A-MPR and RSD are operating band specific. (Qualcomm, R4-2305502)

    - Observation 2: To enable PC2 support for RedCap, RAN4 to at least revisit 5.2I, 6.1I, 6.2I.1 and 7.3I.2 in the RAN4 specs. (MediaTek, R4-2305711)

    - Proposal on PC2 support for RedCap: (Skyworks, R4-2305843)

      - PC2 1Tx is supported by RedCap and 1Tx PC2 MPR applies with PC2 signaling without support for 2Tx signaling

      - PC2 1Tx is supported in a generic way for TDD band and HD-FDD with the corresponding 1Rx/2Rx PC3 REFSENS

      - For PC2 FD-FDD, a generic, equation-based RSD, is specified and derived from PC3 REFSENS de-sense level that can apply to both 2Rx and 1Rx FD-FDD cases.

- Recommended WF

  - TBA

| Company | Comments on Issue 1-2-1: How to specify HPUE including PC2 for RedCap |
| --- | --- |
| China Telecom | We support option 1. |
| MediaTek | These Options could be consolidated in some way since they are not completely mutually exclusive. |
| CMCC | We support to specify HPUE RedCap in band agnostic way. |
| Qualcomm | In some bands PC2 can be introduced in band agnostic manner. For example, for TDD bands which do not have additional emission requirements there is no need for A-MPR and RSD. There are also operating bands which require analysis unless signification relaxations are made for A-MPR and RSD. The required analysis can be reduced for cases for PC2 is already introduced for regular NR, as then A-MPR could be re-used. For receiver sensitivity degradation further discussion is needed whether it is appropriate to re-use values given that PC2 requirements may have been derived with 2 Tx assumption, and additionally RedCap UE may have only 1 Rx in which case RSD may be larger. |
| Meta | For TDD or HD-FDD bands of RedCap HUE, option 1 is good solution. But PC2 FD-FDD bands with 1Tx/2Tx, SKW, QC’s proposal shall be considered. |
| vivo | How to specify the requirement is also related to the conclusion of necessity and applicable form factor discussions. If that can be confirmed, we think a band agnostic way would be proper direction to go. We also support other companies view that given only 1Tx is assumed for RedCap, some requirements should be studied case by case before simply reusing. |
| Huawei | We support Option 1, to specify the PC2 RedCap HPUE requirements in the band agnostic way |
| China Unicom | Support to specify the PC2 RedCap HPUE requirements in the band agnostic way. |
| CHTTL | Would like to clarify that with the so called band agnostic approach, still there is a pre-condition that the PC2 for normal UE needs to be supported first on the band intend to support PC2 RedCap? Seems like some of the requirements might not be band agnostic, but we introduce it as a general way. We also support to revisit some of the section as MTK proposed, at least we think 6.1I needs an update. |
| Skyworks | We can make a precondition that PC2 is only supported by bands that already support PC2 (that solves A-MPR issues and provides a basis for 1Tx RSD). Even with equation based RSD, the A-MPR is a prerequisite if NS is applicable. |
| Nokia2 | We agree with MediaTek comment. |
| Apple | Agree with Skyworks proposal first two sub-bullets. For FD-FDD with 1Rx, we may need to analyze a few bands before deciding whether equation-based approach can be used. |
| ZTE | Referring to the option 2, it seems the supported operating band for PC2 RedCap UE may be indicated/requested first. |
| Sony | More analysis is needed |
| Ericsson | There is a eRedCap Work item ongoing, which focus on PC3 only. For a PC2 HPUE eRedcap, additional analysis for de-sens will be needed. Therefore, whether the eRedcap should include in scope should be discussed. Does the scope cover only PC2 release independent from Rel-17? |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2305039 | Draft CR for enabling PC2 for Redcap UE |
|  | China Telecom: From the procedure perspective, the WI code in the cover sheet is not allowed. |
|  | Qualcomm: In our understanding the task from RAN is to report back to them if HPUE RedCap is feasible to introduce in band agnostic manner, therefore CR discussion is too early. Furthermore, the technical discussion on A-MPR and RSD needs to be done before agreeing CR. |
|  | vivo: too early to discuss the CR. Suggest to focus on RAN task. |
|  | China Unicom: This draft CR proposed the potential spec impact for introducing RedCap PC2 HPUE in band agnostic way. CHTTL: We think some update on 6.1I section can be considered if agreeable. |
|  | Skyworks, before discussing a CR we think RAN4 should first report to RAN how PC2 can be introduced with which pre-requisites. |
|  | Nokia2: We believe it is too early to discuss CRs at this point. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Issue 1-1-1: Enable PC2 for RedCap HPUE Tentative agreements: Candidate options: - Option1: it is feasible to enable PC2 for RedCap HPUE - Option2: it is feasible to enable PC2 for RedCap HPUE, as long as the RF requirements and/or the applicable form factor are specified appropriately. Recommendations for 2nd round: Majority companies support option 1, several companies show their technical concern relative to form factor and requirements. Thus, it is recommended to further discuss with other technical issues such as form factor and spec impacts. Issue 1-1-2: Form factor for PC2 RedCap HPUE Tentative agreements: Candidate options: - Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE - Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification Recommendations for 2nd round: Companies propose their standpoints clearly, a set number of them support option 1 and a few of them support option 2, which shows no obvious preference, it is recommended to continue discuss in the WF in 2nd round. |
| Sub-topic#1-2 | Issue 1-2-1: How to specify HPUE including PC2 for RedCap Tentative agreements: Candidate options: - Option 1: Specify the PC2 RedCap HPUE requirements in the band agnostic way - Option 2: Option 1 is ok for TDD and HD-FDD, more analysis for RF requirements is needed for PC2 FD-FDD bands with 1Rx and 2Rx - Option 3: Specify the PC2 RedCap HPUE requirements based on UE indication/request. Recommendations for 2nd round: It is recommended to continue discuss in the WF in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2305039 | The draft CR is recommended as noted based on the comments and discussion is covered in the WF. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| Title | Source |
| --- | --- |
| WF on RedCap HPUE | China Telecom |

- Way forward

Issue 1: Form factor for PC2 RedCap HPUE

Candidate options for further discussion:

- Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE in the specification

- Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

Issue 2: How to specify HPUE including PC2 for RedCap

Way forward:

- PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD.

  - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

- Further discuss whether the band agnostic way can be applied for PC2 FD-FDD bands with 1Rx and 2Rx, based on more analysis on the RF requirements including A-MPR and RSD.

- Note 1: 1Tx architecture is assumed for RedCap PC2.

- Note 2: PC2 for Rel-17 Redcap is considered under this RAN task.

| Company | Comments on WF on RedCap HPUE |
| --- | --- |
| Qualcomm | Even for TDD and HD-FDD it is necessary to evaluate A-MPR in case there are additional emission requirements. We have made this update to the WF document. |
| Nokia2 | Regarding Issue 1-2-1: There are now two kind of RedCap UEs, one with RB restriction and second with peak rate restriction. It is only for UEs with peak rate restriction, PC2 requirements can be reused. We have updated the WF to reflect this change. |
|  | (Below comments from email discussion [106bis-e][152] RAN_task_UERF - WF on RedCap HPUE) |
| vivo | Thank you very much for the draft WF. Given the applicable form factor is one of the key RAN tasks, so we suggest to slightly refine the wording for issue 1-1-1 to align the discussion status: Way forward:  It is feasible to enable PC2 for RedCap HPUE, as long as the RF requirements and/or the applicable form factor are specified appropriately. |
| China Telecom | To Runxin, I’m ok fine your update. To Toni, On Issue 1-2-1, I added an sub-bullet to clarify that the band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. With this, hope we can go with the main bullet without your modification. Please let me know if my understanding is not correct. Updated version is in: draft WF on RedCap HPUE_v02_QCOM_CTC.docx. Thank you very much. |
| CHTTL | Thank you for the WF. Regarding the changes proposed by vivo: Issue 1-1-1 It is feasible to enable PC2 for RedCap HPUE, as long as the RF requirements and/or the applicable form factor are specified appropriately. By removing "or" we are wondering now this sentence become contradicting to the Issue 1-1-2 in the WF, whether to restrict the applicable form factor is still FFS. And the RAN task is to study instead of to specify. • To study the applicable form factor, e.g., sensor, camera So we are not ok to remove "or" in issue 1-1-1 at this stage. |
| CMCC | We agree with Tank for keep the previous version with "or". Thank you. |
| China Telecom | Thank you very much for the discussion. The updated version is in: draft WF on RedCap HPUE_v05_SKW_Mod.docx. The main updates are: 1) Removed the first issue, considering that companies still have different views on whether to keep the “/or”, and this issue is not explicitly tasked by RAN. 2) Added a note 1 according to Dominique’s comment:  Note 1: 1Tx architecture is assumed for RedCap PC2. 3) Added to note 2 to clarify it is for Rel-17 Redcap  Note 2: PC2 for Rel-17 Redcap is considered for this RAN task. To Nokia expert, you added a sentence that “This is only applicable for eRedCap UEs with peak rate restriction” . I don’t quite understand the intention. To my understanding, the baseline PC3 RF requirements for Rel-18 eRedcap are still under discussion, and only Rel-17 Redcap PC2 is targeted for this RAN task. So, instead, I added a note 2. Sorry for being late, and please let me know if any further comments. |
| Nokia | Thank you for the clarification. My comment was targeted towards Rel18 eRedCap, so it is OK to remove it. |

Topic #1: [8.2] RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL

Companies’ contributions summary

| T-doc number | Proposals / Observations/Abstracts | Company |
| --- | --- | --- |
| R4-2304391 | Observation 1: Case 1 contains the assumption that two SUL carrier (on different bands) cannot transmit at the same time since if this were the case, the question would be moot. Observation 2: Case 2 contains the assumption that SUL carrier and non-corresponding NUL carrier cannot transmit at the same time. Observation 3: Two NUL carriers can transmit at the same time. Observation 4: If UE supports Rel-15 SUL configuration as defined containing SUL and corresponding NUL, both need their own TX chains. Observation 5: Back-to-back transmissions between SUL and non-corresponding NUL and between two SULs are feasible but means new UE implementation assumptions in RAN4. Proposal: RAN4 will respond to the LS [1] that back-to-back transmissions between SUL and non-corresponding NUL and between two SULs are feasible but specifying this means new UE implementation assumptions in RAN4 and in-order to properly specify UE with those new assumptions, new non-spectrum related work item is needed. | Qualcomm Incorporated |
| R4-2304405 | Observation 1: With the big CR for NR CA band combinations with two SUL cells agreed in RAN4 #106, it is clear that the band configurations with two serving cells and each cell with one SUL band have already been supported from RAN4 perspective. Proposal 1: To support the back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period, - From RAN4 specification perspective, three additional uplink configurations need to be added on top of the RAN4 agreed CR. Taking CA_n41A-n95A_n79A-n98A as example, the three UL CA configurations in red need to be added. SUL band combination with CA; Uplink CA configuration or SUL configuration. CA_n41A-n95A_n79A-n98A; SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A CA_n41A-n98A CA_n95A-n79A CA_n95A-n98A. - From UE implementation perspective, there is no issue since the number of bands in uplink configuration is still 2. Proposal 2: It is feasible to support such configuration in the Rel-18 UL Tx switching framework with UE capability based switching period, and without any additional RAN4 specification impact. In additional, there is no UE implementation issue. | China Telecom |
| R4-2304662 | Observation 1: Back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier can be supported without any switching period by some UE implementation, e.g. UE implemented with 2Tx chains and four PLLs. No RAN4 specification impact is observed. Observation 2: Existing RAN4 requirements (i.e. time mask for switching across three or four uplink bands) can support the UL Tx switching for CA band combination with two SUL cells. No RAN4 specification impact is observed. Proposal 1: For a UE configured with two serving cells, each with SUL, ­ Back-to-back transmission between two SUL carriers and back-to-back transmission between SUL carrier and non-corresponding NUL carrier without any switching period can be supported by some UE implementation. ­ UL Tx switching between two SUL carriers and between SUL carrier and corresponding NUL carrier can be supported with Rel-18 UL Tx switching framework. Proposal 2: No RAN4 specification impacts are observed in order to support a UE configured with two serving cells, each with SUL. | CMCC |
| R4-2305128 | Observation 1: For both R16/17 and R18 UL switching, the length of UL switching for the band pair are the same set of values as {35 us, 140 us, 210 us}, and 0us switching period is not supported, which means the band pair within a band combination supporting Rel-16/17 and R18 Tx switching could not be supported with 0us switching period. Observation 2: For NR inter-band CA, the maximum output power requirements are defined for each supported UL CA configuration including only two bands. Observation 3: For SUL combination, the maximum output power requirement of single band is applied to each constitute band. Observation 4: The maximum output power requirement is band combination specific requirement which is involved in the basket WID work. Observation 5: For NR inter-band UL CA, the requirements for PPowerClass,CA, a-mpr and tIB,c may be different for different UL CA configurations. Observation 6: For SUL, the configured transmit power requirements for the corresponding NUL carrier and SUL carrier are applied, and there are no Configured transmitted power requirements for SUL defined for two bands supporting simultaneous Tx. Observation 7: Configured transmitted power requirements are general requirements which are applied for all band combinations, and it is out of the scope of basket WID. Observation 8: The general requirements of Configured transmitted power for inter-band CA or SUL are needed to be changed in the case of a UE configured with two serving cells, each with SUL. | ZTE Corporation |
| R4-2305568 | Observation 1: The NR CA band combinations with two SUL cells in Rel-18 is a RAN4 spectrum related WI, which follows the usual study methodology for band combinations to specify the band combination specific requirements. Observation 2: Switching period between two SUL bands in two cells could be both 0us or based on reported UE capability, which depends on choice of UE implementation even without increasing the hardware complexity. Observation 3: Switching period between SUL band and NUL band in two cells could be both 0us or based on reported UE capability, which depends on choice of UE implementation even without increasing the hardware complexity. Observation 4: The time mask requirements in the endorsed CR can already cover the cases mentioned in the RAN task, and no additional specification impact is foreseen to support a UE configured with two serving cells, each with SUL. Proposal 1: It is proposed to conclude in RAN4 that from both implementation and spec impact perspective, with completion of CRs for two SUL cells band combinations and Tx switching for 3/4 bands, there is no additional work needed to support a UE configured with two serving cells, each with SUL. | Huawei, HiSilicon |

Open issues summary

Sub-topic description: RAN #99 task to RAN4

- Task RAN4 to assess the additional, if any, RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL; report to RAN#100 with the goal of striving for potential normative work supporting the case where a UE is configured with two serving cells, each with SUL

  - E.g., whether back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier could be supported without any switching period, or

  - E.g., whether it is only feasible to support such configuration in the UL Tx switching framework with UE capability based switching period

  - Example band combinations are referred to in RP-223553 (RP-230719)

  - Further check the status in RAN#100

Sub-topic 2-1 assess specification and UE implementation impacts for back-to-back transmissions

Issue 2-1-1: UE implementation impacts to support back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period

- Proposals

  - Option 1: New UE implementation is needed to support back-to-back transmissions without any switching period.

  - Option 2: No UE implementation issue.

- Recommended WF

  - TBA

| Company | Comments on Issue 2-1-1: UE implementation impacts to support back-to-back transmissions |
| --- | --- |
| China Telecom | We support option 2. As discussed in our tdoc R4-2304405, for the band combinations in two SUL cells basket WI, with more uplink configurations added, the back-to-back transmissions in this issue can be supported with still 2 bands configured in the uplink. So, no UE implementation issue is seen based on commercial UE ability. |
| CMCC | Option 1. UE implemented with four Tx chains is not the only UE implementation choice to support this. |
| Huawei, HiSilicon | We support option 2. From the analysis in our contribution R4-2305568, it can be seen that switching period between two SUL bands in two cells could be both 0us, or based on reported UE capability, which depends on choice of UE implementation even without increasing the hardware complexity. Certainly, other implementations, e.g. 4 Tx is also a possible implementation to support the back-to-back transmissions. |
| China Unicom | Support option 2. |
| Nokia | Option 1. As QC wrote RAN4 should respond to the LS that back-to-back transmissions between SUL and non-corresponding NUL and between two SULs are feasible but specifying this means new UE implementation assumptions in RAN4 and in-order to properly specify UE with those new assumptions, new non-spectrum related work item is needed. |
| Qualcomm | Option 1, with the clarification that this needs further discussion on how ran4 proceeds with the normative work, maybe start with the study what is the preferred implementation assumption. To Huawei’s “tuning while transmitting” 2 Tx implementation, how would it work for the two concurrently transmitting NULs? |
| Apple | To our understanding, the current dual SUL basket WID only supports semi-static UL configurations, meaning that at any time, only 1 UL pair is configured. To switch to another UL pair, UL reconfiguration is required. If we are talking about dual SUL cells with dynamic Tx switching, this would be a new feature which we agree with Nokia that a new non-spectrum related work item is needed. |
| ZTE | Depending on the targeted UE implementation choice, a new UE implementation may be needed to enable back-to-back transmissions between SUL and non-corresponding NUL and between two SULs. Further discussion is probably needed to get a consensus on the targeted implementation assumption. |

Issue 2-1-2: RAN4 specification impacts to support back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period

- Proposals

  - Option 1: No RAN4 specification impacts.

  - Option 2: No RAN4 specification impacts except of band combination configurations.

  - Option 3: RAN4 specification impacts are observed.

- Recommended WF

  - TBA

| Company | Comments on Issue 2-1-2: RAN4 specification impacts to support back-to-back transmissions |
| --- | --- |
| China Telecom | We support option 2. For each SUL band combination, three additional uplink configurations need to be added. Taking CA_n41A-n95A_n79A-n98A as example, the last three UL CA configurations need to be added. We can response to RAN plenary that RAN4 recommends to add these three UL configurations for each band combination in the 2 SUL-cells basket WI. SUL band combination with CA; Uplink CA configuration or SUL configuration. CA_n41A-n95A_n79A-n98A; SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A CA_n41A-n98A CA_n95A-n79A CA_n95A-n98A. |
| CMCC | Option 2. We are open to include more UL CA configurations since no impact on RAN4 general requirements. |
| Huawei, HiSilicon | We support option 2, and agree with China Telecom that additional UL/SUL configurations can be added for the proposed band combinations in the two SUL cell WI. |
| China Unicom | Support option 2. |
| Nokia | Option 3 is closest. As we stated in 2-1-1 new non-spectrum related work item may be needed and this implies that we cannot say for certain that specification is not impacted. |
| Qualcomm | Option 3. Specification impact can be discussed when we know what assumptions are made. E.g. the table from CT comment does not include dual SUL UL configuration but there is something new that is a UL configuration with two SUL bands. This is not the same as dual SUL uplink configuration since e.g. SUL_n41A-n95A is one SUL configuration. The RAN task was a question how SUL and non-corresponding NUL interact so that kind of configuration is not listed in CT table. So it seems understanding among companies supporting same option “no specification impacts” is not same even about the band configuration table impact. Conclusion is that further discussions and study/WI is needed for RAN4 to understand what is the objective and what are the side conditions of the normative work. |
| Apple | Option 3. As we commented above, this looks to be a new feature which may have RAN4 specifications impact. |
| ZTE | From our current understanding, it may be more appropriate to carry out the related study in a new non-spectrum WID instead of the current basket WID, if there is consensus on introducing the support of the case where a UE is configured with two serving cells, each with SUL. We are open to further discuss how to proceed with the normative work if needed. |

Sub-topic 2-2 assess specification and UE implementation impacts to support such configurations in the UL Tx switching framework

Issue 2-2-1: UE implementation impacts to support such configuration in the UL Tx switching framework with UE capability based switching period

- Proposals

  - Option 1: No UE implementation issue.

- Recommended WF

  - Option 1.

Nokia: we are aware of RAN1 is discussing switching case. Switching is possible between NUL carriers.

Apple: for current dual SUL basket, only one uplink carrier is configured. We are trying to introduce the switching for all the band combinations.

Qualcomm: What is the switching scheme applied? Question: can UE transmit between two NUL bands? There are restriction on the options which needs be clarified.

Huawei: it is two SUL basket WI. This WI is spectrum related basket. We can discuss the issues from RAN. From implementation perspective, whether it can be supported from UE implementation perspective. From implementation perspective, what is the difference between SUL band combinations and CA band combinations?

CMCC: support Option1. For RAN1 discussion, if RAN1 found something needing be informed, RAN1 can send LS. WI is approved as spectrum WI. RAN task is intended for companies to understand how it can work. There is no difference between band combination types. Does Qualcomm comment that there seems some issue on the implementation?

Qualcomm: to CMCC, your understanding is correct, the limitations need to be known to understand the UE implementation

China Telecom: UE just needs to implement 2Tx. There is no issue. It is based on Rel-16 features. To Qualcomm, we can focus on RAN task of back-to-back case.

Apple: In terms of UE implementation, for single SUL cell, have RAN4 requirements to say switching time 0us between SUL and NUL. You have two SUL bands and two NUL bands. You are trying to run two SUL in parallel and have 0us switching time.

Qualcomm: There is also other RAN task about such configuration and how it operates. What is the intention from proponents?

Mediatek: We also share the view for this configuration with Dual SUL to switching to two NUL. If uplink is configured with two SUL, it needs CA framework. We agree with 0us the implementation issue should be considered.

Huawei: to Apple, for single SUL cell, 0us switching time is supported in Rel-15. In Rel-16 it is allowed for UE to report non-0us. RAN tasks do not include the case mentioned by Apple.

CMCC: to Qualcomm, as RAN task, band combinations requested there is no two SUL transmission simultaneously. We agree focusing on RAN task.

Nokia: it is better to come up with the scenario for evaluations.

Moderator: dual uplink on two SUL bands, if we are discussing based on the existing two SUL bands without any update, we can only discuss the switching case between two SUL.

| Company | Comments on Issue 2-2-1: UE implementation impacts in UL Tx switching framework |
| --- | --- |
| China Telecom | We support the recommended WF. Under the Rel-18 switching framework, up to 2Tx is needed. So, no UE implementation issue. |
| CMCC | Support recommended WF. |
| Huawei, HiSilicon | We support the recommended WF. |
| China Unicom | We support the recommended WF. |
| Nokia | We do not support the recommended WF. As we stated in 2-1-1 new non-spectrum related work item may be needed and this implies that there may be UL implementation impact. |
| Qualcomm | This may have been agreed in the plenary already in principle but it is worth to discuss what does it mean: uplinkTxSwitching-OptionSupport:dualUL may not be supported between the two SUL bands, between SUL and corresponding NUL and non-corresponding NUL, but it can be supported between the two NUL bands. This then limits possible cases and switches between the cases. It would be good to get this information on applicable cases from proponents so that UE implementation can be properly discussed and analysed and proper specification work can be done. |
| Apple | The RF characteristics may be similar to UL Tx switching among 4 bands. But again this looks to be a new feature. What would be the purpose for agreeing on the proposal here? |

Issue 2-2-2: RAN4 specification impacts to support such configuration in the UL Tx switching framework with UE capability based switching period

- Proposals

  - Option 1: No RAN4 specification impacts and Rel-18 UL Tx switching framework can be reused.

- Recommended WF

  - Option 1

| Company | Comments on Issue 2-2-2: RAN4 specification impacts in UL Tx switching framework |
| --- | --- |
| China Telecom | We support the recommended WF. |
| CMCC | Support recommended WF |
| Huawei, HiSilicon | We support the recommended WF. |
| China Unicom | We support the recommended WF. |
| Nokia | We do not support recommended WF. As we stated in 2-1-1 new non-spectrum related work item may be needed. |
| Qualcomm | Specification impact is maybe moderate, configuration tables are needed but in case agreements is such, they would apply only to TX switching. ON-OFF time mask applicability for SUL and NULs in the dual SUL configuration should be specified, once the objective and scope is defined for this work. A good description of the intended applicability should be made available. |
| Apple | During the dual SUL basket WID discussions, it was clarified by the proponent company that the dual SUL operation is different from Rel-18 UL Tx switching. We wonder why it is bundled with Rel-18 UL Tx switching. Is this an enhanced feature for dual-SUL CA? |
| ZTE | Currently, for both R16/17 and R18 UL switching, the length of UL switching for the band pair are the same set of values as {35 us, 140 us, 210 us}, and 0us switching period is not supported. So we are not sure how Rel-18 UL Tx switching framework can be reused for the back-to-back transmission cases without any switching period. For the cases with switching period, then it has to be clarified our targeted scenarios e.g. switchedUL and/or dualUL. Only if these targeted scenarios are clear, we can try to reach common understanding on the potential RAN4 spec impact. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: UE implementation impacts to support back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period Candidate options: - Option 1: New UE implementation is needed to support back-to-back transmissions without any switching period. - Option 2: No UE implementation issue. Issue 2-1-2: RAN4 specification impacts to support back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period Candidate options: - Option 1: No RAN4 specification impacts. - Option 2: No RAN4 specification impacts except of band combination configurations. - Option 3: RAN4 specification impacts are observed. Recommendations for 2nd round: It is recommended to continue discuss in the WF in 2nd round. |
| Sub-topic#2-2 | Issue 2-2-1: UE implementation impacts to support such configuration in the UL Tx switching framework with UE capability based switching period Candidate options: - Option 1: No UE implementation issue. - Option 2: UE implementation issue is observed. Issue 2-2-2: RAN4 specification impacts to support such configuration in the UL Tx switching framework with UE capability based switching period Candidate options: - Option 1: No RAN4 specification impacts and Rel-18 UL Tx switching framework can be reused. - Option 2: ON-OFF time mask applicability for SUL and NULs in the dual SUL configuration should be specified - Option 3: other impact is observed Recommendations for 2nd round: It is recommended to continue discuss in the WF in 2nd round. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| Title | Source |
| --- | --- |
| WF on RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL | CMCC |

Background on proposals

Clarification on the band combinations in approved WID, take CA_n41A-n95A_ n79A-n98A as example:

| NR CA configuration | Uplink CA configuration or SUL configuration | Power Class | contact name, company | Contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n95A_ n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A | PC3 | Xiaoran ZHANG, CMCC | zhangxiaoran@chinamobile.com | CBN, Huawei, HiSilicon | New |

- For SUL configuration, the SUL band of n95 is configured with n41, and n98 is configured with n79

- For UL CA configuration, n41 and n79 can be configured as UL CA and transmit simultaneously.

- No simultaneous transmission on SUL carrier and corresponding NUL carrier, SUL carrier and non-corresponding NUL carrier, two SUL carriers

Clarification on the proposed band combinations in R4-2304405, take CA_n41A-n95A_n79A-n98A as example

| SUL band combination with CA | Uplink CA configuration or SUL configuration |
| --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A CA_n41A-n98A CA_n95A-n79A CA_n95A-n98A |

- For SUL configuration, the SUL band of n95 is configured with n41, and n98 is configured with n79

- For UL CA configuration,

  - n41 and n79 can be configured as UL CA and transmit simultaneously.

  - NUL carrier in n41 and non-corresponding SUL carrier in n98 can be configured as UL CA and transmit simultaneously.

  - NUL carrier in n79 and non-corresponding SUL carrier in n95 can be configured as UL CA and transmit simultaneously.

  - SUL carrier in n95 and SUL carrier in n98 can be configured as UL CA and transmit simultaneously.

Moderator note:

This section is for clarification. If you have any comments on the above clarifications, please provide your comments.

And in section 3, companies are encouraged to provide comments based on the example band combinations in WID first (RAN task), and provide additional comments for the proposed band combination in R4-2304405 if there is any difference.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | The clarifications for the configurations are very clear. The WI proposed in RAN#98e and #99 are for spectrum topic, and the configurations in the tables of WID are for the purpose of specifying the band combination specific requirements. As compromise, some configurations proposed originally were removed. In our opinion, the spectrum related RAN4 WI is to complete the RF requirements based on requests of operators for the proposed combinations as usual for other spectrum/basket WIs. While the RAN task is just asking RAN4 to evaluate from implementation and spec impact perspective for the applicability of Tx switching feature for the RAN4 related band combinations. In our view, the note in the WID is not intended to limit RAN4 not to consider the real deployment requests from operators. And from implementation perspective, we see no difference for a combination supporting two SULs and that supporting inter-band CA with NR bands sharing the same UL frequency range as SUL band. With clear clarification of the configurations, we support to include the new configurations. |
| China Telecom | We agree with the clarification from moderator. |
| Qualcomm | Thank you for the clarifications, together with Huawei comment above this is helpful. Just to repeat and clarify the background for the following comment: there are two independent things discussed above, 1) RAN task and then 2) the proposals from R4-2304405 and they are even in some cases conflicting with each other. The first part is the RAN task, and the second part is something new, that to our understanding has RAN1 impact and certainly a large RAN4 impact. The second part should be discussed separately and in our view is out of scope of this WF. What is not clear, what is the intention of the section 2 as whole, for 1) part, it is background but this new part 2), is the intention to agree something? It would be our preference to label this section 2 as “Background on proposals” etc. As such unclear floating section, we can not accept the WF since this maybe be interpret in many ways. |
| ZTE | For the band combinations in the approved WID, whether it supports simultaneous uplink configuration on the two SUL bands is subject to the discussion in section 3. For the band combinations proposed in R4-2304405, it’s better to clarify the following: - The new added three band combinations CA_n41A-n98A, CA_n95A-n79A, CA_n95A-n98A are not supported in current WID. It was discussed in the past that prioritization may exist among these band combinations e.g. some may not be really needed. We would like to understand whether there is still such prioritization and the corresponding motivation. This may help to reduce the standardization effort in Rel-18 if needed. - It’s not clear to us why the title of the first column is changed from ‘NR CA configuration’ to ‘SUL band combination with CA’ In addition, it seems there are some typos in the following bullet: ‘For SUL configuration, the SUL band of n83 is configured with n41, and n95 is configured with n79’ |

Way forward

Sub-topic 2-2 assess specification and UE implementation impacts to support such configurations in the UL Tx switching framework

Issue 2-2-1: UE implementation impacts to support such configuration in the UL Tx switching framework with UE capability based switching period

For band combinations in approved WID, access UE implementation for dualUL between two NULs, switchedUL between {SUL1,NUL1}, {SUL1,NUL2}, {SUL2,NUL2}, {SUL2,NUL1} and {SUL1,SUL2}.

Candidate options:

- Option 1: No UE implementation issue.

- Option 2: UE implementation issue is observed.

Moderator note: This issue focus on UL Tx switching framework instead of 0us switching period.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We support option 1. Tx switching is a mature mechanism supported from Rel-16. For 3/4 bands, though there are some further enhancements for the feature, but the essential points to enable the feature from implementation perspective is not changed, which still relies on the switching period capability for the switching band pairs resulted from PLL retuning and/or RF parameters loading. Such implementation does not necessarily to distinguish it is a SUL band or a NR band, as SUL band is just the UL frequency range of a NR band, and the switching is for Tx only. Thus, we support option 1. |
| China Telecom | We support option 1. Under the Rel-18 switching framework, up to 2Tx is needed. So, no UE implementation issue, which is with the same situation as Tx switching across 3/4 bands under pure CA framework. In addition, based on the RAN task, we can focus on the switchedUL case, and does not need to discuss the dualUL case under the RAN task. |
| Qualcomm | RAN task was to asses additional UE implementation impact if UE is configured with two serving cells, each with SUL. From TX switching perspective, this is similar than the Rel-18 TW switching Wi where this dual SUL case was precluded in previous RAN1 discussion. One unclear issue for assessing implementation is the applicability of the switching option support. Is the intention to enable dualUL between two NULs but switchedUL only between {SUL1,NUL1}, {SUL1,NUL2}, {SUL2,NUL2}, {SUL2,NUL1} and {SUL1,SUL2}? |
| ZTE | Due to different companies’ comments/understandings in the GTW, it may need more discussion to reach consensus on the UE implementation issue. |

Issue 2-2-2: RAN4 specification impacts to support such configuration in the UL Tx switching framework with UE capability based switching period

For band combinations in approved WID, access RAN4 specification impacts for dualUL between two NULs, switchedUL between {SUL1,NUL1}, {SUL1,NUL2}, {SUL2,NUL2}, {SUL2,NUL1} and {SUL1,SUL2}.

Candidate options:

- Option 1: No RAN4 specification impacts and Rel-18 UL Tx switching framework can be reused.

- Option 2: ON-OFF time mask applicability for SUL and NULs in the dual SUL configuration should be specified

- Option 3: other impact is observed

Moderator note: This issue focus on UL Tx switching framework instead of 0us switching period.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We support option 1. The Tx switching framework based on switching period capability has been studied in detail in previous RAN4 meetings and a draft big CR was endorsed in last meeting. The Tx switching requirements for SUL are also included in the draft CR, which is generic for all applicable switching cases, and can be used for the dual SUL configurations as well. We don’t see the necessity to introduce additional requirements to enable Tx switching for SUL configurations in the example band combinations of the RAN task. |
| China Telecom | We support option 1. For option 2, there is no ON-OFF time mask requirement for SUL and its corresponding NUL in single-SUL configuration case. So, we don’t see the need to add such time mask requirement for dual-SUL configuration as well. |
| Qualcomm | RAN task was to asses additional UE specification impact if UE is configured with two serving cells, each with SUL. From TX switching perspective, this is similar than the Rel-18 TW switching Wi where this dual SUL case was precluded in previous RAN1 discussion. One unclear issue for assessing implementation is the applicability of the switching option support. Is the intention to enable dualUL between two NULs but switchedUL only between {SUL1,NUL1}, {SUL1,NUL2}, {SUL2,NUL2}, {SUL2,NUL1} and {SUL1,SUL2}? Could Huawei clarify what are the band combinations of the RAN task? The WI only has these: So there are no dual SUL UL configurations in the WID. Just asking since the section 2 in this WF is unclear. Also there would need to be clarification on power sharing aspects between the two NULs on such band configuration. The clause 6.2C.1 states that: “When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier(s) or SUL carrier, “ It all in singular, so the specification is unclear when UE is configured for two serving cells with SUL in each. |
| ZTE | The targeted scenarios e.g. switchedUL and/or dualUL should be clarified. Only if these targeted scenarios are clear, we can try to reach common understanding on the potential RAN4 spec impact |

Sub-topic 2-1 assess specification and UE implementation impacts for back-to-back transmissions

Issue 2-1-1: UE implementation impacts to support back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period

For band combinations in approved WID, access UE implementation impacts for dualUL between two NULs, switchedUL between {SUL1,NUL1}, {SUL1,NUL2}, {SUL2,NUL2}, {SUL2,NUL1} and {SUL1,SUL2}.

Candidate options:

- Option 1: New UE implementation is needed to support back-to-back transmissions without any switching period.

- Option 2: No UE implementation issue.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Based on our analysis, for some switching cases, we see even with the same UE implementation assumption, the UE can support back-to-back transmission of SUL carriers and SUL with non-corresponding NUL carrier w/o switching period. As proposed by other companies, more Tx can also be an implementation choice. The issue is not whether new UE implementation is needed or not, but whether UE implementation is feasible to support such back-to-back transmissions. From this perspective, even new UE implementation with more Tx chains could be considered as one alternative solution, it is still a feasible implementation, in other words, no UE implementation issue. |
| China Telecom | Option 1 based on the UL configurations already included in the basket WI. Option 2, if 3 more UL configurations can be added in the basket WI (as moderator clarified in section 2). |
| Qualcomm | Agree with Huawei that this is feasible with some new assumptions about UE, most straight forward is the 4 TX chain assumption and see that the option 1 is the proper option for the issue 2-1-1 as it is written. However, just to clarify the terminology, we are not sure what the meaning of the repeated phrase “UE implementation issue”? The question is about “impact”. “issue” refers to more permanent problem. Not sure how to comment the China Telecom comment, seems separate proposal from this RAN task. |
| ZTE | Due to different companies’ comments/understanding in the GTW, it may need more discussion to reach consensus on the UE implementation issue. |

Issue 2-1-2: RAN4 specification impacts to support back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period

For band combinations in approved WID, access RAN4 specification impacts for dualUL between two NULs, switchedUL between {SUL1,NUL1}, {SUL1,NUL2}, {SUL2,NUL2}, {SUL2,NUL1} and {SUL1,SUL2}.

Candidate options:

- Option 1: No RAN4 specification impacts.

- Option 2: No RAN4 specification impacts except of band combination configurations.

- Option 3: RAN4 specification impacts are observed.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We support option 2. As commented for the clarification of band combinations in section 2, we think the additional band combination configurations should be added in the WI. If there is no switching period, for TDM transmission among operating bands in the band combiantion, the existing requirements are enough to support such kind of operation, no new requirements need to be specified. |
| Qualcomm | Option 3. There would need to be clarification on power sharing aspects between the two NULs on such band configuration. The clause 6.2C.1 states that: “When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier(s) or SUL carrier, “ It all in singular, so the specification is unclear when UE is configured for two serving cells with SUL in each. Detailed changes can be discussed when or if the WI is started. |
| ZTE | From our current understanding, it may be more appropriate to carry out the related study in a new non-spectrum WID instead of the current basket WID, if there is consensus on introducing the support of the case where a UE is configured with two serving cells, each with SUL. We are open to further discuss how to proceed with the normative work if needed. |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| #1 | WF on RedCap HPUE | China Telecom |  |
| #2 | WF on RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL | CMCC |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2304404 |  | On the support of RedCap HPUE | China Telecom | Noted |  |
| R4-2304661 |  | Discussion on RedCap HPUE | CMCC | Noted |  |
| R4-2305039 |  | Draft CR for enabling PC2 for Redcap UE | China Unicom, Huawei, HiSilicon | Noted |  |
| R4-2305109 |  | Discussions on RedCap HPUE | vivo | Noted |  |
| R4-2305127 |  | Views on HPUE RedCap in FR1 | ZTE Corporation | Noted |  |
| R4-2305391 |  | Discussion on RF impacts for RedCap HPUE | Huawei, HiSilicon | Noted |  |
| R4-2305502 |  | On enabling RedCap HPUE | Qualcomm Inc. | Noted |  |
| R4-2305711 |  | Discussion on standardization for RedCap PC2 | MediaTek Inc. | Noted |  |
| R4-2305843 |  | Redcap HPUE support in FR1 | Skyworks Solutions Inc. | Noted |  |
| R4-2304391 |  | Scenarios for dual SUL | Qualcomm Incorporated | Noted |  |
| R4-2304405 |  | Discussion on UE configured with two serving cells and each cell with one SUL band | China Telecom | Noted |  |
| R4-2304662 |  | Discussion on UE configured with two SUL cells | CMCC | Noted |  |
| R4-2305128 |  | On UE configured with dual SUL | ZTE Corporation | Noted |  |
| R4-2305568 |  | Discussion on UE configured with two serving cells | Huawei, HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2306650 |  | WF on RedCap HPUE | China Telecom | Agreeable | The latest version shared by WF moderator is recommended as agreeable |
| R4-2306651 |  | WF on RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL | CMCC | Agreeable | The latest version shared by WF moderator is recommended as agreeable |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents